================================================================

EEEEEEE   MM   MM    SSSSS   EEEEEEE  TTTTTTT  U     U   PPPPPP

E         M M M M   S     S  E           T     U     U   P     P

E         M  M  M   S        E           T     U     U   P     P

EEEE      M     M    SSSSS   EEEEE       T     U     U   PPPPPP

E         M     M         S  E           T     U     U   P

E         M     M   S     S  E           T     U     U   P

EEEEEEE   M     M    SSSSS   EEEEEEE     T      UUUUU    P

================================================================

A program for configuring EMILY and MONICA :

EMSETUP is a program which configures the EMILY and/or MONICA programs

to set the initial values of several features which are loaded when

the program is run.

The EMSETUP command accepts the name of the file to configure as a

command line argument. If no extension is given, EMSETUP assumed

".COM". For example, to edit the settings contained in EMILY52.COM,

you could use the command:

EMSETUP EMILY52.COM

or

EMSETUP EMILY52

If additional filenames are given, EMSETUP will write the new settings to

those files (as well as the original one). This behaviour can be used to

copy settings from one file to another:

EMSETUP emily52 monica52

The above command reads the default settings from EMILY52.COM, allows you

to edit them, and writes the new settings to EMILY52.COM and MONICA52.COM.

If you do not make any changes before exiting and saving the settings, the

effect is to copy the settings from EMILY52.COM to MONICA52.COM.

Once you have started EMSETUP, you will be presented with a menu of

configuration options which can be configured:

Screen colors

Special function registers

Serial I/O defaults

Target interface speed

COMM port assigments

RESET logic level

Default load filename

Exit EMSETUP

Screen colors:

This options permit you to change the colors used in the various

windows presented by EMILY and MONICA. NOTE: Only the color screens

can be altered. When the '/Monochrome' switch is used, EMILY and

MONICA always use a black and white presentation.

Once you select this option, you will be presented with a sample

window which is displayed in the colors of the "Dissassembly" window

in EMILY and MONICA. Press:

PgUp/PgDn   - To switch to the other windows

Up/Down     - To change the foreground color

Left/Right  - To change the background color

Escape      - To return to the main EMSETUP menu

Special function registers:

This option allows you to Add/Modify/Delete special function registers

which appear in the EMILY/MONICA 'F' menu and disassembler output.

The currently defined special function registers will be displayed, along

with each ones address. To change/delete/insert a register, press the

letter beside the one you wish to alter. If more than 1 column of

registers are defined, use the SPACE BAR to shift the selection menu

between each row of register definitions.

Once you have selected a regiser, you will be presented with a menu

of options:

Change address      - Modify the Direct Address of the SFR

Delete entry        - Delete the SFR definition

Insert before       - Insert a new definition before this one

Insert after        - Insert a new definition after this one

NOTE: if you add SFR's to MONICA, or to EMILY when you are using the

'/Hardware' option, you must also add code to the kernal (KERNAL.ASM)

to allow them to be read and written. The kernal uses a "jump" table

of direct addresses (called 'CTAB1' and 'CTAB2') to invoke handlers

which read and write those addresses in response to commands from the

PC. Look at the code for other SFR's ('TL0' etc.) to see how to add

your own handlers.

When you run EMILY or MONICA with additional SFR's defined, use the

SPACE BAR to move the selection menu between each row of SFR's.

Serial I/O defaults:

This option presents you with a serial I/O setup screen which mimics

the ones in EMILY and MONICA. Select the values that you wish to be

in effect when EMILY and/or MONICA is loaded.

Target interface speed:

This option sets the serial I/O speed (baudrate) that is used by MONICA

and EMILY (when '/h' is used) to interface to the target system kernal.

You should not change this value, unless you have modified KERNAL.ASM,

or are using a target crystal frequency which is not 11.059 Mhz. The

EMILY manual contains a detailed discussion on interface speeds available

at various crystal frequencies. Use the UP/DOWN arrows to change the

interface speed, ESCAPE to exit.

Serial port assignments:

This option allows you to set the default function of the PC serial ports.

You may select between COM1 = Serial I/O redirection / COM2 = Target

interface, or COM2 = Serial I/O redirection / COM1 = Target interface.

Use SPACE to swap the COM port operation, and ESCAPE to exit.

RESET logic level:

This option allows you to set the logic level of the RTS "reset" control

line from the PC. You may select RESET when RTS is "asserted" (+12v) or

"deasserted" (-12v). If you are using an older version of our "BD52"

single board computer, you may have to set this value to "asserted".

Default load filename

This option allows you to select the default filename and extension that

will appear in the input area in response to a 'L'oad command. Enter the

name you wish, and press ENTER or ESCAPE to exit.

NOTE: EMSETUP should NOT be used on the DEMO version of EMILY.