==================================================================

MM   MM   OOOO   N     N IIIIIII  CCCC      A     5555555   22222

M M M M  O    O  NN    N    I    C    C    A A    5        2     2

M  M  M  O    O  N N   N    I    C        A   A   555555         2

M     M  O    O  N  N  N    I    C       A     A        5   22222

M     M  O    O  N   N N    I    C       AAAAAAA        5  2

M     M  O    O  N    NN    I    C    C  A     A  5     5  2

M     M   OOOO   N     N IIIIIII  CCCC   A     A   55555   2222222

==================================================================

A

PC Hosted

Monitor for the

8051 and 8052

Release 1.3

Revised 04-Jan-94

Copyright 1991-1994 Dave Dunfield

All rights reserved

MONICA: A PC Hosted 8052 Monitor                                 Page: 1

1. INTRODUCTION

MONICA is a PC hosted  monitor/debugger  for  the  8051  and  8052

family of  processors.  It  provides  a  powerful  on-chip  debugging

environment, using a tiny firmware "kernal" in the target system, and

a easy to use "windowed" interface program running on the an  IBM  PC

compatible computer. MONICA offers many features including:

- Uses no RAM memory from the target system.

- Supports separate code and data space (requires hardware assist).

- Single step, Animate and Full Speed execution modes.

- Simultanious on-screen displays of  program  disassembly,  internal

memory, CPU registers and monitor messages.

- Full screen editors for CPU registers, Special  Function  Registers

(SFR's), and each of the INTERNAL, EXTERNAL DATA and PROGRAM memory

spaces. All of the above may be viewed/altered at any  time  during

the debugging session.

- Multiple breakpoints are transparent to the user program.

- Supports the additional SFR's and internal RAM of the 8052 series.

- Supports single-chip "in circuit emulation" using a DS5000.

1.1 Host System Requirements

MONICA will run on any IBM PC/AT/PS2 compatible system  with  a

MDA/CGA/EGA or VGA display, two COMM ports, and at  least  64K  of

free system RAM.

- If the serial I/O redirect feature is not  used,  only  one  COM

port is required.

- Some CGA displays may exibit "snow" or flicker during high speed

screen updates. This effect will not occur with MDA, EGA or  VGA

displays.

MONICA: A PC Hosted 8052 Monitor                                 Page: 2

1.2 Target System Requirements

MONICA debugs by communicating  with  a  tiny  (less  than  2K)

"kernal" program on the target system. This kernal is  located  in

memory at address $0000, which  allows  it  to  execute  when  the

processor resets, and to gain access to the interrupt vectors. All

vectors  except  for  "Timer  1  Overflow"  are   re-mapped   into

corresponding  locations  at  the  beginning  RAM  program  memory

(normally $0800).

Since the kernal occupies program memory from $0000-$07FF,  the

user program should be origined  to  begin  at  $0800  or  higher.

Interrupt vectors in the user program should be placed at $0800  +

the normal vector offset.

The serial port of the target system must be connected  to  the

COM2 serial port of  the  PC  (Unless  the  '/Swapcom'  option  is

enabled, in which case COM1 is used).

MONICA uses the RTS signal on the COMM port to perform a  RESET

of the target system. MONICA will pulse RTS to  perform  a  reset.

During normal (non-reset)  operation,  RTS  is  maintained  in  an

ASSERTED state.

MONICA lowers the DTR signal whenever she is writing to PROGRAM

memory. For all other operations,  DTR  is  asserted.  The  target

system may use this signal to map a RAM back and forth between the

PROGRAM and DATA address spaces. If this feature  is  implemented,

you will be able to use MONICA with a full  128K  of  memory  (64K

CODE + 64K DATA).

If the hardware does not support the above  mapping,  then  you

must use MONICA on a system that maps RAM into BOTH the  DATA  and

PROGRAM address spaces. This is normally  accomplished  by  ANDing

the RD and PSEN lines from the CPU.

The control software uses TIMER1  internally  to  generate  the

baud rate clock for its serial port,  and  also  as  an  interrupt

source for single-stepping. This means that TIMER1 should  not  be

re-programmed,  or  have  its  interrupt  disabled  by  the   test

software. If your application uses the serial port,  we  recommend

that you also use TIMER1 as the baud rate  generator,  and  during

testing, either do not program it, or make sure to program  it  at

the same speed as MONICA (default = 28800).

MONICA: A PC Hosted 8052 Monitor                                 Page: 3

2. STARTING UP MONICA

Insure that the resident control software (supplied)  is  inserted

into the target system, and that the PC running MONICA  is  connected

to the target systems serial port via COM2. Power on the test  system

before starting MONICA. The resident control software must be  active

when MONICA "looks" for it.

MONICA is invoked by typing her name at the DOS command line.  You

will be greeted with a "welcome" screen which displays a reminder  of

the command line syntax and the copyright message. This  screen  will

remain on the display for about 5 seconds.  Pressing  any  key  while

this screen is visible will bypass this delay, and  proceed  directly

to the main control panel.

If MONICA is given a file name on the command line, she  loads  it

in the same manner as if the 'L'oad command had been used from within

the main control panel.

MONICA also accepts several option switches on the command line:

/DS5000       - Allows MONICA to interact with a DS5000 type

processor for in-circuit single-chip emulation.

/Hardware:nn  - Sets the baud rate for the hardware interface.

Default is 28800.

/Mono *       - Inhibits the  color display, even  if  a  color

adapter is present. This option may  be  useful

if your monitor/adapter combination provides an

unpleasent or unusable display for MONICA.

/Reset        - Causes MONICA to ASSERT the RTS line  when  she

attempts to reset the target system, and to de-

assert it at all other times.  This  option may

be  required  when  using  MONICA   with  older

versions of our "BD52" single board computer.

/Swapcom      - Exchanges the operation of the two serial ports

on the PC.  When this option is used, COM1 will

be used to communicate with the  target system,

and COM2 is used for serial I/O redirection.

* Note: If some of the MONICA windows appear  to  be  an  un-readable

washed-out yellow color, try adjusting the color contrast on

your monitor. Those windows should be an easy to read brown.

If MONICA is unable to establish  communication  with  the  target

system, you will see an error message flashing on the screen.  MONICA

will keep trying, allowing you  to  debug  the  connection.  You  may

cancel MONICA, and return to the DOS prompt by  pressing  the  ESCAPE

key.

MONICA: A PC Hosted 8052 Monitor                                 Page: 4

3. USING MONICA

3.1 The Main Control Panel

Once MONICA is up and running, you will be presented  with  the

"Main Control Panel", which consists of four "windows" displaying:

- A disassembly of program memory       (Upper LEFT)

- The contents of CPU registers         (Upper CENTER)

- The contents of INTERNAL MEMORY       (Upper RIGHT)

- Informational messages from MONICA    (Lower portion)

All of MONICA'S functions are accessed  by  command  characters

entered from this screen:

3.1.1 A - Animate execution

This function begins execution of the user program (from the

address in the PC register). As each instruction  is  executed,

MONICA will display the instruction  being  executed,  the  CPU

registers, and the contents of internal RAM in the  appropriate

windows.

When you enter the 'A' command, MONICA will prompt  you  for

the number of instructions to animate. Pressing return (without

entering  a  number)  will  cause  the  animation  to   proceed

non-stop. Pressing <escape> will return you to the Main Control

Panel.

The animation will terminate when:

- The requested number of instructions have been stepped.

- A breakpoint is encountered.

- The <escape> key is pressed on the PC console.

3.1.2 B - set Breakpoint

This command sets a breakpoint such that  program  execution

under the 'A'nimate, or  'G'o,  commands  will  stop  when  the

breakpoint address is encountered.

Up to 10 breakpoints may be in effect at the same time.

You will be prompted for  the  address  (in  HEX).  Pressing

return (without entering an address) will cause the  breakpoint

to be  set  at  the  current  program  counter  (PC)  location.

Pressing <escape> will return you to the main control panel.

MONICA stores  the  breakpoint  addresses  internally  in  a

simple zero terminated list. This allows her to avoid searching

a fixed size  table,  and  helps  speed  up  the  operation  of

A)nimation, especially when there  are  few  breakpoints.  This

does however cause  the  restriction  that  you  cannot  set  a

breakpoint at address zero ($0000).

MONICA: A PC Hosted 8052 Monitor                                 Page: 5

NOTE: Breakpoints are implemented on the  target  system  by

substituting a LCALL to the  breakpoint  handler  ($0040)  into

your code. This instruction requires  THREE  bytes  of  memory.

When setting breakpoints, you must be  careful  to  place  them

ONLY at addresses which are the beginning  of  an  instruction,

and also to  insure  that  any  instruction(s)  which  will  be

replaced by the remaining two bytes are not  the  target  of  a

'jump' or 'call' instruction.

3.1.3 C - Change register

You will be prompted for the CPU register to change. Enter a

character from the following list, ESCAPE to abort, or F1 for a

reminder:

A   - A accumulator

B   - B accumulator

D   - Data pointer (DPTR)

S   - Stack pointer

P   - Program counter

W   - processor status Word

0-7 - general register 0-7

After you have selected the register, you will  be  prompted

for the new contents (in HEX).

3.1.4 D - edit Data memory

This command allows you to edit EXTERNAL DATA  memory  using

MONICA's built  in  memory  editor.  See  the  "Memory  Editor"

section.

3.1.5 F - display/edit special Function registers

The names and contents of all SFR's are displayed in a menu.

To change a register, enter the character preceeding  the  ':',

immediately to the left of its name. You will be  prompted  for

its new contents.

If multiple columns of SFR's are  defined,  use  <space>  to

select the column to edit.

Pressing any other key while in this menu will  refresh  the

display, and make visible any changes which have occured on the

target system I/O lines etc.

3.1.6 G - Go: execute program

This  function  begins  high-speed  execution  of  the  user

program (from the address in the PC register).

When you enter the 'G'  command,  MONICA  will  ask  if  you

really wish to begin execution. Press 'Y' or ENTER to continue,

'N' or ESCAPE will return you to the main control panel.

MONICA: A PC Hosted 8052 Monitor                                 Page: 6

When serial I/O is set to 'Local', any serial data  that  is

sent by the executing program will be displayed in the  message

window in the lower portion of the screen. Any  data  typed  on

the keyboard will  be  transmitted  serially  to  the  program.

Pressing the ESCAPE key will  bring  up  a  menu  of  execution

options:

Reset target    - Reset and return to main control panel

Send ESCAPE     - Send ESCAPE character to target system.

Zoom window     - Switch to full screen I/O window.

If serial I/O is being redirected, serial data sent  by  the

program is passed on to the PC COM port. Data received  by  the

PC is passed through to the target system serial port. Pressing

ESCAPE will reset the processor and terminate the 'G'o command.

NOTE: When the target system is reset,  MONICA  retains  the

contents of processor  registers  and  the  first  32  byte  of

internal memory that were in  effect  when  the  original  'G'o

command was executed.

Program execution will terminate when:

- A breakpoint is encountered.

- A RESET is performed from the PC console.

- The program calls address $0040. *

- The program calls address $001B. *

* Note1: Address $001B is the same as  $0040,  except  that  it

resets the timer for MONICA's  serial  port,  disables

serial interrupts, and clears  the  8051/52  interrupt

system (by internally performing an IRET).

* Note2: When the user program returns control  to  the  PC  by

calling address $0040 or $001B, MONICA sets  the  user

program counter to the address of the next instruction

AFTER that call. This allow you to  use  another  'G'o

command to continue execution from that point.

3.1.7 I - edit Internal memory

This command  allows  you  to  edit  INTERNAL  memory  using

MONICA's built  in  memory  editor.  See  the  "Memory  Editor"

section.

3.1.8 J - Jump to PC

This command re-positions the disassembly display window (if

necessary) to display the instruction addressed by the  current

CPU Program Counter (PC).

3.1.9 K - Kill all breakpoints

Removes ALL breakpoints after prompting for permission to do

so.

MONICA: A PC Hosted 8052 Monitor                                 Page: 7

3.1.10 L - Load program image

Prompts for the name of a file, and loads  it  into  PROGRAM

memory. The file must  be  in  either  INTEL  or  MOTOROLA  hex

download format.

3.1.11 N - Next disassembly window

Advances the disassembler to display the  following  screen.

Subsequent 'N' commands  will  proceed  to  disassemble  higher

memory addresses.

3.1.12 P - edit Program memory

This  command  allows  you  to  edit  PROGRAM  memory  using

MONICA's built  in  memory  editor.  See  the  "Memory  Editor"

section.

3.1.13 R - Remove a breakpoint

Prompts for the address, and if a breakpoint is set at  that

address, it is removed. Pressing return  (without  entering  an

address) will remove the  breakpoint  at  the  current  program

counter (PC) location.

3.1.14 F1 - Help display

Pressing this key displays a summary of the MONICA  commands

which are available from the Main Control Panel.

3.1.15 F2 - Set Disassembler Address

Prompts for  an  address  (in  HEX),  and  re-positions  the

disassembler window to display instructions beginning  at  that

address.

3.1.16 F3 - Setup Serial Port

Opens a window displaying the (B)audrate,  (P)arity,  (D)ata

bits and (S)top bits which are  currently  configured  for  the

PC's COM1 serial port.

Press B, P, D or S to cycle to the next corresponding entry.

When Baudrate is selected as 'Local', MONICA will  re-direct

the  programs  serial  accesses  to  the  message  window.  Any

characters which are written to SBUF  will  be  output  in  the

message window, and  any  character  which  are  typed  at  the

keyboard will appear as incomming serial data. When  this  mode

is active, Pressing ESCAPE in the 'G'o command brings up a menu

of execution options:

Reset target    - Reset and return to main control panel.

Send ESCAPE     - Send ESCAPE through as a charcter.

Zoom window     - Switch to full screen I/O window.

MONICA: A PC Hosted 8052 Monitor                                 Page: 8

If 'Local' is not selected, MONICA re-directs  the  programs

serial accesses to the PC's  COM1  serial  port,  however,  for

maximum  flexibility,  the  actual  data  format  used  may  be

configured by this menu.

Since the speed of the target interface, and the  PC  serial

port can be different, MONICA buffers data in  both  directions

using two 4K buffers. Up to 4095 characters can be output in  a

single burst without data loss.

MONICA allows you to "fine tune" a baud rate, by  using  the

Up/Down keys. Note that the  available  baud  rate  values  are

determined by the PC COMM  port  hardware,  and  may  not  fall

exactly on the value you wish to use.

Asynchronous communication is specified to work properly  as

long as the device is operating within 5% of the intended  baud

rate.

For example:

Desired baud rate       : 1500

Allowable range (+/- 5%): 1425-1575

Closest PC baud rate    : 1496

In the above example, the baud rate produced is within  0.3%

of  the  desired  value,   and   is   therefore   well   within

specifications.

3.1.17 F4 - Setup DS5000 options

Sets the memory map when using a DS5000 type processor.  The

UP / DOWN keys select the startng address of data  memory,  and

F2 enables / disables the CE2  select  for  the  upper  32K  of

memory. See the section on in-circuit emulation with the DS5000

for more information.

3.1.18 <space> - Single step one instruction

Every time you press the <space> bar,  MONICA  will  execute

one instruction, and update the on-screen displays.

3.1.19 PgDn - Advance IRAM display page

Advances the internal memory display by one page (64 bytes).

3.1.20 PgUp - Backup IRAM display page

Backs up the internal memory display by one page (64 bytes).

3.1.21 Down - Advance IRAM display line

Advances the internal memory display by one line (4 bytes).

3.1.22 Up - Backup IRAM display line

Backs up the internal memory display by one line (4 bytes).

MONICA: A PC Hosted 8052 Monitor                                 Page: 9

3.2 The Memory Editor

When you activate the 'D', 'I' or 'P' commands  from  the  main

control panel, MONICA will invoke its internal "Memory Editor"  to

access the desired memory area. When you are within  this  editor,

the following commands are available:

3.2.1 F1 - Help display

Pressing this key displays a summary of the MONICA  commands

which are available from within the Memory Editor.

3.2.2 F2 - Set Edit Address

Prompts for an address (in HEX), and re-positions the editor

display and cursor to edit at that address.

3.2.3 F3 - Toggle Edit Mode

MONICA allows two edit modes, HEX and  ASCII.  When  in  HEX

mode, your cursor will be positioned in the hexidecimal display

portion of the memory  memory  dump,  allowing  you  to  change

memory by entering a new value as two  hexidecimal  characters.

When in ASCII mode, your cursor will  be  positioned  over  the

ASCII portion of the dump, and  you  can  replace  memory  with

ASCII text typed at the keyboard.

Pressing F3 will toggle between HEX and ASCII mode,  causing

your cursor to switch between the corresponding portion of  the

memory display.

3.2.4 Up - Move up 1 line

Pressing the UP ARROW causes the cursor to move  upwards  on

the screen by one line. This reduces the memory edit address by

16 bytes.

3.2.5 Down - Move down 1 line

Pressing the DOWN ARROW causes the cursor to move  downwards

on the screen by one  line.  This  increases  the  memory  edit

address by 16 bytes.

3.2.6 Left - Move left 1 byte

Pressing the LEFT ARROW causes the cursor  to  move  to  the

left by one byte position. This reduces the memory edit address

by 1 byte.

If the cursor is already at the left hand  margin,  it  will

wrap around to the rightmost position of the previous line.

MONICA: A PC Hosted 8052 Monitor                                 Page: 10

3.2.7 Right - Move right 1 byte

Pressing the RIGHT ARROW causes the cursor to  move  to  the

right by one byte position. This in increases the  memory  edit

address by 1 byte.

If the cursor is already at the right hand margin,  it  will

wrap around to the leftmost position of the following line.

3.2.8 PgUp - Move back 1 page

Pressing the PGUP key causes the editor  to  move  back  one

display page. This reduces  the  memory  edit  address  by  256

bytes.

3.2.9 PgDn - Move forward 1 page

Pressing the PGDN causes the  editor  to  move  forward  one

display page. This increases the memory  edit  address  by  256

bytes.

3.2.10 Home - Move to start of line

Pressing the HOME key causes  the  cursor  to  move  to  the

leftmost position of the line it is currently on. If the cursor

was already in the  leftmost  position,  it  is  moved  to  the

leftmost position of the preceeding line.

3.2.11 End - Move to end of line

Pressing the END key  causes  the  cursor  to  move  to  the

rightmost position of the line  it  is  currently  on.  If  the

cursor was already in the rightmost position, it  is  moved  to

the rightmost position of the following line.

3.2.12 Ctrl-PgUp - Move to start of memory

Pressing the CTRL and  PGUP  keys  re-positions  the  editor

display and cursor to address $0000 at the start of the  memory

block.

3.2.13 Ctrl-PgDn - Move to end of memory

Pressing the CTRL and  PGDN  keys  re=positions  the  editor

display and cursor to the  highest  address  available  in  the

memory block.

3.2.14 space - Refresh display

Pressing the space bar causes MONICA to refresh the  display

window from the target system. This is  useful  for  monitoring

memory mapped I/O locations etc. which may change.

MONICA: A PC Hosted 8052 Monitor                                 Page: 11

3.3 Messages

During  the  course  of  operation,  MONICA  may  display   the

following messages in the message  window  located  in  the  lower

portion of the screen:

3.3.1 All Used!

You have tried to enter a  breakpoint,  when  all  available

breakpoints are in use. MONICA allows up to 10 code breakpoints

to be active at any one time.

3.3.2 Animating at XXXX

Indicates that MONICA has commenced  animated  execution  of

your program, beginning at the address 'XXXX'.

3.3.3 Conflict!

Issued to indicate that the address at which you have  tried

to set a breakpoint is already active  as  a  breakpoint.  Note

that breakpoints on an 8052 require THREE BYTES, and  therefore

this message will result from attempting to  set  a  breakpoint

within three bytes of another one.

This message will also  result  of  you  attempt  to  set  a

breakpoint at address zero ($0000).

3.3.4 Breakpoints: [XXXX ...]

This message shows the currently set  breakpoint  addresses,

and is issued whenever any  command  involving  breakpoints  is

performed.

3.3.5 Executing at XXXX

Indicates that MONICA has commenced full speed  exection  of

your program, beginning at the address 'XXXX'.

3.3.6 FILENAME: Bad checksum

MONICA has  encountered  a  record  containing  a  incorrect

checksum value while attempting to 'L'oad the named file.

3.3.7 FILENAME: Invalid HEX record

MONICA has  encountered  an  improperly  formatted  download

record while attempting to 'L'oad the named file.

3.3.8 FILENAME: N bytes loaded

The  indicated  number  of  bytes  have  been   successfully

'L'oaded from the named file.

MONICA: A PC Hosted 8052 Monitor                                 Page: 12

3.3.9 FILENAME: Unable to access

MONICA was unable to open and read the named file.

3.3.10 Stepping over breakpoint at XXXX

Indicates that a 'G' command would  start  execution  within

three  bytes  of  a  breakpoint.  This  prevents  MONICA   from

implanting the breakpoint before  executing  the  code.  MONICA

single-steps the processor until the program  counter  is  more

than three bytes from any breakpoint, and then proceeds.

3.3.11 Stopped at XXXX

Indicates that MONICA has stopped execution of  you  program

at the indicated address.  This  message  is  appended  to  the

initial message indicating that execution had commenced.

3.3.12 Target system is not responding!!!

Indicates that MONICA cannot  establish  communication  with

the target system. MONICA will keep  trying,  allowing  you  to

attempt to debug the  connection.  You  may  cancel  MONICA  by

pressing the ESCAPE key.

MONICA: A PC Hosted 8052 Monitor                                 Page: 13

4. SINGLE-CHIP EMULATION WITH THE DS5000

Since MONICA downloads the program code and actually  runs  it  on

the target system processor, you must have RAM  from  which  you  can

execute. On the standard 8051 family processors, this means that  you

must have external RAM. This also means that you have to give  up  18

of the 32 available I/O lines to  interface  to  that  memory.  Often

additional I/O  lines  are  used  for  serial  interface,  timer  and

interrupt inputs etc.

Although many designs can make do with the 8 or 10  remaining  I/O

lines, often a particular project requires the additional  I/O  lines

which are available in single chip (no external  memory)  mode.  This

presents a debugging problem.

MONICA solves the single-chip debugging problem  by  providing  an

interface  to  the  DS5000   series   of   processors   from   Dallas

Semiconductor. The DS5000 contains 8, 32 or 64K of internal RAM which

can be allocated to the CODE and/or DATA address  spaces.  Since  the

memory is internal to the chip, all of the pins remain available  for

use as I/O lines.

For most single-chip applications, all you have to do  is  replace

the standard CPU with a DS5000 before running MONICA.

4.1 Loading the kernal

Before you can  use  the  DS5000  processor  as  a  single-chip

in-circuit emulator with MONICA, you must load the kernal into it.

To do this, perform the following steps:

1) Assemble the kernal and generate an intel format  hex  download

file. If you are using our ASM51, the command is:

ASM51 kernal -ti

2) With the DS5000 installed in  a  system,  and  connected  to  a

serial port of the PC. Ground the PSEN pin (29), and  power  up

or reset the DS5000.

3) Run a terminal program on the PC, and hit  ENTER  a  few  times

until you see the DS5000 loader prompt. If you have difficulty,

see the DS5000 data sheets for information on  acceptable  baud

rates for various crystal frequencies.

4) Enter 'U' to unlock the RAM

5) Enter 'L' to load code, and then transmit the  KERNAL.HEX  file

using a simple ASCII send.

6) Enter 'W 18' to set the data address.

7) Release the ground on PSEN, and reset the  DS5000.  The  kernal

should now be loaded. Since the internal memory of  the  DS5000

is non-volitile, you should not have to repeat  this  procedure

unless the kernal code should somehow become corrupted. If ever

the DS5000 appears to stop working with MONICA,  try  reloading

the kernal.

MONICA: A PC Hosted 8052 Monitor                                 Page: 14

4.2 Starting and configuring MONICA

When MONICA is started with the '/DS5000' option, she  performs

additional interfacing to the DS5000 chip (through the kernal)  to

accomodate loading and executing code from the internal RAM.  This

option also enables the F4 (Setup DS5000 options) menu.

The first item in the DS5000 menu is the "data  address".  This

is the memory address above which is assumed to be DATA space, and

below which is treated as CODE space. Memory  contents  below  the

"data address" can not be written, and can be read only  with  the

'MOVC' instruction. Memory at or above the "data address"  can  be

read or written with 'MOVX', but cannot be read with  'MOVC'.  You

should set this value to provide the proper allocation of code and

data memory to suit the program under test.  You  must  also  make

sure that the  programs  memory  setup  is  consistant  with  this

setting.

MONICA will not allow you to set the "data address"  below  the

2K mark, to insure that the kernal code cannot be overwritten (The

DS5000 would also crash if this were done, since there would be no

executable code space). The "data address" is set  to  $0800  when

loading a program, so that  the  RAM  can  be  written.  Once  the

program is loaded,  the  "data  address"  is  set  the  the  value

programed in the F4 setup menu.

The second item on the DS5000 menu is "Toggle CE2 On/Off". This

function (when enabled) permits the DS5000 to access the upper 32K

of internal memory. Unless you have a 64K device, this bit  should

be left disabled.

MONICA: A PC Hosted 8052 Monitor                                 Page: 15

5. INTERFACE CIRCUITS

5.1 RESET circuits

Most low cost 8051/52 boards use a simple reset circuit similar

to the one from the INTEL application notes. This circuit may  not

provide a "bulletproof" reset after very short power  outages  and

brownouts, however it is very  easy  to  implement,  and  entirely

adaquate for most applications.

A reset when RTS is deasserted can be  added  to  this  circuit

with an RS-232 receiver (7489, 75150, MAX232, 233 etc.), a  diode,

and a resistor:

+5V

|  +|| 5uf

RS-232 Receiver   +---||-----+------------ RESET (to CPU)

(inverting)         ||     |

[\     +\ |           |    10K

RTS --[ >O---| >|--/\/\/\/--+--/\/\/\/-- Ground (0 volts)

[/     +/ |  300 Ohm

1N4148

Since common RS-232 receiver IC's have two or more receivers in

each package, single board computers using such an IC may  have  a

spare receiver available.

WARNING! When RTS controlled RESET is  activated,  the  charged

capacitor will try and "push up" the +5V rail (briefly). For  this

reason, the capacitor should be as small as possible. For CMOS and

other low power systems in which the +5V rail may not  be  capable

of absorbing this power,  this  circuit  is  unsuitable,  and  you

should substitute the circuit below.

The following circuit can be used to implement a reset when RTS

is deasserted, and a more  reliable  power-up  reset  (if  RTS  is

unconnected). It does not require an RS-232 receiver:

+---------------+------+--- +5V

|        5K     |      |

|   +--/\/\/\/--+      |

|   |   8.2K       ||+ |

|   +--/\/\/\/--+--||--+

10K    |   |  HP2835   |  || 10uf

+--/\/\/\/--+   |   +\ |    |

|               +---| >|----+---------- RESET (to CPU)

|               |   +/ |    |

|  +\ |    | /--+         -----

RTS --+--| >|----|< 2N3904       /^\ IN4148

+/ |    | \>-+         +---+

1N4148        |           |

+-----------+---------- Ground (0 volts)

MONICA: A PC Hosted 8052 Monitor                                 Page: 16

The following circuit can be used to implement a reset when RTS

is asserted ('/Reset' option enabled), and a  power-up  reset  (if

RTS is unconnected). It does not require an RS-232 receiver:

+-----------+------------- +5 Volts DC

|           |

+---+----+      |+

RTS ---/\/\/\/\---| 2N9606 |    ----- 10uf, 16v

20K      +---+----+    -----

|           |

+-----------+

+-------+----- Reset (to CPU)

|       |

/     --+--

8.2K \      /^\

/     +---+ 1N4148

|       |

+-------+----- Ground (0 Volts)

MONICA: A PC Hosted 8052 Monitor                                 Page: 17

5.2 PSEUDO-ROM Circuit

The  following  decoding  circuit  can  be  used  to  implement  a

debugging environment with 2K  DEBUG  ROM,  up  to  62K  CODE  RAM

(Pseudo-ROM), and 64K DATA RAM, using the DTR line from the PC  to

switch between download and run modes:

PSEN- -----------------------------+-------- DEBUG ROM OE-

|

A15 -[\                          +-------- CODE RAM OE-

[ )--+

A14 -[/   |  74LS32 x 4

+-[\

A13 -[\     [ )--+

[ )----[/   |       +---------------- DEBUG ROM SELECT-

A12 -[/          +-[\    |

[ )---+--[>O----------- CODE RAM SELECT-

A11 ---------------[/      74LS04

0v (Gnd) ----------- DATA RAM SELECT-

RS-232 Receiver

(inverting)

DTR ---[>O-+------[\

|      [ )--------------------- DATA RAM WR-

|   +--[/

WR- -----------+    74LS32 x 2

|   +--[\

|      [ )--------------------- CODE RAM WR-

74LS04 +-[>O--[/

RD- -------------------------------------- DATA RAM OE-

Note: Selects are for 64K devices, additional decoding with A15,

A14, A13 etc. will be required for smaller devices.

DEBUG ROM SELECT is activated (0) only when A11-A15 are  0,  which

occurs when  the  CPU  accesses  the  lower  2K  of  memory.  When

accessing above 2K, A11-A15 will have at  least  1  non-zero  bit,

which causes the CODE RAM SELECT to be activated. The  PSEN  input

on the ROM/RAM OE pins insures that they  will  be  accessed  only

during code fetches.

When DTR is asserted (HIGH), the output of the RS-232 receiver  is

0, which allows WR- to pass through the OR gate to DATA  RAM  WR-.

CODE RAM WR- is held high (OFF) by the inverted  receiver  output.

When DTR is de-asserted (LOW), the output of the  RS-232  receiver

is high, and holds DATA RAM WR- high (OFF). WR- is allowed to pass

through the other OR gate to CODE RAM WR-.

MONICA: A PC Hosted 8052 Monitor

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

1.1 Host System Requirements                                        1

1.2 Target System Requirements                                      2

2. STARTING UP MONICA                                                  3

3. USING MONICA                                                        4

3.1 The Main Control Panel                                          4

3.2 The Memory Editor                                               9

3.3 Messages                                                       11

4. SINGLE-CHIP EMULATION WITH THE DS5000                              13

4.1 Loading the kernal                                             13

4.2 Starting and configuring MONICA                                14

5. INTERFACE CIRCUITS                                                 15

5.1 RESET circuits                                                 15

5.2 PSEUDO-ROM Circuit                                             17