New features in EMILY / MONICA release 1.3 :

- Serial I/O may now be redirected to a window (with ZOOM), or to a

PC COMM port in BOTH EMILY and MONICA. EMILY's F3 serial setup menu

has been added to MONICA, and has received a new 'Local' option which

selects I/O in the PC window.

- Serial I/O "fine tuning" now shows only the actual speeds which are

supported by the PC COMM port hardware.

- MONICA now supports single-chip in-circuit emulation with DS5000 type

processor. New '/DS5000' command line option, and F4 setup menu have

been added.

- EMILY and MONICA now support user defined Special Function Registers,

for use with expanded 8051/52 variants.

- New EMSETUP utility for configuring default setup values in EMILY and

MONICA, including screen colors, Special function registers, COMM port

assignment and configuration, reset mode, default filename etc.

- New '/Reset' option to reverse sense of RTS controlled reset.