HW 30 - Sense Lines

Hardware	M.HW.SenseLines

Revised by: 	Rich Kubota	May 1994

Revised by: 	Rich Kubota	May 1993

Written by:	Rich “I See Colors” Collyer and Blake Roberts	September 1992

Ever wonder how to set up an Apple video card or Macintosh built-in video to support various size monitors? Well, this Technical Note will tell you everything you need to know about what monitors are supported and how.

Changes since February 1993: Updated list to include latest Macintosh Products as of April 1994.  Added discussion of the new type 6 extended sense code.  This new code is in addition to the previously discussed extended sense code protocol which is now designated as type 7.  Show the new type 6 sense codes for three multiple scan monitor configurations.

Changes since September 1992: Corrected the sense line pin descriptions in Figures 1, 2, and 3, plus the associated descriptions. In the previous release of the Technical Note, sense lines 2 and 0 were inverted. In each example, sense line 0 was marked as sense line 2 and vice versa. Modified Figure 3 to show only the relevant combinations of extended sense line combinations as presented in Table 4, “Extended Sense Line Descriptions for each Monitor.” Provided additional description on understanding the regular and extended sense line protocol notation. Added the Spring ’93 CPUs to Table 5, the list of what video hardware supports what monitors.

Sense Lines

The Sense Line Protocol was implemented when Apple recognized the need for a mechanism that would allow a display card to identify the monitor connected to it. For example, the built-in video display circuitry on the Macintosh Quadra and on the PowerBook 160, 165c, and others, can configure themselves according to the monitor that is connected at boot time. The identification scheme works fine, but there is one problem. Three sense lines limit the number of different monitors to seven plus the no-connect case. To overcome this limitation, newer display cards and built-in video use an extension to the sense line scheme that allows for 28 new codes.

The extension is based on the following idea: When the display circuitry senses a configuration that in the original scheme signals “no display connected” (in other words, when all three sense lines are not grounded), the card pulls down each sense line, one by one, and reads back what the other lines return. To return a unique code, the only requirement is that the sense lines be connected, in the cable or the monitor itself, by wires or diodes. The beauty of this idea is that existing monitors are detected correctly. Newer monitors, such as the Apple 16" Color Display, can have their own encoding, and the circuitry for detecting new monitors is relatively simple. Since there are no active components, adding the encoding to new or existing monitors involves only a few inexpensive diodes and a little wire.

The Original Scheme

In the original scheme of things, the display circuitry determines the type of monitor attached by reading three sense lines and comparing the signal value to ground. By convention, the sense lines were identified as 0, 1, and 2. Given the three lines and the two different states, on or off, there were a total of eight possible combinations. For a monitor to be recognized, a sense line on the monitor side was connected to ground to be read as a binary 0, or left unconnected to be read as a binary 1. The first seven monitors listed in Table 3 show the required sense line states for them to be recognized using the original sense line scheme.

It’s important to note that monitors using the original sense line scheme need only have their sense lines read once to determine the monitor type. As new monitor types became available for the Macintosh, the Extended Sense Line Protocol was implemented.

Extended Sense Line Protocol (Type 7)

The Extended Sense Line Protocol is one of those oddities of software engineering that can at first defy logic. One wonders how late in the evening or how many cans of Mountain Dew were consumed before the protocol notation was defined. The extended sense line protocol sets the guidelines for defining a 6-bit binary number that defines how the monitor sense lines must respond in order to be recognized correctly.  Note that this sense code protocol may also be referred to as the Type 7 Extended Sense (no-connect) Code Protocol.

With this unusual introduction, let’s step back. Under the original scheme, sense lines were compared to ground to determine their state. To reiterate, with three pins, there are a total of eight possible combinations. Seven combinations of the sense line states were assigned to the early monitor types. The eighth sense line combination has become reserved for use to signal the extended sense line protocol. In this eighth configuration, none of the three sense lines are tied to monitor ground. When the display system software reads the state of the monitor sense lines and finds that none of the three lines are grounded, the type $07 sense line protocol is assumed. For the type $07 sense line protocol, the sense lines get tied to each other within the monitor using a combination of straight wire and/or diode connections.

Under this sense line protocol, each sense line is asserted and the other two sense lines are read. Instead of being compared to ground, each line is compared to the asserted line. If a sense line is the same state as the asserted sense line, the state equates to a binary 0, otherwise, it is binary 1.

The diagram in Figure 1 shows an example of the Type 7 Sense Line Protocol notation. There are three pairs of binary numbers. Each pair of binary digits represents the relative state of two sense line pins to the third sense line pin, which is held low. Each bit is sense line specific. The idea is that software tells the video card/hardware to hold one line low and read the result of the other two lines.

Figure 1—Extended Sense Line Decode Method

To make “sense” of the protocol notation, we now present a diagram showing some theoretical interconnection between the sense lines. Given the extended sense line notation of 11 00 01, Figure 2 shows how the monitor cable sense lines would be interconnected for the system to detect a specific extended monitor type. In the Extended Sense Line Protocol, the first pair of binary digits represent the sense state of pins 1 and 0 when sense line 2 is held low. The diagram maintains the same sense of order, showing sense line 2 on the left side and sense line 0 on the right.

To read the chart in Figure 2: The three points of the triangle correspond to the three sense lines. The diodes and wires show the required connection. The 6-bit binary number below each diagram (see Figure 3 for all the current valid diagrams) corresponds to the result that will be read when polled. It is read: when 2 is pulled low, 1 and 0 return the first two digits; when 1 is pulled low, 2 and 0 return the second two bits; when 0 is pulled low, 2 and 1 return the last two bits.

Figure 2—Extended Sense Line Decode Sample

Figure 3—Extended Sense Line Decode Diagram (Type 7)

Figure 3 shows the sense line interconnections required for the existing monitors that require Type 7 sense line coding.

It is important to clarify the NTSC (Alternate) and the PAL (Alternate) diagrams for those who might wonder whether a diode is missing. For NTSC (Alternate), the binary extended notation is 01 01 00. One might at first feel that for the last pair of binary digits, 00, to make sense, there must be diode between sense line 0 and 1. In fact since sense line 1 is wired to sense line 2, asserting sense line 0 means that sense lines 2 and 1 will be read at the same state. Similar logic applies to the PAL (Alternate) diagram. This saves the implementation of another diode. For the PAL diagram, any one of the three sense line interconnections could be removed and the same extended sense line logic would be maintained.

Extended Sense Line Protocol (Type 6)

With the implementation of the New Display Manager, the sense line protocol is extended further for a family of monitors.  The type 7 extended sense line protocol takes its designation from the fact that all three sense lines are high relative to ground and can be presented in binary format as 0b111.  The same idea is extended to the three cases where one sense line is grounded.  For example, when sense line 0, (pin 4) is grounded to pin 11, the binary notation is expressed as 0b110 (least significant bit is for sense 0). From Table 3 below, one notes that the sense line configuration for this case, corresponds to the Apple RGB 13" monitor.  To extend the support of monitors in this family, one can add three new codes by connecting sense lines 1 and 2 with 1. a jumper, 2. a diode in one direction, and 3. the same diode reversed.  These new type 6 extended sense codes support three new multiple scan monitors, each of which is capable of supporting the standard 640x480 resolution in addition to other resolutions.  The Extended Sense Line Decode Table for type 6 is presented in figure 4.

Figure 4—Extended Sense Line Decode Diagram (Type 6)

Multiple Scan Monitors

The Power Macintosh VRAM expansion cards include support for several new monitor types which use the Type 6 Extended Sense Codes.  These new types are multiple scan rate monitors which can display in multiple screen resolutions.  The VRAM expansion cards being shipped with the Power Macintosh units will support the multiple scan monitors with the resolutions listed in Table 1.  Note that the Apple Multiple Scan 20 Display is sensed as a multiple scan 21 display type.

Table 1  Supported Multiple Scan Screen Resolutions

Monitor Type	Screen Resolution	Frequency (MHz)

multiple scan 13	640 x 480	67

832 x 624	75

multiple scan 17	640 x 480	67

832 x 624	75

1024 x 768	75

multiple scan 21	640 x 480	67

832 x 624	75

1024 x 768	75

1152 x 870	75

Video Connector

Table 2  Signal Assignments for the DB-15 External Video Connector

Pin	Signal Name	Signal Description

1	RED.GND	Red Ground

2	RED.VID	Red Video Signal

3	/CSYNC	Composite Sync Signal

4	SENSE0	Monitor Sense Line 0

5	GRN.VID	Green Video Signal

6	GRN.GND	Green Ground

7	SENSE1	Monitor Sense Line 1

8	n.c.	Not Connected

9	BLU.VID	Blue Video Signal

10	SENSE2	Monitor Sense Line 2

11	C&VSYNC.GND	Ground for CSYNC & VSYNC

12	/VSYNC	Vertical Sync Signal

13	BLU.GND	Blue Ground

14	HSYNC.GND	HSYNC Ground

15	/HSYNC	Horizontal Sync Signal

Sense Line to Monitors

Table 3  Sense Line Descriptions for Each Monitor

Signal	Sense 0	Sense 1	Sense 2	Frequency (MHz)

RGB 21"	0	0	0	100

NTSC	0	0	1	12.2727 MHz

RGB 12"	0	1	0	15.6672

B&W 12" & RGB 13"	0	1	1	30.24

B&W 15"	1	0	0	57.2834

RGB 15"	1	0	1	57.2834

B&W 21"	1	1	0	100

RGB 16"*	1	1	1	57.2834

RGB 19"*	1	1	1	80

VGA*	1	1	1	25.175

Super VGA*	1	1	1	35.16

NTSC w/convolution*	1	1	1	12.2727

PAL*	1	1	1	14.75

PAL w/convolution*	1	1	1	14.75

Multiple Scan 13"*	0	1	1	variable

Multiple Scan 17"*	0	1	1	variable

Multiple Scan 21"*	0	1	1	variable

* These monitors require extended sense line support.

Note:	The binary values in this table indicate the relative state of the sense pin measured against monitor ground, pin 11, from Table 2 above.

Table 4  Extended Sense Line Descriptions for Each Monitor

SIGNAL	1-0(2 low)	2-0(1 low)	2-1(0 low)

PAL	00	00	00

NTSC (Alternate)	01	01	00

VGA/Super VGA	01	01	11

RGB 16"	10	11	01

PAL (Alternate)	11	00	00

RGB 19"	11	10	10

Multiple Scan 13"	00	00	11

Multiple Scan 17"	00	10	11

Multiple Scan 21"	10	00	11

Note:	1-0(2 low) indicates that the software is driving monitor sense line 2 and reading back monitor sense lines 1 and 0. For example, if you have a PowerBook 160, 165c, 180, or 180c, you can make the CPU recognize that there is an Apple 16" RGB external monitor attached by noticing that the sense line code in Table 4 for such a monitor is 10 11 01. From Figure 3, this sense code equates to sense line 2 being connected to sense line 0. Using Table 2, this means that by tying pin 4 to pin 10, the PowerBook will think that the larger monitor is attached. It’s important to recognize that pins 4 and 10 in this example must not be tied to ground, otherwise, a 12" RGB monitor would be detected.

What Video Hardware Supports What Monitors

Table 5  Monitor Versus Video Hardware Versus Supported Depth

4•8:

Monitor	Max. Bit Depth

NTSC	8

NTSC w/convolution	8

RGB 13"	8

B&W 15"	4

B&W 21"	4

B&W 12"	8

PAL**	8

PAL w/convolution**	8

RGB 16"**	4

RGB 21"**	4

** These monitors are supported on this card if the card has a new  ROM (see your

dealer).

8•24:

Monitor	Max. Bit Depth

NTSC****	1-8 w/conv; millions w/o conv

RGB 13"	Millions

B&W 15"	8

B&W 21"	8

B&W 12"	Millions

PAL****	1-8 w/conv; millions w/o conv

RGB 16"**	8

RGB 21"**	8

**These monitors are supported on this card if the card has a new ROM (see your

dealer).

**** In the Monitors control panel, the display circuitry implements convolution

automatically for bit depths 1–8, and nonconvolution for the millions bit depth

8•24GC:

Monitor	Max. Bit Depth

NTSC****	1-8 w/conv; millions w/o conv

RGB 13"	Millions

B&W 15"	8

B&W 21"	8

B&W 12"	Millions

PAL****	1-8 w/conv; Millions w/o conv

RGB 16"**	8

RGB 21"**	8

** These monitors are supported on this card if the card has a new ROM (see your

dealer).

**** In the Monitors control panel, the display circuitry implements convolution

automatically for bit depths 1–8, and nonconvolution for the millions bit depth

24AC:

Monitor	Max. Bit Depth

VGA	Millions

Super VGA 800 x 600	Millions

Super VGA 1024 x 768	Thousands

RGB 13"	Millions

B&W 15"	8

B&W 21"	8

B&W 12"	Millions

RGB 16"	Millions

RGB 21"	Millions

Apple Multiple Scan 20 Display	Millions

Macintosh LC/LC II

Monitor	Max. Bit Depth

VGA	8

RGB 13"	8

RGB 12"	Thousands

B&W 12"	8

Macintosh IIvx/IIvi

Monitor	Max. Bit Depth

VGA	8

RGB 13"	8

RGB 12"***	Thousands

B&W 12"	8

*** For the 12" RGB monitor, the built-in video does not support 1 bit/pixel mode.

Macintosh IIci/IIsi

Monitor	Max. Bit Depth

RGB 13"	8

RGB 12"	8

B&W 15"	4

B&W 12"	8

RGB 15"	4

Macintosh LC III

Power Macintosh 6100/60 Built-in Video

Power Macintosh 7100/66 Built-in Video

Power Macintosh 8100/80 Built-in Video

Monitor	Max. Bit Depth

VGA	Thousands

RGB 13"	Thousands

RGB 12"	Thousands

B&W 12"*****	Thousands

B&W 15"	8

RGB 16"	8

***** Thousands mode is available because the built-in video circuitry cannot

distinguish between the 12" black-and-white and the 13" RGB monitors. The

12" black-and white display is capable of 256 gray levels; however, when the

Thousands mode is selected, there are only 32 gray levels available due to the

way 16 bits-per-pixel support is implemented.

Macintosh LC 475/Quadra 605

Monitor	Max. Bit Depth

NTSC	Thousands

VGA/SuperVGA	Thousands

B&W 12"	8

RGB 12"	Thousands

RGB 13"	Thousands

B&W 15"	8

RGB 16"	Thousands

RGB 19"	8

B&W 21"	8

RGB 21"	8

PowerBook 160/165c/180/180c/MiniDock/DuoDock

Monitor	Max. Bit Depth

VGA/Super VGA	8

RGB 13"	8

RGB 12"	8

B&W 12"	8

B&W 15"	4

RGB 16"	8

Macintosh Quadra 700/900

Monitor	Max. Bit Depth

NTSC	Millions

NTSC w/convolution	8

VGA	Millions

Super VGA	Millions

PAL	Millions

PAL w/convolution	8

RGB 13"	Millions

RGB 12"	Millions

B&W 15"	8

RGB 16"	Millions

RGB 21"	8

B&W 21"	8

B&W 12"	Millions

Macintosh Quadra 660AV

Monitor	Max. Bit Depth

NTSC	Thousands

NTSC w/Convolution	8

VGA/SuperVGA	Thousands

PAL	Thousands

PAL w/Convolution	8

RGB 12"	Thousands

RGB 13"	Thousands

B&W 15"	8

RGB 16"	Thousands

RGB 21"	8

B&W 21"	8

B&W 12"	Thousands

RGB 19"	8

Note:	For the Quadra 660AV, the maximum bit depth settings assume all VRAM set for use by video out.

Macintosh Quadra 950

Macintosh Quadra 840AV

Monitor	Max. Bit Depth

NTSC	Millions

NTSC w/Convolution	8

VGA	Millions

Super VGA	Millions

PAL	Millions

PAL w/Convolution	8

RGB 12"	Millions

RGB 13"	Millions

B&W 15"	8

RGB 16"	Millions

RGB 21"	Thousands

B&W 21"	8

B&W 12"	Millions

RGB 19"	Thousands

Note:	For the Quadra 840AV, the maximum bit depth settings assume all VRAM set for use by video out.

Macintosh Centris 610/650

Macintosh Quadra 610/650/800

Monitor	Max. Bit Depth

NTSC	Thousands

VGA/SuperVGA	Thousands

PAL	Thousands

RGB 13"	Thousands

RGB 12"	Thousands

B&W 15"	8

RGB 16"	Thousands

RGB 21"	8

B&W 21"	8

B&W 12"	Thousands

RGB 19"	8

Power Macintosh 7100 VRAM Expansion Card

Monitor	Max. Bit Depth

NTSC	Millions

VGA/Super VGA	Millions

PAL	Millions

RGB 12"	Millions

RGB 13"	Millions

B&W 15"	8

RGB 16"	Millions

RGB 21"	Thousands

B&W 21"	8

B&W 12"	8

RGB 19"	Thousands

Apple Multiple Scan 20 Display	Thousands

Power Macintosh 8100 VRAM Expansion Card

Monitor	Max. Bit Depth

NTSC	Millions

VGA/Super VGA	Millions

PAL	Millions

RGB 12"	Millions

RGB 13"	Millions

B&W 15"	8

RGB 16"	Millions

RGB 21"	Millions

B&W 21"	8

B&W 12"	8

RGB 19"	Millions

Apple Multiple Scan 20 Display	Millions

Power Macintosh AV Card

Monitor	Max. Bit Depth

NTSC	Millions

NTSC w/Convolution	8

VGA	Millions

Super VGA 800 x 600	Millions

Super VGA 1024 x 768	Thousands

PAL	Millions

PAL w/Convolution	8

RGB 12"	Millions

RGB 13"	Millions

B&W 15"	8

RGB 15"	Thousands

RGB 16"	Millions

RGB 21"	Thousands

B&W 21"	8

B&W 12"	8

RGB 19"	Thousands

Apple Multiple Scan 20 Display	Millions

Further Reference:

•	Guide to the Macintosh Family Hardware, second edition, Displays

•	develop Issue 3, “Macintosh Display Card 8•24GC: The Naked Truth”

•	Developer Notes (for each CPU)

•	M.HW.ColorMonitors