From: Denis Pelli (3/17/1995)To:   Apple Developer Support,   Apple PCI Bus DevelopmentCC: David NagelSubject:Video Drivers            5:06 PM        3/17/1995March 17, 1995Dear Apple,We, the 41 undersigned visual scientists [and another ten who endorsed theletter after it was sent], are writing to suggest the addition of a new"cscSetModeTiming" video driver control call to the "Display Driver Calls"chapter of "Designing PCI Cards & Drivers".THE NEED:Today's video cards and monitors are mostly capable of running with apractically infinite range of resolutions and field rates (timings).Current MacOS software, recently enhanced by the addition of the DisplayManager, restricts this infinite spectrum to a short list of modes(timings) specified in the video card's ROM and driver. This creates aperennial compatibility problem as old video cards cannot drive newdisplays with unanticipated timings. (In principle a new driver could beinstalled, but this would require writing a new driver for each videocard that is to support the new display, which is impractical.) Newdisplay devices--LCD, projection, HDTV, stereo, etc.--are coming and itseems likely that they will function best with new timings that are hardto anticipate today.One of the best ways of generating stereo imagery (for virtual realityand cyberspace) is to double the monitor's frame rate (e.g. to 134 Hz)and to use LCD glasses that allow each eye to see alternate frames. Manycommercially available video monitors will run at such high frame rates,but, alas, most Mac video drivers have no provision for it, even thoughthe video card is capable.The availability of custom timings would provide a new opportunityfor game developers. Games often achieve exciting effects by pushing thehardware to its limits. Some very interesting effects could be generatedby smoothly changing the timing from spatially coarse high frame rateimages for fast motion sequences to spatially fine temporally slow (e.g.67 Hz) frame rates for intricate imagery.Many vision studies require high frame rates, over 100 Hz, in order tostudy stereo, motion perception, and retinal phenomena withoutartifacts. Many monitors and most video cards would run happily at suchhigh frame rates, but hardly any of today's Mac video drivers will allowit.TESTIMONIALS TO THE NEED:Patrick Cavanagh, patrick@burrhus.harvard.edu, writes "High-frame-ratevideo cards are available for the PC, typically for stereo. None areavailable for the Mac. Many of us and those in computer graphics arebeing forced to the PC market because of this oversight."James Dinunzio, dinunzio@scf.usc.edu, writes, "Hi. I am a graduatestudent at University of Southern Cal in the Computer ScienceDepartment. I am a member of a newly formed group working under facultydedicated to graphics and virtual reality. I am currently working on aproject to produce decent stereopsis on the Macintosh, including stillstereo images, movies, and if possible "live" walkthroughs. I will beusing a pair of LCD shutter glasses controlled through the serial portand running at a nominal 30 fps, 60fps for both eyes. ... Anysuggestions?"Tom Busey, busey@indiana.edu, adds, "I have just finished writing codethat displays color stereo 3-d pairs for use with LCD glasses. This codeuses sequential-frame presentations synched with LCD glasses controlledvia the serial port to provide the illusion of depth. While thistechnology is old, what is new is my code that takes two pict files andreduces the colors from millions of colors to what amounts to 4 bits perred, green and blue channel for each picture (on monitors with millionsof colors) or 2 bits per channel for monitors with thousands of colors.The two pictures are then added together and then I use clut switchingto switch between the two pictures. The stereo 3-d code uses calls from(not surprisingly) the VideoToolbox code library [described below]. Aswith all the code I write, this code is freely available to anyone whowants it. I'm currently working with LCD glasses distributors to have itdistributed free to purchasers of LCD glasses. It may be downloadedfrom:"ftp://ftp.psych.indiana.edu/pub/busey/AdjustStereoPicts.sea.hqx"The bottom line is that this approach makes virtual reality a (virtual)reality on macs, for very low cost (about $150 for the glasses, plussome parts for a very simple interface). However, because of therelatively slow frame rate of my 2-page monitor (67 hz), I am onlygetting about 33 images per eye per second. This causes visible flicker,and I would love to be able to increase the frame rate to 120 hz. Suchan increase would not only reduce the flicker, but it would also makethe glasses and the mac research-grade for doing experiments. Thus Isupport your call for Apple to add an adjustable frame rate option."[The following testimonials were received after the originalletter was sent. 3/22/95]Mats Lind,  mats.lind@cmd.uu.se, writes, "3-D Max, a Swedish company,working with a Korean supplier, has just started to deliver attractivelypriced LCD-shutter glasses and accessories giving binocular stereocapabilities to PCs. Their product received enormous attention at theCe-Bit fair in Hannover last week. The estimated retail price in the USwill be less than 180 USD (including some extra software like 3-D games)and will probably reach a high market share. 3-D Max, is very interestedin entering the Apple market segment as well, but to do so requires theusual 120+ Hz."Chris Tyler, cwt@skivs.ski.org, writes, "We are using the VideoToolboxto develop a two-monitor stereo display system. We would be delightedto know of calls that could control the raster rates and video cardsynchronization. I think that I can speak for many investigators at theSmith-Kettlewell Institute, which probably owns about 50 Macintoshes,that the features you propose would be of great value to many."FIVE SUGGESTIONS TO APPLE:We suggest the following additions to the "Display Driver Calls" chapterof "Designing PCI Cards & Drivers".1. (MAJOR) Add a cscSetModeTiming control call to the repertoire forvideo drivers to allow specification of an arbitrary new timing.Using an extended VDTimingInfoRec struct, or perhaps an entirely newstruct, the calling program would specify the desired timing (i.e. pixelrate, line rate, field rate, frame synch width, frame front porch, frameback porch, frame synch polarity, line synch width, line front porch,line back porch, line synch polarity, synch on green (y/n), combinedsynch polarity, and interlaced/noninterlaced, as well as the desiredrowBytes and page [oops, I meant "pageBytes"]) and issue acscSetModeTiming control call to the video driver. In response, thevideo driver would change the mode to comply, as nearly as possible,with the request, and, before returning, would update the caller'sstruct to reflect the achievable timing. The driver would also beresponsible for updating the mode's other derivative specifications(bounds rect, etc.) that are used by other control and status calls(e.g. cscSwitchMode, cscGetPages, cscGetModeTiming,cscGetNextResolution, cscGetVideoParams). The effect of this changeshould be persistent, lasting at least until shut down.(Timing changes that do not affect rowBytes would not require callingthe Display Manager to repaint the screen, so most timing transitionscould be done smoothly, without any visible glitches. The applicationwould typically call DMSetDisplayMode only at the end of the specialeffect, mainly to update the bounds rect.)(cscSetModeTiming is a powerful call that could create timings thatwould not effectively drive the monitor, and might even harm it.However, this power can be accessed only through a Control call to thedriver, and thus is inaccessible to the casual user. We would expect itto be used only by applications written by the monitor manufacturers anddevelopers of games and virtual reality environments and otherspecial-effects applications.)cscSetModeTiming would allow an application run at start up to customizeANY video card (whose driver supports the call) to drive a noveldisplay, or drive an old display at novel timings. The new timing wouldpersist until shut down or restart.2. (MINOR) It would be helpful to enhance the correspondingcscGetModeTiming status call to return the same timing info.3. (MINOR) Please add a trivial cscGetVBLBitAddress status call toreturn the location of the VBL bit. As far as we know, every Mac videodevice has a VBL bit, somewhere, that is true only during blanking.Unfortunately there is no easy way to find it. For many special effectsit is necessary to synchronize multiple video monitors. Since everyvideo card insists on being a master, and they have independent clocks,monitors normally run asynchronously of one another. In the past we'veused custom hardware and software tricks, specific to particular videocards, to reset multiple cards at once, or to halt the clock on oneuntil it drifts into phase with another. A better, general, solutionwould be to use cscSetModeTiming to tweak the timing of one video cardrelative to the other until they come into synch. However, to do this,we need to be able to continuously monitor the VBL bit of each device.(Using a VBL interrupt service routine to set a bit at a known locationis much less satisfactory because of the extra uncertainty in timing andbecause this gives us only the time of onset of VBL, denying us accessto the time of offset.)4. (VERY MINOR) We'd like to request that a clarification be added tothe documentation of the cscSetGray control call. Most video driversimplement this call correctly. However, a few video drivers (e.g.PowerMac 7100 vram driving an Apple 2-page monochrome monitor, and theRadius PrecisionColor

8 and 8Xj driving a Radius 2-page monochromemonitor), upon detecting the presence of a monochrome video monitor,lock themselves into the monochrome state (i.e. always do luminancemapping) irrespective of calls to cscSetGray. The cscSetGray callsreturn without error and calls to cscGetGray misleadingly imply that themono/color mode is being changed, but the driver stubbornly continues todo luminance mapping. This prevents us from using some very usefulspecial effects that depend on having full control of the CLUT (i.e.color mode) while driving a monochrome display (see Pelli & Zhang,1991). Please tell the developers of video drivers not to get tricky:don't disable cscSetGray.5. (VERY MINOR) A surprisingly large number of video drivers (e.g. MacIIci Built-In Video, Relax 19" Model 200) fail to correctly implementcscGetEntries, even though it is a required status call. Perhaps asentence could be added to the documentation of cscGetEntries, advisingdevelopers to perform a write-then-read (cscSetEntries & cscGetEntries)test of their new driver before releasing it to the world. (TimeVideo,described below, does this test, which revealed the bugs.)Denis PelliApple PartnerProfessor of Neuroscience at Syracuse Universitydenis_pelli@isr.syr.eduKenneth R. AlexanderEye and Ear InfirmaryUniversity  of Illinoiskennalex@tigger.cc.uic.eduMichael BachDept. of OphthalmologyUniversity of FreiburgGermanybach@sun1.ruf.uni-freiburg.deMartin S. BanksSchool of OptometryUC Berkeleymarty@john.berkeley.eduPaul BeckmannPsychology DepartmentUniversity of Minnesotapaul@eye.psych.umn.eduPatrick BennettPsychology DepartmentUniversity of Torontobennett@psych.utoronto.caMargaret BradleyNIMH Center for the Study of Emotion and AttentionUniversity of Florida.BRADLEY@nervm.nerdc.ufl.eduDavid BrainardPsychology DepartmentUC Santa Barbarabrainard@condor.psych.ucsb.eduCatherine BurnsInstitute for Sensory ResearchSyracuse Universitykatey_burns@isr.syr.eduTom BuseyDepartment of PsychologyIndiana Universitybusey@indiana.eduPatrick CavanaghPsychology DepartmentHarvard Universitypatrick@burrhus.harvard.eduFrans W. CornelissenLaboratory for Experimental OphthalmologyCenter for Behavioural, Cognitive and NeurosciencesUniversity of GroningenThe Netherlandsf.w.cornelissen@bcn.rug.nlAndrew DerringtonPsychology DeptNottingham UniversityUKAndrew.Derrington@nott.ac.ukMichael P. EckertSchool of Electrical EngineeringUniversity of Technology, SydneyAustraliameckert@ee.uts.edu.auRhea T. Eskew, Jr.Dept. of PsychologyNortheastern Universityeskew@neu.eduBart FarellInstitute for Sensory ResearchSyracuse Universitybart_farell@isr.syr.eduMark W. GreenleeNeurologische UniversitaetsklinikUniversity of FreiburgGERMANYgreenlee@sun1.ruf.uni-freiburg.deStevan HarnadDepartment of PsychologyUniversity of SouthamptonUKharnad@ecs.soton.ac.ukMary HayhoeCenter for Visual ScienceUniversity of Rochestermary@cvs.rochester.eduAlun JohnsPsychology DepartmentOxford UniversityUKajohns@psy.ox.ac.ukKeith KarnCenter for Visual ScienceUniversity of Rochesterkeith@cvs.rochester.eduStanley KleinSchool of OptometryUC Berkeleyklein@adage.Berkeley.EDUPeter LennieCenter for Visual ScienceUniversity of Rochesterpl@cvs.rochester.eduDennis LeviSchool of OptometryUniversity of HoustonDLevi@UH.EDUDonald I. A. MacLeodPsychology DepartmentUniversity of California S.D.dmacleod@ucsd.eduTony NorciaSmith Kettlewell Eye Research InstituteSan Francisco, CAamn@skivs.ski.orgJoel PokornyEye Research Lab.University of Chicagos+p@chroma.uchicago.eduBertram O. PloogHunter CollegeCUNYploog@envmed.rochester.eduRussell A. Poldrack			Amnesia Research LabBeckman Institutepoldrack@uiuc.edu Manoj RaghavanInstitute for Sensory ResearchSyracuse UniversityManoj_Raghavan@isr.syr.eduAllison B. SekulerPsychology DepartmentUniversity of Torontosekuler@psych.toronto.caShui-I ShihPsychologySouthampton UniversityUKSHUI@psy.soton.ac.ukBen SingerCenter for Visual ScienceU Rochesterbens@cvs.rochester.eduVivianne C. SmithEye Research Lab.University of Chicagos+p@chroma.uchicago.eduJoshua SolomonVision GroupNASA AmesJoshua_Solomon@qmgate.arc.nasa.govHans StrasburgerInstitut fuer Medizinische PsychologieUniversity of MuenchenGermanyhans@groucho.imp.med.uni-muenchen.deJoann M. TaylorColor Technology Solutionsjoannt@teleport.comMarty WachterSchool of Medicine         Johns Hopkins Universitymrw@welchgate.welch.jhu.eduAndrew B. WatsonVision GroupNASA Amesbeau@vision.arc.nasa.govLan ZhangPsychology Dept.Syracuse Universitylzhang@mailbox.syr.eduYi-Xiong ZhouCenter for Neural SystemsNYUzhou@cns.nyu.edu[In addition to the above 41, the following 10 people also askedto be included as signatories, but I received their replies too late for the original letter. 3/27/95]Lawrence K. CormackPsychology DepartmentUniversity of Texas at AustinCORMACK@PSY.UTEXAS.EDUJames DinunzioComputer Science DepartmentUSCdinunzio@scf.usc.eduLewis O. Harvey Jr.Department of PsychologyUniversity of Coloradolharvey@clipr.colorado.eduJu LiDepartment of PsychologyColumbia Universityta-jl134@columbia.eduMats LindCenter for Human Computer StudiesUppsala UniversitySweden mats.lind@cmd.uu.seWalter MakousCenter for Visual ScienceUniversity of Rochesterwalt@cvs.rochester.eduDavid RoseDepartment of PsychologyUniversity of SurreyUKpss1dr@surrey.ac.ukStefan TreueDiv. of NeuroscienceBaylor College of Medicinestreue@bcm.tmc.eduChristopher TylerSmith Kettlewell Eye Research InstituteSan Francisco, CAcwt@skivs.ski.orgHugh R. WilsonVisual Science CenterUniversity of Chicagohrw6@midway.uchicago.eduBACKGROUND--THE VIDEOTOOLBOX:Denis Pelli uses Macs to do vision research. A '91 paper of hisdescribed a simple electronic circuit that combined the three DACoutputs of a color video card to achieve exquisitely precise grayscaleimages on a grayscale monitor. A recent paper, on how people seeletters, was the cover article of the June '94 issue of Nature.(Reprints are available.) A paper on how we see words is currently underreview at Science. Over the years since the Mac II appeared he developeda library of C software (the VideoToolbox) for doing vision research onMacs, and since '91 the complete archive has been available to thepublic. It is electronically available in source form from info-mac,updated several times a year, and is free for research purposes. Weestimate that one hundred vision research labs worldwide are using thissoftware, each with many Macs and students.ftp://sumex-aim.stanford.edu/info-mac/dev/src/video-toolbox-95-01-14-c.sitThe TimeVideo application included in the VideoToolbox checks out thetiming of all video devices in anticipation of their use in criticalreal-time applications, e.g. movies or lookup table animation. It seemsto be the only test program in existence for rigorous testing of Macvideo drivers. It uncovered bugs in drivers from Dome, Radius,RasterOps, Relax, SuperMac, TrueVision, and Apple, including the MacIIci "Macintosh II Built-In Video" (.Display_Video_Apple_RBV1), 4*8 and8*24 "Macintosh Display Card" (.Display_Video_Apple_MDC), "MacintoshDisplay Card 8*24 GC" (.Display_Video_Apple_MDCGC), and Quadra 700, 900,and 950 (.Display_Video_Apple_DAFB versions 0 and 2). Version3 of the .Display_Video_Apple_DAFB driver (e.g. in the Centris 650) isbug-free. All bugs have been reported to the manufacturers. We're happyto report that all of Apple's new drivers published in the last severalyears are bug free. Perhaps the driver-design teams have added TimeVideoto their testing suite.ftp://sumex-aim.stanford.edu/info-mac/Configuration/time-video-363.sitPelli, D. G. and Zhang, L. (1991) Accurate control of contrast onmicrocomputer displays. Vision Research, 31, 1337-1350.Solomon, J. A. and Pelli, D. G. (1994) The visual filter mediatingletter identification. Nature, 369 (6479), 395-397.Pelli, D. G., Farell, B., and Moore, D. C. (1995) The role of letters inrecognizing words. Under review at Science.Burns, C. W., Pelli, D. G., Farell, B., and Moore, D. C. (1995)Identifying letters. Submitted to Vision Research.TYPOS:In the "Display Driver Calls" chapter of interim-release "Designing PCI Cards & Drivers" on the MacOS SDK #2.p. 161 "Too improve ..."p. 163 "... 32-bit address#."p. 163 "lumimance"p. 163 "luminnace-"p. 167 "The Display Manager. also ..."Incidentally, the "Display Driver Calls" chapter would be easier to useif it were a complete document, not requiring reference to the old 68k"Designing Cards & Drivers".ACKNOWLEDGEMENTS:Thanks to Mike Landy for help with the timing specs.======================================================================