CM32.DK Help File

Loading in the file CM32.DK will setup Band-in-a-Box for the Roland CM32

multi-timbral synthesizer module as well as the Roland CM64(in the LA part

of it i.e. midi channels 2-10). It will setup:

1. Drum #'s for the CM32 and CM series.

2. Patch Map (assigned to factory settings of the CM32,CM64-LA section)

3. Startup Patches and Channels in the Midi Settings Window.

A Few Key Points to Know About the CM32:

String Patches are generally too loud so you might want to adjust the

volume of the String channel to your liking and save it to a MYSETUP.DK

file. That way,everytime you load Band-in-a-Box,your CM32 settings will be

intact.

At times you may detect some sounds being 'choked' or notes being cut off

while a style is playing.The reason for this is that the CM32 has a certain

amount of notes that can be played at once and the other is in the way their

sounds are made up. The sounds are made up of 'partials'.Some sounds use 1

or 2 of these partials while others can have up to 4;the latter being very

complex sounds such as "Warm Bell".When too many of these complex sounds are

occurring at once,it's too much for the CM32 to handle so it starts to get

'choked'. There is also only a certain number of notes that can be played at

once so either of these two things can be happening.The best solutions we

can recommend are:

a) Mute an instrument such as Strings(usually 4 partial sounds),

or   b) Select alternative sounds that would be less of a problem.

Once you've loaded CM32.DK, you are ready to start having fun with Band-

in-a-Box!

Have Fun!