PROTFX.DK Help File

Loading in the file PROTFX.DK will setup Band-In-A-Box

for the EMU Proteus FX .  It will setup:

1.  Drums mapped to the Proteus FX General MIDI drum kit

patch (Proteus FX

patch #112)..

2.  Patch Map (assigned to factory settings of the Proteus

FX).

3.  Startup Patches and Channels in the MIDI Settings

Window.

The EMU Proteus FX patches run along similar lines to

General MIDI but

requires mapping to be fully usable in Band-in-a-Box.  This

file uses Bank

2 (the first ROM preset bank) for the patches.  ROM banks are

guaranteed

not to change.  The first bank (Bank 0) is a RAM bank, and is

therefore not

guaranteed to have the expected patches at all times.

MIDI channel 10 is forced to load the above drum kit patch

through the

MIDI Settings dialog.

Your Proteus FX must be in "Multi" mode in order to handle

the multi-

timbral output of Band-in-a-Box. Set your Proteus FX up for

Multi mode

as follows:

1. Press the MASTER button to access the MASTER Menu.

2. Place the cursor at the top left of the display using the < >

cursor

buttons.

3. Turn the DATA knob to change screens until the "MIDI

MODE" screen

appears.

4. Press the > cursor button once to move the cursor under the

mode name.

5. Turn the DATA knob until "Multi" is displayed.

6. Press the MASTER button to leave the MASTER Menu.

Your Proteus FX must be mapped to Bank 2 for MIDI

channels 2, 3, 4, 5,

6, 7, 8, and 10.  This is accomplished through the MASTER

Menu as follows:

1.  Press the MASTER button to access the MASTER Menu.

2.  Place the cursor at the top left of the display using the < or

>

cursor button.

3.  Turn the DATA knob to change screens until the

"BANK/PRST CHANGE"

screen appears.

4.  Press the > cursor button once to move the cursor under the

channel #.

5.  Turn the DATA knob to select "Channel 02".

6.  Press the > cursor button once to move the cursor under the

channel

mapping.

7.  Turn the DATA knob to select "Bank2".

8.  Press the < cursor button once to return to the channel #.

9.  Repeat steps 6, 7 and 8 for channels 3, 4, 5, 6, 7, 8, and 10.

10. Press the MASTER button to leave the MASTER Menu.

To use this drum/patch file:

1. In the "MIDI" pull-down menu select "Load alternate

Drum/Patch file .DK".

2. Select "PROTFX.DK" in the Open dialog.

Have Fun!