***************

This documentation file is actually for the PSR500.

We don't have a DOC file for the PSS480, but we do have

a Drum/Patch kit file for the 480. Hopefully the DOC file

is of some help. Load in the PSR480.DK drum kit file, and

then read this DOC file.

***************************

NOTES ON HOW TO USE A PSR-500 WITH BAND IN A

BOX

Hardware requirements

=====================

The minimum requirement is of course to have a MIDI

interface recognized

by Band-In-A-Box (I use a MPU-401 compatible from Music

Quest) and the

cables to connect the interface and the synthsizer.

On the PSR-500

==============

Make sure you turn on the PSR-500 and configure for multi-

timbral

operation before running BIAB. This isdone by pressing twice

the "mode"

button in the upper right part of the PSR-500 control panel. The

PSR

returns to 'remote' (i.e. mono timbral operation) every time it is

switched off. The right two digits of the LED display should

read '99'.

If you have loaded BIAB before turning on the PSR, you can

nevertheless

correct the situation (after you have switched the PSR on and

set it to

multimbral mode) simply by going to the MIDI SETTINGS

menu and

exiting immediately by pressing the escape key. This should

send the

necessary midi data to the synthesizer.

In Band-In-A-Box

================

The MIDI SETTINGS menu should be set as follows (there are

other

possibilities, but this is the one I use)

Channel    Octave     Patch

Bass             3         0          33

Piano            1         0           1

Drums            2         0          99

Guitar           4         1          27

Horns            5         0          63

Strings          6         0          54

Melody           7         1          83

Through channel  0         0           0

The through channel should never be used since the PSR

cannot operate

with local off, i.e. notes played on the keyboard always

generate a

sound. If through channel is used, 2 notes will sound when a

key is

pressed (usually with a little delay)

General Midi implementation

===========================

The PSR 500 has only 100 sounds whilst GM provides 128.

Numbering is also

different: the PSR uses numbers from 0 to 99 and GM from 1

to 128. The PSR

has only one set of drums while GM has more.

I have not attempted to find an equivalent on the PSR for the

'exotic'

instruments since most of the time I don't even know how these

instruments

are supposed to sound.

PSR 400 compatibility

=====================

Although I could not test it on a PSR 400, I expect that the

configuration

file for the PSR 500 will work as well with PSR 400 since the

Yamaha

documentation I have concerns the 2 instruments and they have

the patch

numbers. The only differences between the two being in the

size of song

memory and the reverb effect available on the 500 but not on

the 400.

I hope this information will be useful for fellow users of these

very good

machines. I find they offer a very good value for money. If you

find anything

incorrect or have any question or suggestion, you can leave me

a message

on Compuserve:

User ID: Denis Wetzel

User # : 100120,355