DIGIFOSSIL V1.1 - Jan 11, 1992

Fossil Interpreter for DigiBoards

=================================

Technique Computer Systems

Ste 110, 1841 Oak Bay Ave

Victoria, BC   V8R 1C4

CANADA

BBS: (604) 598-1546

FAX: (604) 598-4318

What is it?

-----------

DIGIFOSSIL (DF) is  the program YOU  need in order  to get a huge

number of  doors, written for  FOSSIL systems,  working with your

Wildcat! IM system.  (if you need a FOSSIL for a non-IM/Digiboard

system,  try either  X00 or  BNU, which  are available  from many

BBSs)

What is a FOSSIL anyway?

------------------------

FOSSIL  (Fido/Opus/Seadog  Standard Interface  layer) was created

to  provide  a  standard   communications  interface  during  the

evolution of some other BBS  software (namely Fido(tm),  Opus and

Seadog(tm)).  It  provided a  superset of  the serial  port calls

available  on  the early  IBM  PC,  and provided  a  way  to port

programs from  one system  to another,  that may  no use hardware

compatible serial ports to the IBM PC.

By writing programs that use a standard set of calls, all that is

required  to move  the system  to dissimilar  hardware, is  a new

Fossil driver.

What does DIGIFOSSIL do?

------------------------

It provides the translation services so that calls made to FOSSIL

standards are  translated into equivalent  Digiboard calls.  This

allows HUNDREDS of  applications written for  FOSSIL to run under

IM, as if they were running  on a FOSSIL based system.  This list

includes many  popular doors such  as Tradewars  & Scrabble (tm).

Think  of  it as  the  'missing  link' between  your  Wildcat! IM

system, and vast number of existing doors.

- 1 -

DigiFossil (c) Copyright 1992, Technique Computer Systems

Installation

------------

Simple,  part of  your AUTOEXEC.BAT,  before Desqview  is loaded,

should load the TSR driver for DF.  Simply add:

DFTSR

to  your  AUTOEXEC.BAT.   When  DFTSR  starts,  it  will identify

itself,  and indicate  the  type of  Digiboard  installed.  DFTSR

currently  takes about  14k  of memory,  and  can be  loaded into

'HIGHRAM' using QEMM's LOADHI, or DOS 5's LOADHIGH commands.

It must be loaded AFTER the DigiBoard (XIDOS5) driver is loaded.

Using Doors that work with FOSSIL Drivers

-----------------------------------------

Although  DIGIFOSSIL may  not run  all doors,  it will  run those

doors that  require or  support FOSSILs.   If a  program does not

mention that  it will utilize  a FOSSIL, it  probably doesn't use

it, and work with DIGIFOSSIL.

When running a door that  requires FOSSIL support, simply use the

program DF to enable or  disable FOSSIL support in the DIGIFOSSIL

driver (DFTSR which was loaded in your AUTOEXEC.BAT).  To run DF,

simply specify which Digiboard channel,  and whether to enable or

disable support.   If no  port number is  given, DF  will use the

port specified by the environment variable WCPORTID.

** NOTE **

**    Be sure that if you enable FOSSIL support, that you

**    disable it again before re-loading Wildcat!.

**    (I suggest that you add DF OFF to your CAT.BAT)

examples:   DF ON 1     - turn on FOSSIL support for port 1

DF OFF      - turn off support for port WCPORTID

DF LOCK 1   - turn on FOSSIL support for port 1

but don't allow the door to change

the baud rate.

When  using doors  that support  FOSSIL,  such as  Scrabble, COM1

(w/FOSSIL), is the same as Digiboard  channel 1.  That is to say,

if  you set  WCPORTID  to 1,  your should  tell  the DOOR  to use

(FOSSIL) COM1.

If you have a door that does  NOT support a locked DTE rate (that

is when the baud rate to the modem is higher than the actual link

connection speed), AND  you are using  a modem that  has a locked

DTE, you should can use the DF LOCK command rather than the DF ON

command, which will prevent the  DOOR from changing the baud rate

on the modem.

- 2 -

DigiFossil (c) Copyright 1992, Technique Computer Systems

NOTE!

Programmers  writing code  that makes  FOSSIL calls

should  note  COM1, and  WCPORTID=1  both  refer to

channel 0 (DX=0) in the  actual with respect to the

actual FOSSIL and DigiBoard drivers.

Examples

--------

This is an  example DOOR.BAT for  Chris Hall's excellent Scrabble

door.

DOOR1.BAT

@ECHO OFF

REM This batch file is located in your HOME directory,

REM and is copied to the work directory and executed.

REM It needs to change to the right location of the

REM door and run the program

REM

REM  ******* Enable Fossil

DF ON

CD \DOORS\scrab

scrabdor node%WCNODEID%.cfg

CD \WC30

REM  ******* Remember to turn it off again

DF OFF

REM The above example takes its parameters from a

REM configuration file located in the door directory.

REM Others may take command line arguments for information

REM such as CHESS.EXE %WCPORTID% where the %WCPORTID% gets

REM replaced by the com port number by DOS when

REM the program is run.

Files

-----

The following files should be included in this archive, and for

authenticity,   it   should   contain   the   PKZIP  authenticity

verification for TECHNIQUE COMPUTER SYSTEMS.

DF.EXE         - to enable and disable DIGIFOSSIL support

DFTSR.EXE      - the DIGIFOSSIL driver

DF.DOC         - this document file

ORDERFRM.DOC   - Ordering information

Registration & Ordering

-----------------------

DIGIFOSSIL is  NOT free.  Although  it is distributed  in a fully

functional   form,  continued   use   beyond  30   days  requires

registration.   Registered  versions   are  always  available  to

registered users on the PLYWOOD PC BBS at 604-598-1546.

Registration costs $15 (US), or $18 (Canadian).  To order, please

use the order form found in this archive.  You can also order the

product  online on  the Plywood  PC BBS  at 604-598-1546,  or FAX

completed order forms to 604-598-4318.

- 3 -

DigiFossil (c) Copyright 1992, Technique Computer Systems

Support

-------

For further information, or technical support of DIGIFOSSIL, your

best bet is to call the call the Plywood PC BBS at 604-598-1546

(V32bis/HST), and join conference 2 (DIGIFOSSIL support).

Other options are: leave email to SCOTT BRYNEN in the 3rd party

support conference on MSI's Bulletin Board (805-395-0650), or the

MSI-OffTopic conference, that is echoed to many email networks.

Another option is to FAX us at 604-598-4318.

Some FOSSIL applications may not work.  If you have problems with

a specific application  (or have found one  that works well), let

us know, so  that in future  versions we can  list doors that are

known to work.

If  you  have  a  node  that  crashes  during  operation  of  the

DIGIFOSSIL, please make  a note of the  conditions under which it

happened,  and  if  you  are able,  open  another  DOS  window in

Desqview, and type

DF STAT <port-number>

where  <port-number>   is  the   port  that   has  failed.   This

information that this provides will  be most helpful in trying to

diagnose what has happened.

Updates

-------

By  registering the  product  with Technique  Computer,  you will

receive the most recent version by mail (please allow 6-8 weeks),

and be able  to download the  latest version of  it from our BBS.

Once  you are  registered, you  can  download future  versions of

DIGIFOSSIL from the Plywood PC BBS.

Credits

-------

I have a few people to thank.  Firstly, I would like to thank Ron

Green, and Ken O'Connor for their patience, and for letting me at

their Wildcat!  IM baby in  order to get  DIGIFOSSIL working, and

also to Ron Green for saying "but we can't run Scrabble now we've

gone to Wildcat! IM",  and to Chip North (and his co-sysop), for

actually  proving that it works with more than Scrabble.

- 4 -

DigiFossil (c) Copyright 1992, Technique Computer Systems

History

-------

Version 1.1 - Jan 11/1993

- Added support for forcing Locked DTEs.

- Added beg screen to unregistered version.

(lack of registrations caused this)

- Fixed minor typos in the orderform.

- Added better support for detecting errors during

installation.

Version 1.0 - Dec 25/1992

- Initial Release

Legal Stuff

-----------

DIGIFOSSIL  is Copyright  1992-1993, Technique  Computer Systems,

all rights  reserved.  This  product is  provided "AS-IS" without

warranty of any kind.  All risk as to the results and performance

are  assumed  by you.   Technique  Computer Systems  will  not be

liable for  any damages, direct  or indirect,  resulting from the

use, or inability to use, this product.

Wildcat! isn't actually a trademark of Mustang Software Inc (but

is a very good BBS package)

DigiBoard could be a trademark of DigiBoard

Fossil (Fido(tm)/Opus/Seadog(tm) interface layer), is the

combined product and ideas of Tom Jennings, Thom

Henderson, Vince Perriello, & Wynn Wagner (& many

more).

Fido is a trade mark of Tom Jennings

Seadog is a trade mark of Thom Henderson and/or System

Enhancements Associates.

QEMM and Desqview are trade marks of Quarterdeck Office Systems.

- 5 -

DigiFossil (c) Copyright 1992, Technique Computer Systems