Included in this file are the TSLCDR.SYS SCSI driver for PAS Cards, and the

file MSCDEX.EXE version 2.21.

Please note if you are using DOS 6.0, then the MSCDEX that you need to use is

located in your DOS directory. ( ver 2.22 )

Copy both file to the directory where your sound drivers are located.

To install these drivers, add to your config.sys the following as the last

line (make sure it is somewhere AFTER the MVSOUND.SYS line):

Device=c:\<path to sound drivers>\TSLCDR.SYS /D:MVCD001

For example, the default for a PAS 16 card would look like this:

Device=C;\proaudio\TSLCDR.SYS /D:MVCD001

To your autoexec.bat file add the following line as the very first line:

C:\<path to sound drivers>\MSCDEX.EXE /D:MVCD001 /M:10

For example:

C:\proaudio\MSCDEX.EXE /D:MVCD001 /M:10

For a DOS 6.0 User:

C:\DOS\MSCDEX.EXE /D:MVCD001 /M:10

Optional Switches:

For TSLCDR.SYS - /R = reset on boot

/P:n (1-3) Seek parameter - not needed

For MSCDEX.EXE - /E - uses expanded memory if available

/L:X - forces the drive letter to the value X (A-Z valid)

/X - displays memory usage information

The /M:XX  parameter is for the number of 2K buffers allocated - increasing

this number increases the amount of memory used by MSCDEX.