Shadowing:

Some adapter cards have a BIOS chip which contain code necessary

for the proper operation of the adapter. For example, VGA cards have

a VGA BIOS chip with video write routines. The average access time for

a ROM chip is 170 Ns. This is considered very slow. The average access

time for motherboard DRAM is 70 or 80 Ns, normally twice as fast as a

ROM chip. AMI BIOS will set aside an area of memory from C000:0000 to

C7FF:FFFF. The video adapter BIOS is copyed into this area. Instead of

accessing the video ROM for video BIOS calls, the system will access

section of memory to get the proper code. This greatly improves video

speed. The process of copying video ROM to RAM for faster access is

called VIDEO BIOS SHADOWING. AMI BIOS will copy itself to another

section of RAM from F000:0000 to FFFF:FFFF. This is called SYSTEM ROM

BIOS SHADOWING and greatly enhances system performance. Some older AMI

BIOS and all new BIOS (dated2/2/92 and after) will also support shadowing

for other adapters cards which reside at different addresses.

ADAPTER BIOS ADDRESSING:

At any address reserved for adapter BIOS, the first two sections of

the address will contain the numbers '55 AA'. This will be true for any

BIOS. To confirm this, go into DEBUG. Type 'D C000:0' to see the first

block of memory in the VGA BIOS. On the screen, the upper-left-most

characters will be '55 AA'. When having trouble with an adapter, check

in this manner to see that the BIOS is functioning properly. If the BIOS

appears okay, check to see that no other peripherals are set to the same

address. If two adapter cards are suspected to be in conflict, remove

all but the video card and drive controller and try the cards one at a

time to see if they function.

RESET XCMOS:

For older AMI BIOS (pre-1991) the contents of Extended CMOS Setup

may be reset to the default values by holding down the INSert key and

turning on the system. An XCMOS Checksum Error will be generated. This

error can be corrected by entering XCMOS Setup, writing CMOS registers

and exit, and rebooting the system. For newer AMI BIOS, the only way to

load default values for XCMOS is to enter CMOS Setup and select LOAD

DEFAULT VALUES from the menu.

AMI PRODUCT PURCHASES:

AMI Corporate Offices does not do any direct sales of BIOS, manuals,

hardware or software products to the end user. BIOS and manuals are

available from Washburn & Co at (716) 248-3627 or Upgrades Etc at

(800) 541-1943. When calling these companies, proper information must be

given in order to determine the correct BIOS needed. If the machine has

AMI BIOS currently, the distributor will need the reference number

located at the bottom of the AMI copyright screen when the system is

booted. If the machine does not have AMI BIOS, the distributor needs to

know what speed the board is and what chipset (e.g. C&T, OPTI, etc.) is

used. If the machine has cache memory, consult the dealer or manufacturer

of the board for a new BIOS that will support the cache. For information

on AMI motherboards or hardware products, contact the AMI Sales

department to find the nearest dealer.

PASSWORD FEATURE:

New AMI BIOS (2/2/91 and later) are equiped with a password feature

which can be set to ask for a password to enter setup or every time the

system is booted. Normally this password is set by the computer dealer

before the user gets the system. Sometimes users may assemble thier own

system or CMOS information may be lost. In these cases, the following

information is necessary. When the password feature is first enabled,

a CURRENT PASSWORD will not exist. The default password is 'AMI'. In

order to change the password, select the CHANGE PASSWORD option from the

main menu of the CMOS setup routine. The machine will ask for the CURRENT

PASSWORD. 'AMI' must be entered at this point. The new password may then

be entered, then re-entered for confirmation.

CMOS SETUP:

It is a popular misconception among new computer users and people

who are not very involved with hardware that the CMOS is a part of the

BIOS. The setup program in the BIOS is a very basic interface program

which only allows the user to put information into the CMOS memory and

XCMOS registers. The screen for standard setup should be basically self-

explanatory. Drive types, monitor types, time and date should all be

easy to enter for the user. The settings in the extended setup really

have to do with the chipset on the motherboard. Questions on XCMOS should

be directed to chipset or motherboard manufacturers for BIOS dated pre-

1991. Any BIOS dated 1991 is in what is called the HI-FLEX format. A

HI-FLEX BIOS User's Guide is available to help the user understand the

screens. An appendix is also available to help explain the options in

the advanced setup so that the user can pick the options best suited

for his/her environment. The best place to get a good explaination of the

chipset options is of course the chipset manufacturer.

CUSTOMIZATIONS:

Customized BIOS is available from Washburn & Co or Upgrades Etc.

Examples of customizations are personalized copyright screens, changes

to the drive table, and having some options absent from the setup menu.

Some OEM companies have licensed AMI BIOS and are allowed to

customize the main code for use with thier motherboards. Some examples

are Everex and Mylex. The motherboard manufacturer must be consulted for

a BIOS upgrade in these cases. Standard AMI BIOS will not work because

it would not contain the proprietary changes made by the OEM.Megabytes

A Megabyte is 1024 Kilobytes, which is 1024 bytes; thus 1 Megabyte is

1024 * 1024 BYTES. 1 Meg actually equals 1,048,576 bytes. Most drive

manufacturers specify the drive capacity in millions of bytes and call it

Megabytes, possibly to make the drive sound larger than it is. AMI BIOS

calculates drive capacity in Megabytes using the follwing calculation:

Millions of Bytes    =    CYL * HEADS * SEC/TRACK * 512 (BYTES PER SECTOR)

Megabytes             =    MILLIONS OF BYTES / 1024 / 1024

The following example uses a Seagate ST-251:

820 * 6 * 17 * 512 = 42,823,680 million bytes

42,823,680 / 1024 / 1024 = 40.83 MEGABYTES

In the drive specs provided by Seagate, the ST-251 is listed as a

42 Megabytes drive. The average end-user will become confused when he/she

sees the AMI BIOS calculate 40.8 Megabytes.

Maximum parameters supported by DOS are 1024 cylinders, 64 heads, and

64 sectors per track. Exceptions are EDSI and SCSI, which have BIOS on the

controllers for translation. Also, some IDE drives do their own translation.

ESDI drives are ALWAYS drive type 1 when using the controller

translation, normally type 47 when not using controller translation.

SCSI drives are specified as NOT INSTALLED.

IDE drives don't adhere to the AT bus standards. A provision was made

in the 4/9/90 release of AMI BIOS to be able to use IDE drives. (Some of

the read routines were changed. All other info is proprietary)

Drive type 47 (user defined) was put into the BIOS in late 1989 and

must have the ScratchRAM option set to 2 (to protect info).

ERROR NO ROM BASIC can be caused by not having an active partition. The

error means the computer can't find anything to boot from.

In the new HI-FLEX BIOS, most boards will have the option to shadow

the C000 area. Some boards will have the option to cache this area. This

makes for extremely fast video times.

C000 cacheing also has one drawback. The caching is done in the 486

8K internal cache. This cache cannot be write-protected. Whenever a BIOS

test is done with AMIDIAG, the program sees that there is a BIOS present.

The program has no knowledge of the Video ROM Cache and will treat the code

as being a normal shadowed BIOS. The program will see that this BIOS is

NOT Write-Protected. A NON WRITE-PROTECTED BIOS is viewed as an error

condition and the BIOS ROM TEST will fail.   Currently, there is no work-around

to

this problem. The BIOS test must be performed with the C000 ROM CACHE

option disabled.