CLEANING AND RESPOOLING INSTRUCTIONS

Rev 8/93

Now that you realize the necessity of your backups, it's time to

talk about the care and cleaning of your tape backup system.

Periodic cleaning of the end-of-tape sensor and read/write head

will help to keep your tape drive operating at its peak.  These two

procedures are the only routine maintenance you need to perform.

CLEANING THE END OF TAPE SENSOR and READ/WRITE HEAD

When to Clean

1.   After every 8 hours of tape operation, approximately 20

backups or restores.

2.   Or, when dust accumulates.

3.   Or, if a tape comes unspooled.  The end of tape is detected by

light projected through a series of holes towards the end of

the tape.  Dust, dirt or smoke blocking this light can cause

the tape to unspool, or wind to one side of the cartridge.

Although the tape can look as if it is damaged beyond repair,

most tapes can be respooled and often the data will still be

intact.  Follow the instructions to respool your tape.

4.   Or, when errors occur.  Residue buildup on the read/write head

can cause the message:  Caution - excessive read errors. Clean

read/write head -or- Replace tape cartridge to display.

Supplies You Will Need

1.   Aerosol can of compressed air - purchased at an office supply

or computer store.  CAUTION:  Do not use shop air or air from

an oil-lubricated compressor.

2.   Sponge-tipped swabs

3.   90% isopropyl alcohol.   CAUTION:  Do not use cotton swabs as

they may leave behind fibers that can amage the read/write

head.  Do not use rubbing alcohol or any abrasive to clean the

head or apply a lubricant to any part of the drive.

To Clean the End-of-Tape Sensor and Read/Write Head

1.   Turn off the computer.

2.   Hold open the tape drive door.

3.   Note the location of the sensor and read/write head.  They are

approximately 2 inches back from the door.

4.   Use the can of compressed air to blow the dust away from the

two areas.

or

Dip a sponge-tipped swab in 90% isopropyl alcohol and rub the

swab back and forth over the sensor and read/write head in the

direction of tape motion several times until it is shiny.

5.   Let the read/write head dry completely before using the tape

drive.

RESPOOLING A TAPE CARTRIDGE

Your investment in tapes for your backup system is appreciated!  To

retrieve tapes that have come unspooled, follow the outlined

procedure that follows.  PLEASE READ THOROUGHLY BEFORE ATTEMPTING

THIS PROCEDURE!

1.   Remove the two or three screws from the cartridge base plate.

2.   Place the cartridge on a flat surface in a clean area.  While

firmly holding the metal base plate with one hand, carefully

remove the cover.  It may be helpful to hold the door of the

tape down as it remains with the base plate.

3.   Locate and carefully free the loose end of the tape.  Wind all

but approximately 3-4 inches of tape on the full spool.

4.   Thread the tape through its designated tape path using a fully

assembled cartridge as a model.  The tape must be routed

around the tape guides, in the front of the tape capstan, and

in front of the guide pin next to the drive capstan.  Be

certain that hands are clean.  CAUTION:  DO NOT allow any of

the cartridge components to lose their position as they can be

difficult to reposition properly.

5.   Moisten the backside of the tape with freon or water and

attach to the empty tape hub.  Turn the capstan a couple of

turns counter-clockwise until the tape is secure on the empty

tape hub.  Make sure the tape clings to the tape hub rather

than traveling toward the tape capstan.

On some tapes it may be necessary to hold a finger on the top

of the tape hub to prevent the tape from riding up on the hub.

6.   Hold one tape hub securely and turn the other slightly to

remove the slack in the tape.

7.   Use a pointed object, such as a pen, to turn the belt capstan

counter-clockwise and watch for a set of two holes in the

tape.  NOTE:  Some tapes may unspool at the end of the tape

rather than the beginning, making it necessary to turn the

belt capstan in a clockwise direction.  The first set of holes

should be about 12 inches (about 8 turns of the capstan) into

the tape.

Another 12 inches should reveal a second set of two holes and,

finally, a single hole should appear approximately 36 inches

(23 turns of the capstan) from the end of the tape.  The tape

is wound sufficiently when this single hole becomes visible.

8.   Make sure that the mirror is pushed all the way down on its

mounting posts then carefully replace the cover on the base

plate.  Insert the screws in the base plate to complete the

assembly.

To get a diagram showing the parts listed in the above

instructions, please call our QICFAX service and request document

#2020.