The Universal VESA TSR

----------------------

Copyright (C) 1993 Kendall Bennett

All rights reserved.

What is it?

-----------

The Universal VESA TSR is a small Terminate and Stay Resident (TSR) program

that provides extends the Video BIOS routines of SuperVGA video cards

to make them compatible with the Video Electonics Standards Association

(VESA) recommended extensions.

By having a video card that has a VESA compatible BIOS, or by installing

this small TSR, programs that support the VESA programming interface

will work correctly for your SuperVGA. This helps to solve a lot of

problems with compatability between different SuperVGA cards on the

market.

Why use it?

-----------

You may already have a VESA TSR for you video card, or you video card

may have a VESA compatible BIOS already, so why would you want to use

this program?

The simple answer is that most of the TSR's that exist today, and a lot

of the not-so-new SuperVGA cards that are around implement an older version

of the VESA programming interface (if any at all). For this reason, some

programs that use the advanced features of the new VESA standard will

not work with the TSR or BIOS that you currently have.

So what advanced features does this TSR provide that other's don't? The

Universal VESA TSR implements the VESA Version 1.2 programming interface,

which supports the following features:

- VESA page flipping. High performance animation programs can

use your card to full potential using the VESA BIOS to implement

extended page flipping (imagine, 1024x768x16 double buffered

animation, or even 800x600x256 and 640x400x32k on a 1Mb video

card!!).

- Support for the 32k and 16 million color video modes. As well

as supporting the industry standard 16 and 256 color video

modes.

Hardware Requirements

---------------------

The Universal VESA TSR contains '386 dependant code, so it will only

run on machine that have and 80386 or better CPU. Currently the

Universal VESA TSR supports the following SuperVGA chipsets (note that

not all of these have been fully tested):

- ATI Technologies 18800, 28800

- Ahead A & B

- Chips & Technologies 82c451/452/453

- Everex

- Genoa Systems GVGA

- OAK Technologies OTI-037C, OTI-067, OTI-077

- Paradise PVGA1A, WD90C00/10/11/30/31

- NCR 77C20/21/22E

- Trident 88/8900

- Video7 V7VGA versions 1-5

- Tseng Labs ET3000, ET4000

- AcuMos AVGA2

- S3 86c911/924/801/806/928

- Advance Logic AL2101 SuperVGA

- MXIC 86010 SuperVGA

- Primus 2000 SuperVGA

- RealTek 3106 SuperVGA

- Cirrus CL-GD 5422 SuperVGA

The Universal VESA TSR seems to have problems with QEMM, since QEMM does

something weird with the Video BIOS. In order to fix this problem,

you should use the QEMM 'exclude' options to exclude the Video BIOS

area. Do this by including the following in the QEMM options:

EXCLUDE=C000-C1FF

How do I install it?

--------------------

You can install the TSR from the command line, or from your autoexec.bat

file. Don't worry about installing it twice, it will automatically detect

if you have already installed it.

To install it simply type:

univesa

from the command line. It will determine what type of SuperVGA card you

have installed and install itself accordingly.

When installed the Universal VESA TSR takes up approximately 7k of memory.

All of the code required to detect the video adapter and available video

modes is removed after the program is installed, saving on the amount

of memory required. You may also load the TSR high.

Can I get the source code?

--------------------------

Yes, the full source code for this library is available, along with a

number of test programs (with source) for programming SuperVGA cards

in a device independant manner in a SuperVGA Test Kit Library. You can

probably download this from Simtel-20 or garbo (or one of it's mirrors)

or you can ftp the absolute latest version from:

godzilla.cgl.rmit.oz.au: kjb/MGL/svgakt??.zip

How can I contact the Author?

-----------------------------

If you have any corrections or updates to the code in this package, or

you have any questions you would like asked, feel free to contact me

through one of the following methods.

Internet:   kjb@citri.edu.au

Snail Mail:

Home:       Kendall Bennett

15 Stevenson Street

Kew Victoria 3101

AUSTRALIA

Work:       Kendall Bennett

RMIT Advanced Computer Graphics Centre

CITRI Building, 723 Swanston Street

Carlton Victoria 3053

AUSTRALIA