From: tici@uni-paderborn.de (Thomas Thissen)

Newsgroups: comp.graphics

Subject: PCX-FORMAT: Here it is !!

Keywords: pcx,paintbrush

Date: 13 Jun 90 13:51:17 GMT

Organization: Uni-GH Paderborn, West Germany

Lines: 540

Technical Reference Manual

Including Information For:

Publisher's Paintbrush~

PC Paintbrush~ Plus

PC Paintbrush

FRIEZE Graphics~

ZSoft Corporation

450 Franklin Rd. Suite 100

Marietta, GA  30067

(404) 428-0008

Copyright 1988 ZSoft Corporation

Table of Contents

Introduction      			3

Image File (.PCX) Format		4

Decoding the .PCX File Format		6

Palette Information Description		7

PC Paintbrush Bitmap Font Format	9

Sample "C" Routines	        	10

FRIEZE Technical Information		13

Pre-7.00 FRIEZE Specifications		14

Pre-7.00 FRIEZE Function Calls		15

Pre-7.00 FRIEZE Error Codes		16

7.00 and later FRIEZE Specifications	17

7.00 and later FRIEZE Function Calls	18

7.00 and later FRIEZE Error Codes	19

The .PCX Programmer's Toolkit		21

Introduction

This booklet was designed to aid developers and users in understanding the technical aspects of the .PCX file format and the use of FRIEZE.

Any comments, questions or suggestions about this booklet should be sent to:

ZSoft Corporation

Technical Support Department

ATTN: Technical Reference Manual

450 Franklin Rd. Suite 100

Marietta, GA  30067

IMAGE FILE (.PCX) FORMAT

The information in this section will be useful if you want to write a program to read or write PCX files (images).  If you want to write a special case program for one particular image format you should be able to produce something that runs twice as fast

as "Load from..." in PC Paintbrush.

Image files used by PC Paintbrush product family and FRIEZE (those with a .PCX extension) begin with a 128 byte header.  Usually you can ignore this header, since your images will all have the same resolution.  If you want to process different resolutions

or colors, you will need to interpret the header correctly.  The remainder of the image file consists of encoded graphic data.  The encoding method is a simple byte oriented run-length technique.  We reserve the right to change this method to improv

e efficiency.  When more than one color plane is stored in the file, each line of the image is stored by color plane (generally ordered red, green, blue, intensity), As shown below.

Scan line 0: 	RRR...

GGG...

BBB...

III...

Scan line 1: 	RRR...

GGG...

BBB...

III...

(etc.)

The encoding method is:

FOR  each  byte,  X,  read from the file

IF the top two bits of X are  1's then

count = 6 lowest bits of X

data = next byte following X

ELSE

count = 1

data = X

Since the overhead this technique requires is, on average,  25% of the non-repeating data and is at least offset whenever  bytes are repeated, the file storage savings are usually considerable.

The format of the file header is shown below.

ZSoft .PCX FILE HEADER FORMAT

Byte	Item	        Size	Description/Comments

0	Manufacturer	1	Constant Flag  10 = ZSoft .PCX

1	Version 	1	Version information:

0 = Version 2.5

2 = Version 2.8 w/palette information

3 = Version 2.8 w/o palette information

5 = Version 3.0

2	Encoding	1	1 = .PCX run length encoding

3	Bits per pixel	1	Number of bits/pixel per plane

4	Window  	8	Picture Dimensions

(Xmin, Ymin) - (Xmax - Ymax)

in pixels, inclusive

12	HRes    	2	Horizontal Resolution of creating device

14	VRes    	2	Vertical Resolution of creating device

16	Colormap	48	Color palette setting, see text

64	Reserved	1

65	NPlanes	        1	Number of color planes

66	Bytes per Line	2	Number of bytes per scan line per

color plane (always even for .PCX files)

68	Palette Info	2	How to interpret palette - 1 = color/BW,

2 = grayscale

70	Filler  	58	blank to fill out 128 byte header

All variables of size 2 are integers.

Decoding .PCX Files

First, find the pixel dimensions of the image by calculating [XSIZE = Xmax - Xmin + 1] and [YSIZE = Ymax - Ymin + 1].  Then calculate how many bytes are required to hold one complete uncompressed scan line:

TotalBytes = NPlanes * BytesPerLine

Note that since there are always an integral number of bytes, there will probably be unused data at the end of each scan line.  TotalBytes shows how much storage must be available to decode each scan line, including any blank area on the right side of the

image.  You can now begin decoding the first scan line - read the first byte of data from the file.  If the top two bits are set, the remaining six bits in the byte show how many times to duplicate the .MDUL/next.MDNM/ byte in the file.  If the top

ts are not set, the first byte is the data itself, with a count of one.

Continue decoding the rest of the line.  Keep a running subtotal of how many bytes are moved and duplicated into the output buffer.  When the subtotal equals TotalBytes, the scan line is complete.  There will always be a decoding break at the end of each

scan line.  But there will not be a decoding break at the end of each plane within each scan line.  When the scan line is completed, there may be extra blank data at the end of each plane within the scan line.  Use the .MDBO/XSIZE.MDNM/ and .MDBO/YSI

M/ values to find where the valid image data is.  If the data is multi-plane BytesPerLine shows where each plane ends within the scan line.

Continue decoding the remainder of the scan lines.  There may be extra scan lines at the bottom of the image, to round to 8 or 16 scan lines.

Palette Information Description

EGA/VGA 16 Color Palette Information

The palette information is stored in one of two different formats.  In standard RGB format (IBM EGA, IBM VGA) the data is stored as 16 triples.  Each triple is a 3 byte quantity of Red, Green, Blue values.  The values can range from 0-255 so some interpre

tation into the base card format is necessary.  On an IBM EGA, for example, there are 4 possible levels of RGB for each color.  Since 256/4 = 64, the following is a list of the settings and levels:

Setting		Level

0-63		0

64-127		1

128-192		2

193-254		3

VGA 256 Color Palette Information

ZSoft has recently added the capability to store palettes containing more than 16 colors in the .PCX image file.  The 256 color palette is formatted and treated the same as the 16 color palette, except that it is substantially longer.  The palette (number

of colors x 3 bytes in length) is appended to the end of the .PCX file, and is preceded by a 12 decimal.  To determine the VGA BIOS palette you need only divide the values read in the palette by 4.

To access a 256 color palette:

First, check the version number in the header, if it contains a 5 there is a palette.

Second, read to the end of the file and count back 769 bytes.  The value you find should be a 12 decimal, showing the presence of a 256 color palette.

CGA Color Palette Information

For a standard IBM CGA board, the palette settings are a bit more complex.  Only the first byte of the triple is used.  The first triple has a valid first byte which represents the background color.  To find the background, take the (unsigned) byte value

and divide by 16.  This will give a result between 0-15, hence the background color.  The second triple has a valid first byte, which represents the foreground palette.  PC Paintbrush supports 8 possible CGA palettes, so when the foreground setting i

ded between 0 and 255, there are 8 ranges of numbers and the divisor is 32.

.MDUL/

CGA Color Map.MDNM/

Header Byte #16

Background color is determined in the upper four bits.

Header Byte #19

Only upper 3 bits are used, lower 5 bits are ignored.  The first three bits that are used are ordered C, P, I.  These bits are interpreted as follows:

c: color burst enable - 0 = color; 1 = monochrome

p: palette - 0 = yellow; 1 = white

i: intensity - 0 = dim; 1 = bright

PC Paintbrush Bitmap Character Format

The bitmap character fonts are stored in a particularly simple format.  The format of these characters is as follows:

Header (2 bytes)

font width	db	0a0h + character width (in dots)

font height	db	character height (in dots)

Character Widths (256 bytes)

char widths	db	256 dup(each char's width +1)

Character Images

(remainder of the file)

The characters are stored in ASCII order and as many as 256 may be provided.  Each character is left justified in the character block, all characters take up the same number of bytes.

Bytes are organized as N strings, where each string is one scan line of the character.  See figure 2.

For example, each character in a 5x7 font requires 7 bytes.  A 9x14 font uses 28 bytes per character (stored two bytes per scan line in 14 sets of 2 byte packets).  Custom fonts may be any size up to the current maximum of 10K bytes allowed for a font fil

e.

Sample "C" Routines

The following is a simple set of C subroutines to read data from a .PCX file.

/* This procedure reads one encoded block from the image file and stores a count and data byte. Result:

0 = valid data stored

EOF = out of data in file */

encget(pbyt, pcnt, fid)

int *pbyt;     /* where to place data */

int *pcnt;     /* where to place count */

FILE *fid;     /* image file handle */

{

int i;

*pcnt = 1;     /* safety play */

if(EOF    ==    (i    =    getc(fid))) return(EOF);

if(0xc0 == (0xc0 & i))   {

*pcnt = 0x3f&i;

if(EOF == (i=getc(fid)))

return(EOF);

}

*pbyt = i;

return(0);

}

/* Here's a program fragment using encget.   This reads an entire file and stores it in a (large) buffer, pointed to by the variable "bufr". "fp" is the file pointer for the image */

while (EOF != encget(&chr, &cnt, fp))

for (i = 0; i ~			*bufr++ = chr;

The following is a set of C subroutines to write data to a .PCX file.

/* This subroutine encodes one scanline and writes it to a file */

encLine(inBuff, inLen, fp)

unsigned char *inBuff;  /* pointer to scanline data */

int inLen;			/* length of raw scanline in bytes */

FILE *fp;			/* file to be written to */

{  /* returns number of bytes written into outBuff, 0 if failed */

unsigned char this, last;

int srcIndex, i;

register int total;

register unsigned char runCount; /* max single runlength is 63 */

total = 0;

last = *(inBuff);		runCount = 1;

for (srcIndex = 1; srcIndex  inLen; srcIndex++) {

this = *(++inBuff);

if (this == last)	{

runCount++;	/* it encodes */

if (runCount == 63)	{

if (!(i=encput(last, runCount, fp)))

return(0);

total += i;

runCount = 0;

}

}

else	{   /* this != last */

if (runCount)	{

if (!(i=encput(last, runCount, fp)))

return(0);

total += i;

}

last = this;

runCount = 1;

}

}	/* endloop */

if (runCount)	{		/* finish up */

if (!(i=encput(last, runCount, fp)))

return(0);

return(total + i);

}

return(total);

}

/* subroutine for writing an encoded byte pair

(or single byte  if it doesn't encode) to a file */

encput(byt, cnt, fid) /* returns count of bytes written, 0 if err */

unsigned char byt, cnt;

FILE *fid;

{

if(cnt) {

if( (cnt==1) && (0xc0 != (0xc0&byt)) )	{

if(EOF == putc((int)byt, fid))

return(0); /* disk write error (probably full) */

return(1);

}

else		{

if(EOF == putc((int)0xC0 | cnt, fid))

return(0); 	/* disk write error */

if(EOF == putc((int)byt, fid))

return(0); 	/* disk write error */

return(2);

}

}

return(0);

}

FRIEZE Technical Information

FRIEZE Information

FRIEZE is a memory resident utility that allows you to capture and save graphic images from other programs.  You can then bring these images into PC Paintbrush for editing and enhancement.

FRIEZE was rewritten for use in PC Paintbrush Plus, and so the technical information about FRIEZE has changed dramatically.   To easily provide technical information for all versions of FRIEZE, we have split this section of the manual into two parts, one

about PRE-7.00 versions of FRIEZE, and one about the current versions (7.00 or higher).

FRIEZE 7.10 and later can be removed from memory (this can return you almost 85K of DOS RAM, depending on your configuration).  To do this, you can choose to release FRIEZE from memory in the PCINSTAL menu, or at any time by changing directories to your P

C PAINTBRUSH product directory and typing the word "FRIEZE."

Pre-7.00 FRIEZE Specifications

FRIEZE Print Option Settings

FRIEZE can easily adapt to incomplete printer cables (missing IBM specified status lines) and will drive either serial or parallel devices.  Note that FRIEZE always uses the standard BIOS calls, so a non-handshaking device will time out, but can be told t

o ignore such things as paper out.

The FRIEZE command syntax is:

FRIEZE Xnaarr

Where:

X = either Parallel or Serial

n = port number

aa = a two digit hexadecimal code for which return bits cause an abort

rr = a two digit hexadecimal code for which return bits cause a retry

Examples:

FRIEZE P1 - use the default settings of Parallel output, port number 1, abort mask of 28h, and retry mask of 01h

FRIEZE P2 - use printer port #2

FRIEZE S1 - use serial port #1, and Xon/Xoff handshaking

FRIEZE P10028 - use printer port #1, abort mask of 00 (nothing is read as an error) and retry mask of 28h

Interpreting the codes:

On return from the parallel printer call, the bit interpretations are:

80h - busy signal (0=busy)

40h - acknowledge

20h - out of paper

10h - selected

08h - I/O error

04h - unused

02h - unused

01h - time out

FRIEZE Function Calls

FRIEZE is operated using software interrupt number  10h (the video interrupt call).

To make a FRIEZE function call, load 75 (decimal) into the  AH register, the function call number into the CL register and then, either load AL with the function argument or load ES and BX with a segment and offset which point to the function argument the

n do an int 10h.

FRIEZE will return a result code number in AX--zero means  success, other values show error conditions.  All other registers are unchanged.

No.	Definition 	Arguments

0	Print Window	AL = mode: 0 - character,

1 - normal, 2 - sideways

1	Read Window	ES:BX - string

(filename to read from)

2	Write Window 	ES:BX - string

(filename to write to)

3	Print Width	AL = width in 1/4 inches

4	Print Height	AL = height in 1/4 inches

5	Reserved

6	Set Left Margin	AL = printout margin in

1/4 inches

7	Set Window Size	ES:BX - 4 element word

vector of window settings:

Xmin, Ymin, Xmax, Ymax

8	Reserved

9	Set Patterns	ES:BX - 16 element vector

of byte values containing the

screen-to-printer color

correspondence

10	Get Patterns	ES:BX - room for 16 bytes as

above

11	Set Mode	AL = mode number

(See SETMODE command)

12	Reserved

13	Reserved

14	Reserved

15	Get Window	ES:BX - room for 4 words of

the current window settings

16 	Set Print Options	ES:BX - character string of

printer options.  Same format

as for the FRIEZE command.

17	Initialize	ES:BX - 3 word array

containing data from

PC Paintbrush Disk 1 file

CARDS.DAT (Hres, Vres,

optional code number)

All character strings are ended by a zero byte (ASCIIZ format).

FRIEZE Error Codes

When FRIEZE is called using interrupt 10 hex, it will return an error code in the AX register.  A value of zero shows that there was no error.  A nonzero result means there was an error.  These error codes are explained below.

0	No Error

1	Printout was stopped by user with the ESC key

2	Reserved

3	File read error

4	File write error or printer error

5	File not found

6	Invalid Header or can't create file

(not a picture or wrong screen mode)

7	File close error

8	Disk error - usually drive door open

9	Not used

10	Invalid command - CL was set to call a nonexistent

FRIEZE function

11	Not used

12	Not used

7.00 and Later FRIEZE

The newer versions of FRIEZE have a different number of parameters on its command line.  The new FRIEZE command line format is:

FRIEZE {PD} {Xnaarr} {flags} {video} {hres} {vres} {vnum}

Where:

{PD}	Printer driver filename (without the .PDV extension)

{Xnaarr}

X=S for Serial Printer X=P for Parallel Printer

n = port number

aa = Two digit hex code for which return bits cause

an abort

rr = Two digit hex code for which return bits cause

a retry

{flags}	Four digit hex code

First Digit controls Length Flag

Second Digit controls Width Flag

Third Digit controls Mode Flag

Fourth Digit controls BIOS Flag

NOTE:	The length, width and mode flags are printer driver specific.

See PRINTERS.DAT on disk 1 for correct use.  In

general width flag of 1 means wide carriage, and

0 means standard width.  Length flag of 0 and

mode flag of 0 means use standard printer driver

settings.

{video} Video driver combination, where the leading digit

signifies the high level video driver and the rest

signifies the low level video driver

Example = 1EGA - uses DRIVE1 and EGA.DEV

{hres}	Horizontal resolution of the desired graphics mode

{vres}	Vertical resolution of the desired graphics mode

{vnum}	Hardware specific parameter (usually number of color planes)

Note: The last four parameters can be obtained from the CARDS.DAT file, on Disk 1 of your PC Paintbrush diskettes.

Parallel printer return codes:

80h - Busy Signal (0=busy)

40h - Acknowledge

20h - Out of paper

10h - Selected

08h - I/O error

04h - Unused

02h - Unused

01h - Time out

FRIEZE Function Calls

FRIEZE is operated using software interrupt number  10h (the video interrupt call).

To make a FRIEZE function call, load 75 (decimal) into the  AH register, the function number into the CL register and then, either load AL with the function argument or load ES and BX with a segment and offset which point to the function argument then do

an int 10h.

FRIEZE will return a result code number in AX--zero means  success, other values show error conditions.  All other registers are unchanged.

No.	Definition 	Arguments

0	Reserved

1	Load Window	ES:BX - string

(filename to read from)

2	Save Window 	ES:BX - string

(filename to write to)

3	Reserved

4	Reserved

6	Reserved

7	Set Window Size	ES:BX - 4 element word

vector of window settings:

Xmin, Ymin, Xmax, Ymax

8	Reserved

9	Set Patterns	ES:BX - 16 element vector

of byte values containing the

screen-to-printer color

correspondence

10	Get Patterns	ES:BX - room for 16 bytes as

above

11	Set Mode	AL = mode number

(See SETMODE command)

12	Reserved

13	Reserved

14	Reserved

15	Get Window	ES:BX - room for 4 words of

the current window settings

16 	Set Print Options	ES:BX - character string of

printer options.  Same format

as for the FRIEZE command.

17	Reserved

18	Reserved

19	Reserved

20	Get FRIEZE Version.	AH gets the whole number portion

and AL gets the decimal portion of

the version number.  If AH=0, it

can be assumed that it is a

pre-7.00 version of FRIEZE.

21	Set Parameters	ES:BX points to an 8 word table

(16 bytes) of parameter settings:

TopMargin, LeftMargin, HSize,VSize,

Quality/Draft Mode, PrintHres,

PrintVres, Reserved.

Margins and sizes are specified in

hundredths of inches.

Q/D mode parameter values:

0 - draft print mode

1 - quality print mode

2 - use Hres, Vres for output

resolution.  Print resolutions are

specified in DPI.  Any parameter

which should be left unchanged may

be filled with a (-1) (0FFFF hex).

The reserved setting should be filled

with a (-1).

22	Get Parameters	ES:BX points to an 8 word table

(16 bytes) where parameter settings

are held.

23	Get Printer Res	ES:BX points to a 12 word table

(24 bytes) where printer resolution

pairs (6 pairs) are held.

NOTE: All character strings are ended by a zero byte

(ASCIIZ format).

FRIEZE Error Codes

When FRIEZE is called using interrupt 10 hex, it will return an error code in the AX register.  A value of zero shows that there was no error.  A nonzero result means there was an error.  These error codes are explained below.

0	No Error

1	Printout was stopped by user with the ESC key

2	Reserved

3	File read error

4	File write error

5	File not found

6	Invalid Header - not an image, wrong screen mode

7	File close error

8	Disk error - usually drive door open

9	Printer error - printer is off or out of paper

10	Invalid command - CL was set to call a nonexistent

FRIEZE function

11	Can't create file - write protect tab or disk is full

12	Wrong video mode - FRIEZE cannot capture text screens.

--

tici@uni-paderborn.de

�