B A L L I S T I C

Version  3.20

(C) Copyright 1988 by William R. Frenchu

DISCLAIMER

Reloading  small arms ammunition can be a DANGEROUS pastime.  The

purpose of this program is NOT to teach the fundamentals of  this

fascinating hobby but rather to aid the hobbyist in assessing the

performance of the ammunition he has produced. It is assumed from

the outset that the user of this program is acquainted with basic

reloading  techniques. For review, a list of  references  wherein

may be found the necessary instruction is included in Appendix  B

of this document. DO NOT ATTEMPT TO PRODUCE AMMUNITION FOR  YOUR-

SELF  OR  OTHERS UNTIL YOU ARE COMPLETELY FAMILIAR WITH  ALL  THE

FUNDAMENTALS OF RELOADING. ALWAYS START WITH THE MINIMUM LOAD AND

WORK SLOWLY UP TO (NEVER EXCEED) THE MAXIMUM LISTED LOAD.

Although  every reasonable care was taken to insure the  accuracy

of this program, it is provided "AS IS" and without warranties as

to performance of merchantability or any other warranties whether

expressed  or implied, including, but not limited to warranty  of

fitness for a particular purpose. Good data processing  procedure

dictates that any program be tested with non-critical data before

relying on it. The user must assume the entire risk of using  the

program. The author will assume NO responsibility for any  damage

or injury that may occur.

User Supported Software

This  program may be copied and freely distributed for  any  non-

commercial use provided all accompanying files and  documentation

are  included  WITHOUT MODIFICATION. See Appendix E  for  further

information (and the usual pitch for cash).

TABLE OF CONTENTS

-----------------

1. About the Program ....................................... 1

A. General ............................................... 1

B. Program files ......................................... 1

C. System requirements ................................... 1

D. Program limitations ................................... 1

E. Notation .............................................. 2

F. What's new ............................................ 2

G. The future ............................................ 2

H. Acknowledgments ....................................... 3

2. Using the Program ....................................... 4

A. Starting the program .................................. 4

B. Command line arguments ................................ 4

C. Calculating the ballistic coefficient ................. 5

D. The ballistic table ................................... 5

E. Sight (trajectory) tables ............................. 5

F. Plotting the trajectory ............................... 6

G. Modifying the ballistic coefficient ................... 7

H. Changing the cross wind value ......................... 7

I. Changing the input function table ..................... 7

J. Miscellaneous functions menu .......................... 7

K. Calculating recoil .................................... 8

L. Calculating minimum twist ............................. 8

M. Calculating the point blank range ..................... 9

N. DOS shell ............................................. 9

O. Instrumental velocity correction ...................... 9

P. Velocity statistics ................................... 9

Q. Target statistics ..................................... 10

3. The Ballistic Table ..................................... 11

A. The table header ...................................... 11

B. Energy ................................................ 11

C. Momentum .............................................. 11

D. Maximum height ........................................ 12

E. Deflection ............................................ 12

F. Drop .................................................. 12

G. Lead .................................................. 12

H. Time of flight ........................................ 12

I. Trajectory ............................................ 13

4. The Configuration File .................................. 14

A. Using the configuration file .......................... 14

B. Allowable parameters .................................. 14

5. Sources of Data ......................................... 16

A. Altitude .............................................. 16

B. Ballistic coefficient ................................. 16

C. Bullet weight, length and caliber ..................... 16

D. Gun and powder weight ................................. 16

E. Specific gravity of bullet alloys ..................... 17

F. Temperature, atmospheric pressure and wind ............ 17

G. Velocity .............................................. 17

6. Using GENTABLE and READTABLE ............................ 19

A. Using the pre-programmed functions .................... 19

B. Creating custom function tables ....................... 19

C. Checking the function file ............................ 20

Figures .................................................... 21

Appendix A, Glossary ....................................... 26

Appendix B, Information Sources ............................ 29

Appendix C, Coefficients for Several Factory Cartridges .... 30

Appendix D, Specific Gravities of Bullet Alloys ............ 33

Appendix E, User Supported Software ........................ 34

Registration Form .......................................... 35

1.  ABOUT THE PROGRAM

A. General

BALLISTIC  is a Turbo C (v1.5) program written  to  generate

ballistic  tables for shooters. Given certain  easily  obtainable

data,  it  calculates such useful information  as  the  ballistic

coefficient  and sectional density, remaining  velocity,  energy,

momentum,  drop, wind deflection, maximum height, time of  flight

and lead (for a moving target) at a given range. In addition,  it

can  be  used to determine the amount of recoil  generated  by  a

cartridge and gun, estimate the minimum twist necessary for  bul-

let  stabilization, calculate point blank ranges generate  target

and  velocity statistics, and produce trajectory tables for  est-

imating "holdover". Finally, BALLISTIC will produce a plot of the

bullet's  trajectory versus range. It does all this by  comparing

the  bullet in question to a "standard bullet" with a well  known

trajectory.  As this data is calculated before use by a  separate

program (GENTABLE), the user's ballistic tables are produced very

quickly.

B. Program files

The  following files should be included in any  distribution

of BALLISTIC:

BAL-GEN.EXE  -GENTABLE  :Creates ballistic function tables.

BAL-READ.EXE -READTABLE :Optionally checks function table.

BAL-READ.ME! -Describes version, etc.

BALISTIC.EXE -BALLISTIC :Executable file.

BALISTIC.DOC -This file.

BALISTIC.REV -BALLISTIC Revision history.

C. System Requirements

BALLISTIC  should run on most IBM BIOS-compatible  computers

using PC/MS-DOS 3.0 or above. It requires 256k of free memory and

at  least one disk drive.  Non-graphic functions are directed  to

the  default  display. Graphic output  (for  Shareware  versions)

requires  a  VGA, MCGA, EGA, CGA or Hercules  compatible  display

card. Registered versions can be supplied that support the higher

resolution modes of the AT&T (400 line), Toshiba 3100, IBM  8514,

PC-3270 or Everex EGA+.

BALLISTIC  has been tested and found to operate properly  on

the following systems:

PC/XT with Hercules monochrome or ATI "EGA Wonder" adapter

PC/AT with CGA or EGA adapter

PS/2 System 30 with VGA adapter

NEC MultiSpeed Portable

BIOS Model G (V40 CPU)

AT&T 6300

Compaq Plus with Compaq CGA-type adapter

Compaq DeskPro 386 with Compaq EGA-type adapter

D. Program limitations

Any  program of this type will have certain limitations  re-

garding its use and accuracy and BALLISTIC is no exception. These

discrepancies  between  actual and calculated  performance  arise

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 1

primarily  because of the inability of the user to measure  input

values exactly and certain approximations the program uses in its

calculations. Assuming that the input values are known to 1%  the

average user should find agreement with published data to be 1-2%

for all velocity and trajectory calculations. The recoil section,

due to the more approximate nature of the calculation,  shouldn't

be  considered more than 10-15% accurate. In any case, while  the

numbers  produced  by  the program are believed  to  be  accurate

enough for all practical purposes, they are meant only as a guide

to ammunition performance. How a particular load performs with  a

given gun can ONLY be determined by actual firings.

While  Ingalls'  ballistic table is  historically  the  most

commonly used for this sort of work, several others are likely to

be encountered. (Many manufacturers, for instance, favor the "G1"

table.) For the types of bullets most often used by reloaders the

results  will be very similar regardless of the table  used.  In-

deed, the variation between handloads or factory ammunition  ran-

domly  selected from the same lot will probably be  greater  than

the  discrepancies  noted between different  tables  over  normal

shooting ranges. For best results, one should try to use the same

input  function to produce the table that was initially  used  to

calculate  the ballistic coefficient. However, this isn't  always

possible,  as in the case where a published coefficient was  cal-

culated using a table to which the program doesn't have access.

E. Notation

Throughout  this document, keys to be pushed in response  to

program  prompts  are enclosed in angle  brackets.  For  example,

"press  <1>" means to press number 1 on the keyboard. For  "press

<ENTER>",  use the key labeled either "return" or  "enter".  "En-

tering" a value will require that the <ENTER> key be pressed when

finished, i.e., to "enter 10" press <1><0><ENTER>.

F. What's New

There have been several additions to the program since  ver-

sion 3.10. Some of these include:

1. Added sectional density calculation to table.

2. Added  calculation for correction of instrumental  velocity

to true muzzle velocity.

3. Added  calculation of velocity statistics, including  mean,

extreme spread, and standard deviation.

4. Added  calculation  of target statistics,  including  group

center, group size and indication of "true flyers".

5. Added Epson FX80 support for graphics screen dump.

6. Fixed bug in DOS Shell command.

7. Added helix angle calculation to twist section.

8. Added multiple plot save and restore.

G. The Future

BALLISTIC  isn't perfect (or even necessarily finished).  It

is  hoped, for instance, that future releases will incorporate  a

"Powley  Computer"  type computation for  estimating  the  powder

charge necessary for a given velocity. The ability to make calcu-

lations based on distances given in meters or yards would also be

a  welcome improvement, as would an on-line database relating  to

commercially loaded cartridges. Additional features the author is

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 2

considering include: context sensitive help, reading/writing load

data  to  dBase III or Lotus 123/Symphony compatible  files,  and

plotting  of  functions other than trajectory  (i.e.,  energy  or

momentum vs. range).

The  author is interested in adding new function  tables  if

reliable data can be found. He is currently on the lookout for  a

copy  of BRITISH TEXTBOOK OF SMALLARMS or listings of the "J"  or

"G5" tables. Also of interest is additional specific gravity data

for  commonly used bullet casting alloys, and information on  the

types  of materials being used in the "new" non-lead  bullet  de-

signs (i.e., bronze) AND the "old" ones (steel). All things  will

come  to  those users who wait (and support  BALLISTIC  with  the

voluntary contribution!)

All in all, BALLISTIC seems likely to fill the needs of many

shooting  buffs.  Included  in the  documentation  is  a  section

listing sources for further reading on small arms ballistics  and

reloading. The local gun shop or public library may have  others.

READ some of them!.. AND the disclaimers. Then enjoy the program.

H. Acknowledgments

The  author  is indebted to the following people  for  their

help, support, and the many suggestions they made. Without  dedi-

cated  users such as these, the program would never have  reached

its current level of sophistication:

Bart Carfizzi

Hugh Roberts

Jim Kreiser

James Roberts

Donald Terwilliger

Dave Zarodnansky

Special thanks, of course, are due to the author's wife  and

family,  Susan, Billy and Karen, for understanding (or  at  least

putting  up  with!) the author's wild mood  swings,  ranting  and

raving  as  various parts of BALLISTIC (which were  working  just

fine  yesterday!) stopped working  for no apparent reason.  Thank

you.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 3

2.  USING THE PROGRAM

The goal in writing BALLISTIC was to produce a program which

was at once versatile and relatively easy to use. While there  is

still room for improvement, some success in both of these  direc-

tions has been achieved. The program is primarily menu driven and

prompts  whenever input data is required. Additionally, data  en-

tered  (or calculated) in one section of the program need not  be

re-entered  unless the user wants to change the value.  Data  the

program  knows about are displayed during the prompts. To  accept

the displayed value press <ENTER>. To change it, simply type  the

new  data over the old, then press <ENTER>. (The table title  and

uphill/downhill  shooting  angle must ALWAYS  be  entered  expli-

citly.)  To  correct typing errors on the current line,  use  the

<BACKSPACE>  key, NOT the <DELETE> key. Improper entries on  pre-

vious lines can be corrected by using the <ESC> key to "back  up"

to the bad line, then retyping the data. In most cases, obviously

improper data will be refused and an acceptable range  displayed.

Input  filenames  will be checked for existence before  they  are

accepted.

A. Running the Program for the First Time

Prior  to  running BALLISTIC for the first  time,  the  file

containing  the  function table required by the program  must  be

created using GENTABLE (v3.00 or above, see Section 6). This file

will  contain the version number of the GENTABLE used to  produce

it,  the  function name, some atmospheric data, the  maximum  and

minimum velocity and S and T value pairs. Once this file has been

created (and optionally checked using READTABLE) BALLISTIC can be

invoked.  Only BALISTIC.EXE and a function file need actually  be

present on the disk when running the program.

B. Command Line Arguments

BALLISTIC is started by simply typing its filename (BALISTIC)

at  the DOS prompt, optionally following it with one or  both  of

the  following arguments. "-BIOS" will cause all video output  to

be  done using BIOS calls rather than direct screen writes.  This

will significantly slow screen I/O, but may be necessary in cases

where the computer being used isn't 100% IBM compatible, or  when

the program is being operated remotely. The second argument, "-F"

followed  (with  NO spaces) by the name of a  BALLISTIC  function

file, causes the named function file to be loaded instead of  the

default file BALISTIC.TAB.

BALLISTIC will first attempt to read the configuration  file

(BALISTIC.CFG, see Section 4), then search for the default  func-

tion  file  (either BALISTIC.TAB or the  command  line  specified

file),  prompting the user for the name of an alternate  if  it's

not  found.  The default file type is ".TAB" which  need  not  be

entered. (To enter a filename with no extension enter "filename."

with the trailing period.) BALLISTIC will next read the  function

file,  make a check to determine its validity (a simple  checksum

procedure), and present the user with the main menu. (Fig. 1)

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 4

C. Calculating a Ballistic Coefficient

The first step in developing a ballistic table is to  deter-

mine  the  ballistic coefficient. If this value cannot  be  found

explicitly  in one of the sources described in section  5B,  main

menu  choice  2 may be used to calculate it. Press  <2>  and  the

program will prompt for an initial and final velocity, the  range

and some atmospheric data. After this has been entered, the  cal-

culated value of the effective ballistic coefficient (as well  as

the  coefficient at standard conditions for the function  chosen)

will be displayed on the screen (Fig. 2). Having found the  coef-

ficient, BALLISTIC can now calculate the table. Press the <ENTER>

key  to  redisplay the main menu then <1> to bring up  the  input

section.

D. The Ballistic Table

Figure  3 shows the input data for main menu choice 1,  cal-

culating  the ballistic table. Since the initial velocity,  range

and  ballistic coefficient have been entered or calculated  on  a

prior  screen, these values needn't be re-entered unless  a  dif-

ferent value is required. In this case, entering a range value of

500  will  create a table to 500 yards based on  the  performance

noted over the first 100. Entering a value of zero for the  range

will cause the program to switch to metallic silhouette mode. The

user will then be prompted for his choice of standard  silhouette

rifle  or pistol ranges rounded to the nearest yard. As  long  as

the  input  value for range remains zero these  alternate  ranges

will be used. The bullet diameter prompt should be answered  with

the  actual  diameter in inches although input of  the  firearm's

nominal caliber (with leading decimal point, i.e., .22) is  often

close  enough.  The final necessary data item is  the  number  of

intervals into which the total range should be divided. The  max-

imum  number allowed is the smaller of 50 or the total range  (in

yards). Data entry is concluded with an optional table title.

Throughout  the  program, entering a value of  0.0  for  the

"effective" ballistic coefficient, will cause the program to  re-

prompt  for the value of the coefficient at STP. The  "effective"

value  will then be calculated from the STP value using the  cur-

rent  values of temperature and pressure. (Temperature and  pres-

sure  may be changed from either main menu choice 3, or the  con-

figuration file.)

Figure 4 shows the table itself and the program's prompt for

generation  of a sight table. At this point, should it be  desir-

able to get a printed copy, press <1>. The program will  redirect

ballistic table output to the system's LST: device and return  to

the  prompt. A form feed (page eject) can be sent to the  printer

by pressing <2>.

The "New Data" choice, <4>, will return the user to the data

entry  screen to calculate another ballistic table,  while  "Main

Menu"  (<5>  or <ESC>) will jump directly back to  the  program's

initial screen.

E. Sight (Trajectory) Tables.

Sight (trajectory) tables can be produced by pushing <3>  in

response to the prompt at the bottom of the ballistic table.  The

program  will  require  that distance for  which  the  weapon  is

"sighted  in"  (this value need NOT have appeared  in  the  range

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 5

column of the original ballistic table), the height of the  sight

above the centerline of the bore, and any angular correction  for

uphill or downhill shooting. In silhouette mode, entering a value

of zero for the targeted range will cause the prompt to take  the

form of which silhouette (chicken, pig, turkey or ram) the weapon

is  zeroed for. The user should respond with the first letter  of

the  appropriate target. Output of this table can be diverted  in

exactly  the  same manner as the ballistic  table.  Pressing  <3>

allows the user to enter new data for another sight table  (using

the current ballistic conditions), and <5> (or <ESC>) returns  to

the ballistic table and prompt.

F. Plotting the Trajectory.

Pressing <4> at the sight table prompt will present the user

with a graphic representation of the bullet's trajectory on  sys-

tems  with an appropriate graphics card. Range, in yards will  be

indicated along the bottom, with tic marks appearing at distances

corresponding to the entries in the ballistic table. Dotted lines

will  appear vertically at the distance for which the  weapon  is

zeroed and horizontally at the line of sight. The trajectory  (in

inches above or below the line of sight) is indicated on the left

scale.  Pressing  <A>  (for axes) will allow the user  to  set  a

"viewport".  The  program  will prompt for  starting  and  ending

range,  and  the maximum and minimum trajectory, then  plot  only

that portion of the trajectory that falls in the described  "win-

dow". Pressing <S> (for start) will reset the plot to the initial

(program chosen) limits. <G> will toggle the appearance of a grid

on the plot and <H> will provide a short help screen.

If  <W> is pressed, trajectory data (for the  current  range

viewport ONLY) will be written to a file. The user will first  be

prompted for a filename and any existing file with that name will

be overwritten. (There is NO default extension.) ONLY the trajec-

tory  plotted with a solid line will be saved to the  file.  This

data may be "merged" with the current trajectory plot at any time

by  pressing <R>. When read back, it will be scaled  and  plotted

using a dotted line to distinguish it from the current plot.  The

number  of  files that can be read back is limited  only  by  the

user's ability to tell them apart on the graphics screen.

Hardcopy  of the plot (actually, a screen dump) may  be  ob-

tained  by  pressing <P> if an appropriate  graphics  printer  is

present.  (Supported printers are discussed in the  configuration

file  section under PTYPE.) Depending on the display adapter  and

printer,  this may require several minutes. (A typical  EGA  gra-

phics  screen dump is about 30k, roughly the size of a  ten  page

document, and takes just as long to print.) Printing may be aban-

doned  at  any time by pressing a key. Users with a  printer  not

listed  may want to try using the <PRINT SCREEN> key  which  will

usually require that the program GRAPHICS.COM (from the DOS  sys-

tem diskette) be run before BALLISTIC is started. Neither  method

will work with all display and printer combinations.  (Registered

users may, of course, contact the author and request support  for

their  particular  printer.) Pressing <X> or the <ESC>  key  will

return the user to the sight table prompt.

From  the sight table press <5> (or <ESC>) to return to  the

ballistic table, (Figure 4 again) then a <5> (or <ESC>) to return

to the main menu.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 6

G. Modifying the Ballistic Coefficient

Those  living at high altitude or in areas of extreme  temp-

erature  will want to adjust published (sea level) values of  the

ballistic  coefficient to more accurate local values.  Main  menu

choice  3 may be used to accomplish this (Figure 5). The  program

will  need to know the altitude, barometric pressure and  temper-

ature  in addition to the value of the ballistic  coefficient  at

STP  (Standard  Temperature and Pressure, the  published  value).

Bear in mind that these meteorological values (especially temper-

ature)  ALSO  affect the initial velocity of a given  load.  This

effect  is  beyond the ability of this program to  calculate  but

descriptions of it may be found in many reloading manuals.

H. Changing the Cross Wind Value and Angle

Menu choice 3 also allows for adjustment of the speed of the

cross  wind  used to calculate wind deflection. By  default,  the

program  sets this value to 10 miles per hour,  blowing  directly

across  the  path of the bullet (0 degrees). It should  be  noted

that  deflection  is a linear function, i.e., a 5 mph  wind  will

deflect a bullet by exactly half the amount of a 10 mph wind. The

angle  of the cross wind may be changed here as well. In  program

coordinates,  0 degrees (the default) is blowing directly  across

the  bullet's  path, either left to right or right  to  left.  90

degrees  would be either a head or tail wind with no  cross  wind

component. These values are used ONLY for calculating deflection.

Their  effect  on other aspects of the  trajectory  is  currently

ignored.

I. Changing the Input Function Table

Starting  with version 2.00, it is possible to switch  input

function tables without restarting the program. Pressing <4> from

the  main menu will cause a prompt for the new file name.   If  a

filename containing wildcard characters ("*" or "?") is  entered,

the  directory  of all matching filenames will be  displayed.  As

noted  in section A, the default table type is ".TAB" which  need

not  be entered. Simply press <ENTER> to abort and return to  the

main menu.

This option doesn't alter the fact that BALISTIC.TAB (or the

filename  supplied  with the "-F" command line  option)  will  be

searched  for (and loaded if present) when the program  is  first

started.  It  is provided only for users who routinely  use  more

than one function table. Even so, program startup will be simpli-

fied if the table most commonly used is copied to BALISTIC.TAB.

It  should  be noted here that loading a new  function  file

will  not reset the temperature and pressure to the defaults  for

the  new table. Thus, some care should be taken if switching  be-

tween two tables that use different standard conditions.

J. Miscellaneous Functions Menu

Pressing  <5> at the main menu prompt will present the  user

with  the miscellaneous functions menu. From this menu  the  user

may  elect to calculate recoil, shot statistics, the minimum  ne-

cessary twist for bullet stabilization, or the point blank  range

for a given target size.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 7

K. Calculating Recoil

The recoil energy and velocity of a weapon can be calculated

using  miscellaneous menu choice 1, provided its weight  and  the

weight of the powder charge are known (Figure 6). This number can

be deceptive as the perception of recoil by different shooters is

VERY subjective. In addition to the physics involved, there  must

be some consideration given to the fit of the gun to the individ-

ual,  shape  and  material  of the  stock  or  grips,  etc.  Many

shooters,  for  instance, fear the recoil of a 12  gauge  shotgun

even with target loads while thinking nothing of using a  6 pound

30-06 despite the fact that the "calculated" recoil can be  quite

similar.  Such  recoil is often in the eye (or shoulder)  of  the

beholder.

This  section also calculates whether a "muzzle brake"  type

device  will  be useful in reducing felt recoil by  allowing  the

user  to determine the rough percentage of recoil caused  by  the

bullet  itself  (which a muzzle brake cannot affect).  The  value

indicated is the level to which recoil energy would be reduced if

there  were no powder at all. This is a crude estimate of what  a

shooter  might achieve with a perfect brake. (Bear in mind,  how-

ever,  that  NO  muzzle brake is 100% effective  and  the  actual

amount of reduction will be less.)

The  recoil  calculation used by BALLISTIC is a  very  rough

approximation, similar to that discussed in the March 1988  Amer-

ican Rifleman. It would be extremely challenging to write a  com-

pletely accurate description of the recoil process. It would also

be  very  difficult  to use as many variables  are  involved  in-

cluding, but not limited to bore cross sectional area, length  of

barrel,  atmospheric  pressure and temperature, type  of  powder,

loading  density, etc. The values calculated here should  not  be

considered  accurate to more than 10-15%. Barrel/powder  combina-

tions that produce excessive muzzle flash will be even less accu-

rate.

L. Calculating Minimum Twist

The slowest twist that will stabilize a bullet in flight may

be calculated using miscellaneous menu choice 2. The formula used

here is the approximation credited to Sir Alfred George Greenhill

in 1879. Necessary input data includes the bullet's  diameter and

length  in  inches and specific gravity. The  default  value  for

specific  gravity  is the average density of a  typical  jacketed

bullet.  Also listed on the input screen (for reference)  is  the

specific gravity of pure lead. Specific gravities (or  densities)

of  other  bullet  alloys seem difficult to find.  It  should  be

pointed out that these specific gravities can NOT be added algeb-

raically. That is, an alloy containing 50% lead and 50% tin  will

NOT  have a specific gravity that is the average of the two  pure

metals.  Appendix  D of this document contains  specific  gravity

data for a small selection of common alloys.

The  angle made by the rifling grooves and the axis  of  the

bore  is  known as the "helix angle". As some  European  manufac-

turers describe the twist using this value rather than the custo-

mary "1 turn in xx inches", BALLISTIC provides both numbers.

It has been pointed out to the author, that this  approxima-

tion was originally intended for use with artillery  projectiles.

More  recent findings indicate that, for typical small  arms  am-

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 8

munition,  a Greenhill number smaller than the default  of  150.0

(100  has  been suggested) may be  appropriate  especially  under

adverse  atmospheric conditions such as rain and snow. A  discus-

sion may be found in the June 1988 issue of Rifle. ("Shooting the

Kalashnikov",  pp.  28 - 31.) This number  may  be  "permanently"

changed using the configuration file.

M. Calculating the Point Blank Range

Given the velocity, sight height and ballistic  coefficient,

a point blank range may be determined for a specific target size.

The point blank range is the maximum range for which a bullet can

be  counted upon to strike a target by simply aiming  the  weapon

directly at it. For example, given a 6 inch target, a point blank

range may be calculated so that the bullet is always less than  3

inches  (above or below) from the line of sight.  BALLISTIC  will

calculate this distance for a given target size and tell the user

for  what range the weapon should be sighted and what  trajectory

the bullet will have at 100 yards. (If the targeted range is less

than  100  yards, the trajectory at 25 yards will  be  given  in-

stead.)  This  figure may be used to sight the  weapon  when  the

targeted range is either unrealistically far away, or an  unusual

number  of yards. For instance, in the example  screen,  sighting

the weapon 2.9 inches high at 100 yards will allow the shooter to

hit  a 6 inch target out to 257 yards, without allowing  for  any

holdover. However, the bullet will only strike the CENTER of  the

target at 219 yards.

It  should  be noted that the point blank  range  is  ALWAYS

calculated  for  an  uphill/downhill angle of  zero  degrees.  As

before,  entering a value of zero for the  "effective"  ballistic

coefficient will re-prompt for the STP value.

N. DOS Shell (Escape to DOS)

Users operating under MS/PC-DOS version 3.00 or higher  have

the  option of shelling out to DOS from the main menu.  The  only

other  requirement  for this option is that there be  a  copy  of

COMMAND.COM  in  the place indicated by the  COMSPEC  environment

variable.

O. Correction from Instrumental to True Muzzle Velocity

When  bullet velocities are measured by chronograph,  it  is

necessary to set the first (or start) screen far enough away from

the  muzzle to avoid blast effects. The second (or  stop)  screen

must  be set a fixed distance from the first so the elapsed  time

between the two can be measured. Unfortunately, the speed  calcu-

lated  under these conditions is NOT the actual muzzle  velocity,

but  rather  the average speed of the bullet  over  the  interval

between  the two screens. While the difference is usually  small,

it  is possible to make the correction to the true  muzzle  velo-

city, provided the screen distances and the ballistic coefficient

are  known. Miscellaneous menu choice 4 provides for this  calcu-

lation.

P. Calculation of Velocity Statistics

Not all bullets from the same lot will leave the barrel of a

gun at exactly the same speed. Some statistical variation is also

to be expected in the measurement of any physical quantity. Anal-

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 9

ysis of a string of measured velocities (up to 25) may be accomp-

lished  using miscellaneous menu choice 5. Output  data  includes

the maximum and minimum velocity values (and extreme spread), the

average  velocity for the string, and the standard  deviation  of

the  sample. BALLISTIC updates these values as each point is  en-

tered, allowing the user to determine the effect of each shot  on

the statistics. Pressing <ESC> at the prompt for "Next  Velocity"

will display the statistics menu, allowing the user to correct or

delete  the entry list. From here, pressing <5> (or  <ESC>)  will

return to the miscellaneous menu.

A  complete discussion of statistics is beyond the scope  of

this document, but it should be noted that standard deviation  of

a  velocity string is a measure of data or sampling scatter,  and

NOT  necessarily an indication of the target accuracy of a  part-

icular  load. (Accurate loads will, however, tend to  have  lower

standard  deviations.) Examination of the standard deviation  can

indicate  whether a particular measurement should be included  in

the average. For example, in a string of 10 shots or less, it  is

usually safe to delete any velocity that deviates by more than  2

standard  deviations from the average as a  statistical  anomaly.

BALLISTIC  will mark each shot that exceeds the  current  average

value by more than one standard deviation (SD). A single star (*)

indicates  difference of more than one but less than two SD.  Two

stars (**) indicate a value more than 2 SD from the mean.

Q. Calculation of Target Statistics

BALLISTIC  will  also provide some statistical  analysis  of

targets, including group center, group size and an indication  of

"true  flyers".  This section is most easily used  when  the  x,y

coordinates  of  each shot can be read directly from one  of  the

"sighting  in" targets, commonly supplied by some firearms  manu-

facturers. (These targets are marked off in small squares  rather

than the concentric circles used for "bullseye" shooting.)  Enter

the  coordinates  of each shot (up to 10 total) and  the  program

will  display  the x,y coordinates of the group center,  and  the

size  of  the  group as measured across the  two  shots  farthest

apart. As with the velocity statistics section, pressing <ESC> in

response to a coordinate prompt will cause the statistics menu to

be displayed.

"Flyers" will be marked in a manner similar to that used for

velocity  statistics. A single star will indicate that  the  shot

exceeds  one  standard deviation from the group center in  the  x

direction, the y direction or both. Two stars indicates a  devia-

tion of more than 2 SD from the current group center.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 10

3.  THE BALLISTIC TABLE

The primary purpose of BALLISTIC is to produce a  "ballistic

table"  showing the remaining velocity at given ranges. The  pro-

gram  is also capable of supplying a wealth of additional  infor-

mation derived from these values. (Refer to Fig. 4)

A. The Table Header

The ballistic table header merely provides a "data echo"  of

the  conditions used to produce the accompanying table. In  addi-

tion to the table title and other user supplied data, two calcul-

ated values are displayed here.

Sectional density of the bullet is defined as the weight  of

the  projectile in pounds, divided by the square of its  diameter

(in  inches). Sectional density can provide the reloader  with  a

handle on two important pieces of information. First, it provides

a measure of how fast a bullet can be safely "pushed".  Certainly

for a given caliber, bullets with a high sectional density should

not be loaded to as high a velocity as bullets with a low  value.

Secondly, a bullet with high sectional density will tend to  have

greater  penetration than a projectile with a smaller  value.  An

excellent  discussion  of sectional density and its  relation  to

hunting  bullets may be found in June 1988 issue of The  American

Rifleman.  ("From the Loading Bench: Sectional  Density  Counts",

page 16)

The IPSC Power Factor is a number used by the  International

Practical  Shooting  Confederation to determine whether  a  comp-

etitor's handgun load is to be scored as "major" or "minor"  cal-

iber.  It can be calculated by multiplying the bullet  weight  in

grains  by  the initial velocity and dividing by 1000.  Any  load

whose  power factor is over 175 is scored as major.  Factors  be-

tween 125 and 175 are scored as minor caliber. Loads with a power

factor of less than 125 are not legal for IPSC competition.

B. Energy

The  kinetic  energy of a moving body is  described  by  1/2

times  the mass of the body multiplied by the square of its  vel-

ocity.  It may be thought of as the amount of work the bullet  is

able to do on its target. The unit (foot-pounds) is the amount of

work necessary to lift an object weighing 1 pound to a height  of

1 foot. Many hunters feel this quantity is a good one to use when

determining the suitability of a given load for a particular game

animal  as (under certain conditions) it is probably  related  to

the "killing power" of a projectile. One cannot rely entirely  on

this simplified picture, however, as terminal ballistics is  also

concerned with bullet and target construction.

C. Momentum

As,  in  any collision of moving bodies,  momentum  is  com-

pletely  conserved,  it may be thought of as  the  "striking"  or

"knockdown"  power  of a projectile. This can  be  slightly  mis-

leading,  however, as the conservation applies only to the  total

"system"  of  particles. If, for example, the  bullet  completely

penetrates the target, it may retain a large portion of its orig-

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 11

inal momentum, transferring to the target only that part it  act-

ually  lost. It is defined as the mass of a body times  its  vel-

ocity.

D. Maximum Height

The  maximum height calculated by BALLISTIC is the  greatest

distance  that a projectile will pass above a straight  reference

line  drawn from the muzzle to its current position at any  given

range. (Many books refer to this number as the maximum ordinate.)

It  is NOT the same as the mid-range trajectory which is  defined

as the bullet's trajectory (above the line of sight) exactly half

way to the indicated range. Typically, the maximum height  occurs

slightly  after the half way point and is very  slightly  larger.

For  average shooting conditions, however, the two values may  be

taken as equal without causing any great error.

E. Deflection

Deflection, as used in BALLISTIC, is the amount of  sideways

motion  which can be attributed to the action of a  crosswind  on

the  projectile.  This  value is directly  proportional  to  wind

speed, that is, if the wind at the site is 5 m.p.h. and the  fig-

ures calculated are for 10 m.p.h., simply divide the table values

by 2. Deflection should not be confused with drift, the  sideways

motion  caused  by the bullet's spin. Deflection is  typically  a

much larger figure. For example, at 1000 yards the drift of a .30

caliber service bullet is only about 7 inches. Compare this value

with  the roughly 175 inches of deflection produced by  the  same

bullet in a 10 mph cross wind.

F. Drop

From the instant a bullet leaves the barrel, gravity and air

resistance conspire to bend its path from that along which it was

originally  projected (the bore line, NOT the line of  sight).  A

bullet's  drop is the distance it has fallen away from that  ini-

tial path, known as the line of departure. This is that  quantity

to  which shooters refer when speaking about the "flatness" of  a

particular loading.

G. Lead

Lead  is  primarily of interest to those who hunt.  By  now,

most experienced hunters will have realized that aiming  directly

at  an  animal running across the path of the  bullet  is  almost

guaranteed  to produce a miss. By the time the bullet has  gotten

to  the  point of aim, the animal is somewhere else.  The  trick,

then, is to know how far ahead of the target one needs to aim  so

that bullet and target arrive in the same place at the same time.

In  BALLISTIC,  the units used for this figure  are  "inches  per

m.p.h. of target speed" and are used as follows. Imagine a target

moving  10 m.p.h. across the path of the bullet at a distance  of

100  yards.  Examining  the table (Fig. 4) produces  a  value  of

2.0 in/m.p.h.  at that distance, so the appropriate  target  lead

would  be (10 m.p.h.) times (2.0 in/m.p.h.) or 20.0 inches.  Wind

speed is NOT taken into account during the calculation of lead.

H. Time of Flight

Time  of flight is simply that; the amount of time, in  sec-

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 12

onds,  necessary  for a bullet to travel the  distance  from  the

muzzle to the indicated range.

I. Trajectory

Loosely  speaking,  trajectory  is simply the  path  of  any

moving  projectile. BALLISTIC provides both tabular  and  graphic

representations  of this path, relative to the weapon's  line  of

sight,  i.e.,  the line drawn directly from  the  sights  (either

"iron" or telescopic) to the target. It should be noted, that the

actual trajectory of a small arms bullet coincides with the  line

of  sight at only two places. Once, a few yards from the  muzzle,

(when  its  motion relative to the line of sight is  upward)  and

again at the targeted (or "sighted in") distance (when it crosses

in a downward direction). Trajectory may be described in  several

ways,  but  the  two most common are  "inches"  and  "minutes  of

angle". When expressed in inches it is the actual distance of the

projectile  above or below the line of sight. This value  may  be

converted to the necessary angular correction (minutes of  angle,

or M.O.A.) by applying the conversion factor of 1 M.O.A. =  1.047

inches/100  yds. This value is particularly useful as most  sight

adjustments are calibrated in minutes of angle. Target scopes are

usually  calibrated at 1/4 minute per "click", while iron  sights

may be as much as 1 minute per click. (In this notation,  minutes

do NOT refer in any way to units of time, but rather to 1/60th of

an angular degree.) BALLISTIC provides both systems in its  traj-

ectory tables.

When  the  uphill/downhill angle is not equal to  zero,  the

bullet will follow a higher trajectory than it would during level

firing.  That is, shooting either uphill or downhill  will  cause

the  bullet to shoot higher than it is aimed. Under these  condi-

tions, the projectile doesn't make a second crossing of the  line

of  sight  until  AFTER the targeted distance.  The  term  "level

firing"  used here may be slightly misleading. Almost  all  small

arms  are sighted so the initial angle of the bullet's  departure

is very slightly up. (This angle is known as the "angle of depar-

ture", and for a typical rifle, is less than one degree.)  Firing

uphill or downhill usually produces a tilt that dwarfs this small

value, so for practical purposes, the uphill/downhill angle  used

in  the  program  may be thought of as the  angle  of  departure,

rounded to the nearest degree.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 13

4.  THE CONFIGURATION FILE

Version  3.20  of BALLISTIC will allow certain  program  de-

faults  to be overridden with the aid of the  configuration  file

BALISTIC.CFG.  This option was originally provided for  users  in

the  CP/M environment. The file is searched for only in the  cur-

rent directory when BALLISTIC is first started and is NOT  neces-

sary for the operation of the program.

A. Using the configuration file

The  configuration file is an ascii text file each  line  of

which  contains the name of the parameter to be changed,  one  or

more  spaces,  and the new value. Parameters listed as  "must  be

integer" may NOT contain a decimal point or exponential notation.

Certain parameters may only accept ON and OFF (or similar  string

choices) as values. Entries are NOT case specific.

The following file would turn off the bell, set the default

altitude to 2000 feet and set the video write mode to BIOS:

BELL OFF

Altitude 2000

video bios

B. Allowable parameters

The following parameters are allowable for BALLISTIC v3.20:

ALTITUDE - The  altitude to be used in the ballistic  table

and  ballistic coefficient calculations. May  be  REAL.

The default value is zero.

BELL - Controls whether the console bell is rung for errors

detected  by the program. Allowable values are  ON  and

OFF. The default is BELL ON.

BKG - Determines  the background text screen color.  Allow-

able  values are 0 through 7. Must be INTEGER. The  de-

fault value is 0. This parameter is illegal for systems

having only a monochrome text/graphics card.  Following

are the colors corresponding to each value:

0 - Black         4 - Red

1 - Blue          5 - Magenta

2 - Green         6 - Brown

3 - Cyan          7 - Lt. Gray

COLOR - The  text color. Default is 7. See BKG for  a  des-

cription of allowable values and restrictions.

GREENHILL - This  option specifies a new  Greenhill  number

for  use in the minimum twist  calculation.  Acceptable

values range from 75.0 to 200.0. The default is 150.0.

GRID - This  option  informs the program whether  the  grid

should initially appear in the Plot function. This  may

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 14

also  be toggled from the Plot function itself.  Allow-

able values are ON and OFF.

PRESSURE - The atmospheric pressure used for the  ballistic

table  and ballistic coefficient calculations.  May  be

REAL.  The default value is dependent on  the  function

file chosen.

PRINTER - Line  width in characters of the system  printer.

Must be an INTEGER value from 80 to 200. Default is 80.

PTYPE - Type of graphics printer connected to system.  This

option is only used by the graphic screen dump routine.

Default is 1, for the HP Laserjet. Currently  supported

values are:

1 - Hewlett Packard Laserjet

2 - Star SG-10 (in IBM mode)

3 - IBM Proprinter

4 - IBM Graphics Printer

5 - Epson FX-80

TEMPERATURE - The  temperature  to be  used  for  ballistic

table  and ballistic coefficient calculations.  May  be

REAL.  The default value is dependent on  the  function

file chosen.

VIDEO - Determines the method used to write to the  screen.

Allowable  values are BIOS and DIRECT. BIOS causes  all

output to be done through the use of system BIOS calls.

This mode may be useful for computers/graphics adapters

that are not 100% compatible with the IBM PC family. It

does,  however,  significantly slow output.  Using  the

DIRECT  option causes all screen output to  be  written

directly to video memory. Default is DIRECT.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 15

5.  SOURCES OF DATA

Much  of  the data necessary for ballistic  calculations  is

readily available. Even the ballistic coefficient can usually  be

calculated from the information at hand. Following are  suggested

sources for specific input data to the program.

A. Altitude

The  calculation  doesn't depend on altitude  directly,  but

rather  on the way air temperature and pressure vary  with  alti-

tude.  The program uses this value to determine what  the  "stan-

dard"  temperature and pressure are for a given  ballistic  func-

tion. Users can look up the actual altitude of their location  in

an  almanac or one of the U.S. Department of the Interior  "Quad-

rangle Maps" available at many sporting goods or camping  stores.

Either  jot it down near the computer so it can be  entered  when

the  program  is run or add it to the configuration file  as  de-

scribed in section 4.

B. Ballistic Coefficient

This  will  probably be the most difficult number  to  find.

Those who reload will usually be able to get this value from  the

manufacturer  of their bullets. (Speer and Sierra, for  instance,

print this data in their reloading manuals.) The value for  other

bullets  may sometimes be estimated by comparing  bullet  shapes,

provided the bullet is of the same caliber. Comparing the  bullet

in question to a specially prepared chart of shapes is often more

accurate.  The  DuPont Company at one time produced a  series  of

these  and  other useful charts, called "A Short Cut  To  Ballis-

tics".  The complete set is supplied with Ackley's  handbook  and

the  specific chart for estimating ballistic coefficients is  re-

printed in HATCHER'S NOTEBOOK.

The program itself will calculate the ballistic  coefficient

in  certain cases. Both the initial and remaining velocity  at  a

given  range  must be known. This data is available  for  factory

loaded  ammunition in manufacturers' literature and sources  such

as  SHOOTERS BIBLE. (A short section of coefficients for  several

factory loadings calculated using this program and Ingalls' table

may be found in Appendix C.)

C. Bullet Weight, Length, and Caliber

These are easy. On factory-loaded ammunition or  components,

bullet weight (in grains) and caliber will be written on the box.

Those who cast their own bullets will need to weigh several  sam-

ples  prepared  with the actual alloy used. Take the  average  of

half  a dozen or so and use that value. A  sufficiently  accurate

value for bullet length may be found with a ruler.

D. Gun, Powder Weights

These two are needed only to calculate the amount of  recoil

generated  by a given load. The weight of a rifle can usually  be

found accurately enough on a bathroom-type scale. Actual  weights

of  most  firearms can be found in manufacturers'  literature  as

well as in such sources as SHOOTER'S BIBLE, GUN DIGEST, and  pro-

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 16

duct reviews in many magazines. Don't forget to add in the weight

of scopes, slings, custom stocks or any other accessories.

Powder weights will be known if performing calculations  for

handloads. If using factory loads, try estimating by comparing to

available  reloading  data. Find a load that moves  a  bullet  of

similar  construction  (jacketed, soft lead, etc.) to  about  the

same muzzle velocity.

E. Specific Gravity

This  value  is only required for  calculating  the  minimum

necessary  twist  for stabilization. Two more  or  less  standard

values are 11.34 for pure lead, and 10.90 for a typical  jacketed

bullet.  Other values are very difficult to find in  the  litera-

ture.  Hatcher,  describes a fairly simple way  to  measure  this

quantity,  but the two values above should be sufficiently  accu-

rate for most practical purposes.

Appendix D of this document contains a short compilation  of

specific  gravity data for some common Lead/Tin/Antimony  alloys.

These were taken primarily from volume one of the American  Soci-

ety of Metals METALS HANDBOOK.

F. Temperature, Atmospheric Pressure, Wind Speed and Direction

Suitably accurate values for temperature and pressure can be

obtained  from a weather report or newspaper. As  with  altitude,

the  calculation  isn't terribly sensitive to these  factors,  so

unless  the range is in the Sahara Desert or Antarctica they  can

probably be ignored. (It CAN be interesting to set up a ballistic

table and vary them just to see what happens... Experiment!)  The

same  suggestion applies; try the program with and  without  cor-

recting and see if the difference is big enough to  realistically

notice.  Bear  in mind that this program examines the  effect  of

these  variables only on the ballistic coefficient.  Temperature,

for instance, will also change the initial velocity of a load,  a

factor  beyond the ability of this program to calculate.  (Tables

describing  this effect can be found in both the Speer  reloading

manual and Ackley's handbook.) Temperature and pressure are  also

valid parameters for use in the configuration file. (Section 4)

Wind direction can only be estimated at the actual  shooting

site.  Zero degrees indicates a wind blowing across the  bullet's

path to either the right or left. Ninety degrees indicates a wind

blowing  either  directly behind or directly  into  the  bullet's

path. As this value is only used for calculating wind deflection,

a signed value isn't necessary. Also, since deflection is  linear

with  wind  speed, leaving the value set for the default  10  mph

allows for easy interpolation at the range.

G. Velocities

Muzzle velocities of most factory ammunition can be found in

periodicals  such  as the SHOOTER'S BIBLE. Those who  reload  are

aware  that estimated or reported velocities are  usually  listed

with the loading data for a particular cartridge.

The most effective way to find the velocity for a completely

unknown  load is, of course, the chronograph. Unfortunately,  few

people  have access to one and their price puts them  beyond  the

reach of the casual buyer. (The author once built an  inexpensive

attachment  that allowed a Commodore-64 home computer to be  used

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 17

as  a simple chronograph, but its usefulness was limited since  a

source of house current was still needed nearby... also, the C-64

isn't exactly portable. The technique SHOULD be adaptable to many

other home computers and the author would be happy to discuss the

project with registered users.) Ackley, in his book HANDBOOK  FOR

SHOOTERS  AND RELOADERS, describes a simple "ballistic  pendulum"

that  can be used to estimate projectile velocities. No  external

power is necessary; it relies on the kinetic energy of the bullet

to swing a heavy mass.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 18

6.  USING GENTABLE AND READTABLE

A. Using the pre-programmed functions

Before  running  BALLISTIC for the first time  a  file  con-

taining  the chosen set of function values the program  will  use

during  its calculations must be produced. GENTABLE Version  3.00

makes the following functions available:

FUNCTION           VELOCITY         ATMOSPHERIC

max.   min.     temp.  pressure

British 1909          4000    0        59      29.53

"G1"                  4094   300       59      29.53

Ingalls'              3600    0        60      30.00

Ingalls' (extended)   4094    0        60      30.00

The  British  1909 and Ingalls'  functions  are  practically

identical until low velocities are reached. Both are well  suited

to  calculations  involving typical hunting  or  pistol  bullets,

i.e.,  rounded or slightly pointed (roughly two  caliber  radius)

spitzers. Results for sharply pointed spitzers and boattails will

not  be  as accurate with these functions as they might  be  with

some others. Less accurate results will also be obtained if  bal-

listic coefficients calculated with one set of functions are used

with  another, i.e., using the coefficients listed in Appendix  C

(calculated with Ingalls' table) with the British 1909 tables.

The  extended  Ingalls' table is simply the  standard  table

continued to the maximum allowable program velocity following the

method  used in Hatcher. As values above 3600  are  extrapolated,

they may not be as accurate as those below.

Many  of the companies producing reloading  components  feel

that the G1 table is the best choice for all around use.  Indeed,

most  of  the major ammunition manufacturers seem to use  the  G1

drag function for calculating their own "factory" ballistics  and

duplication  of  these tables is most easily  accomplished  using

this selection. This function is used by SAAMI in calculating the

tables they supply to the manufacturers as well.

GENTABLE is started by typing its name (BAL-GEN) at the  DOS

prompt, optionally following it with a command line argument. The

user will be presented with a short menu to which he should  res-

pond with the number of the function he desires. The program will

next  prompt for a filename to contain the new data, then  create

the  table, overwriting any previous file with that  name.  Users

that  plan to routinely use more than one table  should  probably

create  them all ahead of time, renaming them to  something  more

descriptive. Copy the one used most often to BALISTIC.TAB.

GENTABLE will accept only one argument on the command  line.

If the flag "-BIOS" is used all video output will be sent to  the

screen via BIOS calls. The default is to write directly to screen

memory.

B. Creating custom function tables

The  user is not limited to using the functions produced  by

GENTABLE. Tables may be produced by any program or language  that

stores  numbers  in IEEE format, including Microsoft  C  v5.0  or

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 19

Quick  C.  (There will be a problem  calculating  checksums  when

using  Microsoft  5.0 unless the -Op optimization is  used.)  The

file format and data types used are as follows:

1.  Truncated  revision number for version of  GENTABLE  used

multiplied by -1. (int)

2.  The table name. (char[32])

3.  Standard temperature for table data. (double)

4.  Standard pressure for table data. (double)

5.  Maximum velocity in table (<=4094). (unsigned)

6.  Minimum velocity in table. (unsigned)

7.  S_table, T_table pairs as described in HATCHER'S NOTEBOOK

for velocities (max >= v >= 0). (float)

8.  Checksum  calculated  as the sum of (S_table  -  T_table)

values. (float)

C. Checking the function file

After  creating  the  function file  it  may  be  optionally

checked  with  READTABLE. READTABLE was originally written  as  a

debugging aid and is included only to satisfy user's curiosity as

to what is contained in a ballistic function table. It might also

be of some use to those contemplating creation of their own func-

tion files. Once started, it prompts the user for the name of the

function  file to be examined, displays the  header  information,

then  prints the values to the screen. The speed of  the  display

may  be altered by pressing either the <-> or <+> keys.  Pressing

any other key will cause the display to pause at the next  value.

Once paused, pressing <ESC> causes execution to abort. Any  other

key will cause the program to resume.

As  with GENTABLE, READTABLE is started by typing  its  name

(BAL-READ)  at the DOS prompt, optionally followed by any of  the

following flags:

-BIOS - Causes  the program to perform all writes using  BIOS

calls. This will significantly slow output.

-F - (From) - Start listing the table to the screen with  the

value  immediately following the argument, i.e.,  -F2000

will start the listing at 2000 fps.

-T - (To) - End the listing at the value following this argu-

ment.

-N - (Noprint) - Don't print the table, calculate the  check-

sum only.

-G - (Graphics) - Don't  use the IBM graphic  character  set.

(Use ASCII characters instead.)

Flags  requiring numeric arguments should NOT have  a  space

between the flag and input value.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 20

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- MAIN MENU -

1 ........... Calculate Ballistic Table.

2 ..... Calculate Ballistic Coefficient.

3 ....... Adjust Atmospheric Conditions.

4 ........... Select New Function Table.

5 ............. Miscellaneous Functions.

6 ........................... DOS Shell.

7 .................... Return to System.

Your Choice ? 1

---------------------( Fig. 1 - MAIN MENU )----------------------

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE BALLISTIC COEFFICIENT -

Please input:

Initial velocity ....(fps)  2820

Final velocity ......(fps)  2507

Range .............(yards)  100

Altitude ............(ft.)  0

Temperature ......(deg. F)  59.0

Atmos. pressure ..(in. Hg)  29.53

Effective Bal. Coeff. =  0.288

(Bal. Coeff. at STP = 0.288

-------( Fig 2. - Calculating the Ballistic Coefficient )-------

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 21

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE BALLISTIC TABLE -

Please input:

Initial velocity ........(fps)  2820

Bullet weight ........(grains)  165

Bullet diameter .........(in.)  0.308

Effective Bal. Coeff. ........  0.288

Range .................(yards)  500

Number of intervals ..(50 max)  10

Chart title ?

.308 Winchester - 165 gr. Spire pt.

-----------------( Fig. 3 - MAIN MENU choice 1 )-----------------

.308 Winchester - 165 gr. Spire pt.

(Calculated using G1 table)

Bullet weight ......... 165 grains   Bullet Caliber ........ 0.308

Sectional Density ..... 0.248        IPSC Power Factor ..... 465.3

Effective Bal. Coeff... 0.288        Bal. Coeff. at STP .... 0.288

Cross wind ............ 10.0 m.p.h.  Altitude .............. 0    Ft.

Atmospheric pressure .. 29.53 in.    Temperature ........... 59.0 F

Range  Velocity  Energy   Momentum  Mx. Ht.  Defl.   Drop   Lead   Time

yards   f.p.s.   ft-lb.   lb.-sec.    in.     in.     in.  in/mph  sec.

0     2820    2913.2    2.0661     0.0     0.0     0.0    0.0   0.000

50     2661    2593.5    1.9494     0.1     0.3     0.6    1.0   0.055

100     2507    2302.4    1.8368     0.6     1.1     2.4    2.0   0.113

150     2358    2037.4    1.7278     1.5     2.6     5.5    3.1   0.175

200     2215    1797.1    1.6228     2.8     4.8    10.3    4.2   0.240

250     2076    1579.4    1.5213     4.7     7.8    16.8    5.5   0.310

300     1942    1382.2    1.4231     7.2    11.6    25.3    6.8   0.385

350     1814    1205.8    1.3292    10.5    16.3    36.2    8.2   0.465

400     1693    1050.3    1.2406    14.7    22.0    49.7    9.7   0.550

450     1578     912.5    1.1563    20.0    28.8    66.2   11.3   0.642

500     1470     791.2    1.0767    26.7    36.7    86.2   13.0   0.741

1=Print  2=Form Feed  3=Sight Table  4=New Data  5=Main Menu  ? 3

------------( Fig. 4 - The completed ballistic table )-----------

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 22

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- ADJUST ATMOSPHERIC CONDITIONS -

(Modify ballistic coefficient)

Please input:

Bal. Coeff. at STP ........ 0.288

Altitude .............(ft.) 5000

Temperature .......(deg. F) 41.2

Atmos. pressure ...(in. Hg) 25.44

Wind speed ........(m.p.h.) 10.0

Angle from broadside (deg.) 0

Effective Ballistic Coefficient = 0.323

--------( Fig. 5 - Modifying the ballistic coefficient )--------

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE RECOIL -

Please input:

Initial velocity .....(fps)  2820

Weight of gun ........(lbs)  9.50

Weight of bullet ..(grains)  165

Weight of powder ..(grains)  42.0

Recoil Velocity =  9.5 fps

Recoil Energy =  13.4 ft-lbs

(Recoil due to bullet =  7.2 ft-lbs or 54%)

-----------------( Fig. 6 - Recoil calculation )----------------

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 23

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE POINT BLANK RANGE -

Please input:

Initial velocity ............(fps)  2820

Sight height .............(inches)  0.90

Max. dist. from sight line ..(in.)  6.00

Effective Bal. Coeff. ............  0.288

Targeted range for +/-3.00 inch path is 219 yds.

Point-blank range is 257 yds.

(100 yd. trajectory = +2.9 in.)

-----------( Fig. 7 - Calculating Point Blank Range )-----------

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE TWIST -

Please input:

Bullet diameter ......(in.)  0.308

Bullet length ........(in.)  1.20

Greenhill number ..........  150.0

Spec. gravity of bullet ...  10.90

Slowest twist allowing for stabilization is

1 turn in 11.9 inches.

(Helix angle = 4.7 degrees)

------------------( Fig. 8 - Calculating twist )----------------

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 24

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE TRUE MUZZLE VELOCITY -

Please input:

Distance to first screen ..(ft.)  3.00

Distance between screens ..(ft.)  1.00

Instrumental velocity .....(fps)  2820

Effective Bal. Coeff. ..........  0.288

Actual Muzzle Velocity =  2824 fps

---( Fig. 9 - Calculating muzzle velocity from instrumental )---

--==<  B A L L I S T I C  v3.20  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE VELOCITY STATISTICS -

1 - 2820.0     6 - 2852.0

2 - 2825.0     7 - 2827.0

3 - 2830.0     8 - 2818.0

4 - 2835.0

5 - 2850.0

Maximum = 2852.0,  Minimum = 2818.0,  Extreme Spread =  34.0

Average Velocity = 2832.1,  Sample Standard Deviation = 12.8

1=Clear All  2=Delete  3=Correct  4=Add Entries  5=Exit ?  4

----------( Fig. 10 - Calculating velocity statistics )----------

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 25

Appendix A

Glossary

BALLISTICS - The  study of moving projectiles. Commonly, what  is

meant is "exterior" ballistics, that portion of the bullet's

flight  between  muzzle and  target.  "Interior"  ballistics

refers  to  the  portion between primer  ignition  and  free

flight  and "terminal" ballistics to what happens  when  the

bullet strikes its target.

BALLISTIC COEFFICIENT - The  ratio of the sectional density of  a

projectile to its coefficient of form. A measure of how well

a  bullet  retains its velocity. Commonly  supplied  by  re-

loading manufacturers, or it may be derived from their  bal-

listic  tables. Two types are used by the program.  The  STP

value  is that which the bullet would have at  the  standard

temperature  and  atmospheric pressure for the  table  being

used.  The "effective" value is the STP value  modified  for

the actual atmospheric conditions.

BULLET - The actual projectile in small arms ammunition. Commonly

corrupted to mean the entire cartridge.

BULLET WEIGHT - Weight of the actual projectile. Usually given in

grains.

CALIBER - Technically, the diameter of the barrel measured across

the lands. In practice, the diameter of the bullet.

CARTRIDGE - Fixed ammunition, including case, powder, primer, and

bullet, for small arms.

COEFFICIENT OF FORM - A number relating the ballistic  efficiency

of  a  given shape to the shape of the  projectile  used  to

calculate the ballistic table.

CROSS WIND ANGLE - The  angle  between the wind and line  of  de-

parture.

DRIFT - Technically,  the distance a bullet will travel  horizon-

tally  due to its spin. This effect is usually quite  small,

amounting to about 7 inches at 1000 yards for a military  M2

(30-06) cartridge. Often confused with wind deflection.

DEFLECTION - See wind deflection.

DROP - The  distance a bullet will fall due to the  influence  of

gravity.  Measured from the line of departure, not the  line

of sight.

ENERGY - Here,  the  kinetic energy of a moving  bullet,  usually

given  in ft-lbs. Equal to one half the mass of  the  bullet

multiplied by the square of the velocity.

F.P.S. - Feet per second. A unit of velocity.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 26

FT.-LBS. - Foot  pounds.  A unit of energy equal  to  the  effort

required to raise one pound to a height of one foot.

GRAINS - A unit of weight equal to 1/7000 of a pound.

INGALLS' TABLES - A  set of ballistic tables first calculated  by

Col.  J. M. Ingalls in 1918. Probably the most  widely  used

tables  for small arms calculations. Other  tables  commonly

used include the British tables of 1909, 1929,  Winchester's

G tables and the Ordnance Department's J table.

I.H.M.S.A. - International  Handgun  Metallic  Silhouette  Assoc-

iation.

I.P.S.C - International Practical Shooting Confederation.

LEAD - How far ahead of a moving target a shooter must aim to  be

assured  of hitting it. In BALLISTIC, the lead is  given  in

inches/m.p.h. of target speed. If the entry in the table  is

"1.2",  the lead for a target moving at 5  m.p.h.,  (perpen-

dicular to the bullet's path) would be (5 * 1.2) 6.0 inches.

Wind speed is NOT taken into account in this calculation.

LINE OF DEPARTURE - An  imaginary  line formed by  extending  the

muzzle  of the weapon. It coincides with the  bullet's  path

only while the bullet is actually in the barrel.

LINE OF SIGHT - An  imaginary line from the center of the  sights

to the point of aim. The bullet typically crosses this  line

only twice; once, a few yards from the muzzle, and again  at

the targeted distance.

MAXIMUM HEIGHT - The highest vertical distance above a line  from

the muzzle to the bullet's current position. Sometimes  con-

fused with the mid-range trajectory due to the fact that  it

typically  occurs about halfway between muzzle  and  target.

Maximum  height  is  usually  somewhat  higher  and   occurs

slightly later than the MRT.

M.O.A. - Minute of Angle. Equal to 1/60 of an angular degree.  At

100 yards, 1 M.O.A. is approximately 1.047 inches.

MOMENTUM - Equal to the mass of a bullet multiplied by its  velo-

city.  Indicative  of the striking or knockdown power  of  a

projectile.

N.R.A - National  Rifle  Association. If you  shoot,  you  SHOULD

belong to this organization.

POINT-BLANK  RANGE - That  range  for  which  the  bullet's  path

doesn't  vary  from the line of sight by more than  a  given

amount.

POWER FACTOR - A  number  used in IPSC competition  to  determine

whether  a handgun cartridge may be scored as a  "major"  or

"minor"  caliber. Defined as the bullet weight  (in  grains)

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 27

divided  by 1000 and multiplied by the velocity.  Cartridges

with  a power factor equal to or greater than 175  are  con-

sidered "major". Cartridges ranking between 125 and 175  are

"minor"  and those below 125 are not legal for  IPSC  compe-

tition.

S.A.A.M.I - Sporting Arms Ammunition Manufacturers Institute.

SECTIONAL DENSITY - The  weight of a projectile (in  pounds)  di-

vided by the square of its diameter (in inches).

SIGHT HEIGHT - The  measured distance between the centerlines  of

the sights and muzzle of a gun.

SPECIFIC GRAVITY - The ratio of the mass of a material to that of

an equal volume of water.

TARGETED RANGE - That  distance  where  the path  of  the  bullet

crosses the line of sight. (The "sighting in" distance.)

TIME OF FLIGHT - The  time necessary for a bullet to travel  from

the muzzle of a gun to a given range.

TRAJECTORY - The path of a moving projectile. Here, the number of

inches above or below the line of sight.

TWIST - Here,  the rate of twist in the rifling of a gun  barrel.

The minimum amount of twist necessary to stabilize a  bullet

in  flight can be determined from its length,  density,  and

caliber. Usually expressed as "1 turn in xx inches" where xx

may range from about 8 inches up to over 30. "Faster"  twist

translates  to  a  smaller  number.  Minimum  twist  is  the

"slowest" twist that will stabilize a given bullet.

WIND DEFLECTION - The amount of horizontal motion attributable to

the action of the wind. Often mistakenly called drift.

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 28

Appendix B

Information Sources

BALLISTICS:

Any intermediate physics (mechanics) text.

BALLISTICS ON THE HOME COMPUTER. American Rifleman, June

1983

BALLISTICS ON YOUR POCKET CALCULATOR. American Rifleman,

June 1987

BALLISTIC TABLES AND HOW TO USE THEM. American Rifleman,

December 1963

CALCULATING RECOIL. American Rifleman, March 1988

EXPLORING BALLISTICS WITH YOUR COMPUTER, Byte Magazine

September 1980

EXTERIOR BALLISTICS, McShane, Kelly and Reno, University of

Denver Press, 1953

HATCHER'S NOTEBOOK, Julian S. Hatcher, Harrisburg: Stackpole

Books, 1962

TRAJECTORY COMPUTATION SYSTEM FOR DIGITAL COMPUTER, SAAMI

publications, 1976

RELOADING:

CARTRIDGES OF THE WORLD, J.T. Amber, Northfield: Digest

Books, 1972

COMPLETE GUIDE TO HANDLOADING, Philip B. Sharpe, New York:

Funk & Wagnells Co., 1953

HORNADY HANDBOOK OF CARTRIDGE RELOADING, RIFLE-PISTOL, Grand

Island: Hornady Manufacturing Co., 1973

HANDBOOK FOR SHOOTERS AND RELOADERS, VOL I & II, P.O. Ackley,

Salt Lake City: Publisher's Press, 1965

LYMAN RELOADING HANDBOOK #46, Middlefield: Lyman Products

for Shooters, 1970

NRA HANDLOADER'S GUIDE, Wash. D.C.: The National Rifle

Association of America, 1969

PRINCIPLES AND PRACTICE OF HANDLOADING, Georgetown: Small

Arms Technical Publishing Co., 1954

SIERRA BULLETS RELOADING MANUAL, Santa Fe Springs: Sierra

Bullets, 1971

SPEER RELOADING MANUAL #10, Lewiston: Speer, Inc., 1979

STATISTICS:

DATA REDUCTION AND ERROR ANALYSIS FOR THE PHYSICAL SCIENCES,

Philip R. Bevington, New York: McGraw-Hill Co., 1969

ASSOCIATIONS:

THE NATIONAL RIFLE ASSOCIATION - 1600 Rhode Island Ave. NW,

Washington, D.C. 20036

S. A. A. M. I. (SPORTING ARMS AMMUNITION MANUFACTURERS

INSTITUTE) - PO Box 838, Branford, Conn. 06405

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 29

Appendix C

Ballistic Coefficients for Factory Bullets

The  coefficients  listed here were calculated  from  tables

found  in  SHOOTER'S BIBLE No. 75 (1984) using  the  program  and

Ingalls'  function.  No attempt was made to include  every  load,

instead, representative loads for each caliber were chosen.

Part 1 - Centerfire Pistol and Revolver

Cartridge      Bullet       Coefficient    Manufacturer

================================================================

.25 ACP        50 FMC          .174            F

.25 ACP        50 FMC          .097            R,W

.380 ACP       85 STHP         .105            W

.380 ACP       90 JHP          .073            F

.380 ACP       95 FMC          .082            F,R,W

9 mm          95 JHP          .101            F,R,W

9 mm         115 FMC          .128            W

9 mm         115 JHP          .131            R,F

9 mm         115 STHP         .131            W

9 mm         123 FMC          .140            F

.38 SPEC.     110 STHP         .147            W

.38 SPEC.     148 WC           .062            F,R,W

.38 SPEC.     158 LRN          .162            F,R,W

.357 MAG.     110 JHP          .093            F,R,W

.357 MAG.     125 JHP          .122            F,R,W

.357 MAG.     158 JSP          .135            F,R,W

.44 MAG.      180 JHP          .120            F,R

.44 MAG.      210 STHP         .125            W

.44 MAG.      220 MCP          .193            F

.44 MAG.      240 JHP          .156            R

.44 MAG.      240 LGC          .139            R

.45 ACP       185 MCWC         .082            R,W

.45 ACP       185 JHP          .152            F

.45 ACP       185 STHP         .137            W

.45 ACP       230 FMC          .165            R,W

.45 COLT      225 STHP         .167            W

.45 COLT      225 SWCHP        .172            F

.45 COLT      250 LRN          .157            R

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  FMC=Full  Metal  Case, STHP=Silver  Tip  Hollow  Point,

JHP=Jacketed  Hollow Point, MCWC=Metal Case Wadcutter,  WC=Wad-

cutter,  LRN=Lead Round Nose, LGC=Lead Gas Check,  JSP=Jacketed

Soft Point, MCP=Metal Case Profile, SWCHP=Semiwadcutter  Hollow

Point

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 30

Part 2 - Centerfire Rifle

Cartridge      Bullet       Coefficient    Manufacturer

=================================================================

.22-250 REM.     55 PSP          .231            F,W

.22-250 REM.     55 PSP          .253            R

.223 REM.        55 FMC          .207            R

.223 REM.        55 FMC          .278            W

.223 REM.        55 MCBT         .350            F

.243 WIN.        80 SP           .259            F,R,W

.243 WIN.       100 PSP          .371            F,R,W

.270 WIN.       130 PSP          .378            F,W

.270 WIN.       130 PSP          .345            R

.270 WIN.       150 SP           .267            F,R

.270 WIN.       150 PSP          .356            W

7mm MAUSER     140 PSP          .433            F,R

7mm MAUSER     175 SP           .280            F,W

7mm REM.MAG.   150 PSP          .354            F,R,W

7mm REM.MAG.   175 PSP          .453            F,R,W

.30 CARBINE     110 FMC          .180            F,W

.30 CARBINE     110 FMC          .164            R

30-30 WIN.     150 SP           .221            F,W

30-30 WIN.     150 SP           .195            R

30-30 WIN.     170 SP           .259            F,R,W

30-06          150 PSP          .329            F,R,W

30-06          180 PSP          .398            F,R,W

30-06          200 BTSP         .592            F

30-06          220 SP           .299            R,W

.300 WIN.MAG.   180 PSP          .462            F,R,W

.308 WIN.       150 PSP          .323            F,R,W

.308 WIN.       180 PSP          .390            F,R,W

.35 REM         200 SP           .193            F,R,W

.45-70          300 JHP          .302            F,W

.45-70          405 SP           .269            R

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  FMC=Full  Metal Case, PSP=Pointed Soft  Point,  SP=Soft

Point, BTSP=Boat Tail Soft Point, MCBT=Metal Case Boat Tail

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 31

Part 3 - Rimfire

Cartridge              Bullet   Coefficient  Manufacturer

==================================================================

.22 LR "VIPER"           36 TC       .110           R

.22 LR "YELLOW JACKET"   33 TCHP     .101           R

.22 LR HV                40 LRN      .128           R

.22 LR HV                36 LHP      .115           R

.22 LR TARGET            40 LRN      .136           R

.22 LONG                 29 LRN      .095           R

.22 SHORT                29 LRN      .098           R

.22 SHORT                27 LHP      .091           R

.22 SHORT TARGET         29 LRN      .097           R

.22 WIN. MAG.            40 JHP      .108           W

.22 WIN. MAG.            40 FMC      .108           W

.22 LR "XPEDITER"        29 LHP      .079           W

.22 LR "H.P."            37 LRN      .117           W

.22 LR "DYNAPOINT"       40 LDP      .127           W

.22 LR "T22"             40 LRN      .135           W

.22 LR                   40 LRN      .127           W

.22 LONG                 29 LRN      .095           W

.22 SHORT "H.P."         27 LRN      .091           W

.22 SHORT "T22"          29 LRN      .097           W

.22 SHORT                29 LRN      .098           W

.22 LR "HI POWER"        40 LRN      .126           F

.22 LR "HI POWER"        38 LHP      .122           F

.22 LR "CHAMPION"        40 LRN      .135           F

.22 LONG "HI POWER"      29 LRN      .094           F

.22 SHORT                29 LRN      .100           F

.22 SHORT                29 LHP      .092           F

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  LRN=Lead  Round Nose, LHP=Lead Hollow  Point,  LDP=Lead

Dynapoint, FMC=Full Metal Case, TC=Truncated Cone, TCHP=  Trun-

cated Cone Hollow Point

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 32

Appendix D

Specific Gravities of Bullet Alloys

Part 1 - Lead/Tin/Antimony Alloys

Percent Composition      Specific Gravity      Trade or

Pb      Sn       Sb         of alloy          Common Name

=================================================================

100                           11.340        pure lead

100                    7.298        pure tin

100           6.62         pure antimony

30      70                    8.32         soft solder

37      63                    8.42         eutectic solder

50      50                    8.89         50-50 solder

75      10       15           9.73         lead babbit-SAE 14

80      20                   10.20         20-80 solder

80       5       15          10.04         alloy 8

83       2       15          10.09         alloy 10

85               15          10.28         alloy 11

85       5       10          10.24         lead babbit-SAE 13

91                9          10.66         9% antimonial lead

92                8          10.74         8% antimonial lead

94                6          10.88         hard lead (bullets)

95       5                   11.00         5-95 solder

96                4          11.04         hard lead

99                1          11.27         1% antimonial lead

92        8           7.28         white metal

Part 2 -  Other Alloys

Composition         Specific Gravity      Trade or

of alloy          Common Name

=================================================================

100 Cu                         8.96         pure copper

100 Fe                         7.87         pure iron

95 Cu, 5 Zn                   8.86         gilding

90 Cu, 10 Zn                  8.80         commercial bronze

70 Cu, 30 Zn                  8.53         cartridge brass

98.75 Cu, 1.25 Sn             8.89         1.25% phos. bronze

90 Cu, 10 Sn                  8.78         10% phosphor bronze

97 Cu, 3 Si                   8.53         silicon bronze (A)

98.5 Cu, 1.5 Si               8.75         silicon bronze (B)

99.5 Fe,.06 C,.38 Mn,.01 Si   7.87         .06% carbon steel

99.0 Fe,.23 C,.64 Mn,.11 Si   7.86         .23% carbon steel

98.7 Fe,.44 C,.69 Mn,.20 Si   7.84         .435% carbon steel

---------------

SOURCES:

Metals Handbook, Vol. 1, "Properties and Selection of Metals"

(8th ed.), American Society for Metals

Metal and Alloys Data Book, Samuel L. Hoyt

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 33

Appendix E

User Supported Software

User  Supported Software is based on the premise that  if  a

someone  gets a chance to try out a program, likes it, and  finds

he uses it on a regular basis, he will voluntarily contribute  to

the  support of that program. (Where else do you get  the  oppor-

tunity  to check out a product BEFORE you buy it?) Users are  en-

couraged to copy and distribute User Supported Software for  non-

commercial  purposes. Commercial users should be aware  that  the

author of the program often retains commercial rights.

BALLISTIC  is being distributed as User Supported  Software.

If after an initial trial period you find you're using  BALLISTIC

and  you like it, please send the registration form,  check  (and

diskette if indicated) to:

William R. Frenchu

79 Taylor Terrace

Hopewell, NJ  08525

(Please label the diskette with your name and address.)

Contributors  will receive a copy of the latest revision  of

the  program,  as well as the TURBO C source code.  In  addition,

they  will be informed by mail of any major updates  or  improve-

ments. Major revisions will probably not be undertaken without  a

positive  user response. If you have any questions,  problems  or

suggestions  about  the program, (or need support for  a  special

printer, etc.) please contact the author.

All  commercial  rights  to BALLISTIC are  reserved  by  the

author. Anyone wishing to make any commercial use of this program

(or the tables it produces) is urged to contact the author at the

above  address. This includes people who use the program  in  any

commercial  or governmental environment. Site and commercial  li-

censes are available at very reasonable rates.

Non-commercial  users are encouraged to make and  distribute

as  many UNMODIFIED copies as they like, provided  the  following

files are included:

BAL-GEN.EXE

BAL-READ.EXE

BAL-READ.ME!

BALISTIC.EXE

BALISTIC.DOC

BALISTIC.REV

Comments  or  suggestions may be left for the  author  (Bill

Frenchu) at any of the following places:

Bullet 'n Board BBS    (703)-971-4491

Road To Utopia BBS     (609)-799-5418 (CoSysop)

CompuServe                   74575,61

BALLISTIC v3.20 - (C) Copyright 1988  W.R. Frenchu         Page 34

REGISTRATION FORM FOR BALLISTIC v3.20

Name _____________________________________   Date _______________

Mailing Address:

Street __________________________________   Phone ______________

City  ________________________  State ___   Zip ________________

Computer type (& BIOS)________________________ Printer __________

Display and adapter type ________________________________________

Where did you find out about BALLISTIC? _________________________

(If CompuServe, please list forum.)

Please register my copy of BALLISTIC and send the latest version

(as well as the TURBO-C source code) to the above address.

_____ I've enclosed $15.00 and a 5 1/4" DSDD (360k) or 3.5"

(720k) diskette. (No 1.2 or 1.44 Meg diskettes please!)

_____ I've enclosed $20.00. Please send  5 1/4"  3.5" (circle)

The author is always trying to improve BALLISTIC. You can help!

Comments about BALLISTIC v3.20:

Comments about the documentation:

Suggestions for future versions of BALLISTIC:

Types of shooting you do (How will you use BALLISTIC?):

Mail to: William Frenchu, 79 Taylor Terrace, Hopewell, NJ  08525

########################################