][][][][][][][][][][][][][][][][][][][][

][          BROUGHT TO YOU BY         ][

][      COMPUTER PIRATES OF UTAH      ][

][      COURTESY OF THE SAFEHOUSE     ][

][           (801)-264-8201           ][

][][][][][][][][][][][][][][][][][][][][

[+][+][+][+][+][+][+][+][+][+][+][+][+]

[+]                                 [+]

[+]     L O C K   P I C K I N G     [+]

[+]                                 [+]

[+][+][+][+][+][+][+][+][+][+][+][+][+]

[+]                                 [+]

[+]     WRITTEN BY: DARC DEATHE     [+]

[+]                                 [+]

[+][+][+][+][+][+][+][+][+][+][+][+][+]

THIS TUTORIAL WILL DEMONSTRATE HOW TO

"PICK" A PIN TUMBLER LOCK.  USE OF THIS

MATERIAL IS FOR LOCKSMITHS ONLY, ANY

USE OF THIS INFORMATION FOR ILLEGAL

PURPOSES IS FORBIDDEN AND AGAINST THE

LAW. (AS LONG AS WE ARE AT IT, DO YOU

WANT TO BUY SOME LAND IN FLORIDA?)

IN ORDER TO PICK A PIN TUMBLER LOCK,

YOU WILL REQUIRE FOUR ITEMS: A LOCK,

YOU, A PICK, AND A TENSION WRENCH. YOU

CAN USSUALLY GET THESE AT A LOCKSMITH

STORE, IF YOU CAN NOT FIND ONE NEAR YOU

THERE WILL BE AN ADDRESS AT THE END OF

THE ARTICLE THAT YOU CAN ORDER THEM

FROM. HERE IS AN ILLUSTRATION OF A PICK

AND A TENSION WRENCH:

________/   !________

PICK         TENSION WRENCH

MOST PEOPLE KNOW OF THE NEED FOR THE

PICK, BUT HAVE NO IDEA WHAT THE WRENCH

IS FOR.  IT IS VERY IMPORTANT AND WITH-

OUT IT IT WOULD BE IMPOSSIBLE TO PICK

A LOCK.

IN ORDER TO PICK A LOCK, WE MUST COUNT

UPON THE IMPERFECTION OF THE LOCK.

BEFORE WE LOOK AT HOW TO ACTUALLY PICK

THE LOCK, WE WILL LOOK AT THE PARTS OF

IT AND HOW THE IMPERFECTION PART FITS

IN.  HERE IS A DISSASSEMBLED LOCK:

/   /   /   /

\   \   \   \

SPRINGS -> /   /   /   /

\   \   \   \

_   _   _   _

! ! ! ! ! ! ! !

! ! ! ! ! ! ! !

DRIVERS ->! ! ! ! ! ! ! !

!_! !_! !_! !_!

_           _

! !  _      ! !

BOTTOM PINS ->! ! ! !  _  ! !

! ! ! ! ! ! ! !

\_/ \_/ \_/ \_/

_____________________

!   : : : : : : : :   !

HOUSING ->!   : : : : : : : :   !

!   : : : : : : : :   !

!___: :_: :_: :_: :___!

!    : : : : : : : :   !

PLUG ->!    : : : : : : : :   !

!______________________!

!                     !

!                     !

!                     !

!_____________________!

___

/   \

!     !__      _   _

!        \__  / \_/ \__

!           \/         \

\__/-------------------  <- KEY

WHEN YOU INSERT A KEY INTO A LOCK, THE

BOTTEM PINS ARE PUSHED UP, AND IF IT

IS THE PROPER KEY, THE TOPS OF THE

BOTTOM PINS WILL MATCH WITH THE SPOT

WHERE THE PLUG AND HOUSING MEET, THUS

ALLOWING YOU TO TURN THE PLUG, AND OPEN

THE DOOR, ETC.. WHEN YOU INSER THE KEY,

THE BOTTOM PINS GO INTO THE VALLEYS OF

THE KEY, THUS MEANING THAT THE KEY MUST

HAVE THE RIGHT HEIGHT VALLEYS TO MAKE

THE LOCK OPEN. PRETTY ELEMENTRY, RIGHT?

WELL NOW WE CAN MOVE ON TO HOW TO PICK

A LOCK.

IN ORDER TO PICK A LOCK WE (AS I SAID

BEFORE) DEPEND ON THE INACCURACY OF THE

MANUFACTURING PROCESS. THE FIRST THING

TO DO IS TO INSERT THE TENSION WRENCH

INTO THE LOCK AND APPLY A SLIGHT

PRESSURE TO THE LEFT (OR RIGHT IF YOU

WISH) SO THAT IF YOU COULD LOOK INSIDE

THE LOCK AT WHERE THE PLUG AND THE

HOUSING WOULD MEET, IT WOULD LOOK LIKE

THIS:

! !*! !

HOUSING   ! !*! !

! !*! !

__________! !*! !___________

__________  !*! ____________

!  !*!!

PLUG     !  !_!!

!   _ !

!  !*!!

!  \_/!

NOW A SLIGHT PRESURE IS ON THE PINS.

BECAUSE THE PINS CAN NOT BE PRODUCED

EXACTLY THE SAME, THERE IS ONE PIN

WHICH IS THE WIDEST AND THERE FORE HAS

MORE TENSION ON IT, AND ONE WHICH IS

THE THINNEST AND HAS ALMOST NO PRESSURE

ON IT. WE NOW USE THE PICK TO >GENTLY<

PUSH EACH PIN UP (AND TRY TO FEEL IT

WHEN YOU LET IT DOWN) UNTIL WE FIND

WHICH IS THE TIGHTEST ON AND WHICH IS

LOOSEST. GETTING THE FEEL FOR THIS IS

THE HARDEST PART OF LOCK PICKING. NOW

THAT YOU HAVE FOUND THE LOOSEST ONE,

GENTLY PRESS IT UPWARD UNTIL YOU FEEL

A SLIGHT REDUCTION IN TENSION ON THE

TENSION WRENCH. THIS WILL HAPPEN WHEN

THE TOP OF THE BOTTOM PIN BECOMES

EVEN WITH THE JUNCTION OF THE PLUG AND

THE HOUSING. DO NOT RELEASE ANY TENSION

FROM THE WRENCH NOW! THE DRIVER WILL

NOW BE TRAPPED IN THE HOUSING AS

ILLUSTRATED HERE: (DON'T I DRAW PRETTY)

! !*! !

HOUSING  ! !*! !

! !*! !

___________! !_! !___________

_______________     ___________

! !*! !

PLUG       ! !*! !

! \_/ !

!     !

NOW YOU CONTINUE THIS PROCESS WITH EACH

OF THE PINS UNTIL YOU WORK YOUR WAY UP

TO THE ONE THAT IS WIDEST. WITH SOME

PRACTICE YOU CAN GET FAIRLY FAST AT

THIS. I SUGGEST PRACTICING ON A FOUR

PIN TUMBLER LOCK THAT IS BOUGHT FROM A

HARDWARE STORE, THE CHEAPER THE BETTER.

I WOULD LIKE TO DISCUSS A PATICULAR

CONFIGURATION OF THE PINS NOW THAT MAY

PRESENT A PARTICULARLY HARD JOB TO

PICK. THIS IS GRAPHICLY SHOWN HERE BY

THE TWO MIDDLE PINS:

!*! !*! !*! !*!

!*! !*! !_! !*!

!*! !*!  _  !*!

!_! !*! !*! !_!

_  !*! !*!  _

!*! !*! !*! !*!

!*! !_! !*! !*!

!*!  _  !*! !*!

!*! !*! !*! !*!

\_/ \_/ \_/ \_/

\_______________

WHEN YOU TRY TO PUSH THE 2ND PIN FROM

THE LEFT UP, YOU WILL UNAVOIDABLY BE

PUSHING THE ONE IN FRONT OF IT UP BE-

CAUSE OF IT'S LONG BOTTOM PIN. THE ONLY

SOLUTION FOR THIS IS TO GET A SPECIAL

PICK THAT LOOKS LIKE THIS:

\

\   _______________

\_/

THE MAJOR PROBLEM WITH THIS IS THAT IT

IS HARD TO INITIALLY DETECT. THE REASON

THAT IT MAKES IT HARDER IF IT IS NOT

IMMEDIETLY APPERANT IS THAT YOU UN-

AVOIDABLY PUSH THE 3RD PIN FROM THE

LEFT UP INTO THE HOUSING, GETTING IT

JAMMED:

! !*! !

HOUSING   ! !_! !

!  _  !

! !*! !

__________! !*! !________

___________ !*! ________

!!*!!

PLUG     !!*!!

!\_/!

I WOULD ALSO LIKE TO ADDRESS A

TECHNIQUE CALLED RAKING. IT USES A

TOOL LIKE THIS:

\/\/\/\___________

BASICLY YOU "RAKE" IT BACK AND FORTH

ACROSS THE PINS, HOPING THAT COMBINED

WITH THE TENSION IT WILL GIVE YOU THE

RIGHT COMBINATION. THIS WAY HAS BEEN

KNOWN: FAST SOMETIMES, BUT IS

NOT VERY RELIABLE, AND I WOULD SUGGEST

LEARNING TO ACTUALLY "PICK" THE LOCK.

EARLIER I PROMISED AN ADDRESS TO ORDER

LOCKSMITHING MATERIALS FROM, SO HERE IT

IS:

GARRISON PROTECTIVE ELECTRONICS

BOX 128

KEW GARDENS, NEW YORK, 11415

SOURCES: PERSONAL PRACTICE AND MANY

EXCELLENT BOOKS FROM MENTOR PRESS, IF

YOU WOULD LIKE THEIR CATALOG, SEND A

SASE TO:

THE INTELLIGENCE LIBRARY

MENTOR PULICATIONS

135-53 NORTHERN BLVD.

FLUSHING, NY  11354

AND ASK FOR ANY INFORMATION AVAILABLE

ON THE INTELLIGENCE LIBRARY.

THIS CONCLUDES OUR EXTRAORDINAIRELY

GRAPHIC ARTICLE ON LOCK PICKING. IF YOU

HAVE ANY QUESTIONS, LEAVE E-MAIL FOR

DARC DEATHE ON MOST NATIONAL BBS'S.

Edited by : Quasimoto