WB8EOH Gizmo Report

(Very Long)

The Ramsey FM-10 is a low power FM stereo transmitter kit that is easy

to build and has many practical applications around the home and

ham/swl shack.  After living with one for several weeks, I could never

go back to life before I acquired this little "freedom machine".  The

audio quality and stability of the signal give the unit many HI FI

type applications, and indeed I am already planning to buy another

unit to patch into the Audio/Video system.

Egad, you mean you have to BUILD this thing?

When my XYL saw that this package I had spent fifty bucks on consisted

of a couple of polyethylene bags full of plastic bugs and little

metalic button like pieces, she shook her head in dismay.  Though I

tinker with computer and radio gizmos all day long, I will admit that

the past 20 years have seen me become an "appliance operator" more

content to sit back and PLAY than get in there and tear into the

circuitry, much less build something from scratch.  Well, the folks at

Ramsey have honed the art of kitbuilding, and even a klutz like me CAN

have the satisfaction (like in the old Heath ads) to say "I built it

myself".

Constructing the FM-10 was in part a happy trip down memory lane.  In

my distant youth, one of my happiest memories was the construction and

operation of an Allied Radio Knight Kit #83Y706 three tube AM Phono

Oscillator.  This wonderful little gadget paved the way not only for

my interest in Ham radio, but also got me interested in broadcasting,

a field which put bread on the table for many years.  Well in the more

than 30 years between the two kits, things sure have changed.

First right off, don't even think about dragging out the old Weller

soldering gun and the old roll of Kester solder.  First thing I had to

do was borrow a low wattage iron, and get some super thin solder.  The

Lifetime Supply of solder I bought in 1968 (five pounds) was fine for

antenna work, and gobbing up audio connectors, but the old stuff is

actually WIDER than many of the separate connections on the FM-10 PC

board.  EGAD these components are TINY.

Kudos to Ramsey for the way their whole concept.  The manual is very

well written.  My only problem was I did not get the companion booklet

on how to build a kit (the generic HOW TO SOLDER book).  Well I hoped

that the instructions that came with my 1957 Knight Kit still applied,

because I remember them.  Heat the component, don't glob it all over

the place, and try not to melt the PC board.  I was on my own to

develop a technique.  Ramsey even takes this into account by having

you mount some of the larger "landmark" components first, namely three

RCA jacks (left and right audio and the antenna jack).

Now I wanted this thing to work.  I would not be able to face Elaine

if it became necessary to send the completed unworking mess back to

Ramsey for their $18 an hour bail-you-out plan.  So I took absolutely

extrordinary steps.  With a (borrowed) digital meter, I measured every

resistor, even though Ramsey gives the color code for each one in the

manual as you install it.

One of the nicest touches is the GIANT print of the PC board, upon

which you place all the components in the same place they will go in

the final kit.  I did this with most of the components, except for the

multitude of .01 capacitors, which I left neatly together (Ramsey

uses a masking tape type type medium for keeping similar value

components together).

The scariest moment for me was soldering in the 18 pin DIP socket that

houses the heart of the kit, a ROHM stereo transmitter on a chip.  We

are talking TEENY TINY little pins separated by seeming microns.

EGAD.  After each dot of solder, I held the PC board a quarter inch

from my eyeballs to make sure there were no solder bridges.  And I

measured all the components with the Digital meter a second time

before installing each one.  You probably won't take these

extrordinary steps, which make the simple kit an all night project.

But it was a FUN night.  The scent of melting tin/led/rosin... The

occaisional absolutely PERFECT joint...  Kitbuilding is not only an

art, it is positively a cosmic experience.  It is relaxing, creative,

and there is the anticipation of all the wonderful things you will do

with your kit upon completion.

3:25AM

Ah.  The moment of truth.

Connect the nine volt battery, push the power switch, and TUNE around

on the FM radio to find the carrier.

And I found.......

NOTHING.

It did not work.

Now this is where we separate the men from the boys.  How well you

manage not to throw the thing through the window.

So I went back through the whole manual.  checked EVERY joint.

Eyeballed EACH component.  I could find NOTHING wrong.

Elaine came into the shack as I was holding the PC board.  I hated to

admit defeat.  But I told her it did not work.... yet.

She held the board and looked at it with wonder.  She could not

believe that I had placed each of the little parts in their places.

She told me I would figure out what was wrong with it.

The next day, I went through the manual again.  I took voltage

measurements, and found all the proper voltages on the chip and RF

amplifier.  I was stymied.

Then I looked very closely at the OTHER side of the board.  I stared

at the ROHM chip, which Elaine had called a "train trestle".  Hmmm.  I

wonder if I pushed on the chip if it would go in any farther.

Push.  Click.  It snapped into place.

I pushed the power switch, turned on the FM radio, and found a nice

clean carrier at 102.3, with the stereo light blazing away in pure

clear silence.

Hot DAWG it WORKS!

First thing I did was to move the frequency.  Since it was

transmitting on top of one of the local stations, this seemed in

order.  Ramsey's manual stresses the importance of selecting a clear

channel so as not to bring the wrath of neighbors and the FCC.  Up

here in the boonies, there is a tremendous clear swath from about 98

Mhz to 102 mhz.  (I plan to pouplate it).

The next thing required is adjustment of the subcarrier frequency and

stereo balance.  Surprisingly mine was already right on for the

subcarrier adjustment, and I found out that this adjustment is

actually pretty critical.  There is a test point if you have a freq

counter to get it exactly on 19khz, but it can be done by just turning

the little variable capacitor until the stereo light goes on.

The "stereo balance" control takes a bit of explaining.  This is not a

simple LEFT-RIGHT adjustment, but actually adjustment of the level of

left MINUS right.  It is more of a separation adjustment, and also

seems to have a real effect on tonal quality.  It took quite a bit of

diddling to get it right, and there seems to be some interaction

between the two controls, as the stereo light will go out just when

you think you have the sound right...  The end result is a surprising

quality signal with amazing separation.  However, one thing to make

note of...  The RM-10 is designed with NO audio level controls.  This

is a bit of an oversight, because when using it with components that

have fixed level output, the unit is prone to overmodulation.  You

MUST be able to lower the audio level of whatever it is you are

feeding to the tranmitter, as it is too sensitive with EVERY component

I tried connecting direct.  Once you get the level under control,

though, it actually sounds better than many local FM stations because

the signal is not run through all sorts of "Enhancers" that

broadcasters use to be the loudest thing on the band.  I did find

though that adding an audio limiter (an old DBX 119) really helped

tame the overmodulation problem.

I finished the project by mounting it in the $12.95 Ramsey Kit

cabinet.  OK, this is where the Ramsey guys make a few bucks.  It does

give the kit a finished look, but I would have to say that this is a

bit dear for a simple plastic case.   I think the next one will be

built into a VIDEOTAPE plastic case or other cheapo cabinet.

One oversight is that there is no hole in the cabinet for the whip

antenna which mounts to the circuit board.  Ramsey suggests

constructing a dipole or groundplane antenna, which I did initially in

the final installation in my hamshack.  I have since stopped using

this antenna however (more on why later)

OK NOW WHAT CAN I DO WITH THIS LITTLE GADGET

Ramsey gives many practical applications in the manual.  I found the

unit handy for listening to MDS stereo TV on a little sony walkman in

the wee hours without having to run headphone cords.  Remote listening

of ANYTHING on your main stereo system in any room in your home by

just tuning the radio is just ONE thing the kit can be used for.

My own application is a bit unique.  I have the FM-10 in my ham/swl

shack, and it is connected to a stereo mixer, to which I have several

receivers patched in.  I am a communications junkie and often in the

shack I listen to several things at once.  The only problem is that

one can not stay in ones hamshack all day long.  Well with the FM-10

and a pocket stereo receiver, you can monitor whatever you wish in

your home or yard.  Now since the Ramsey Kit is a STEREO transmitter,

you can do what I do (if you are insane enough)  I have a 2 meter rig

on the LEFT channel, scanner on the RIGHT channel, and the HF rig

panned dead center.  The ears and the brain manage to sort it all out.

I feel sorry for any of the neighbors who happen to tune in when in

this configuration (when I am in my active monitoring mode).

No Code, No License, No Kidding It's LEGAL

Now speaking of neighbors, lets focus for a moment on the LEGALITY of

using this kit under part 15 of the FCC rules.  In 1989, the FCC

revised part 15, changing the way the measurements are taken to

determine if a device is legal.   The new standard is: 250 microvolts

per meter.  A calibrated Field Strength Meter is needed to make sure

the signal complies with this regulation (FCC rule 15.239).  Beyond

this, part 15 requires that the unit produce no interference to

licensed stations.  Basically those are the rules.  What you put on

the device is YOUR business.  It comes under the same type of

regulation as cordless phones, baby monitors, and walkie talkies.  the

only difference is that this unit operates in a BROADCAST band rather

than a semi-hidden part of the spectrum like the others.

The Ramsey manual has a chart that shows that even if one complies

FULLY with this measurement, the transmitter has a surprising range.

Doing the math of the inverse square law, we find that there is still

.41 microvolts at 5000 feet from the transmitter, nearly a mile.

Since the Ramsey kit has a FINAL AMPLIFIER, it is much more powerful

than a similar kit sold by another manufacturer which uses the chip

output only (that one can be heard well within only 20 feet).  I found

that the kit with a dipole antenna cut to the operating frequency has

TOO MUCH range for my own use.  Those who wish to "Play DJ" might be

interested in running the recommended dipole antenna (making CERTAIN

they make the Field Strength Measurement) to get the maximum range out

of the unit.  I found that just a small piece of wire gives me all the

coverage in my home that I need.

For someone interested in providing a broadcast type service, to a

college dorm, appartment complex or local neighborhood (Legally the

signal can be quite loud over a quarter mile away) It can be legally

done under part 15 of the present FCC rules (as ammended in June of

1989) by simply adding a mixer, microphone, and sound sources to the

Ramsey Kit.

An application I am considering is connecting the audio output of my

new satellite system and tuning it to the BBC audio feed and just

leave it run that way when I am not using the dish for other purposes.

This way I can enjoy BBC in FM quality and so can my immediate

neighbors.  Now before the flames begin, please remember that

copyrights, etc, do not apply to part 15 transmissions.  You can put

on ANYTHING you want.  Just as if you were listening on closed circuit

speakers.  The only difference is that your immediate neighbors CAN

also enjoy the transmissions as well.

FCC RULE 15.215(a) Says:  "Unless otherwise stated, there are no

restrictions as to the types of operations permitted under these

sections."  This general provision *APPEARS* to leave you free to use

the fm transmitter for just about ANY type of operation you desire,

including becoming a "legal low power broadcaster".

Now the nitty Gritty:

RAMSEY FM-10 FM STEREO TRANSMITTER KIT

Price:  Circuit Board and Components -- $29.95

Cabinet (black plastic)      -- $12.95

Shipping add 6%.

Ramsey Electronics

793 Canning Parkway

Victor, NY 14564

(716) 924-4560 (Voice)

(716) 924-4555   (FAX)

o  Operates from internal 9 volt battery

o  Choice of onboard whip or external antenna

o  Stable output, from 88 to 108 MHz

o  Left and Right channel RCA line audio input jacks

o  Use with Mixers, cassette or CD decks etc.

o  Clear, step-by-step assembly instructions

o  Helpful information on FCC rules included

TYPICAL USES:

o  Extension of home stereo system without wires

o  Student-operated school radio station

o  Home or neighborhood radio station

o  College dorm favorite music broadcast service

o  Listening aid for auditoriums, churches

NOTE1:  I am in NO WAY affiliated with Ramsey Electronics other than

being one of their very satisfied customers.

NOTE2: If you plan to use the kit as a "broadcasting" service, I would

STRONGLY SUGGEST you have the output level CERTIFIED by an

engineer assertaining for SURE it is no more than 250

microvolts per meter.  This kit has an almost amazing range,

and I imagine it could very easilly exceed LEGAL SPECIFICATIONS

if you are not careful.

=============== WB8EOH = The Eccentric Old Hippie = WB8EOH ================

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X