Show-Vectors, Version 1.00, Copyright (c) 1992 SoftCircuits

Redistributed by permission

Title: SHOWVECT.DOC

Overview

========

This package provides a simple program to display the vectors in

the interrupt table on IBM-compatible computers. The assembly

language source code is also included.

Description

===========

IBM-compatible computers use interrupts. An interrupt acts like a

call. It causes the CPU's thread of execution to jump to another

location and then return to where it left off when the interrupt

is finished.

An interrupt can be invoked using the INT instruction. Interrupts

are also triggered by the hardware. In fact, by saving the

current state of the CPU and then restoring it when the interrupt

returns, hardware interrupts are able to occur without the

currently executing program even being aware it was interrupted!

In fact, there are many such interrupts in most PC's including

the system timer interrupt which occurs about 18 times a second.

When an interrupt occurs, the CPU saves the current address and

the flags register, and then jumps to the handler routine for

that interrupt. It determines the address, or vector, of the

handler by a table that starts in low memory (0000:0000). The

vector for interrupt 0 is at this address. Since each vector

requires a segment (2 bytes) and an offset (2 bytes), the vector

for interrupt 1 is at address 0000:0004. So in response to an

interrupt, the CPU calculates the vector address by multiplying

the interrupt number by 4. It then reads the vector from that

address and finally jumps to where the vector pointed.

Show-Vectors displays the vectors for each of the 256 interrupts

on IBM-compatible computers. It also gives a description of what

each interrupt is normally used for. Although it's a very simple

program, it can be useful for troubleshooting, and getting low

level information about your machine.

Distribution Information

========================

This program was created and released by:

SoftCircuits Programming

P.O. Box 16262

Irvine, CA 92713

It is provided "as is" with no warranty of any kind, expressed or

otherwise. SoftCircuits assumes no responsibility as to the

performance or usability of this program.

You may use and distribute this program freely on the condition

that the complete package is distributed without modification and

that no fee is charged for such distribution with the exception

of reasonable shipping and media charges.

This package contains the following files:

SHOWVECT.COM   Program executable file

SHOWVECT.ASM   Program assembly language source code

SHOWVECT.DOC   Program documentation (the file you're

reading)