"BEAM" - QUICK, SIMPLE BEAM ANALYSIS: EVERYTHING IS BUILT-IN !   12/93

BEAM.EXE = Executable Quick Basic Program that analyses 10 different

load conditions, 6 different materials (Pine, Oak, Steel,

Stainless, Aluminum, Brass, Concrete, and User-Defined).

Predefined cross sectional shapes of Angle, I-Beam, Pipe,

Rectangular Tube, Solid and T-Beam (same as Angle).

Sample Problem 1: 150 Lb man standing on 8 foot long 2x4.

---------------------------------------------------------

RUN: Shape ?       B  (A 2x4 is a 'bar' of wood)

Span ?       96  (Inches, Ends of 2x4 nailed to boards)

Height ?     3.5

Width ?      1.5

Material ?    6  (Pine)

Condition ?   1  (Center load, ends basically free to rotate)

Load ?       150

Add Inertia ? 0  (No added strength from floor or other boards)

Stress     = 1170 Psi    Stress Limit = 1400 Psi  ; Beam will

Deflection = .30 Inch    Defl. Limit  = .53 Inch    not break...

------------------------------------------------------------------------------

Sample Problem 2: 16 1/2 Foot Garage Door Beam

----------------------------------------------

Info: House is 28 feet wide. 24 Inch Snow = 2.4 Inch Water

Water Weighs 5 Lb/Inch Depth

Total Load on Roof: Truss Weight ......... 2.2 Lb/Ft2

Insulation (12 Inches) 1.2

|--- 14'--|      Ceiling .............. 1.5

/\               1/2" Roof Sheeting ... 1.1

/    \             Shingles, 2 Layers ... 5.2

/        \           Snow (24 Inches deep) 12.0

/            x         --------------------------

|            | Garage        Roof Pressure = 23.2 Ib/Ft2

|____________| Beam          (23.2/144 = .16 Psi)

RUN:   Shape ?      B  (bar)

---    Name ?       2 x 12 - Three of Them

Span ?       198  (inches)

Material ?   6  (pine)

Height ?     11.5  (actual height of 12" beam)

Width ?      4.5   (Three 2 x 12's, 1.5" wide each)

Inertia ?    0     (no help from roof sheeting)

Condition ?  3     (evenly distributed load)

Pressure ?   .16   (Psi)

Load Width ? 168   (14 feet wide roof x 12 In/Ft)

(width of the load)

Answer:  Stress = 1333 Psi      Stress Limit = 1400 Psi ... Good !

Deflection = .5 Inch   Defl Limit = (1/180 x L)

or 1.10 Inch

-----------------------------------------------------------------------------

Sample Problem 3: Beam under Floor of 3' x 8' Steel Water Tank

---------------------------------------------------------------

Hint: Angle iron welded underneath the tank in the "L" position

----  effectively gives a channel section and is very strong...

||

||

Closeup of Floor ---> ==========================================

||

____________                      ||

/|          /                      ||_______

/  |_______ / | 5' Deep               -------- Angle

/___________/   |                             ( L2x2-1/4 )

|           |  /

|___________|/  3' Wide

L

<---8'------>

3            2

If the Tank is filled it will have 5 Ft x 62.4 Lb/Ft or 312 Lb/Ft

of pressure on the floor. 312/144 In2 = 2.166 Psi Floor Pressure.

The beam could be expected to support 1/2 the total floor pressure:

|   wt beam supports     | outer wall supports 1/2 of floor

|      <--------->       |

Floor: ==========================

<---4'---> |_ <---4'--->

RUN:  Shape ?     C     (angle welded to flat plate = channel)

Span ?     36     (width of tank in Inches)

Height ?    2.25  (Ht of angle plus 1/4 floor plate)

Width ?     2     (width of angle iron)

Thick ?    .25

Material ?  1     (Steel)

Condition ? 3     (Uniformly loaded with free ends)

Pressure ?  2.166 (Psi on the tank floor)

Press Width ? 48    (width of supported area)

Stress = 17000 Psi    Stress Limit     = 18000 Psi... Good design!

Defl   = .068 Inch    Deflection Limit = .20 Inches