�����������������������������������������������������������Ŀ

�     BNU Rev 5 FOSSIL Compatible Communications Driver     �

�                        Release 2.02                       �

�������������������������������������������������������������

15th October, 1992

Copyright (C) 1992

David Nugent and Unique Computing Pty Limited

Melbourne, Australia.

FidoNet  3:632/348.0

AlterNet  7:833/387.0

FreeNet  23:2/3.0

BNU is 'freely'  available copyrighted software, and supplied

no charge to system operators  of  Fidonet(tm) and 'Othernet'

public access amateur network systems.  It is  NOT  available

in this form  for  commercial  use  or use outside of amateur

EMail systems.

Please read the License details  at  the end of this document

on contacting the author regarding commercial  application of

BNU and associated products.

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

BNU is a  FOSSIL  compatible  Communications driver, suitable

for use with the following (among  others) bulletin board and

FidoNet(tm) mailer software:

Fido v11,12  Tom Jennings, Fido Software

BinkleyTerm  Vince Perriello, Bob Hartman, Alan Applegate,

& Bit Bucket Software

Opus-CBCS    Wynn Wagner III, The POLE of Dallas

QuickBBS     Adam Hudson

TPBoard      John Schnieder & Rick Peterson

SEAdog       Thom  Henderson, System Enhancement Associates

D'Bridge     Chris Irwin

FrontDoor    Joaquim Homrighausen, Interzone Software, Inc.

It handles hardware  specific  aspects  of communications, to

enable the above applications to run in multiple environments

which are supported by a FOSSIL  compatible  driver.   BNU is

designed for the IBM PC, XT, AT, AT/386 and PS/2  range under

the MS-DOS operating  system  version 2.10 and above, OS/2 in

'DOS compatibility' mode and under  PC-MOS/386.   It may also

be compatible with other environments which emulate MS-DOS.

The term 'FOSSIL'  is  short  for "Fido Opus SEAdog  Standard

Interface Layer".  It  is  a  generic hardware interface used

mainly by bulletin board systems  and Fidonet compatible mail

software which evolved from the efforts of Wynn  Wagner,  Bob

Hartman and Vince   Perriello.    The  current  "Revision  5"

standard makes it possible for  a lot of software to run on a

wide variety of hardware platforms capable of  running MS-DOS

but with distinct and incompatible hardware.

BNU's main task  is  communications  handling.  It is a fully

interrupt driven communications   driver   which  effectively

handles communications "in  background",  and   continues  to

receive and transmit   characters   asynchronously  with  any

program or application running.   The application itself need

not be concerned   about   the  time  critical   aspects   of

communications, nor the capabilities, limitations or features

of the hardware itself.  These are all handled by BNU itself.

BNU is one  of many FOSSIL drivers available.  BNU implements

many features found in other(tm) FOSSIL drivers, sometimes in

a slightly different  way.    Known   differences   will   be

highlighted.

This package contains BNU.COM, with the driver itself being a

TSR program (Terminate and Stay Resident) which may be loaded

and unloaded, activated  or  made  dormant "on-the-fly";  and

also BNU.SYS, a  device  driver  implementation  loaded  from

CONFIG.SYS.  This is not as 'flexible'  as using BNU.COM, but

in general conserves some memory and allows loading  in  some

environments where the TSR implementation cannot be used.

- 1 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

Some of BNU's unique and more interesting features are:

o    BNU  has  been  built  to  handle up to 4 COM ports

concurrently in up to 16 different configurations,

o    It has an "open architecture"  which  will allow is

adaptation later  in  its  development  to  support

multiport shared    interrupt    and    intelligent

asynchronous IO boards,

o    It supports the 16550 USART in fully buffered mode,

providing the capability of much higher baud rates,

performance and  significantly  less  CPU  overhead

especially in multitasking environments,

o    The driver is extremely FAST, optimised  for speed,

and takes up very little memory,

o    Incorporates  an optional "diagnostic" mode, so you

(the user) can inspect  the  state  of  the  FOSSIL

driver itself;    assisting   in   debugging    and

optimisation of   software  being  written  to  use

FOSSIL calls,  and   in   attempting  to  determine

problems in  software  using a FOSSIL,  such  as  a

mailer, BBS or BBS 'door',

o    Has  'hooks'  for  an installable driver to support

network and  multitasking  software,  forcing  ill-

behaved processor intensive programs  to  give more

of their  time-slice to the system when inactive or

when simply "polling"  communications  or  keyboard

functions for activity or status reports,

o    Allows 'fast' routing of screen output directly via

the ANSI  driver,  making  "FOSSIL"  screen  output

calls significantly   faster  than  going  via  DOS

itself,

o    Allows for problems  in  some  XT  &  AT BIOS' when

16550AN's are installed:  very often  a  BIOS  will

not 'see'  the  communications  chip during startup

diagnostics -  so   BNU  will  force  the  BIOS  to

recognise it automatically,

o    Provides a full range of utility functions  in  the

same .COM  file,  including built-in carrier watch,

system reboot, lock baud rate and/or communications

parameters and allowing  some  PC  BIOS  compatible

non-FOSSIL aware  software to take advantage  of  a

interrupt driven communications,

o    Allows   resizing  of  communications  buffers  and

locking/unlocking   of   baud   rates   and   other

communications parameters "on-the-fly",

o    Provides a means of displaying the currently loaded

driver's "status" and active parameters.

- 2 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

BNU.COM is both a FOSSIL driver (TSR) and a utility which can

manipulate the resident   version   of   itself.    On  first

successful loading, it will terminate  and  leave  the driver

portion of itself   (only   about  5K  or  so   plus   buffer

requirements) resident, and  allocate  communication  buffers

for each of the enabled ports (typically  using 14K for a one

port install when  'diagnostics' mode is enabled).   It  then

captures and utilises  the  system's  INT 14H (communications

services) vector and services  requests  for  all  FOSSIL and

non-FOSSIL aware programs using the INT 14H interface.

BNU will NEVER  load  itself  more than once,  unless  within

different tasks when    using    a   multitasker,   such   as

PC-MOS/386(tm), DESQview(tm), DoubleDOS(tm) or VM/386(tm).

This documentation assumes a working knowledge of MS-DOS, and

no effort is made to explain MS-DOS  functions,  commands  or

setup.  Some familiarity  with  BBS or mail capable  software

and other FOSSIL drivers will definitely be an advantage, but

not required.

USING BNU

When running BNU,   avoid  using  any  other  software  which

purports to enhance   communications    products   (such   as

16550.EXE).  These WILL NOT work, and could cause your system

to crash or work unreliably.  BNU already uses  the 16550 and

16550AN to full advantage.

Avoid loading DOS's   MODE   command   with   BNU.    BNU  is

incompatible with it's communications  handling  -  if loaded

after BNU, then  both may fail to operate reliably.   Printer

redirection is usually  ok,  provided  MODE  is loaded before

BNU.

If you need to set a COM port's  baud rate (which is why MODE

is most often  used),  then you can get BNU  to  do  that  by

"locking" the baud  rate  (using  BNU /L as documented below)

which sets the COM port immediately  to  that rate.  Remember

to "unlock" the  baud  rate  if  not  using a constant  speed

between computer and modem.

BNU.COM would normally  be  loaded  in your AUTOEXEC.BAT, and

installed as part of your normal  system  startup.  A typical

command line might be:

BNU /F

[Loads BNU.COM with default settings and fast ANSI]

BNU will respond  with  a  short  copyright   notice  and  an

indication that it  has been successfully loaded.  The number

of ports installed for will also be shown.

- 3 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

BNU.SYS must be  loaded  from CONFIG.SYS (and probably can be

using QuarterDeck's DEVICE.COM under DESQview but I have only

run it this way for short periods)  using  a  "device"  line,

such as:

device = C:\BNU.SYS /P2 /L0:19200,8N1 /F

[Loads BNU.SYS  for  2  ports, port 0 (COM1)  locked  at

19200 baud  8  data  bits 1 stop bit normal parity, fast

ANSI]

A similar copyright message is  displayed  as  an  indication

that the driver has been installed.

If BNU encounters  an error in the command line,  the  driver

will NOT be  installed.   The  device driver version displays

the command line parameter in error.

BNU has been successfully run  loaded  "high"  above the 640K

mark using both   QuarterDeck's   QEMM(tm)   and    Qualitas'

386^MAX(tm) 386 memory  management  products.   It  should be

noted that there may be performance  differences  as a result

of either 386i  virtual  mode  switching  or  differences  in

hardware RAM access  speeds between mother-board RAM and that

used on an extended RAM card.  You should test the results on

your own hardware before deciding  which  method  is  better,

since generalisations without  considering  all  the  factors

involved can be misleading.

Other enhanced memory  products may or may not work, but I am

either not familiar with them  or  do  not  know enough about

them to comment.

Since BNU.COM can  be  unloaded  at  any  time   (unless  BNU

considers it unsafe to do so for various reasons), you should

be aware that  doing  so sometimes creates "holes" in memory.

This is not  a  problem  so   far   as   MS-DOS   or  various

applications running in it are concerned, but  the  effect of

releasing more memory  for  DOS  to use may be lost.  This is

almost always the  case when unloaded  from  within  a  batch

file: the memory freed will not be available until  after the

batch file terminates.

If the desired  effect  is to "disable" BNU completely, refer

to the /Q and /C command line  switches  below.   This leaves

BNU loaded, but  provides  a  means of disabling  the  driver

completely and forces  use of the BIOS INT 14H functions or a

previously installed driver.

One note regarding PS/2 hardware  and  applications which use

its specific INT  14H  functions  (though I haven't  actually

seen any).  A PS/2's extended BIOS contains a superset of INT

14H services, which  were designed after FOSSILs first become

available for PC's.    The  additional   services   are   NOT

compatible with the extended functions supported  by  FOSSIL,

and you should  therefore  either  unload  (/U) or deactivate

(/Q) BNU prior running such software.

- 4 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

DEFAULT SETTINGS

By default, BNU  uses  1K  (1024 byte) communications buffers

for both transmit and receive.  This should be sufficient, if

not excessive, for most applications.   This  may  need to be

varied for systems  running  at slow (less than  6MHZ)  clock

speeds, use modems  with  higher  baud  rates,  make  use  of

special modem technology, run under  multitasking software or

run on a  network.   Some applications (for example  terminal

emulation systems and bulletin boards) require more buffering

because of the nature of how they communicate.

The fast 'ANSI' output is DISabled by default.  The option to

use the faster  output  method  will normally work well under

standard MS-DOS, DESQview  or  most  DOS-based  environments.

This will NOT  work under PC-MOS/386 as it makes  use  of  an

undocumented MS-DOS feature  not  supported by that operating

system.

After installation (only the first  successful  invocation of

BNU.COM actually loads the TSR portion), the FOSSIL  driver's

extended interrupt driven  functions  are  inactive, and only

the basic BIOS   services  are   provided   (by   BNU).    An

application must specifically   "initialise"   a   port   for

communications to make it become active and use the interrupt

driven functions.

When BNU is  installed,  it  does a brief and simple hardware

diagnostic to detect what communications ports are valid.  If

errors are found  (most  PC BIOS'  miss  detection  of  16550

USARTS, cannot detect  non-standard  communications  hardware

and do not  look  for  anything  above COM2), the appropriate

BIOS tables and hardware bits  are  patched.  This means that

if you install and use non-standard port and IRQ settings for

a communications card,  and  tell  BNU about these  (via  the

BNUPORT utility) your  BIOS  will be changed to reflect this,

allowing other BIOS-compatible    programs    to    use   the

communications port (COM1 through COM4 only since the PC BIOS

can only support up to 4 ports).

COMMAND LINE OPTIONS

Following is a  list  of  command line options  available  in

either or both   BNU.COM  and  BNU.SYS.   These  allow  "fine

tuning", setup and utility functions.

Command line switches can be given  with a preceding dash (-)

or forward slash   (/).   Some  parameters   show   a   colon

separating the switch itself and a numeric parameter: this is

optional, and is  included  only  for readability.  An equals

(=) or comma (,) are NOT optional  and must be included since

these separate different parts of a command line switch.

- 5 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

There are basically  two  "groups"  of command line switches.

The first are  configuration   switches,   which  affect  the

tuning, performance   and  behaviour  of  the  communications

driver.  These can  generally be combined on the same command

line (and invocation) without problems.   Exceptions (if any)

are noted where relevant.

The second group are "utility" functions.  These switches are

normally used EXCLUSIVELY of all other switches.  That is, if

you use them, they must appear SEPARATELY on a  command line.

The switches in this group are:

/C        Recapture INT 14H vector

/B        Reboot system unconditionally

/S        Display driver status

/H        Display help for command line switches

/U        Unload driver from memory

/W        Enable/disable carrier watch

Remember that you  can  invoke BNU.COM multiple times with no

risk in loading the communications  driver  portion more than

once in any  logical  "task".   All  utilities   relating  to

BNU.COM are contained within the same .COM file.

The device driver  version  works  a little differently.  Its

command line (specified  in  CONFIG.SYS)  must  contain  only

configuration items, since the utilities are  not  supported.

These are contained   within  a  separate  executable  called

BNUHELP.COM, and have a similar  -  if not identical - syntax

to the utility   functions   in   the   .COM  version.    Any

differences are noted below.

~~~

/T        Set Transmit buffer size

Available in .SYS and .COM versions

eg.  /T:512

Sets 512 byte transmit buffers

The default  transmit  buffer size is 1024 bytes.  Since

file transfers are "transmitter  driven" so far as speed

is concerned, this should be large enough to service the

interrupt driven transmitter without it "waiting"  to be

filled by the application.  These should be increased in

size where  multitaskers,  networks or faster baud rates

are being used.

- 6 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

/R        Set Receive buffer size

Available in .SYS and .COM versions

eg.  /R:2048

Sets RX buffer size of 2048 (2K) bytes

The default  RX  (receive)  buffer  size  is 1K, or 1024

bytes.  In practice, only  a portion of this is actually

used on a constant basis, but this will be  dependant on

how fast an application can service the buffer (and keep

it empty).

Buffer sizes  CAN  be  changed  "on-the-fly".  But at no

time can  both  the  receive  and  transmit  buffers  be

increased in size beyond the combined amount  when first

installed.  For  example,  if  BNU was installed with 2K

transmit and 4K receive buffers, only 6K is available to

each port in total.  Combined  buffer size cannot exceed

that 6K.  An attempt to exceed this amount  will  result

in an error.

Changing buffer  sizes  whilst  communications tasks are

active will be disallowed.

When modifying the buffer  sizes,  it  is  advisable  to

first specify  the  one  which is to "shrink"  in  size,

before the  one  which  will "grow".  For example, if 1K

buffers were active (with  a total of 2K available), and

you wished  to  change  these  to .5K receive  and  1.5K

transmit:

BNU /T1536 /R512

would result in an error (Maximum buffer size exceeded).

The result  could  be  achieved by changing the order of

the command line arguments to:

BNU /R512 /T1536

/P        Enable concurrent ports

Available in .SYS and .COM versions

eg.  /P:2

Enables two ports for use concurrently

This sets the number of ports which can be active at the

same time.  Up to 4 ports  can  be used (in this current

release), in up to 16 different configurations.

By default, BNU enables one port for 'interrupt  driven'

use.  This  is the most memory conservative approach and

fits the requirements of the majority of installations.

The current  version  does   not   support  shared  port

arrangements or  multi-port  'intelligent'   serial   IO

controllers.

- 7 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

This switch  does  not  determine  WHICH  ports  will be

valid, only HOW MANY can run  FOSSIL driven at one time.

The driver  will  support  whichever  port   (0-15)  the

application selects.

/L        Lock ports baud rate and/or settings

Available in .SYS and .COM versions

eg.  /L:0=19200

Locks baud rate on port 0 (COM1) to 19200

/L:1=38400,8N1

Locks port 1 (COM2) to 38400, and forces 8

data bits, no parity and 1 stop bit.

/L:0=0,7E1

Unlocks port 0 (COM1) if previously locked,

or leaves it 'floating' if not, then forces

7 data bits, even parity and 1 stop bit.

The baud  rate given must be valid for a standard FOSSIL

driver, and can be one of  0  (unlock)  300,  600, 1200,

2400, 4800,  9600,  19200 or 38400.  High  speed  modems

locked at a baud rate of 9600 or above also have CTS/RTS

hand-shaking forced since these use the modem's internal

buffering which make use of CTS/RTS flow control.

A baud  rate  specification  of  zero "unlocks" the baud

rate and resets handshaking  to  default  (whatever  the

application sets).

When using  this  switch,  your  modem  MUST  support  a

constant modem  to  computer  speed,  otherwise any baud

rate change requested by an application will be ignored,

resulting in an incorrect  setting  between  your system

and remote.  The result will normally be  garbled  input

and output.

When the  baud  rate  is locked, the port's baud rate is

set IMMEDIATELY.  The baud  rate  is  maintained  at the

locked rate  even  though  FOSSIL  is   not   active   -

applications making  use  of  BIOS  calls to change baud

rate will not succeed.   This  allows  non-FOSSIL  aware

applications to  be run without fear of  the  baud  rate

being changed   even  though  these  do  not  explicitly

support higher baud rates.

/F        Enables/disable 'fast' screen writes

Available in .SYS and .COM versions

eg.  /F+       Enables 'fast' screen writes

/F-       Disables 'fast' screen writes

- 8 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

This switch  is  used  for versions of DOS which support

faster character output  via  the  undocumented  INT 29H

method (used to call the ANSI driver directly).   To  my

knowledge, this  will  work  on  all  DOS  versions 2.00

through 4.01, but since it  is  "undocumented", there is

no guarantee that it will work.

This should NOT be enabled under PC-MOS/386  version 4.0

or below.

/O        Enable function 0x1c compatibility

Available in .SYS and .COM versions

eg.  /O

This switch  is used to enable "compatible" operation of

BNU in situations where either  X00  or Opus!Com worked,

but BNU  doesn't.   Due  to  a misunderstanding  of  the

FOSSIL specification,  X00 and Opus!Com return 0x1c (28)

whenever the highest function  number is requested by an

application instead  of  0x1b  (27)  as   required   the

Revision 5 specification (FSC-0015).

Several applications  rely  on  the  'feature'  of these

drivers and require a 0x1c  return  to  verify  that the

functions used by the application are present.   At this

time, a  function  0x1c  is  not actually implemented by

X00.SYS, Opus!Com or BNU.

When given, this switch tells BNU to return 0x1c, and so

allows applications relying  on  0x1c  to  work with it.

Typically, such  programs will return with  "FOSSIL  not

present" or  "FOSSIL  version too old" (or words to that

effect) when BNU is loaded without using '/O'.

/Z        Sets 16550 FIFO size/trigger levels

Available in .SYS and .COM versions

eg.  BNU /Z0

Enables full 16550 FIFO buffering (16/14)

BNU /Z5

Disables 16550 FIFO buffers

This switch provides the ability to 'fine tune' a system

equipped with  a National  Semiconductor  16550AN  UART.

The UART  is  the  device used in PC's for  asynchronous

communications handling,  and  the 16550AN is a superior

form of these (as compared  to  an  8250  or  16450 with

which it is pin-compatible), since it provides  on-board

FIFO (first-in   first-out)   buffers   or   queues  for

receiving and transmitting.

- 9 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

Software must be built to specifically take advantage of

this UART's  features in order to gain any real benefit.

Normally, the  16550's  FIFO  buffers  are  disabled  on

power-up, and   remain  that  way  unless   specifically

enabled.  The software itself must service interrupts by

emptying and  filling  "queues"  rather  than the normal

single character receive and transmit holding registers.

Having this  ability  to   service   multiple  character

requests in  one  hardware  interrupt   call   makes  it

possible to   SIGNIFICANTLY   reduce  interrupt  service

overhead and thus CPU usage  on  any  machine.  This has

very obvious   advantages   when   operating   under   a

multitasker, on   a  network,  concurrently  with  other

heavily interrupt-driven devices or with modems or other

communications devices running at high baud rates.

The /Z switch takes a single  digit parameter - a number

between 0  and  5.   This  number affects  the  internal

settings of both the UART itself (its internal 'trigger'

levels, which  define  how  "full"  the  FIFO's  will be

allowed to get before triggering an interrupt to the CPU

for service) and how full BNU will let the transmit FIFO

be queue on each transmit 'request'.

Valid numeric parameters to the /Z switch are:

/Z0            16 bytes       14 bytes (max)

/Z1            14 bytes       14 bytes (max)

/Z2            08 bytes       08 bytes

/Z3            04 bytes       04 bytes

/Z4            01 byte        01 byte

/Z5                - FIFOs disabled -

Where 16550AN's  are  not   present,  BNU  automatically

ignores this switch completely.   The default  value  is

'/Z0', which activates the full 16550AN capability.

/M        Enable/disable initialisation message

Available in .SYS and .COM versions

eg.  /M+  Enable init message

/M-  Disables init message

Normally the   driver   outputs   a   message   when  an

applications initialises  the  driver for its use.  This

may be suppressed or reenabled by using this switch.

/U        Unload driver from memory

Available in .COM version only

eg.  BNU /U

Attempts unloading resident driver from memory

- 10 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

This switch  tells BNU.COM to unload itself from memory.

This forces all ports to  be  deactivated  (if  any  are

active).  This  will fail if programs loaded  after  BNU

have "captured"  interrupt vectors which are used by BNU

(and which BNU cannot restore  as  a  result).   In this

case, BNU will not load, but will indicate  this  in  an

error message.    You  will  have to either unload these

later programs (such as FOSSIL  appendages),  or  reboot

your system.

Since BNU.SYS is a device driver, it cannot  be disabled

or unloaded from memory.

/Q        Disable communications driver interface

Available in .SYS and .COM versions

eg.  BNU /Q

Disables resident driver

This switch  is normally given when BNU installs itself,

and the effect is to "delay"  capturing  of  the INT 14H

vector until  a  '/C' command is issued.   This  may  be

useful if  loading  with  a  multitasker and using other

incompatible communications drivers in other tasks.

If issued while BNU is already  loaded,  the  previously

active INT   14H  communications  services   vector   is

restored.  BNU's    extended    FOSSIL    services   are

effectively "dropped" out of the chain.

/B        System reboot

eg.  BNU /B0

Select default reboot method

BNU /B

Reboot (warm)

This switch has two uses.   When  a digit is given after

the 'B', the reboot method is selected.  Only two values

are valid 0 or 1.

Method 0 selects a standard BIOS method  of  jumping  to

the system's  reset  vector.  This should work with most

DOS environments on a wide  range of hardware, including

PC/XT/AT and 386 compatibles.

Method 1 makes use of the AT's keyboard  reset  line  to

cause a  system  reboot.   It  will  work only on AT and

386AT systems.  However,  this  method  will  work under

some 386 specific software where the first  method  does

not.  (for  example,  PC-MOS/386).   In short, it "plays

rough".  Some 386 machines  and almost all PC/XT systems

cannot and should not use this method.

- 11 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

If no  digit  is  given  after  the 'B', BNU proceeds to

reboot the machine - unconditionally.

/H or /?  Display help information

Available in .COM version only

This switch  displays  information  about  command  line

switches and  usage  for  those who don't  believe  that

reading manuals is a very useful exercise.  Pity they'll

probably never discover or know about this switch unless

told....

This switch  much  be  used  alone,  and  will  not load

BNU.COM into memory.  The  help  screen  displayed  will

look like:

BNU Revision 5 FOSSIL Compatible Asynchronous Communications Driver v2.02

Copyright (C)1992, David Nugent & Unique Computing Pty Ltd FidoNet 3:632/348

�  Usage: BNU [options]     � accepts / or - as switch character

/U          � Uninstall (if already resident)

/T=nnnn     � Set TX buffer size to <nnnn> (decimal)

/R=nnnn     � Set RX buffer size to <nnnn> (decimal)

/P=nn       � Enable nn concurrent ports (1-16 decimal)

/D[=n+|-]   � Enable or install diagnostics panel

/N          � Disable diagnostics panel (may be reenabled)

/A=nn       � Set screen attribute for diagnostics window

/Lp=n[,lps] � Lock port <p> (0=COM1) baud rate to <n> (0=Unlock)

Optional: l=Character length, p=Parity, s=Stop Bits

/W=n+|-     � Enable(+)/disable(-) carrier watch on port <n> (0=COM1)

/Q          � Prevent INT 14H capture (install) or release INT 14H

/M+|-       � Enable(+)/disable(-) FOSSIL "init" message

/I=n+|-     � Initialise(+)/deinitialise(-) FOSSIL on port <n>

/F+|-       � Enable/dssable faster screen writes via ANSI driver

/B[0|1]     � Select reboot method, or perform system reboot

/Z[0-5]     � Sets 16550 FIFOs [0=Max(16) 4=Min(1) 5=Disabled]

/O          � Enable function 0x1c "compatibility"

/C          � Recapture INT 14H vector

/S          � Display driver status

/H or /?    � Display this help screen

/C        Recapture INT 14H vector

Available in .COM version only

eg.  BNU /C

Recapture INT 14H

(must not be used with any other switches!)

This forces BNU to takes a look at the current  location

of INT  14H  and  makes  sure  that  it  points into the

resident portion of BNU.   If not, it resets the INT 14H

address to the correct location.

This is most often necessary when using a  multitasker -

such as  DESQview(tm) - which revectors the interrupt on

- 12 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

loading.  Since  one  standard  method  of detecting the

presence of a FOSSIL driver is to examine memory shortly

after the location pointed  to  by the INT 14H interrupt

vector, the  'stealing'  of the interrupt  in  this  way

causes many  applications  to  fail to locate the FOSSIL

driver.  Applications which  use  the  'init  signature'

method of detecting a FOSSIL driver will not normally be

affected by this behaviour, but there  is  a  risk  that

whatever 'stole'  the vector will unintentionally change

CPU registers used by the  application  for  the  FOSSIL

call, causing any number of complications  (and probably

a system crash!).

Another use  of  this switch might be to enable toggling

between two or more communications  drivers  which  make

use  of  INT  14H  (such  as packet radio communications

drivers).  '/Q'  can  be  used  to  switch BNU "off" and

restore a previous vector, and '/C' used to recapture it

again for FOSSIL use.  The  following  sequence  is  the

suggested method of doing this:

BNU /Q [other switches]

(Install but don't vector INT 14H)

<install other communications driver>

BNU /C

(Capture INT 14H saving previous vector)

To use the secondary driver (which could even be another

FOSSIL driver,  but cannot be BNU), use  "BNU  /Q",  and

"BNU /C" to use BNU again.

NOTE: This  switch  MUST NOT be combined with any others

(since they will be ignored)!

/W        Enable/disable carrier watchdog

eg.  BNU /W0+

Enables carrier watchdog on port 0 (COM1)

BNU /W1-

Disables carrier watch on port 1 (COM2)

This switch  enables or disables  carrier  watch  for  a

particular communications port.  If carrier detect drops

(the caller  hangs  up) while the watchdog  function  is

active, the system will be rebooted.

This feature  is most often used with programs which are

not "carrier smart" and do  not monitor carrier detect -

some of these were probably never intended  to  run with

output directed  to  a communications port.  If a caller

hangs up while such a program  requests input, it may be

effectively "stuck", unable to continue until  its input

is satisfied.    It is possible that this situation also

presents a security risk.

- 13 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

The carrier  detect watchdog is not an elegant nor ideal

solution to this problem,  and  especially not useful in

multitasking or networked computing environments,  where

rebooting of   a   whole   machine   could  upset  other

concurrently running tasks  and workstations.  There are

some very clever "watchdog" programs which are much more

suitable to do this, such as Angelo Besani's(++) WABIRD,

which gracefully terminates the program and restores the

system to the state it was in when originally invoked.

Ommission of  the  '+'  or '-' is considered  an  error.

This switch MUST be used exclusive of all others.

(++) Angelo Basani is contactable at:

Amnesia CBCS, Varese - Italy

Fidonet 2:331/101.0, +39-331-263425

/S        Display BNU status

Available in .COM version only

eg.  BNU /S

Displays BNU's current status

The resulting  display  will depend on the current state

of the resident version of  the driver in memory, but if

loaded will normally look similar to:

Driver Version: 2.02    Installed

Ports Available: 1

Ports Active: 0

FOSSIL Revision: 5

TX Buffer Size: 2048

RX Buffer Size: 2048

Max Buffer Size: 4096

Diags Module: Inactive

Init Message: Enabled

ANSI Output: Fast

Port Locked: 1 @ 19200

If the driver is not yet installed, only  the first line

will be  shown,  with  the  text  "Not  Installed".  The

diagnostics module (explained below) is one of "Active",

"Inactive" (present but disabled)  or "Not loaded".  Any

ports whose  baud  rate  has  been "locked"  are  listed

sequentially.

Note that  port  numbers are ZERO-BASED.  Port 0 is what

DOS refers to COM port 1  (ie.  the device COM1), port 1

is COM2  etc.    DOS does not define devices  for  ports

above port 3, though BNU can utilise them.

- 14 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

/D        Enable/disable or install diagnostics module

eg.  BNU /D2+

On BNU install: Installs the diagnostics panel with

a default 'refresh' rate of 2, and

enabled (the '+' here is optional)

Otherwise:      Provided the diagnostics module has

been installed, enables it and sets

the 'refresh' rate to 2

BNU /D-

Disables diagnostics screen updating (but does not

deactivate them)

This is   an   advanced   option,  useful  for  software

development using FOSSIL  calls  as  a  means of viewing

what the current state of the FOSSIL driver  is in 'real

time'.  When  enabled  and  a  port is active for FOSSIL

extended calls, a panel is  continually refreshed on the

screen during each timer tick which displays  a  summary

of various FOSSIL driver states.

Information for   up   to   4  ports  can  be  displayed

concurrently on the screen (thought this might get a bit

messy!), overlaying anything currently displayed.

By default, the "refresh rate" of the panel(s) is set to

0, which means it is refreshed EVERY timer tick, or at a

rate of about 18.2 times per second.  Refresh rates of 1

means every second tick, 2  every third, 3 every fourth,

and so  on.   Since video memory is slower  than  normal

RAM, slowing  down  the  refresh  rate will decrease the

overall detrimental  effect   that  diagnostics  has  on

system performance.

It should  be  noted  that  running  diagnostics   quite

significantly impedes  system  performance  and  adds  a

great deal  of  CPU  overhead   in   servicing  what  is

effectively a "background" task.  You can therefore only

run diagnostics  on a system equipped with  a  fast  CPU

(286 or  above), running at lower baud rates and/or with

a 16550AN UART.  Some general 'minimal' guide-lines are:

� 2400 baud or below

� 6-8MHZ 286 class

� 16450 UART or better

� 9600 baud

� 10-16MHZ 286 class  -or-   � 20MHZ 386 class

� 16550 UART                  � 16450 UART

� 19200 baud

� 16-20MHZ 286/386 class

� 16550AN UART

It takes quite a bit of  processing  power  to  drive  a

'background' routine  to  refresh video  RAM  (which  is

relatively slow)    without    impacting    on    system

performance.

- 15 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

If you  are  having  any  difficulties  at all using the

driver and have enabled the  diagnostics  panel, disable

them immediately and try again.  I have found this to be

a major cause of problems in the past.  This  feature is

officially 'not  supported'  under any conditions, and I

certainly don't need to hear that it "does" or "doesn't"

work on your system.  Before  ANY problem report is made

for any reason whatsoever, please try whatever  you  are

trying to achieve WITHOUT DIAGNOSTICS LOADED!

Usual symptoms  of  the diagnostics panel impacting in a

detrimental way on a system's ability to communicate are

failed handshaking   between   Fidonet   mailers,   poor

resilience in file transfers when "task" switching under

multitasking software, some spurious character loss from

remote during terminal sessions, failure to initialise a

modem and  inability  to detect incoming  calls  from  a

modem returning    verbose   response   codes   (amongst

others...).

When loaded (which must be  done  via the /D switch when

BNU is initially installed), you can selectively  enable

or disable  the diagnostics display from a keyboard "hot

key".  The window is in an  initial  "on" state, but can

be toggled  on or off by pressing LeftShift-Ctrl-5  (the

'5' located  on the numeric keypad).  The 'refresh rate'

can also  be adjusted on-the-fly  using  LeftShift-Ctrl-

Grey+ and LeftShift-Ctrl-Grey-.  The panel itself can be

shifted on the screen by holding down the LeftShift-Ctrl

combination and pressing Up, Down, Left and  Right arrow

keys.  Note  that  the background screen is NOT restored

(since it is never saved).

In summary:

Left-Shift     Action

Numeric 5      Toggle panel "off" and "on"

Grey+/Grey-    Speed up/slow down panel refresh rate

Arrow keys     Move diagnostics panel on screen

Note that these 'hot-keys'  work, no matter if the panel

is visible  or  not, but not if disabled  using  the  /N

switch.

Toggling the  panel  "off"  using LS-5 does not entirely

defeat the CPU overhead required  to  service the panel.

In that case, the background task is still  active,  and

only the  panel  is  not displayed.  To entirely disable

the diagnostics  panel, either  unload  and  reload  BNU

without the  /D  switch, or use "BNU /N" to  effectively

turn it off.

The following  diagram shows a typical diagnostics panel

display, and explains the  fields  shown.    All numeric

values are hexadecimal (base 16).

- 16 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

��� Port 0001 � ������� Port number, zero based (0=COM1)

� TX Cnt 0001 � ������� Chrs currently in transmit buffer

� TX Max 003F � ������� Maximum chrs found in transmit

� TX Bse 141E � ]       buffer (this is cleared whenever

� TX Top 241D � ] �Ŀ   the port is initialised)

� TX Nxt 144A � ]   ��� Transmit buffer pointers/limits

� TX End 1449 � ]

� RX Cnt 0002 � ������� Chrs currently in receive buffer

� RX Max 0019 � ������� Maximum chrs found in receive

� RX Bse 241E � ]       buffered (cleared on init)

� RX Top 341D � ] ����� Receive buffer pointers/limits

� RX Nxt 249A � ]  ���� Current flow control state

� RX End 249A � ]  �    highbyte=remote, lowbyte=local

� FlwFlg 0080 � ������� Active flow control settings

� FlwCtl 0002 � �����   highbyte=remote, lowbyte=local

� Status 6238 � ������� Current line/modem status

� FifoSz 0010 � ������� 16550AN FIFO buffer (TX) size

� Errors 0000 � ������� Number of FOSSIL errors registered

� ErrFun 0000 � ������� Function (AX value) of error call

� LastAX 0300 � ������� Last Int 14H function call (AX)

� Int14H 5F73 � ������� Accumulated Int 14H calls

� ISRInt 0D4F � ������� Accumulated hardware interrupts

���BNU�vX.XX���

If you want to load diagnostics but have them completely

inactive by   default,  use  the  following  batch  file

sequence:

REM Load diagnostics module when installing BNU

BNU -D <other switches ...>

REM Immediately disable them

BNU -N

The option to load without  screen  refresh "/Dn-" still

uses clock cycles: only the screen update is inhibited.

The "/N"   switch   actually  disables  the   background

processing and   will   not  cause  any  degradation  in

performance.  "/D" can be  used  to  reenable the module

again.

/N        Disable diagnostics panel

eg.  BNU /N

This switch disables output of diagnostic information to

the screen, and also the background processing  required

to update it.

- 17 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

/A        Set diagnostics panel attribute

eg.  BNU /A=79

Causes the  diagnostics  panel  to  become  intense

white on red

This allows the physical screen attribute to be set.  By

default, a colour screen uses  dark blue on white, and a

monochrome screen reverse video.

/I        Init/deinit FOSSIL enhanced functions

eg.  BNU /I0+

Initialise port 0 (COM1) for FOSSIL processing

BNU /I1-

Deinitialise port 1 (COM2)

This provides the facility to force normally  non-FOSSIL

aware BIOS  compatible  programs to use the basic FOSSIL

character input/output functions.

This has  specific  advantages   when  running  under  a

multitasker, since  all  input/output is from  interrupt

serviced buffers,   and   not   directly   polling   the

communications port for activity.

Not all non-FOSSIL aware  programs  will  be  compatible

with use  of this option.  Only strictly  BIOS-compliant

programs will benefit.  Some program will have their own

interrupt driven  communications  functions,  and others

will poll the port directly.  Initialising the FOSSIL in

this way MAY interfere with  correct  operation of these

programs.

GENERAL INFORMATION

The following sections outline how best to utilise the driver

and get the best performance from it on your hardware setup.

INSTALLATION

BNU.COM is usually installed in your AUTOEXEC.BAT  or at some

time before you    load    your    communications   software.

Alternatively, MS-DOS 4.xx has  an option to allow loading of

some 'compatible' TSRs using an "install=" statement  in your

CONFIG.SYS.  A major  benefit  of  this  is  that  BNU can be

loaded before any environment variables are set, and so takes

up the absolute minimum of RAM.   However, as with the device

driver when loaded   from  CONFIG.SYS,  BNU.COM   cannot   be

unloaded from memory using the "/U" option.

- 18 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

ERRORLEVELS

The following errorlevels are returned by BNU (this also

documents some of the possible errors which can be returned):

0    No error - operation(s) successful

1    Status was requested and displayed

2    A FOSSIL driver other than BNU is present

3    Incompatible version of BNU is already loaded

4    Can't modify parameters with ports active

5    BNU is not yet resident or loaded

6    Not used

7    DOS memory allocation error - fatal abort

8    Invalid command line option

9    Can't unload - FOSSIL appendages are loaded

10   System reboot failure

11   Error enabling or disabling carrier watch

12   Unable to recapture INT 14H vector

13   Can't unload if installed from CONFIG.SYS

14   Maximum combined TX/RX buffer size exceeded

15   Can't disable/enable diags panel - not loaded

16   Unable to unload due to interrupt chaining

17   Problem initialising FOSSIL (with /I)

MULTITASKERS

Prior versions of BNU directly supported various multitaskers

by allowing for  "poll" detection on status and character I/O

calls made to the FOSSIL.  When BNU sensed a task was "idle",

it automatically gave  time slices  up  during  "polling"  to

other tasks in the system.

This internal multitasker  support  has  been   removed  from

BNU.COM and BNU.SYS,   and   the   /X  switch  is  no  longer

implemented.  However, the current  version  of BNU does have

'hooks' for an installable driver to link into  BNU and carry

out this task.   This  eliminates  much  of  the  unnecessary

"baggage" suffered by users  of  BNU  who  do  not  need this

feature.  It also  eliminates  code  made  redundant  by  not

having all multitasker   time   slice   resident   at   once,

irrespective of which specific  multitasker  is in use.  Only

that code which is actually used and plays a part  in support

of the multitasker itself is loaded.

While this current release (2.02) is not accompanied by these

drivers (which are   now   [October,  1992]  in  beta  test),

multitasking appendages for DESQview/TopView  and  PC-MOS/386

will soon be available.

- 19 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

MEMORY

BNU.COM is approximately  11K  in size.  It is made up of the

following main parts:

Description           Size  Used by

Resident

-------------------------------------------------------------

ISR Device Driver     0.5K  Common 4 ports   Yes

INT 14H Despatcher    4.5K  Common 4 ports   Yes

Diagnostics Model     1.0K  Common 4 ports   Optional

Initialisation/Setup  1.0K  Run time only    Discarded

Command line utility  4.0K  Run time only    Discarded

-------------------------------------------------------------

What eventually remains resident  depends  upon  what command

line options are used.  Initial overhead is about  5K for the

ISR and despatcher.   If  diagnostics are used (rarely), then

add 1K. Add to this, the sum  of  the  transmit  and  receive

buffers times the number of communications ports selected.

THE DIAGNOSTICS PANEL

The ability to  view the state of the FOSSIL driver  is  very

useful when developing  software  utilising FOSSIL calls.  It

can also be an invaluable aid in  "tuning" the driver to your

machine.  Memory is precious - especially in  a  multitasking

environment, and most  of  the time, the standard 1K transmit

and receive buffers are far too  much  to allocate.  This can

only be empirically tested.

Instead of "guessing" and using trial and error  to  see  how

well the driver  performs,  it  is  much better to enable the

diagnostics window.  This provides a 'snapshot' of the driver

18.2 times per second (each clock  tick).   Particular fields

to look at are:

"TX Cnt"  Indicates how full the transmit  buffer  is in

bytes.

"TX Max"    This  is  the  current  'maximum'  count  if

characters in the  transmit buffer detected by

the diagnostics routine (it views  the  driver

in 'snapshots'   so   cannot   be   completely

accurate.

"RX Cnt"  Indicates how full  the  receive  buffer is in

bytes.

"RX Max"   Maximum count of characters  in  the  receive

buffer (see TX Max).

- 20 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

"Int14h"  Particularly  useful  for  monitoring  program

activity.  This  counter ticks over every time

the running  application  requests   a  FOSSIL

service.

"Errors"  Locates  possible  problems in the program you

are running.  Any  time  the  program makes an

"illegal" call  (usually  incorrect   register

values).  The   last   FOSSIL   function  code

invoked which  triggered  the  error  will  be

displayed next to "ErrFun".

By observation, it is possible to calculate the  optimum size

for both receive and transmit buffers.

16550 USART SUPPORT

BNU fully supports  the  16550 USART in buffered mode.  Using

the /Z command line switch, you  can  decrease  the amount of

FIFO buffering used, or disable the FIFO's completely.

This option has  been  included for "compatibility"  reasons.

Previous releases of   BNU   used  the  equivalent  of  "/Z2"

buffering, whereas this release  defaults  to full use of the

FIFO queues.  If you have problems using the default "/Z0"

setting, try reducing  first to "/Z1", then "/Z2"  or  lower,

and once you've found a setting that works, please contact me

(I'm interested in  know what sort of setup this DOESN'T work

on).

BETA VERSIONS

Versions of BNU will either be  BETA  TEST  versions  or full

releases.   Public release versions are numbered  ending with

a zero and  carry  no  "alpha",  "beta", "gamma" etc. suffix.

Beta test releases carry an suffix  after the version number,

and will be  numbered  ending  with  a  non-zero  digit  (for

identification of incremental versions).

Beta releases of  the  BNU driver are not publicly available.

These are generally requestable from a number of systems, but

are be restricted for use by beta testing sites only.  If you

would like to assist in beta testing  future beta versions of

BNU, request "BNUINFO"  (magic  name)  WaZoo   from   Fidonet

3:632/348 (or Alternet 7:833/387).

Beta releases are,   by  nature,  less  stable  than  release

versions.  If you obtain one, please  do NOT distribute it by

posting it for download or file request.

- 21 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

PROBLEM REPORTS

If you have  problems  in  getting BNU to run on your system,

remember that I cannot solve that  problem if you do not take

the time to  let  me know.  If you do decide  to  pursue  any

problem, please give  as much detail regarding your system as

possible so that I can more readily  evaluate  why  something

may or may  not  be  occurring.  Please keep the  information

relevant (ie. I  don't need to know your shoe size, but I may

want to know what's in your CONFIG.SYS!).

Some information I'll probably need:

o    Machine type/model and class (XT/AT/386 etc.)

o    Type of USARTs installed (8250/16450/16550 etc.)

o    Communications software you're running

o    Contents of your AUTOEXEC.BAT/CONFIG.SYS or other

batch files which may be relevant

o    Switches used to load BNU.COM

o    Whether  you've  used   BNUPORT   to   change  port

assignments

o    Brief description of modem if at all unusual ie.

high speed, non-standard or sub-standard<grin>

o    Version of MS-DOS in use

o    Multitasker in use (if any), including version

o    Other TSR's loaded during communications activity

o    What other FOSSIL drivers have worked in the past

o    An accurate and complete (oh, I wish!)  description

of your problem

o    Contents of a diagnostics window when the problem

is occurring (if at all possible)

o    If  you  have  TurboPower's TSR utilities or Andrew

Wylie's DOSMAP program  (or some similar DOS memory

control block  mapping  utility),   then   redirect

output of  this  to a file and include it with your

report.  Try to duplicate the exact state of memory

when the problem occurs.   Then  execute a command,

something like:

"mapmem -v -h >output.mem"

- 22 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

DEVELOPERS!

BNU is currently being restructured to use an alternative far

call model, instead  of  the  INT  14H  interface.  This will

follow the same lines as a FOSSIL  appendage, and is expected

to yield significant performance benefits for  DOS as well as

benefits for the developer by allowing more direct high level

language access.

Enquiries regarding this project should be directed to any of

the electronic addresses   listed   at   the  start  of  this

document.

SUPPORT

An echomail conference originating  from  my  system (Fidonet

3:632/348) called "BNU"  is  available  for general  use  for

support of BNU   and   related  programs.   Topic  areas  are

generally BNU related but also  cover a wider area, including

FOSSIL communications, general  usage,  software  development

using FOSSIL and 'tricks'.

Please forward any enquiries regarding this conference to me.

THANKS!

Thank you to  the  many  people involved in helping the "BNU"

project, especially those of you  willing  (and silly enough)

to run beta  versions  on your own systems and  tolerate  the

problems.  Without the  support of these people, I would have

given up long ago.

Special thanks to  Paul  Marwick   of   3:640/820   for   his

painstaking assistance in beta testing, the high quality (and

quantity!) of his reports and observations and by helping

by proof-reading this documentation.  And to  his  poor users

for putting up with "would you like to try something for

me?" ...  :-)

- 23 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

LICENSE OF USE

This license applies  ONLY  to  non-commercial  use of public

release versions.  Special terms  cover  use of BNU, BNU beta

test versions, any   accompanying  programs  (including   any

supplied source code)  and  its  documentation  in commercial

environments.  "Commercial" here  is  defined  loosely as any

environment in which  software assists - either  directly  or

indirectly - in   any   venture   concerning  making  profit,

financial or other (material) gain.

Vendors of commercial (for sale)  packages  should contact me

regarding distribution arrangements.  Special versions of BNU

are generated for  specific  commercial  products,  and  that

product name is   displayed  as  a  part  of  BNU's  standard

copyright message.  Any version of BNU distributed as part of

another package or  product   which   does  not  display  any

indication of it being a "special" version in  this way is in

breach of this license.

Programs and documentation  are supplied free of charge on an

as is basis to the Fidonet (and othernet) community.  You are

free to use them in any non-profit venture with no obligation

on either myself or my company  to  support or be responsible

for any such use (or misuse).

BNU, any accompanying programs, source and this documentation

are the exclusive property of Unique Computing  Pty  Ltd, and

is protected by international copyright laws.  By obtaining a

copy of the  package,  you  do  not  "own"  a copy - simply a

license to use it.  All property  rights  are retained by the

owner.

You may use the package and/or give copies of  the package to

others including posting  for  download  on a bulletin board,

but only in a form that permits  inclusion  of  all  files as

found in the  original,  archieved  form  in   which  BNU  is

distributed.  You are  not permitted to modify any portion of

the package in any way, including,  but not limited to adding

files to or  removing  files  from the distribution  package,

changing or editing  the  documentation,  and/or  changing or

modifying the executable program.   Permission  is granted to

repack the distribution  archive  using  a different  packing

method, provided that the uncompressed contents EXACTLY match

the uncompressed contents of the distribution archive (I have

no particular affiliation   with   or   preference   for  any

particular archiving method.

BNU is not provided with a warranty  of  any kind, including,

but not limited  to  fitness  for any particular  purpose  or

consequential loss.

This package may  be  posted  for download or file request by

any FidoNet or compatible network  system  in  archived form,

and in adherence  to  the  terms  of the License  above.   No

profit may be   realised  directly  from  such  distribution,

- 24 -

BNU Revision 5 FOSSIL Communications Driver

Version 2.02

although BNU may  be  provided for download on "pay systems."

Distribution on diskette form  is  strictly prohibited unless

done for free  for  a charge not exceeding the  cost  of  the

diskettes.

- End -

- 25 -