16550 - A UART Control Program

Version 1.0

DESCRIPTION:

16550 is a shareware program designed to allow the 'unlocking'

of the internal fifo buffer present in the UART chip of the same

name. The program will scan all four COM ports (COM1: - COM4:)

available on the PC and report the presence (if any...) of the

16550 UART. The user may optionally set the various parameters

necessary to utilize this buffered mode of operation.

DISCUSSION:

The original PC serial interface specification utilized the

8250 Programmable Communications Interface, or UART. This device

converts data from the microprocessor into a format that is

easily transmittable via a single-wire connection. To accomplish

this task, the UART takes data from the computer in a parallel

format (ie: the bits are presented side-by-side) and shifts them

out via a single pin in a serial (one bit directly after another,

daisy-chain fashion) data stream. Various extra bits are added

such as the start, parity and stop bits to provide the UART on

the other end of the serial data connection with an accurate

means for the re-assembly of the original data format.

In the early days of serial communications, particularly that

involving data transmission via telephone lines, the speed at

which the data was transmitted wasn't particularly fast. This is

especially true with telephone lines. The normal voice-grade line

is extremely limited in it's sound-reproduction qualities. This

low bandwidth implied that any data must be transmitted very

slowly to avoid 'overloading' the line. For voice-grade lines,

the maximum reliable data rate was 300 baud, using the Bell 103

modem specification. (A modem translates the on/off binary pulses

into a tone so that the telephone line will treat it as regular

'voice' information.)

The UART, of course had to be able to translate data at the

same rate in order for the system to work. Other serial devices

such as terminals and printers did not require a modem, but in

most cases the data rate was still quite slow, usually under

19,200 baud.

In the PC, when using the serial port(s) directly, such as

sending a file to a serial printer, the 8250 UART (or the updated

16450 version) was more than able to keep up. Even in a

telecommunications link, the UART was still fast enough, since

the data over the phone line was relatively slow.

Another point was that the early PC's weren't terribly fast.

This was made worse when running a communications program such as

ProComm or CrossTalk. Since the data rates were slow, there was

enough time for the computer to run the program instructions

Page 1

16550 - A UART Control Program

while fetching the individual bytes as they came in over the

serial link. Even at 9600 baud, a very-well

written comm program was able to keep up with the data.

Time progressed and technology improved serial communications

(particularly that of modems) to the point where extremely high

data rates could be achieved. The new V32.bis modems are capable

of transfer rates of up to 14,400 baud, and with the V42.bis

compression, data burst rates of over 57,000 baud are feasible.

Of course, software became more complex as well, but with a

price. The more complicated the software, the more time it takes

to handle the program instructions. Couple this with the fact

that the data is coming in at an extremely high rate, and you

have a potential problem on your hands!

What typically happens is a data overrun occurs. This means

that there was still a byte of data left in the UART when the

next byte came in. Naturally, a loss of just one single byte is

enough to crash a program, so this is a very important concern!

One way around the problem is a handshake scheme, whereby the

transmitting computer holds off sending data until the receiving

end is ready. This works very well, but one major drawback is

that a great deal of time is wasted signalling instead of

transferring data.

The 16550 UART gets around this bottlenecking by incorporating

a buffer directly in the chip itself. This buffer is designed to

hold up to 16 bytes (or characters) before an overrun occurs. The

buffer is arranged in a FIFO manner, meaning First In, First Out.

In programming terminology this is known as a queue. The 16550

UART also has improved circuitry which allows it the ability to

transmit data some 4 times faster than the old 8250! This high-

speed circuitry helps in the PC environment mainly in interrupt

timing, though the time saved is rather marginal. Of more

importance is the FIFO. By having a 'reserve' storage area on the

chip, if the processor is momentarily busy, (say, executing an

ANSII graphic sequence) the data is not lost, but held in the

buffer until the processor is free to receive it.

The problem now lies in the fact that the majority of older

communications programs do not check for, and utilize the FIFO

capabilities of the 16550 chip. This is where the 16550 program

comes to the rescue!

PROGRAM OPERATION:

The 16550 UART initially powers up in a non-FIFO state. This

makes the UART software-compatible with the 16450, and the older

8250. In most cases, the communications software doesn't know the

difference. At least until the data comes in faster than the

program can process it!

The 16550 program goes in and 'unlocks' the FIFO capabilities

in the UART, and even in a buffered state, the 16550 still looks

Page 2

16550 - A UART Control Program

like the 16450 to older programs! If buffering occurs, then the

data is held, and subsequently read out at a relaxed pace

automatically, providing full compatibility with older software,

yet giving the high-speed protection of a data buffer!

PROGRAM PARAMETERS:

The 16550 automatically detects a 16550 UART, and provides

control for the COM port address, enabling and disabling of the

FIFO, and the settings for a special interrupt based upon the

number of characters being held in the FIFO buffer.

Executing the program without specifying any parameters results

in a scan of all COM ports, and reports the presence of a 16550

UART in each. If the user knows that a 16550 device exists,

additional parameters may be specified to access the FIFO. These

parameters are as follows:

COMn - Tells the 16550 program to use COM port n, where n

indicates ports 1 through 4. Any value outside of this

range results in an error. NOTE: A COM port MUST be

specified when setting the UART!

? - Echos a brief help text on the screen. No action is

taken.

/f - Enables the 16550 internal buffers.

/r - Disables the 16550 internal buffers and resets the UART

to the 8250/16450 compatibility mode.

/tn - Allows the adjustment of the FIFO-full interrupt. n

specifies the number of characters allowed in the

buffer before an interrupt occurs. the values for n are

1,4,8 and 14. NOTE: this is a very sensitive setting!

For normal operation with old communications software,

this parameter should not be used!

The program defaults to FIFO disabled, and a /t parameter of 1.

If the command line parameters are valid, the program will verify

that a 16550 UART is present, and that the parameters have been

set. Unless another program that recognizes a 16550 UART is

executed, the UART should remain in the current state.

SHAREWARE CONCEPT:

Shareware is a non-conventional approach to software

distribution. Unlike regular commercial programs, shareware

allows a 'try-before-you-buy' approach. In most cases, the

programs are just as bit as good (if not better in some

instances!) as the programs you buy at a computer store.

Shareware ISN'T freeware! In most cases, the software author is

a professional who makes at least part of his or her living on

Page 3

16550 - A UART Control Program

the registration of the software program. Typically the user is

enticed into registration by added features, free updates, etc.

Shareware depends upon the user to make it work. It's up to

you!

Registration:

This program is 'donate-ware', which is similar in concept to

shareware. The author makes no claims as being a professional

software engineer, rather, the author is a disabled hardware

engineer who enjoys tinkering with programs, and making the

hardware work better.

If you find that this program is useful, and saves you some

time and headaches, the author would appreciate a $5.00 (US)

suggested donation. This amount would include a 'registered' copy

of the program (without the shareware notice), and the eternal

thanks from the author. Considering that the cost of new

communications software that recognizes the 16550 UART far

exceeds 5 bucks, you get a pretty good deal! Of course, any

amount is appreciated. Oh, please specify whether you want a

360K, 5-1/4" or 720K, 3-1/2" floppy when you register.

.....and the address is:

Brent Turner

P.O. Box 3612

Fullerton, CA, 92634-3612

NOTE: Foreign (non-US) users please include a few extra bucks

for the postage! Thanks.

THE LEGAL STUFF:

This software is distributed on an as-is basis. The author has

made reasonable attempts to ascertain it's performance, but

cannot guarantee that it will work in every instance. The author

is not responsible for any damages, or loss of data resulting

from the use of this program.

The shareware version of this program may be freely distributed

as long as all files are included in the package. The registered

version may not be copied or duplicated.

Shareware vendors may distribute this program as long as the

cost per disk does not exceed $5.00. This program may be included

(bundled) with other programs of like nature, so long as the

above cost-per-disk stipulation is met.

The original source code is not available.

Page 4