This README file of PS2CMB.ZIP for NetWare v2.2 and v2.15C.

The following files are included for this revision:

COMB     OBJ    12645   5-23-91   1:59p

DUMMY    OBJ      138   5-23-91   1:59p

SCSICOMB DSK      564   9-10-91  10:20a

ESDICOMB DSK     1292   9-10-91  10:22a

Novell ESDI/SCSI Combination Driver Set

WARNING: Before you begin this installation, back up your

system to avoid losing any important data.

If you are installing on a v2.2 network, use the Advanced

installation.  Do not use Basic Install.

To install the new ESDI/SCSI combination driver set in

your PS/2 Model, complete the following steps:

1.    Follow the instructions for installing drives and

disk controllers in your IBM documentation.

Don't autoconfigure the system. Continue with Step 2.

2.    Run REFERENCE.

3.    From the main menu, choose Option #2 ("Set

Configuration")

4.    From the "Set Configuration" menu, choose Option #1

("View Configuration") and record the following

information:

o     I/O addresses

o     DMA

5.    From the "Set Configuration" menu, choose

Option #6 ("Set and View SCSI device

configuration").

Check to be sure the SCSI configuration matches your

recorded configuration information.

6.    When configuration is completed, choose Option #7

("Display Memory Map").

Record the hexadecimal addresses for each driver

as shown on the screen.

Note: If your system includes more than 2 hard drives and you are

running DOS 4.0, you must install the DOS v4.0 Corrective

Services Diskette (CSD #UR29015). This diskette is available

from IBM.  The system must be booted with DOS 4.0 or

higher.

7.    Follow the instructions on the screen to exit

REFERENCE.

The following installation instructions are divided into two

categories:

o     v2.15

o     v2.2

Scroll through to find the section that applies to your specific

system.

You are now ready to begin the installation process. These

procedures vary between v2.15 and v2.2 systems. There are

no defaults available for this installation.  You must load the

ESDI and the SCSI shared int 14 drivers.  The operating

system will be unable to link if both drivers are not loaded.

Note: All driver configuration options must correspond to

actual hardware settings.  Software adjusts must be

accompanied by hardware adjustments and vice versa if

either are necessary.

NetWare v2.15 Installation (Custom)

Refer to the NetWare v2.15 Maintenance manual for

installation instructions. Before running NETGEN complete

the following steps:

1.    NETWORK DRIVE:  Create a directory called

DSK_DRV_.031.

FLOPPY DRIVE:  Format a diskette with the label

DSK_DRV_031. Use the DOS FORMAT /V command.

2.    NETWORK DRIVE:  Copy the .DSK in the AUXGEN

directory and the .OBJ files into the directory you

created in Step 1.

FLOPPY DRIVE:  Copy the .OBJ files and the .DSK

files to the newly formatted diskette you created in

Step 1.

3.    Run NETGEN.

4.    NETWORK DRIVE:  Choose "Select Loaded Item" from

the menu and continue with Step 5.

FLOPPY DRIVE:  Complete the disk driver installation

instructions contained in the Maintenance manual.

Then continue with Step 5 below.

Note: As you install new disk drivers, you may see a message

similar to the following:

"Driver 1 has the same type number as Driver 2.

Replace Driver 1 with Driver 2?"

The drivers do not always appear in the same order,

indicating that Driver 1 is not necessarily the new driver.

Be sure you select the ESDI shared int 14 driver.

5.    Choose the ESDI shared int 14 driver from the list and

press <Enter>.

6.    Highlight Configuration Option and press <Enter>.

Highlight the option you recorded earlier and press

<Enter>.

7.    Repeat Steps 5 and 6, this time choosing the SCSI

shared int 14 driver.

8.    Continue with the file server installation as usual,

following the screen prompts.

NetWare v2.2 Installation (Advanced)

1.    Use the DOS LABEL command to format a blank

diskette.  Label the diskette DSK_DRV_031 and

copy the following files onto it:

o   .OBJ

o   .DSK

2.    Copy the driver and description files to the

DSK_DRV_031 diskette.

3.    Complete the installation instructions contained in the

documentation for NetWare v2.2, through Step 12 on

page 36.  Then continue with Step 3 below.

4.    Press <Insert>.

5.    Insert the DSK_DRV_031 diskette into a floppy

diskette drive and press <Enter>.

6.    Choose the ESDI shared int 14 driver from the list and

press <Enter>.

7.    Highlight Configuration Option and press <Enter>.

Highlight the option you recorded earlier and press

<Enter>.

Note:  An asterisk (*) next to a selected driver indicates that the

option conflicts with other hardware configurations selected.

All driver configuration options must correspond to

actual hardware settings.  Software adjustments must be

accompanied by hardware adjustments and vice versa if

either are necessary.

8.   Repeat Steps 5 through 7, this time choosing the

SCSI shared int 14 driver.

9.   Press <F10> when the form is complete.

This step saves the configurations as entered. Continue

the installation as usual from this point.

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************