TABLE OF CONTENTS

cia.resource/AbleICR

cia.resource/AddICRVector

cia.resource/RemICRVector

cia.resource/SetICR

cia.resource/AbleICR                                     cia.resource/AbleICR

NAME

AbleICR -- Enable/disable ICR interrupts.

SYNOPSIS

oldMask = AbleICR( Resource, mask )

D0                 A6        D0

WORD AbleICR( struct Library *, WORD );

FUNCTION

This function provides a means of enabling and disabling 8520

CIA interrupt control registers. In addition it returns the

previous enable mask.

INPUTS

mask            A bit mask indicating which interrupts to be

modified. If bit 7 is clear the mask

indicates interrupts to be disabled. If

bit 7 is set, the mask indicates

interrupts to be enabled. Bit positions

are identical to those in 8520 ICR.

RESULTS

oldMask         The previous enable mask before the requested

changes. To get the current mask without

making changes, call the function with a

null parameter.

EXAMPLES

Get the current mask:

mask = AbleICR(0)

Enable both timer interrupts:

AbleICR(0x83)

Disable serial port interrupt:

AbleICR(0x08)

EXCEPTIONS

Enabling the mask for a pending interrupt will cause an

immediate processor interrupt (that is if everything else is

enabled). You may want to clear the pending interrupts with

SetICR() prior to enabling them.

NOTE

The CIA resources are special in that there is more than one

of them in the system. Because of this, the C language stubs

in amiga.lib for the CIA resources require an extra parameter

to specify which CIA resource to use. The synopsys for the

amiga.lib stubs is as follows:

oldMask = AbleICR( Resource, mask )

D0                 A6        D0

WORD AbleICR( struct Library *, WORD );

SEE ALSO

cia.resource/SetICR()

cia.resource/AddICRVector                           cia.resource/AddICRVector

NAME

AddICRVector -- attach an interrupt handler to a CIA bit.

SYNOPSIS

interrupt = AddICRVector( Resource, iCRBit, interrupt )

D0                        A6        D0      A1

struct Interrupt *AddICRVector( struct Library *, WORD,

struct Interrupt * );

FUNCTION

Assign interrupt processing code to a particular interrupt bit

of the CIA ICR.  If the interrupt bit has already been

assigned, this function will fail, and return a pointer to the

owner interrupt.  If it succeeds, a null is returned.

This function will also enable the CIA interrupt for the given

ICR bit.

INPUTS

iCRBit          Bit number to set (0..4).

interrupt       Pointer to interrupt structure.

RESULT

interrupt       Zero if successful, otherwise returns a

pointer to the current owner interrupt

structure.

NOTE

A processor interrupt may be generated immediatly if this call

is successful.

In general, it is probably best to only call this function

while DISABLED so that the resource to which the interrupt

handler is being attached may be set to a known state before

the handler is called. You MUST NOT change the state of the

resource before attaching your handler to it.

The CIA resources are special in that there is more than one

of them in the system. Because of this, the C language stubs

in amiga.lib for the CIA resources require an extra parameter

to specify which CIA resource to use. The synopsys for the

amiga.lib stubs is as follows:

interrupt = AddICRVector( Resource, iCRBit, interrupt )

D0                        A6        D0      A1

struct Interrupt *AddICRVector( struct Library *, WORD,

struct Interrupt *);

***WARNING***

Never assume that any of the CIA hardware is free for use.

Always use the AddICRVector() function to obtain ownership

of the CIA hardware registers your code will use.

Note that there are two (2) interval timers per CIA.  If

your application needs one of the interval timers, you

can try to obtain any one of the four (4) until AddICRVector()

succeeds.  If all four interval timers are in-use, your

application should exit cleanly.

If you just want ownership of a CIA hardware timer, or register,

but do not want interrupts generated, use the AddICRVector()

function to obtain ownership, and use the AbleICR() function

to turn off (or on) interrupts as needed.

Note that CIA-B generates level 6 interrupts (which can degrade

system performance by blocking lower priority interrupts).  As

usual, interrupt handling code should be optimized for speed.

Always call RemICRVector() when your code exits to release

ownership of any CIA hardware obtained with AddICRVector().

SEE ALSO

cia.resource/RemICRVector(), cia.resource/AbleICR()

cia.resource/RemICRVector                           cia.resource/RemICRVector

NAME

RemICRVector -- Detach an interrupt handler from a CIA bit.

SYNOPSIS

RemICRVector( Resource, iCRBit, interrupt )

A6        D0      A1

void RemICRVector( struct Library *, WORD, struct Interrupt *);

FUNCTION

Disconnect interrupt processing code for a particular

interrupt bit of the CIA ICR.

This function will also disable the CIA interrupt for the

given ICR bit.

INPUTS

iCRBit          Bit number to set (0..4).

interrupt       Pointer to interrupt structure.

RESULT

NOTE

The CIA resources are special in that there is more than one

of them in the system. Because of this, the C language stubs

in amiga.lib for the CIA resources require an extra parameter

to specify which CIA resource to use. The synopsys for the

amiga.lib stubs is as follows:

RemICRVector( Resource, iCRBit, interrupt )

A6        D0      A1

void RemICRVector( struct Library *, WORD, struct Interrupt *);

SEE ALSO

cia.resource/AddICRVector()

cia.resource/SetICR                                       cia.resource/SetICR

NAME

SetICR -- Cause, clear, and sample ICR interrupts.

SYNOPSIS

oldMask = SetICR( Resource, mask )

D0                A6        D0

WORD SetICR( struct Library *, WORD );

FUNCTION

This function provides a means of reseting, causing, and

sampling 8520 CIA interrupt control registers.

INPUTS

mask            A bit mask indicating which interrupts to be

effected. If bit 7 is clear the mask

indicates interrupts to be reset.  If bit

7 is set, the mask indicates interrupts to

be caused. Bit positions are identical to

those in 8520 ICR.

RESULTS

oldMask         The previous interrupt register status before

making the requested changes.  To sample

current status without making changes,

call the function with a null parameter.

EXAMPLES

Get the interrupt mask:

mask = SetICR(0)

Clear serial port interrupt:

SetICR(0x08)

NOTE

The CIA resources are special in that there is more than one

of them in the system. Because of this, the C language stubs

in amiga.lib for the CIA resources require an extra parameter

to specify which CIA resource to use. The synopsys for the

amiga.lib stubs is as follows:

oldMask = SetICR( Resource, mask )

D0                A6        D0

WORD SetICR( struct Library *, WORD );

***WARNING***

Never read the contents of the CIA interrupt control registers

directly.  Reading the contents of one of the CIA interrupt

control registers clears the register.  This can result in

interrupts being missed by critical operating system code, and

other applications.

EXCEPTIONS

Setting an interrupt bit for an enabled interrupt will cause

an immediate interrupt.

SEE ALSO

cia.resource/AbleICR()