TABLE OF CONTENTS

potgo.resource/AllocPotBits

potgo.resource/FreePotBits

potgo.resource/WritePotgo

potgo.resource/AllocPotBits                       potgo.resource/AllocPotBits

NAME

AllocPotBits -- Allocate bits in the potgo register.

SYNOPSIS

allocated = AllocPotBits(bits)

D0                       D0

UWORD AllocPotBits( UWORD );

FUNCTION

The AllocPotBits routine allocates bits in the hardware potgo

register that the application wishes to manipulate via

WritePotgo.  The request may be for more than one bit.  A

user trying to allocate bits may find that they are

unavailable because they are already allocated, or because

the start bit itself (bit 0) has been allocated, or if

requesting the start bit, because input bits have been

allocated.  A user can block itself from allocation: i.e.

it should FreePotgoBits the bits it has and re-AllocPotBits if

it is trying to change an allocation involving the start bit.

INPUTS

bits - a description of the hardware bits that the application

wishes to manipulate, loosely based on the register

description itself:

START (bit 0) - set if you wish to use start (i.e. start

the proportional controller counters) with the

input ports you allocate (below).  You must

allocate all the DATxx ports you want to apply

START to in this same call, with the OUTxx bit

clear.

DATLX (bit 8) - set if you wish to use the port associated

with the left (0) controller, pin 5.

OUTLX (bit 9) - set if you promise to use the LX port in

output mode only.  The port is not set to output

for you at this time -- this bit set indicates

that you don't mind if STARTs are initiated at any

time by others, since ports that are enabled for

output are unaffected by START.

DATLY (bit 10) - as DATLX but for the left (0) controller,

pin 9.

OUTLY (bit 11) - as OUTLX but for LY.

DATRX (bit 12) - the right (1) controller, pin 5.

OUTRX (bit 13) - OUT for RX.

DATRY (bit 14) - the right (1) controller, pin 9.

OUTRY (bit 15) - OUT for RY.

RESULTS

allocated - the START and DATxx bits of those requested that

were granted.  The OUTxx bits are don't cares.

potgo.resource/FreePotBits                         potgo.resource/FreePotBits

NAME

FreePotBits -- Free allocated bits in the potgo register.

SYNOPSIS

FreePotBits(allocated)

D0

void FreePotBits( UWORD );

FUNCTION

The FreePotBits routine frees previously allocated bits in the

hardware potgo register that the application had allocated via

AllocPotBits and no longer wishes to use.  It accepts the

return value from AllocPotBits as its argument.

potgo.resource/WritePotgo                           potgo.resource/WritePotgo

NAME

WritePotgo -- Write to the hardware potgo register.

SYNOPSIS

WritePotgo(word, mask)

D0    D1

void WritePotgo( UWORD, UWORD );

FUNCTION

The WritePotgo routine sets and clears bits in the hardware

potgo register.  Only those bits specified by the mask are

affected -- it is improper to set bits in the mask that you

have not successfully allocated.  The bits in the high byte

are saved to be maintained when other users write to the

potgo register.  The START bit is not saved, it is written

only explicitly as the result of a call to this routine with

the START bit set: other users will not restart it.

INPUTS

word - the data to write to the hardware potgo register and

save for further use, except the START bit, which is

not saved.

mask - those bits in word that are to be written.  Other

bits may have been provided by previous calls to

this routine, and default to zero.