S P L O T  -  The Pen Plotter Simulator V2.52

---------------------------------------------

Copyright (C) 1989-94 by Alexandr NOVY

All rights reserved.

The latest changes not described in SPLOT.DOC file

--------------------------------------------------

July 1994

1. BATCH MODE

-------------

The SPLOT program can now operate in the so-called 'batch'

mode. This non-graphic mode is designated for automatic

exporting of the HP-GL files without user intervention.

If you want to run the SPLOT program in the batch mode, you

must use the /BAT option on the SPLOT command line. The

complete command line syntax is:

SPLOT /BAT [options] [input_file_mask] [output_file_mask]

All parameters are case insensitive.

The input file mask parameter is optional, however you must

use it if you use the output file mask parameter. The default

input mask is * and the default input extension is the

standard plot file extension set in the DEFAULT EXTENSIONS

dialog in the SPLOT program. The default output mask is *

and the	default output extension is the	standard extension

according to the type of export	set in the DEFAULT EXTENSIONS

dialog in the SPLOT program.

The options parameters are voluntary. All options parameters

must be preceded by the / (slash) character. The only options

parameters supported now are:

1. The EXPORT TYPE parameter can be:

parameter		    export type

----------------------------------

HPGL                  HP-GL

HPGL2                 HP-GL/2

DXYGL                 DXY-GL

PCALL                 PLOTCALL

DXF                   DXF

PSBW                  PostScript black&white

PSC                   PostScript color

IBMGR		    IBM Graphics printer

IBMX24		    IBM X24 printer

ELX		    EPSON LX printer

EFX		    EPSON FX printer

EEX 		    EPSON EX printer

EEXC                  EPSON EX printer color mode

ELQ 		    EPSON LQ printer

ELQC                  EPSON LQ printer color mode

NEC24		    NEC 24 pin printer

NEC24C                NEC 24 pin printer color mode

HPLJ		    HP LaserJet printer

HPDJBW		    HP DeskJet printer B&W mode

HPDJC                 HP DeskJet printer color mode

HPDJ550C              HP DeskJet 550C color mode

HPPJBW                HP PaintJet printer B&W mode

HPPJC		    HP PaintJet printer color mode

HPPJXBW		    HP PaintJet XL300 B&W mode

HPPJXC 		    HP PaintJet XL300 color mode

HPQJ		    HP QuietJet printer

HPTJ                  HP ThinkJet printer

Example:

SPLOT /bat /hplj *.plt prn

exports all files having a PLT extension to the DOS

printer port in the HP LaserJet format.

If the export type option is not used, the SPLOT

program uses the last used export type saved in the

SPLOT.INI file. Also all other export parameters

(which cannot be set from the command line yet) are

loaded from this initialization file. If the SPLOT.INI

file does not exist, the default values are used.

Note: When running SPLOT program in batch mode the

program parameters are not saved to the SPLOT.INI file

after program exit.

2. The APPEND parameter can be used if you want to append

the output data to an existing file(s). Normally the

SPLOT program in batch mode truncates all the output

files to zero length before exporting. If the output

file is common to all input files, the output data

stream is appended automatically to this output file.

However if you don't use the APPEND parameter the output

file is first time opened with overwrite flag.

Examples:

SPLOT /bat /append *.plt *.prn

appends all output data to the *.prn files.

SPLOT /bat *.plt splot.prn

truncates the SPLOT.PRN file to zero length, then

appends all output data to it.

SPLOT /bat /append *.plt splot.prn

appends all output data to the SPLOT.PRN file.