About MEM2 (in library 2 as MEM2.ZIP).

Why MEM2?

---------

MEM2 was written to solve a mystery:  Some time ago we noted that there were

apparent speed variations between different CPUs which had the same nominal

clock rate.  Yet speed-measuring utilities (like Norton) reported that these

machines were all operating at the same clock rate, so we were mystified.

A Word On Computer Architecture

-------------------------------

The chips used to create PC memory typically operate at speeds between

80 and 120 ns.  If your CPU is operating at 33 Mhz, that's about 30ns per

machine cycle.  Therefore, your RAM chips operate between 2.5 and 3.5 SLOWER

than the CPU.

One of the jobs a computer architect has is to somehow cause your machine to

run AS IF its memory was fast enough to keep up with the CPU.  This is

accomplished by hardware caches consisting of small amounts of very high speed

memory.  Such caches are designed with widely varying degrees of sophistication

and competence.

Now most CPU speed-measuring utilities perform rather naive tests of CPU

speed.  And, you may be sure that the computer vendors take care to insure

that the most commonly used programs show their machines in a good light.

However, in REAL applications major differences emerge.

On a well engineered machine like, say, a Compaq, the nominal CPU clock rate

will actually be achieved in normal operation.  On some cut-rate machines with

small or badly implemented caches, extra memory wait states must be inserted

to compensate for shoddy or inexpensive (cheap?) designs.

The bottom line:  you don't always get the performance you're paying for.

What MEM2 Does

--------------

MEM2 tests memory performance by performing 4095 byte block moves across all

of memory.  Of course, memory references across a broad range of addresses

are much more typical of real operation than more narrow tests.

The exact number of machine cycles required for these operations is calculated

in advance.  Then the operation is timed using the high-resolution timer.

Next, this time is divided by the known number of machine cycles required to

execute the operation.  This yields the effective CPU clock rate which is

reported.

Most machines show effective clock rates in the vicinity of the nominal clock

rate;  the best engineered machines match exactly. Unfortunately however,

we've seen effective clock rates as poor as 1/3 the nominal clock rate

(i.e. a supposed 33MHz machine which actually performs like an 11 or 12 MHz

machine).

The results are automatically scaled to reflect the differences between 386

and 486 processors.  However, if you run on a 386sx you'll have to multiply

the clock rate the program calculates by 2.5 to scale it properly for a 386sx.

(On a 486 the instruction we're timing takes 3 cycles per iteration, on a 386

it takes 4 cycles, and on a 386sx it takes 10 cycles).

We've found that MEM2 yields results that closely match our subjective

impressions of machines we've tested.