____________________________________________________________

IBM AT or Compatible

Adaptec AHA-1540/1540A/1542A/1540B/1542B/1640/1740  SCSI  Adapter

Board

SCSI Hard Drive/Tape Drive

MS-DOS 3.3

This utility will not operate while the 80386 is in virtual mode.

Attempting  to  do  so  will  cause  the  program  to  abort.  In

addition, this program will not run when  another  device  driver

has  been  loaded  that talks directly to the adapter such as the

ASW-1410 ASPI Manager.

NOTE:  The 1740 must be  in  standard  mode  in  order  for  this

utility to function.

SCSICNTL is a program that runs  under  MS-DOS  allowing  you  to

reprogram your SCSI device to meet the needs of your environment.

Reprogramming your SCSI device allows you to achieve the  highest

possible  throughput for the given operating environment you will

be working in.

This program assumes a basic  knowledge  of  SCSI  and  makes  no

attempt  to  teach  you  about  SCSI.   Although,  there  is help

available for all options in each submenu window, it  is  advised

that   you  obtain  the  programmers  reference  guide  for  your

particular device, or get the ANSI SCSI Standard Rev 17B document

to  gain full knowledge of the parameters in each window.  If you

do not have access to either of these documents, read this manual

carefully and use the help system that is built into the program.

Through the mode sense pages you can  reprogram  a  SCSI  device.

This  program  does  have a knowledge of certain SCSI devices and

for those devices,  the  defaults  are  listed  in  each  program

window.   For  those devices that are not known, the defaults are

listed as "Unknown".  Unknown devices can  still  be  programmed,

but  it is up to you to have the necessary documentation relating

to the device you are attempting to re-program.

With some experimentation, you can find  the  best  settings  for

your  environment.   All  but two of the menu options may be used

without risk to data stored on a SCSI device.  Where options that

could effect data integrity, there are many warnings.  In some of

the menus,  helpful  recommendations  for  a  specific  operating

environment   are  available.   Take  these  recommendations  for

starters  and  tune  them  to  meet  your  needs.   Some  options

available  can  reduce  the throughput of your device, but give a

higher level of data integrity.  While others will  yield  higher

throughput  and  reduce  the  data  integrity.  Reduction of data

integrity is up to you.  If you have a high level  of  confidence

in your SCSI device, this is not a risk, but it is your option.

If you have very little understanding of  SCSI,  study  the  menu

choices  and  take the time to go through each option and ask for

help.  The help provided is sometimes technical  in  nature,  but

what you are doing is highly technical in nature.  If you are not

comfortable with this program, it is advised that you do not  use

it.   But  keep  in  mind  that the program will not allow you to

destroy data unless you make the effort to do so.

The program may be run off of a floppy drive or  off  of  a  hard

drive.   The disk does not have any copy-protection.  Use the MS-

DOS 'copy' utility to install the programs on to your hard  drive

or another floppy drive.  See your MS-DOS documentation for usage

of the 'copy' command.

To run this program on the in your CPU, you must  have  the  host

adapter base I/O address set to 0x330h.  This is the default, but

if you have changed this default it will be necessary for you  to

change   it  back.   Please  refer  to  your  SCSI  Host  Adapter

Installation guide for help.

To start the program, type SCSICNTL <ENTER>.

You will be presented with a window that  contains  a  disclaimer

which  will require you to press ENTER to continue, next a window

with Copyright information and the version number of the  program

is displayed.  You will asked to press ENTER to continue again.

The screen will clear and you will be  presented  with  a  window

along with a window that states, "Looking for devices on the SCSI

bus".  If you have a SCSI tape drive in the system and  the  tape

is not loaded in the tape drive (Tape drive LED is off), an error

message window pops up at the bottom of  the  display  showing  a

"Not Ready" message.  This message can be ignored as it is simply

stating that the tape drive does not have  a  tape  in  it.   For

almost  all  the  options  available  a  tape does not need to be

loaded in the drive.  If any other error appears,  there  may  be

something wrong with the device that was being reviewed.

After the search for SCSI  devices  is  complete  the  main  menu

window  pops  up,  shown below.

Drive 0                         MAIN MENU               [Modify]

1.  Select Device

2.  Format Utility

3.  Read Device Defect List

4.  Read/Write Error Recovery Page

5.  Disconnect/Reconnect Control Page

6.  Direct Access Device Format Page

7.  Rigid Disk Drive Geometry Page

8.  Cache Control Page

9.  Unique Control Page

10. Set mode of operation (View/Modify)

11. SCSI Adapter Configuration

12. Additional SCSI Device Information

13. Exit

Enter Selection:

The program uses menus to guide you  through  the  programmable

pages  of a SCSI device.  Along with the listed options, as shown

above, is a window that contains the information about  the  SCSI

device that is currently selected.  The program defaults the SCSI

device to drive 0 as displayed in the upper left hand  corner  of

the  main  menu  window.   If  device  zero is not available, the

program will find the first available device on the SCSI bus  and

use that device as the default.

The information, in the window to  the  left  of  the  main  menu

selections  contains  the  manufacturer, model number of the SCSI

device, part number, firmware level, firmware  date,  and  serial

number.   Take  note  that  not  all  SCSI  devices have all this

information available and those fields will be blank.

In the upper right hand corner of  the  window,  is  the  current

state  the  program is in.  It defaults to "Modify".  You can use

option 10 to change it to "View" if you like.

Options 4 through 9 have  a  similar  window  format.   The  menu

heading  will  reflect  the  option you have chosen from the main

menu.  Like the main menu, the drive that has  been  selected  is

displayed  in  the  upper  left  hand corner of the display.  The

sub-menu  headings  or  fields  are,  "Parameters",   "Defaults",

"Current",  "Options", and "Selected".

Parameters

This  field  is  a  listing  of  descriptions  for  the  possible

programming options.

Defaults

This field contains a listing of defaults for the  known  device,

if  the  device is of an unknown type, the word "Unknown" appears

for all of the parameters.

Current

This field lists all of the parameters that are currently set for

the selected device.

Options

This field lists all of the available options for a known device.

If  the  device is not known, the word "Unknown" is displayed for

all the parameters.

Selected

This field is where the cursor is placed for you  to  enter  your

choice for each programmable option that is supported for a known

device.  Not all devices support all the parameters  of  a  given

main menu selection.  If the device is an unknown type of device,

you will need to have the SCSI Reference manual for  that  device

in order to select the correct options for that device.

For a given device, the cursor will be placed in  the  "Selected"

field.  Some options are not available for some devices.  In this

case, the cursor will be placed accordingly.  If the device is  a

known  device,  the  options  available  for  that device will be

listed and the cursor will be placed in the "Selected" field  for

you  to  enter the option.  Unless you are in 'view' mode.  If in

'view' mode, all the  current  settings  for  a  device  will  be

displayed  and  you will be asked to "Press <ENTER> to continue",

doing so will return you to the main menu.

When in 'Modify' mode, you may enter the listed option  for  that

parameter,  press  a  '?' to get help, or press <ENTER> to select

the current setting.  If you press the '?', a  help  window  will

describe  each option for the field you are currently in and what

the effect of each option is.  If the program is only expecting a

single  character  response,  you will not have to press <ENTER>.

For instance, if the option for the field you are in is  (E)nable

or  (D)isable,  pressing  '?',  'e',  or 'd' will not require the

<ENTER> key to be pressed, but if the field is expecting a  range

of  values  such  as  0-65535,  pressing  the <ENTER> key will be

required before action will occur.

At the end of each page, you will receive another  prompt  asking

if  you  wish to 'S'tore, or make 'A'ctive, or 'C'ancel the above

changes.

If you choose to 'S'tore the parameters as they appear under  the

'Selected'  field,  these  parameters  will  become the 'current'

parameters until you use this program to change them again.

If you choose 'A' to make the page  active  only,  the  currently

selected  parameters  will  be sent to the drive and will stay in

effect until the system is re-booted.  This  is  helpful  if  you

only  wish  to  make changes, for instance, under MS-DOS and when

you reboot the system, you may enter  XENIX  and  the  parameters

that were previously installed will take effect.  This allows you

to reprogram a device for one environment, while you are in  that

environment,   and  reboot  the  system  to  return  to  the  old

parameters for another environment.

Answering 'C' will simply return you to the  main  menu  aborting

any changes you have made.

Options '6' and '2' from the main menu are the only options  that

could  have  any  effect  on data you have on the hard drive.  Be

sure you have selected the correct device before using either  of

these options.

The options available at the main menu are as follows:

This allows you to select the device that will be used during the

program.   You  may  choose from any of the devices in the window

and you may reselect devices as often as you like.   The  default

device is zero.

Selecting this option will open a  window  on  the  display  that

shows a warning about the device that is currently selected and a

prompt that asks, "DO YOU REALLY WANT TO  CONTINUE?".   Answering

this  prompt with anything other than a 'Y' will abort the format

and you will be returned to the main  menu.   If  you  choose  to

continue  (answering with a 'Y'), the drive will be formatted and

ALL USER DATA ON THE DRIVE WILL BE DESTROYED.   While  formatting

the  drive the system will appear as if it is doing nothing.  The

only message that will be displayed will  be,  "Formatting  Drive

x",  where 'x' is the drive you have selected.  If the drive is a

tape drive, the message, "Erasing Drive x",  will  be  displayed.

If you are erasing a tape, you will need to have a tape loaded in

the drive, or you will get a "Not Ready" error message displayed.

Some devices are hard sectored and  will  only  check  for  added

media flaws when the format command is issued.  In this case, the

format will be done in a matter of seconds.  This is due  to  the

fact that these devices keep track of potentially bad sectors and

only reassign those  sectors  for  a  low-level  format.   If  no

sectors  are  found  to be defective, a low-level format of these

devices will not result in loss  of  user  data.   In  all  other

cases, user data will be lost for any SCSI device.

When you use this option, you will be  presented  with  a  window

that  asks,  "Which list do you want (P,G,A)?".  In the middle of

the screen is a window that outlines each of  the  options.   The

"P"  list  contains  the original manufacturers defect list.  The

"G" list contains a list of defects that  have  been  mapped  out

during a format command.  Choosing "A" will display both lists on

the screen.

This window allows you to alter the error recovery parameters  of

a  SCSI  device.   You  can control things like ECC and Automatic

Read/Write  Reallocation;  which  allows  the  SCSI   device   to

automatically reallocate bad sectors dynamically.

This  window  allows  you  to  set  the  levels  that  SCSI   bus

arbitration  will  occur, based on how full/empty the SCSI buffer

is for the device  that  you  have  selected.   These  parameters

really only effect the XENIX/OS2 operating environments.

This page allows you to set the SCSI sparing scheme for a device.

SCSI  devices  have  spare  sectors  and tracks that are used for

replacing bad sectors and tracks.   By  reducing  the  number  of

spares  in  this page you can increase your storage, but you lose

the spares that may be required someday.  Increasing  the  number

of  spares will decrease the total storage available on your SCSI

device, but will allow the device to have  a  greater  number  of

spare  sectors  to  use  for  replacing  bad  ones.  Not all SCSI

devices allow these parameters to be altered.

You will be warned that altering this information will require  a

low-level  format  to  be  done.   Also  the only storage options

available are 'S'tore and  'C'ancel.   You  have  the  option  of

aborting  the  format,  but  if  you  leave  the  program without

formatting the device, the parameters you have  altered  will  be

changed back to thier original values.

This page is available for viewing only and  cannot  be  altered.

It  contains the physical specifications for a SCSI device.  Such

as the number of heads and cylinders for a SCSI hard drive.

This page is only available for devices that  this  utility  will

support.   If  the device is not supported, an error message will

be displayed in the center of the main window stating so.

If the device is a supported device, and the  drive  has  a  true

cache,   you   will   have   windows   appear   for  some  option

recommendations for specific  operating  environments.   You  can

choose   any   options   you   like,   but   if  you  follow  the

recommendations you will get the most you can out  of  your  SCSI

device.   Feel  free  to  experiment  with these options for your

application to get the most out the drive that you can.

For devices that only have a read ahead buffer, you will  not  be

presented  with  any suggestions for setting this option as it is

either on or off.  Drives such as CDC and Maxtor have read  ahead

buffers, that may or may not be enabled from the factory.

This page is only available for devices that  this  utility  will

support.   If  the device is not supported, an error message will

be displayed in the center of the main window stating  so.   This

window will display unique control parameters of a SCSI device.

This allows you to set the mode of operation for the utility.  If

you are in 'view' mode, you may view the current settings for the

device you have selected.  This option is a toggle and remains in

effect until you press it again.  For some SCSI devices, the mode

may be changed from Modify to  View  depending  on  if  the  SCSI

device  you  have  selected will actually allow the altering of a

page.

This will display information relating to  the  SCSI  adapter  in

your  system.   Under  the heading of the window will be the part

number of the SCSI  host  adapter  you  have  installed  in  your

system.   The rest of the window shows the base I/O port address,

DMA channel, Interrupt channel, and  the  SCSI  ID  of  the  host

adapter.   It  will  also  display  a  number associated with the

firmware level of the adapter board if you are using a  AHA-154xA

or latter.

If you  are  using  this  program  with  the  micro-channel  host

adapter,  the  DMA  channel will not be displayed.  This is not a

hardware configuration option, but is  instead  set  in  the  POS

registers of the system.

Another window  is  also  displayed  to  show  further  operating

information that is stored on the host adapter.  This window will

only  appear  for  the  AHA-154xA  or  latter.   The  information

indicates  whether  the  host  adapter  is  set  to negotiate for

synchronous or not, whether parity checking on the  SCSI  bus  is

enabled, the host adapter DMA write/read pulse widths, the strobe

off time, the bus on/off times.  The window next  to  this  shows

the  status regarding the synchronous negotiation per device.  If

the adapter succesfully negotiated for synchronous protocol,  the

synchronous  period  and  offsets  are  displayed.  If the device

could not support synchronous, the message asynchronus appears.

This window provides detailed information about the  SCSI  device

according  to  what  was  recieved  from  the device via the SCSI

INQUIRY command.  This information as displayed is in accord with

the SCSI-2 specification.

Used to return to MS-DOS.  A message is displayed indicating that

the SCSI is being cleaned up.  You may hear noises emanating from

your SCSI devics as they recalibrate.  This is due  to  the  SCSI

bus  being  issued  a reset, which forces the SCSI devices into a

power on state.

SCSI:  Usually pronounced  'skuzzy'  stands  for  Small  Computer

Systems Interface.

SCSI Adapter:  This is the board that occupies a slot in the host

CPU.   It  is  not a SCSI controller.  It is a device on the SCSI

bus.

SCSI Bus:  This can be thought of as the cable that is  connected

between the SCSI adapter and the SCSI devices.

Target:  This is the device that a SCSI command has been sent to.

Target ID:  The actual ID of the SCSI controller on the SCSI bus.

The  SCSI  host adapter uses target ID 7, for instance.  All SCSI

controllers must have a unique target ID to work on the SCSI bus.

The  target  ID  ranges  from  0  to 7.  The higher the target ID

number, the higher the priority a device has  on  the  SCSI  bus.

This is akin to the drive select on ST-506 drives.

Initiator:  This the same as 'host'.  This  is  the  system  that

issued  the SCSI command.  While it is possible for more than one

computer to be on the SCSI bus, this  feature  is  currently  not

supported.

LUN:  This stands for Logical Unit Number.  All SCSI devices  are

accessed by the target/LUN of the device.  SCSI devices that have

a SCSI controller imbedded in them, like SCSI  hard  drives,  are

assigned  a  target ID and LUN ID of 0.  The LUN can be from 0 to

7.  This is used only if a SCSI  controller  has  more  than  one

device attached to it.

Mode Sense:  This is a  SCSI  command  that  allows  an  external

program call up and modify parameters for that device.

Page:  This refers to the page of the mode  sense  request.   The

programmable parameters of a SCSI device are kept in pages.  Each

page has a unique set of parameters associated  with  that  page.

Consider  a  page  like  a  small window into the heart of a SCSI

device.

Disconnect:  A SCSI device that has to seek  to  read/write  data

sent  to/from  the host system, will notify the host that it must

relinquish the SCSI bus to complete the requested command.   This

action allows the host system to issue another command to another

SCSI device while the other SCSI device is working  on  the  last

command  sent.   This  action  is  best  noticed in the XENIX/OS2

environments.

Reconnect:  This concerns the action of a SCSI  device  that  has

disconnected  from  the  SCSI  bus  due  to  a seek command being

issued.  Once the device has a certain percentage of data in  its

local  buffer, it will request access to the SCSI bus to send the

data to the host system.  This is best  noticed  in  a  XENIX/OS2

operating   environment.   These  operating  systems  are  multi-

threaded (multi-tasking) and do not have to wait for  a  specific

command to be completed before another command is issued.

Priority:  The priority of a SCSI device  is  determined  by  the

target  ID  of the device.  The higher the number, the higher the

priority a device has on the SCSI bus.  This is how SCSI  devices

arbitrate for the SCSI bus.

Bus   Arbitration:     This    refers    to    the    transparent

disconnection/reconnection  of  a SCSI device.  SCSI devices will

get off of the SCSI bus if they have to seek to get  data.   This

allows  the  SCSI adapter to issue other commands to another SCSI

device that is not doing anything.   Another  term  for  this  is

multi-threaded.   A  SCSI device that has data in the buffer will

ask for the SCSI bus again,  and  send  the  data  for  the  last

command that it received.  If two or more devices attempt to grab

the SCSI bus, it falls to the priority of the device to determine

who shall win the bus.

REQ:  The SCSI bus uses this signal for  the  synchronization  of

data  on  the  SCSI bus.  Broken down, it simply is a signal that

means a SCSI device is asking for a byte of data.

ACK:  This SCSI signal is the inverse  of  the  REQ  signal.   It

means  that  a SCSI device has received the byte of data that was

REQuested.

Asynchronous:  This is  a  term  that  applies  to  how  data  is

transmitted to/from the SCSI bus.  For each byte requested, a REQ

signal is generated, and once the byte  has  been  sent,  an  ACK

signal  is  generated.   This allows SCSI devices to send/receive

data at the fastest rate possible for each  given  device.   Data

rates  up  to  3MBytes/sec  are  allowed  for  this  type of data

transfer.  All SCSI devices support this type of operation.

Synchronous:  This is similar  to  asynchronous  data  transfers,

except the ACK and REQ signals are sent in groups.  For instance,

if the offset negotiated for is 7, then the SCSI device will send

seven REQ's and the seven bytes of data and then it will wait for

the 7 ACK's before sending more data.  The pulse  widths  of  the

REQ's/ACK's  can  be very short as there is no need for a ACK for

each REQ.  This allows the  highest  throughput  possible  for  a

SCSI-1 device.  Data rates up to 5MBytes/sec are allowed for this

method of data transfer.   Not  all  SCSI  devices  support  this

feature.

Automatic Read/Write Reallocation:  SCSI  devices  give  you  the

appearance  of  having  no flaws.  While in truth there are media

flaws,  SCSI devices hide  these  flaws  by  keeping  a  list  of

sectors that are not available to you.  These sectors are used by

the SCSI device to replace bad sectors.  If you have reallocation

turned on, the SCSI device will automatically map out bad sectors

as they are found.  This  is  accomplished  in  the  same  manner

during  a  low-level  format.   That  is  how  a  SCSI device can

maintain the same capacity after many years of use.   Degradation

of  the  capacity  will  not  occur  until  the  device  has  run

completely out of spares.

Firmware:  All SCSI devices have programs that interface with the

SCSI  bus  and  the  hardware  of a particular SCSI device.  This

program is contained in a ROM on the SCSI device and is  referred

to as firmware.

Sense Key:  This is referring to the main error code that a  SCSI

drive  will  send  to  the host system.  It is a single byte that

represents a number associated with the list of  error  codes  in

the back of your SCSI hard drive installation guide.

Sense Data:  This is the data associated with a sense key.   This

data  further  qualifies  the  error  that has occurred on a SCSI

device.

Message System:  SCSI uses a message system for initializing  the

device  as  well as action upon reconnecting/disconnecting.  This

message system is a well defined set of bytes in  the  SCSI  ANSI

standard.

Check Condition:  This data tells the host CPU that an error  has

occurred  on the previous command and the SCSI device expects the

host to ask for the sense key/data before  any  further  commands

can  be sent to the device.  Basically, the SCSI device is asking

the host, "Please check my condition?".

Zone:  Some  SCSI  devices  have  the  media  split  into  zones.

Consider  zones  to  be  partitions of the SCSI hard drive media.

Each partition may contain a  different  number  of  sectors  per

track.   These  partitions also contain spare sectors and tracks.

This keeps the SCSI device from putting  all  spare  sectors  and

tracks in one spot on the device.

Identify Message:  This  message  is  used  when  more  than  one

initiator  is  on the SCSI bus.  It allows multiple initiators to

know which device on the SCSI bus is asking for service from that

initiator.  This is usually disabled for SCSI devices as multiple

initiators are not a supported feature.

Prefetch:  This term is used in reference to  SCSI  devices  that

have  the  ability to get data into the local device buffer, that

has not actually been requested.   Some  SCSI  devices  have  the

ability  to  do  a  read  ahead,  while  others have a true cache

mechanism.  In both implementations, the device  will  read  more

data  into  the  device buffer than was actually requested.  This

helps only when data is requested in a sequential manner, as  may

be done in the MS-DOS/OS2 operating environments.

ECC:  ECC is a mechanism employed by many types of  devices  that

allows  the  device  to do error correction on data that has been

misread from the media.  This helps to ensure data integrity  for

a device.  It also adds some overhead.

All SCSI error messages will be displayed at the  bottom  of  the

screen  in  a  window.  All other errors will be displayed in the

center of the screen with the appropriate error message.

Unknown:  An unknown error has occurred.

Recovered Error:  The last command  completed  successfully  with

some recovery action performed by the device.

Not Ready:  The device selected is not ready.  For a  tape  drive

it  may  indicate  that the tape is not loaded in the drive.  For

another type of removable media device it could indicate that the

device is not up to speed or the media is not loaded.

Media Error:  The last command terminated  with  a  non-recovered

error  condition  that  was  probably  caused by some flaw in the

media.

Hardware Error:  The device detected a  non-recoverable  hardware

failure while performing the command.

Illegal Request:  There was an illegal parameter in  the  command

or  in  the  additional data parameters supplied for the command.

The command will be terminated without altering the device.

Unit Attention:  The device has received a hard reset  since  the

last command has been issued.

Aborted Command:  The device has aborted the  command.   You  may

try the command again.

Miscompare:  The source data did not match the data read from the

media.

Could not get mode sense data from SCSI device:  The device  that

has  been  selected  did  not  return  the  requested information

related to the main  menu  choice.   This  error  indicates  that

either  the mode page selected is not supported by this device or

the device is in such a state that the data cannot be sent.

Test Unit Ready:  This is used to test to  see  if  a  device  is

available.

Request Sense:  This command is used to get error  codes  from  a

SCSI device.

Format Unit:  This command is used to do a  low-level  format  of

the SCSI device.

Inquiry:  This command returns information about a  SCSI  device.

Such as the manufacturer, firmware level, and so on.

Mode Select:  This command returns the tunable information  about

a  SCSI  device  so  that it may be modified and sent back to the

device via the mode sense command.

Mode Sense:  This command is used to  send  information  about  a

particular section of tunable parameters to the device.

Start/Stop Unit:  This command is used to start a device that has

spun  down.   It is also used to load a tape when a tape drive is

present in the system.

Read Capacity:  This command  returns  the  capacity  of  a  SCSI

device.   The  capacity  is  returned  in  total  number  of user

accessible blocks on the device.

Read Defect List:  This command returns the  defect  list  for  a

SCSI hard drive only.  Tape drives have no defect list.