TIMESET TM

Version 6.00A

A program to set computers to the world's most accurate clocks

Copyright 1987-1991, Life Sciences Software

8925 271st N.W., Suite 112

P.O. Box 1560

Stanwood, Washington 98292

206-387-9788

"TIMESET" is a trademark of Life Sciences Software TM

CONTENTS

FEATURES OF TIMESET 6.00A                                                ii

SHAREWARE NOTICE                                                         iv

Toll-Free Registration by Modem -- using TSREGSTR                   iv

INTRODUCTION                                                              1

CONFIGURATION                                                             3

CONFIGURING TIMESET FOR HARDWARE CLOCKS                                   6

"Slotless Clocks" and Tandy's SmartClock                             6

CMOS Clocks                                                          6

Expansion Slot Add-On Clocks                                         6

Using CLKDRV10                                                       7

RUNNING THE PROGRAM                                                       9

Dialing                                                              9

Standard and Daylight Time                                           9

Monitor Color                                                        9

Pulse Generation                                                     9

Changing the Configuration                                          10

Additional Help                                                     10

THE TIME DATA STRINGS                                                    12

The Naval Observatory's Data String                                 12

The NIST's Data String                                              13

LINE DELAY MEASUREMENTS                                                  14

Line Delay Correction by NIST                                       14

Line Delay Correction with the Naval Observatory                    16

When Should Line Delay Measurement Be Used?                         16

A Note About MNP Modems                                             17

THE FINAL DISPLAY                                                        19

MORE ABOUT ACCURACY                                                      19

Recommendation:  Get RIGHTIME.COM                                    20

TIMESET 6 AND THE WORDPERFECT LIBRARY                                    21

PROGRAMMING NOTE AND ACKNOWLEDGEMENTS                                    22

CUSTOM PROGRAMMING SERVICES AVAILABLE                                    23

DISCLAIMER                                                               24

YOUR SUPPORT IS IMPORTANT                                                24

TIMESET 6 MAIL REGISTRATION FORM                                         25

- i -

FEATURES OF TIMESET 6.00A

Version 6.00A of Professional TIMESET is a top-to-bottom revision and

major enhancement of the classic program for setting computer clocks with

high accuracy and precision with a telephone call to the most accurate

time sources in the world.  The program has many new features, but one of

the most notable is its ability to get time not just from one time source,

but from two.  TIMESET 6.00A does alone what used to require two different

programs from two different suppliers.  Yet the price has not changed,

it's still $35.

All the popular features of earlier TIMESET versions are still here,

including the big running time display, interactive or automatic

operation, immediate timesetting or after a countdown to zero seconds

(done offline), and rejection of noise-corrupted data strings.

The new features include:

1.   Accurate time from two alternative sources.  TIMESET 6.00A can set

the computer clock by calling cesium beam atomic clocks of either the

U.S. Naval Observatory in Washington, DC or the National Institute of

Standards and Technology (formerly the National Bureau of Standards)

in Boulder, Colorado.  Previous versions could call and process time

signals only from the Naval Observatory.  Correction for line delay

can be made with both  services.  If line delay is not requested,

estimated delay values are used for both services that are very close

to what could be  measured for most localities in the continental

United States.

2.   Line delay correction with any type of Hayes- compatible modem.

Previous versions could get this correction only through the Naval

Observatory with modems capable of remote digital loopback testing

through the &T6 and &T0 commands.

3.   Very fast action.  It normally takes only 2-6 seconds online,

depending on the service called, if no line delay measurement is

requested.

4.   Very high accuracy.  TIMESET 6.00A even measures the time it requires

to process the received data string, and it corrects for that

internal delay before setting the time.  String processing is one of

the slowest things a computer does, and the time it takes is

measurable.  Yet it's overlooked by every other timesetting program

we know of.

5.   Self-configuration, with a detailed help screen available for each

configuration item and intuitive operation, making this the least

manual-dependent version of TIMESET so far.  The help screens amount

to an online tutorial, and you may not even need the manual.

Previous versions of TIMESET needed a separate  program to configure

for serial port, modem, time zone, time season, telephone dialing

- ii -

procedures, type of display, etc.  All that is self-contained in

Version 6.00A and available at the press of a function key.  The

configuration is written to TIMESET's own source file on disk (there

is no separate configuration data file).  Most choices are made by

pressing arrow keys, and editing functions are available for the few

configuration items that require typing.  Everything is done on a

single configuration screen.

6.   One-key time season adjustment.  The press of a single key toggles

the configuration between daylight or standard time and

simultaneously adjusts the computer clock to reflect the time season

change.  Likewise, a single key lets you override the program's video

display configuration and toggle between color and forced monochrome

display (very useful for some laptop computers).

7.   Numerous fixes, including substantial changes to eliminate connection

and synchronization problems with certain modems.  The program now

works with PC, XT, AT, PS/2, and better computers -- on serial ports

1 to 4, at dialing speeds from 1200 to 38400 bps, with local modem

speed locked or unlocked.  It automatically detects a PS/2 and

assigns the correct COM3/COM4 port addresses for that type of

computer.

8.   Greatly reduced file size -- under 52 kbytes, less than half the

combined total of 105 kbytes for TIMESET plus CONFIGTS in version 5.3

and smaller than other programs that can call only one of the two

services.

9.   Completely redesigned displays to convey more information, plus the

big running digital clock display that has always been part of the

program.  TIMESET has always been a good-looking program, because

esthetics and functionality go hand in hand.  This version is the

most esthetically pleasing yet and by far the most functional.

10.  Advance notice of time season changes.  Version 6.00A can notify you

weeks in advance when a time season change is coming.  And each time

you use it on succeeding days it can tell you the number of days

remaining before the change actually takes effect.  On the eve of the

season change it prints a special announcement to remind you that

it's almost time to reconfigure for daylight or standard time.  On

the day of the change it tells you, in effect, "If you haven't

already reconfigured, you'd better do it now" (which you can do by

merely pressing a function key, with simultaneous automatic

adjustment of your clock).

We think all these enhancements put version 6.00A of Professional TIMESET

far ahead of any other program for computer timesetting.  TIMESET 6.00A is

not a "bait" program intended to entice you to send away for something

we're holding back that's more complete, more professional, and expensive.

It IS the complete professional version.  Nothing has been held back.

It's all here.  You already have the maximum product.  And it's still only

$35.

- iii -

SHAREWARE NOTICE

TIMESET Version 6.00A is a copyrighted commercial program developed and

owned by Peter L. Petrakis (doing business as Life Sciences Software of

Stanwood, Washington and formerly of Annapolis, Maryland) and is

distributed as shareware.

"Shareware" does not mean free and it does not mean public domain.

Shareware is a method of software distribution that lets you try a program

out and pass it along to others to try out.  Those who don't find the

program useful are free to junk it (although they could pass it along to

somebody else).  Those who like TIMESET, find it useful, and continue

using it after a reasonable trial period of 21 days are expected to pay

$35 for each copy in use (plus 8 percent sales tax for residents of

Washington State).

Payment makes you a registered user.  Registration for TIMESET version

6.00A brings benefits of technical help by telephone, a printed manual, a

disk copy of the program containing any recent changes, and a 20 percent

discount on the next major upgrade.

Organizations such as corporations, other enterprises, government

agencies, educational institutions, etc. should not expect to pay $35 for

a single copy of TIMESET then put more than one copy into use.  That is as

unfair to an author as buying a single copy of his book and using the

Xerox machine to duplicate it for the office staff.  Payment of $35 is

required for each copy of TIMESET in use within an organization.  That's

not unreasonable, and discounts are available for quantity.

If your organization requires the formality of purchase order and invoice,

don't let that be a deterrent to payment and registration; contact your

purchasing agent then let us know the procedures.  We've worked that way

with several organizations.  (Please contact us about a discount for

organizational use of more than six copies.  The discounting rate will

depend on how many additional copies your organization plans to use.)

We concede that there is no practical way to make anyone pay for

shareware.  Few shareware writers have the stomach for that sort of thing

anyway. Shareware is an honor system.  And it's a valuable system that is

bringing many high-quality programs at modest prices from talented people

who work diligently to evolve and perfect their products.  It should not

be exploited through abuse or forgetfulness.

Toll-Free Registration by Modem -- using TSREGSTR

Experience suggests that many people intend to pay for a shareware program

they continue using.  But they postpone it.  Then they forget to do it.

Eventually they might not even have the registration form and the address

around anymore.  Because that's human nature, the TIMESET version 6.00A

- iv -

package includes a companion program, TSREGSTR, that lets you register by

modem -- with the phone call paid by us.  It's simple, fast, and

convenient, allowing good intentions to be carried out promptly before the

memory cogs slip.  All you need to do is type in some information -- your

name, address, phone number, etc. -- and press a key to send it to our

electronic mailbox automatically.  A screen editor in TSREGSTR lets you

move around freely in the data entry field so you can make any necessary

corrections before pressing F10 to send the information.  Your phone

number gives us a chance to call you back and confirm the order and take

care of problems such as a garbled transmission.  We will then send you

the latest version of TIMESET, a printed manual, and an invoice.

It takes only a few seconds and the call is free, so there is no real

reason not to do it if you keep and use TIMESET.

To use TSREGSTR you need a 1200 or 2400 baud modem.  To keep this limited

purpose program simple, we did not provide any support for serial ports

other than COM1 and COM2 or for high-speed locked dialing (although

TIMESET itself is much more versatile and can handle COM1-COM4 and locked

or unlocked dialing speeds up to 38,400 bps).

(NOTE:  If you pass the TIMESET version 6.00A package along to others,

please be sure to include TSREGSTR.)

If you prefer to register by mail, using a check or money order, please

use the form at the end of this document.

Our voice line, if you have any general questions about registration,

licensing, quantity discounts, or the program's capabilities, is

206-387-9788.

- v -

INTRODUCTION

One of the least reliable components in any computer is the clock.

Computer clocks are notoriously inaccurate, gaining or losing as much as

several seconds a day, and it's not unusual for them to be off by minutes

within a few weeks or even days after they are set.  This doesn't matter

to everyone who uses a computer, of course, but it matters a great deal to

a lot of people.  This includes businesses that need an accurate record of

the time of data entry, sometimes for legal reasons.  Technological and

research organizations need accurate time for numerous serious purposes

related to physical measurements.  Radio and television stations and cable

TV systems need a dependable source of time so they can switch in and out

of networks or link up with remotes at the right time.  These and many

other organizations have found previous versions of TIMESET not only

useful but necessary.

TIMESET is used in universities, NASA, the Jet Propulsion Laboratory,

the Federal Communications Commission, Federal Aviation Administration,

Veterans Administration, the military, police departments, municipal

agencies, the Federal Reserve Bank of New York, commercial banks, power

companies, Hewlett-Packard and similar high-tech companies, radio and TV

stations, cable networks, and by computer bulletin board operators,

satellite tracking hobbyists, professional and amateur astronomers, and

many people who simply want to know the time is at least close to what

their computer says it is.

There are many TIMESET users of all kinds in the United States and

abroad who are dismayed that the clock in their expensive computer, which

is driven by a high-frequency crystal-controlled oscillator, cannot keep

time nearly as well as a ten dollar wind-up watch from K-Mart.  They think

a computer should provide a standard for setting other timepieces, not the

other way around, and they are right.  Nor are all of them necessarily

interested in split-second accuracy for their computer clocks; many users

simply want a convenient way to set their computer to a standard they know

is right.  TIMESET provides that convenience, and the Naval Observatory

and NIST clocks are as right as any clocks can be.

The first version of TIMESET was released to computer bulletin boards in

July 1987.  It was to be the only version (it didn't even have a version

number), but the response was astonishing.  Phone calls came day and

(alas) night from people all over the country who were either delighted

with the program or had problems getting it to work with their systems,

particularly their modems.  That interest has stimulated continued

development ever since, and successive versions have been released to make

the program work reliably for more and more people and to add new

features, many of them suggested by users.  It was very gratifying to see

Version 5.3, the most recent version until now, written up by PC/Computing

Magazine in its October 1990 issue in an article devoted to good ways to

enhance a DOS computer's operating environment.

- 1 -

Version 6.00A is a top-to-bottom revision that includes, for the first

time, the ability to get atomic time from either of the world's two

preeminent sources for timing by telephone -- the National Institute of

Standards and Technology (NIST) and the U.S. Naval Observatory.  There are

programs,including previous versions of TIMESET, that can call one or the

other of these two services, but TIMESET 6.00A so far appears to be the

only one that can dial both of them (we expect to be corrected if that's

not right or ceases to be so) and make full use of the capabilities of

each.  Thus users from one end of the country to the other can now reduce

phone costs by using the nearest service.

Users of previous versions will notice numerous other changes.  The

program looks and feels completely different.  Though it has more

features, it is much smaller on disk.  Configuration procedures are now

built into the program itself (there is no external configuration data

file and no external configuration program) and are very easy to do.  You

can enter configuration mode at any time to change things by pressing a

function key at the main menu.  Help for each configuration item is

available by pressing another function key when the item is highlighted on

the configuration screen.

You can now toggle between standard and daylight time configuration by

pressing a function key at the main menu -- and it simultaneously sets the

computer clock one hour earlier or later depending on the season.  You can

generate a usable pulse for synchronizing an external device at the

instant of timesetting by pressing another function key before dialing.

Command line options are displayed by typing TIMESET followed by a space

and a question mark at the DOS command.  A quick overview of internal

command options is available by pressing a function key at the main menu.

Finally, TIMESET 6.00A can measure and correct for line delays with either

the Naval Observatory or the National Institute of Standards and

Technology.

CONFIGURATION

Because configuration data for TIMESET 6.00A are written directly to its

source file on disk, the program needs to know where its source file is;

it reads its configuration each time it's loaded and each time you change

the configuration.  With DOS versions 3 and up it's no problem; those

versions allow TIMESET to find out where it came from, making it possible

to configure or run the program from any drive and directory on the system

path (which is set up with the DOS PATH command).  That's not the case

with earlier versions of DOS.  With those DOS versions, TIMESET 6.00A

needs to be told where its source file is.  That's done by using the DOS

SET command to put its location in the DOS environment, like this:

SET TIMESET=[drive]:\[directory\]TIMESET.COM

The drive, directory (if any), and ".COM" are essential.  If you're

using a version of DOS earlier than 3.0 and are going to be using TIMESET

- 2 -

6.00A regularly, it makes sense to put that command in your AUTOEXEC.BAT

file, which should also include a path to its location using the PATH

command.  Otherwise you'll always have to be in the drive and directory

where TIMESET is located before you can use it.

THIS IS IMPORTANT!!:  If the copy of TIMESET version 6.00A that you

received has never been configured before, it will automatically load in

configuration mode so you can tailor it to your own system.  If you got it

from a computer bulletin board, however, the chances are it will have

already been configured, probably by the bulletin board operator who

tested it before making it available for downloading or by the person who

uploaded it to the bulletin board.  In that case, it will load in

operating mode.  In fact, if it was configured to run in non-prompted

(automatic) mode it will immediately start dialing.  For this reason, if

you obtained TIMESET 6.00A from a computer bulletin board or other

indirect source, you should put /C on the command line the first time you

run it, like this:

TIMESET /C

This will force the program to go to configuration mode as soon as it's

loaded so you can tailor the configuration to your own system.  Using /C

will not be necessary if your copy of TIMESET 6.00A came directly from

Life Sciences Software, because we do not preconfigure the program before

shipping it.

The configuration screen comes up with values for phone numbers, time

season, serial port, baud rate, etc. already listed.  Please understand

that if TIMESET 6.00A has never been configured before, those values are

not yet in the program file's configuration field; they are what WILL be

written to that field if you press F10.  On the other hand, if TIMESET has

already been configured, they represent the actual current configuration

that the program read from its own disk source.

After the program has been configured, you can always enter

configuration mode to check or change your settings by pressing F2 at the

main menu.  If you are just checking and not changing anything, you don't

have to press F10 to resave what's on the screen.  Instead, you can get

back to the main program by pressing F2 again.  If you make a change, you

should of course press F10 to save it.

If you are configuring for the first time and the values you see are

what you want, press F10 to save them to TIMESET.COM on your disk.  There

is a good chance, however, that you'll need to change some of the default

values so they'll correspond to your actual computer/modem setup.  That's

what configuration of version 6.00A is mostly about -- changing

configuration values that are already displayed.  It's very easy to do,

and it's likely that you'll only need to make minor changes in two or

three items.

Each configuration item has its own help screen, which can be displayed

by pressing F1 when that item is highlighted.  The highlighting can be

- 3 -

moved up or down by pressing the up or down arrow key on your keypad.

Before doing any configuring, we strongly recommend that you use the up

and down arrow keys to go through the entire list of configuration items

one by one and press F1 to get the help screen for each.  That will give

you a good overview of what the configuration procedure is about.  The

help screens collectively amount to an online configuration manual.  In a

way, they ARE the manual, because few particulars are discussed here.

Version 6.00A has been designed to reduce the need to consult a printed

manual to the minimum.  As result, this manual can devote more attention

to some theoretical matters that are worth knowing about.

Everything is done on a single screen, mostly by pressing the right,

left, up, and down arrow keys on your keyboard.  Pressing the up or down

arrow key highlights a new configuration item.  The right and left arrow

keys toggle the options that are available for a given configuration item.

For example, on the configuration line for baud rate, you can go upward

from 1200 baud -- 1200, 2400, 4800, etc. -- to 38400 baud by repeatedly

pressing the right arrow key.  Pressing the left arrow key takes you in

the opposite direction.

Only four of the configuration items involve typing anything, and even

that might not be necessary if the default or preexisting settings for

those items are what you want.  Three of the items that may require typing

or editing pertain to telephone numbers, the fourth pertains to the user's

modem initialization string.  All the other items are toggled with the

right-left arrow keys.  To change the phone numbers or the modem

initialization string, you have to get into editing mode.  That's done by

pressing Return when the editable configuration line is highlighted (a

message also appears telling when a line can be edited).  In editing mode

you can insert characters or delete or type over existing characters.  A

description of editing functions (inserting, overstriking, deleting,

cursor movement, etc.) pops onto the screen at the right as soon as you

press Return, so not much needs to be said about them here.

For example, the default phone number for the Naval Observatory is shown

as 12026530351.  If you live in Washington, D.C., the Naval Observatory is

a local call and you obviously need to get rid of 1202.  You do that by

scrolling to the appropriate line with the up or down arrow key, pressing

Return, pressing the Delete key four times, then pressing Return again.

Or suppose your company requires you to add an account number to the end

of a long-distance phone number when you dial out.  Scroll to the phone

number, press Return, press the End key to move the cursor to the end of

the phone number, type in your account number, and press Return again.

Insert a comma to generate a pause if you need one.  The default pause in

virtually all modems is 2 seconds for each comma.

(IMPORTANT:  Please just modify  the phone numbers that are there, and

NEVER configure to dial our phone number.  The program will refuse to run

if you put our number in it, and we don't have an atomic clock anyway.)

Editing works the same way for the user's modem initialization string.

It's especially important to study the help screen for that one, because

- 4 -

it has some crucial information about what to use with different types of

modems.

Configuration for the type of clock in your computer is a special topic

and is discussed in the next section.

Be sure that no configuration fields are left blank and none contains

the word "(edit)".  The word "(edit)" appears on a couple of the editable

configuration items when you press the right or left arrow key enough

times.  When you see it, you are supposed to get into editing mode and

replace it with actual configuration data.  Rest assured that the program

will not work properly if configuration fields are left blank or contain

"(edit)".

When you've checked or changed all the configuration items and are

satisified that everything is correct, press F10 to save the configuration

to TIMESET's source file on disk.  As soon as the configuration is saved,

the program jumps into operating mode, reads its new configuration, and is

ready to call either of the time services.

CONFIGURING TIMESET

FOR HARDWARE CLOCKS

Every computer comes equipped with at least one clock, called the DOS

clock/calendar, which can be read or set with the TIME and DATE commands

in DOS or by a program such as TIMESET.  Its major limitation is that it's

volatile; that is, the time and date settings are lost whenever the

computer is turned off, and both have to be reset each time the computer

is restarted.

Because of this volatility, battery-powered clocks have been developed

to maintain the current time and date while the computer is off and pass

them to the DOS clock/calendar when the computer is turned back on.  There

are three kinds of nonvolatile clocks:  (1) the CMOS clock found on the

motherboards of AT and later computers, (2) add-on clocks that plug into

an expansion slot in the computer, and (3) so-called slotless clocks that

plug into a socket under the ROM chip or some other socketed chip.

TIMESET can work with nearly all of the first and second types, but not

with the third.

"Slotless Clocks" and Tandy's SmartClock

TIMESET can work with CMOS clocks and most add-on clock boards.  However,

it will not work directly with Tandy's SmartClock nor with slotless

clocks.  With those clocks you'll have to configure to set the DOS

clock/calendar with TIMESET then later on use your own clock's software to

reset the add- on clock.

- 5 -

CMOS Clocks

Depending on the version of DOS that is installed in a computer, a CMOS

clock may need special software to be reset or to make it update the DOS

clock/calendar.  With DOS 3.3 or later, you need no special software,

because those DOS versions automatically adjust the CMOS clock when you

change the DOS clock/calendar; changes made in the DOS clock/calendar are

simultaneously made in the CMOS clock as well.  That automatic

synchronization doesn't occur with DOS versions earlier than 3.3, however.

Therefore, if your computer has a CMOS clock, the version of DOS you're

using will determine how to configure TIMESET 6.00A for clock type.  If

it's DOS 3.3 or later, you should configure TIMESET to set only the DOS

clock/calendar, because the operating system will automatically update the

CMOS clock when TIMESET changes the DOS time and date.  If you have an

earlier version of DOS, you will have to configure TIMESET 6.00A so it can

directly set both the DOS clock/calendar and the CMOS clock, thus

eliminating the need to run DOS's SETUP program.  You can select the

correct option by using the right or left arrow key in configuration mode.

If you need help, press F1 while that item is highlighted.

Expansion Slot Add-On Clocks

Add-on clocks that plug into expansion slots present a mixed situation.

Some require you to run a special program each time you want to update the

DOS clock/calendar from the add-on clock or update the add-on clock from

the DOS clock/calendar.  Others come with a memory-resident program or

driver that takes care of these time adjustments between the two clocks

automatically, making them behave very much like the CMOS clock under DOS

3.3 or higher.  Configuring TIMESET 6.00A for these add-on clocks

therefore depends on the kind of software that comes with the clock.

Better known brands of add-on clocks these days come with memory-

resident software to keep the add-on clock and the DOS clock/calendar

synchronized automatically.  An example of such a program is ASTCLOCK, for

managing the clock on AST's Six-Pack multifunction board.  Running

ASTCLOCK with the /R switch on the command line (ASTCLOCK /R) makes it

memory-resident so it can automatically keep the two clocks synchronized.

If your add-on clock came with a memory-resident driver program or a

driver that goes in your CONFIG.SYS file, install that driver and

configure TIMESET to set only the DOS clock/calendar.  Your driver

software will take care of resetting the add-on clock when TIMESET sets

the time and date in the DOS clock/calendar.

Using CLKDRV10.SYS with Board Clocks

There are a number of slot-type add-on clocks that do not come with

memory-resident software and therefore can't pass time and date changes

back and forth between the two clocks automatically.  These clocks, with

mail-order prices ranging from $7.50 to $15, have obscure brand names and

- 6 -

often no brand name at all.  Despite their low cost and generic nature

they hold accurate time surprisingly well.

The major limitation of these cheap add-ons is lack of memory-resident

software to allow automatic movement of time and date between the two

clocks.  Instead they require you to run a special program to update the

add-on clock each time you change the time or date in DOS (e.g.,

SETCLOCK), or to update the DOS clock/calendar from the add-on clock

(e.g., GETCLOCK).  With some of these clocks, both functions are handled

by a single program, with the desired function chosen with a command line

switch.  Having to go into DOS to run external programs like these is

utterly impractical for a program like TIMESET, which requires timesetting

to occur in both clocks at virtually the same instant for maximum

accuracy.  Furthermore, some of them insist on prompting for the time,

which of course means the time will be wrong no matter how fast a typist

you are.

For people with these obscure brands and that clumsy kind of software,

the TIMESET 6.00A package includes a small driver called CLKDRV10.SYS,

written by Tom Strickland of Alexandria, Virginia.

To use it, put the driver in your boot directory and the following line

in your CONFIG.SYS file:

device=clkdrv10.sys

Loading CLKDRV10.SYS in effect erases the barrier between your DOS

clock/calendar and your add-on clock and eliminates the need for any other

software to manage the two clocks and keep them synchronized.  Thus, if

you change time or date in DOS, CLKDRV10.SYS sees to it that the add-on

clock gets the same change, and if you ask for time or date in DOS or a

program, CLKDRV10.SYS automatically updates the DOS clock/calendar from

the add-on clock an instant before DOS displays the time or date.  With

CLKDRV10.SYS installed, you must configure TIMESET to set only the DOS

clock/calendar, and let CLKDRV10.SYS take care of updating the add-on

clock.

CLKDRV10.SYS is designed to work with board-type add-on clocks that have

a base clock address of 240h, 2C0h, or 340h, and it automatically checks

to see which of those ports is being used by your clock.  Nearly all

expansion slot add-on clocks use one of those three base addresses, and

some of them allow you to choose between at least two of them by means of

a jumper on the clock board.  If your clock does not use one of these

three port addresses (which is possible but infrequent), CLKDRV10.SYS will

not load and you will have to be content with letting TIMESET adjust only

the DOS clock/calendar.

If you install CLKDRV10.SYS, you should put your clock's regular

software aside, because CLKDRV10.SYS has its own method of managing the

add-on clock.  You won't need your original software anyway, because

CLKDRV10.SYS makes your add-on clock directly accessible through the DOS

TIME and DATE commands or from any program. CLKDRV10.SYS is mainly for

- 7 -

people whose add-on clock did not come with its own memory-resident driver

program.  If your add-on clock came with its own memory-resident software,

use it rather than CLKDRV10.SYS.

It's important to know when not to use CLKDRV10.SYS.  You should NOT use

it when: (1) your computer has a CMOS clock, or (2) you have an add-on

clock board that came with its own memory resident software to accomplish

what CLKDRV10.SYS does, or (3) you have only the DOS clock/calendar and no

battery-powered clock board at all.  Under no circumstances should you

load both CLKDRV10.SYS and another driver that accomplishes the same thing

-- the two drivers will get into a contest that could paralyze the system.

Also, CLKDRV10.SYS will be of no help if you have Tandy's SmartClock or

a "slotless" clock.  (The manufacturers of those clocks should be

pressured to develop their own memory-resident drivers for them.)

RUNNING THE PROGRAM

OPTIONS AT THE MAIN MENU

Dialing

If you configured for interactive mode, dialing starts when you press W,

Alt-W, B, or Alt-B.  W stands for Washington (Naval Observatory) and B

stands for Boulder (NIST).  Pressing W or B alone gets the time without

any adjustment for measured line delay.  In combination with the ALT key,

pressing W or B gets the time with a line delay adjustment from Washington

(Alt-W) or Boulder (Alt-B).  Line delay measurements are discussed in more

detail later on.

If you configured the program to run in noninteractive mode (no

prompts), you won't have to press any keys; TIMESET 6.00A will load,

immediately dial the time service you chose, set the time, and exit to

DOS.  The choice of interactive or noninteractive mode is yours.  If you

plan to have TIMESET in your AUTOEXEC file so it can update you computer

clock during the boot- up process, it's best to configure for

noninteractive mode so timesetting will be automatic and booting can

continue after the clock has been set.  TIMESET skips the main menu when

it's in noninteractive mode and there are no pauses for input.

Standard and Daylight Time

One way to switch between daylight and standard time is to press F2 at the

main menu to get into configuration mode, scroll down to the time season

line, toggle daylight or standard with the right or left arrow key, and

press F10 to save the change.  You'll need to do it that way during

initial configuration since you are automatically put into configuration

mode when you run a fresh copy of TIMESET 6.00A, and the program won't run

until it's fully configured.  However, after you have configured the

- 8 -

program and it's time to reconfigure for daylight or standard time there

is a very simple way to do it from the main menu:  just press F7.  That

will not only change change the configuration, it will simultaneously

reset your computer clock for the new season.  You should do that

reconfiguration before dialing one of the time services on the day the

time season changes.

Monitor Color

Although you can specify color or monochrome display during configuration,

a function key toggle is also available at the main menu for controlling

monitor color.  Pressing F8 at the main menu switches the display between

color and monochrome.  You may need to force monochrome display this way

if you run TIMESET 6.00A on a laptop computer that shows colors as shades

of "grey"; some colors have poor contrast on certain laptops.

Pulse Generation

F9 is a special key that will probably not be needed by many people.

Pressing it at the main menu allows a single pulse to be generated at pin

1 of the parallel printer port at the instant of timesetting.  The pulse

can be used to synchronize external devices, which could include

specialized (and expensive) clocks.  It can also be used for calibrating

instruments.  According to the NIST, its time mark typically has

repeatability of about 1 ms from day to day when obtained with line delay

correction.  This allows the delay-corrected pulse generated by TIMESET to

be used in combination with a digital counter to check or calibrate an

oscillator with high accuracy.  The uncertainty would be on the order of 1

part in 100 million for oscillation counts at 1/second made a day apart

(0.001 second repeatability divided by 86,400 seconds/day).

For reference, here are the characteristics of the pulse:

Change:  ~5 volts

Direction:  from ~+5 volts to 0 volts (relative to pin 25)

Duration:  ~55 ms (controlled by the system timer)

Depending on the external device's requirements, it may be necessary to

invert the pulse or modify it in other ways with appropriate electronic

circuitry (which is beyond the scope of this manual).

A companion program, PULSEGEN, is available for $15 from Life Sciences

Software for generating continuous pulsing at the parallel port at 1/min,

1/sec, 10/sec, or 100/sec with any computer, and up to 1000/sec with

computers equipped with slot-type add-on clocks using base I/O port

addresses of 240h, 2C0h, 340h, or 3C0h and capable of millisecond

resolution (probably all such clocks are, although their port for

thousandths is usually not used since the DOS clock resolution is no

better than 0.01 sec).  The pulse widths produced by PULSEGEN depend on

CPU speed and are in the order of microseconds; i.e., no intentional delay

- 9 -

is inserted between pulse on and pulse off.  The running time display is

to three decimal places and is written directly to the video buffer for

maximum speed.  If the computer clock has been freshly set with TIMESET,

the pulses, which can be used for driving external devices continuously,

will occur approximately in real time, subject to clock drift.

Pulses should not be generated at the parallel port if a printer is

attached.

Changing the Configuration

If TIMESET 6.00A is configured for interactive operation and you need to

change or check the configuration, press F2 at the main menu to get the

configuration screen.  If you make any changes, be sure to press F10 to

save them, otherwise you can press F2 to go back to the program without

making any changes.  Getting to the configuration screen can require an

extra step if you have already configured TIMESET to run in automatic

mode, because it then bypasses the main menu and starts dialing as soon as

you run it.  In that case you can press the Backspace key to interrupt

dialing and go to the main menu, where you can press F2 (there's plenty of

time to do that before the call goes through).  A better alternative is to

load TIMESET 6.00A with /C on the command line, which forces it to come up

in configuration mode so you can make your changes.

Additional Help

Pressing F1 at the main menu brings a help screen that summarizes your

keyboard options within the program.  There are also some command line

options for TIMESET 6.00A, including options for temporarily overriding

some of the configurations or delaying dialing until a specified time.

You can see these command line options by typing TIMESET ? at the DOS

prompt.

THE TIME DATA STRINGS

The data strings from the Naval Observatory and the National Institute of

Standards and Technology have a few similarities and several differences.

The pattern for both services is a string of data that announces the next

second, followed by a brief pause, then a character that marks the second.

This goes on endlessly, every second, 24 hours a day, year round.  When

you are connected to either service with TIMESET 6.00A these data strings

will be displayed on your screen as they are received for as long as it

takes to get a clean data string to set your computer clock.  With clean

lines and no line delay measurement, that's about 2 or 3 seconds with the

Naval Observatory and about 5 or 6 seconds with the NIST.  If you request

line delay measurement, it takes about 11 seconds online with the NIST and

about 21 seconds with the Naval Observatory.  These times will increase if

- 10 -

there is a lot of noise on the phone lines, because TIMESET will reject

data strings that are corrupted by noise and don't match the required

pattern.

Both services limit your time online.  The Naval Observatory gives you a

maximum of 60 seconds.  The NIST gives you up to 55 seconds but will

reduce that to 15 seconds if the service is in heavy use.  Except for

extremely noisy lines (which may indicate a problem with your own phone

system), these times are more than adequate for TIMESET 6.00A.

The Naval Observatory's Data String

The pattern of the Naval Observatory's time data string is:

JJJJJ DDD HHMMSS UTC (pause) *

JJJJJ is the last five digits of the julian day, which is simply a

count of the number of days since January 1, 4713 B.C.  The first two

digits, 24, are left out because they will not change for a long time

to come (140 years or so) and can be assumed in calculations.

TIMESET uses the julian day to calculate the current month, day of

the month, and year.

DDD is the day of the year (number of days since December 31 of the

previous year).  It is only displayed by TIMESET, at the final

screen.

HHMMSS is what the hours, minutes, and seconds will be when the time

mark -- the asterisk (*) -- is sent.

UTC stands for Universal Time Coordinated, and it means that the

values for julian day, day of the year, and the time all apply to 0

degrees longitude, the longitude of the Royal Observatory at

Greenwich, England.  TIMESET corrects these values for your time zone

and season before setting your computer clock (unless you want your

computer on universal time, which is desired by some in the aviation

industry and the military, for example, and have configured TIMESET

for it).

The NIST's Data String

The NIST time data string is more complex:

JJJJJ Yr-Mo-Da HH:MM:SS Sn L UT1 msADV UTC(NIST) (pause) * (or #)

Again, UTC means the time and date values all pertain to Greenwich,

England.

- 11 -

JJJJJ means exactly what it means in the Naval Observatory string.

Unlike the Naval Observatory, the NIST does not provide the day of

the year, but TIMESET 6.00A calculates it from this number and

displays it on the final screen.

Yr-Mo-Da means current year, month, and day of the year.

HH:MM:SS means hours, minutes, and seconds.

Sn is a two-digit number that indicates whether the continental

United States is on standard or daylight saving time or is

approaching daylight or standard time.  Except for a 48-day period

before a time season change, the number is either 00 (standard time)

or 50 (daylight time).  In the fall, the number begins to decrement

daily, starting 48 days before standard time officially begins in

late October, when it reaches 00.  In the spring, 48 days before

daylight saving time officially begins, the number jumps to 98 then

decrements daily until it reaches 50, indicating that daylight saving

time is in effect.  During the periods when Sn is decrementing,

TIMESET 6.00A prints a special message on its final screen announcing

the number of days (including the current day) before the time season

is due to change in the continental United States (the days are based

on UTC).  At all other times it displays the current time season in

the continental U.S. on the final screen.

L is a flag that indicates whether a leap second will be added or

subtracted at midnight on the last day of the current UTC month to

compensate for irregularity in the earth's rotation.  If a leap

second is to be added, the number is 1.  If L is 2 it means that a

second will be subtracted at midnight on the last day of the current

UTC month, but the NIST says a negative leap second is not likely to

occur any time soon.  The L flag is on for the entire month before a

second is added or subtracted, which will usually be in June or

December.  Most of the time the L flag is set at 0, meaning no

change.  The flag is merely an announcement and is not needed by

TIMESET 6.00A.

UT1 is a correction factor for converting UTC to an older form of

universal time that is still used in navigation.  It is the tenths of

a second to be added or subtracted from UTC and is preceded by the

appropriate sign.  It is only displayed by TIMESET 6.00A.

msADV is the NIST's line delay correction and is discussed as a

separate topic in the next section.

- 12 -

LINE DELAY MEASUREMENTS

It is physically impossible for the time mark character at the end of the

data string to arrive at the same instant it leaves the time source.

There is always a delay, and it is the sum of three components:  (1) the

time it takes for data bits moving at the speed of light to travel the

distance between the time source and your computer, plus (2) the time it

takes for the time source's modem to serialize and send the time mark

character, plus (3) the time it takes your modem to process the incoming

data and feed it to your computer.  With land line communication in the

continental United States, the time the signal spends traveling between

the modems is almost negligible compared to the processing delay that is

inherent in modems.  With a satellite link, however, the time the signal

spends traveling becomes very significant and accounts for most of the

lag.

Both the NIST and the U.S. Naval Observatory are equipped for

measurement of line delay, but each uses a different method.  TIMESET

6.00A can handle either method.  When TIMESET 6.00A asks for line delay

measurement from the NIST, the NIST itself measures the delay and advances

the departure of the time mark to compensate so the mark arrives on time.

However, with the Naval Observatory, TIMESET 6.00A itself measures the

line delay by means of a remote digital loopback test, in which the

program sends a stream of characters to the Naval Observatory and measures

the time for the Naval Observatory's modem to echo them back.  It then

captures a time data string and adds the measured one-way delay to the

time as a correction before setting your computer's clock.

Notice the essential difference in methodology here:  delay measurements

with the Naval Observatory are made by TIMESET; delay measurements with

the NIST are made by the NIST.

Because of limitations in standard computers, the line delay

measurements the NIST makes are inherently more precise than the ones

TIMESET 6.00A makes with the Naval Observatory.  The NIST's equipment can

measure line delay to the tenth of a millisecond.  In contrast, TIMESET

must round the line delay it measures with the Naval Observatory to the

nearest 0.01 sec since that's the smallest unit of time that a standard

computer can accept as a correction.  This is a 100-fold difference in

precision.  It's important to realize, however, that this difference in

precision has absolutely no effect on the accuracy of timesetting in your

computer.  Precision and accuracy are not the same thing.  Furthermore,

the "granularity" in computer clocks, which is based on the ticking rate

of the system timer (18.2 times per second), limits the accuracy with

which they can be set to within about 0.055 sec of true time, which means

that timesetting accuracy ends up the same with either service.  (Delay

correction and accuracy are discussed in more detail later.)

- 13 -

Line Delay Correction by NIST

msADV in the NIST's data string is the milliseconds (ms) that the

departure of the time mark was advanced to compensate for line delays.  If

no line delay measurement is requested, the NIST routinely advances the

departure of the mark by 45 ms (0.045 sec), which is displayed in the data

string as 045.0.  The standard 45 ms is an estimated value for the United

States that includes the 8 ms it takes to send all the bits of the time

mark character at 1200 baud, plus an assumed 7 ms for travel from the NIST

to the average user in the United States, plus 30 ms to compensate for the

inherent delay in 1200 baud modems (the modem delay is an average value

for several modem brands).

If an actual line delay measurement is requested by initiating the call

with Alt-B, the NIST itself calculates the delay, then advances the

departure of the time mark to compensate for the measured one-way lag so

the time mark can arrive at your computer at the right instant.  As soon

as the NIST has measured and adjusted for line delay, the time mark

changes from an asterisk to a pound sign (#).  TIMESET watches for the

pound sign and sets the computer clock the instant it is received.

The NIST says that if your modem has the same internal delay as theirs,

you should receive the # time mark within +-2 ms of the correct time.  If

it does not have the same internal delay, the mark should arrive within

+-10 ms of the correct time, with different modem brands varying within

that range.  Repeatability of line delay correction is excellent,

typically within 1 ms from day to day according to the NIST.  Accuracy of

setting the time in your computer is another matter, however, and is

discussed separately below.

The measured line delay is likely to be slightly larger than the

estimated 45 ms that the NIST routinely uses when you don't request an

actual measurement and are using land lines, and it's much larger if the

communication is by satellite.  It will be influenced by the speed of your

computer's CPU in echoing characters to NIST as well as by distance and

the characteristics of your modem.  With a Hayes Smartmodem 2400 and a

computer running at 8 Mhz, the delay measured between the Seattle area and

Boulder, Colorado is always around 62 ms (0.062 second).  Beta testers

using TIMESET 6.00A in New Jersey, Maryland, and Illinois with 20-25 Mhz

386 computers and different modem brands have reported delays around 54

ms.  A tester in Alaska, with a 386 computer and a U.S. Robotics modem

reported a delay of just over .3 second, a magnitude of delay that

indicates a satellite link.  Our tests show that the measured line delay

is also affected by the choice of long distance carrier.  The delay is

consistently greater with MCI than with AT&T -- about 62 and 54 ms

respectively calling from the Seattle area with a land line link in each

case.  The accuracy of setting is the same in either case, however, since

the NIST adjusts for the lag it measures.

In the usual, bidirectional, satellite link, the one-way signal delay is

in the range of 260 to 300 ms (.26 to .3 seconds).  The communications

satellites are parked in stationary orbits about 26,000 miles from earth,

- 14 -

which requires the NIST's time mark to travel 52,000 miles or more to

reach the caller's computer.  This alone takes over a quarter of a second

with the signal traveling at the speed of light (186,000 miles/sec).

Modem processing adds further delay.

There is one situation where the NIST is unable to report a line delay

measurement.  That is when the path is by satellite (a long path) in one

direction and by land line (a shorter path) in the other.  In that

situation, which the NIST detects by a delay measurement of 90-260 ms, it

is impossible to know whether it's the long path or the shorter one that

is carrying the time signal to the user, so a delay adjustment can't be

applied; the NIST time mark remains an asterisk and the standard 45 ms

advance is used.  According to the NIST, a split-path transmission like

this is rarely used by telephone services.

Line Delay Correction with the Naval Observatory

If you dial the Naval Observatory without asking for line delay

measurement, TIMESET 6.00A will routinely add 50 ms (0.05 sec) to the time

it feeds to the computer clock.  This value is close to the standard 0.045

sec time mark advance the NIST makes when no actual line delay measurement

is requested, has the same purpose, and includes the same estimated delay

elements (see first paragraph in the preceding section).  Why is the

estimate for the Naval Observatory 0.05 second instead of 0.045 second?

The answer is that it's impossible to set a computer clock to the third

decimal place; 0.045 sec must therefore be rounded up to 0.05 sec.  The

practical effect of this rounding up is nil; if you dial the two services

alternately and in quick succession without requesting line delay

measurements, the before/after time readings on the final screen will

usually agree within 0.01 sec. each time.

Actually, it makes sense to have the default line delay correction

slightly larger for the Naval Observatory since Washington, D.C., on the

east coast, is farther away from the statistical population center of the

nation than more centrally located Boulder, Colorado.  The default

correction will therefore be about right most of the time for most users

in the United States. An actual line delay measurement for Naval

Observatory data is made by means of a remote digital loopback test, which

not all modems can do.  Even if your modem can do remote digital loopback

testing, if it doesn't use the &T6 command to initiate the test and the

&T0 command to terminate it, you will not be able to make this measurement

with TIMESET.  An example of a modem that can do remote digital loopback

but uses commands other than &T6 and &T0 is the U.S. Robotics HST.  There

is no reliable way for a program to detect a modem's ability to handle &T6

and &T0, so it's up to you to check your modem manual to see if you can do

it.  If you ask for remote digital loopback and your modem can't do it,

you'll just sit there online wasting money and accomplishing nothing until

the Naval Observatory hangs up on you or until you press the backspace key

to hang up and go back to the main menu.  People with such modems who need

line delay measurement should use the NIST's service.

- 15 -

In a remote digital loopback test, TIMESET instructs the local modem to

send a request for the test to the Naval Observatory's modem.  This

conditions the Naval Observatory's modem to echo back characters that

TIMESET sends it.  TIMESET 6.00A calculates the one-way line delay by

measuring the time it takes for 10 characters to travel to the Naval

Observatory and return.  When 10 characters have been sent and echoed

back, TIMESET turns off the remote digital loopback test, calculates the

average lag per character, reverts to normal operation, and grabs one of

the Naval Observatory's regular time data strings.  The calculated one-way

line delay per character, rounded to the nearest 0.01 sec, is added to the

zone- and season-adjusted time just before it is sent to your computer's

clock.

When Should Line Delay Measurement Be Used?

The idea of being able to measure and correct for line delay so you can

set your computer clock with maximum accuracy is fascinating to many

people.  It intrigues us, too, which is why we worked very hard to

incorporate that feature in TIMESET for both time services.  However, it

has the potential of being overused or used unnecessarily.

A major purpose for measured line delay adjustment is for satellite

communications, where line delay is a significant factor.  For ordinary

purposes within the continental United States, where telephone

communications are virtually always by land lines, the default standard

estimates should normally be adequate, because they are so close to what

can be obtained by actual measurement.  For example, TIMESET's built-in

default estimate of line delay for the Naval Observatory's signal is 0.05

seconds, whereas the measured delay from Washington, D.C. to Seattle,

Washington is typically around 0.0700 -- a difference of 0.02 sec., and

usually less.  Similarly, the standard estimated line delay the NIST

provides is 0.045 second, whereas the NIST's measured delay between

Boulder, Colorado and Seattle is typically around 0.062 sec --less than

0.02 second difference.

These are very small discrepancies that are so close to the smallest

time unit that can be read with a computer clock (0.01 sec) that they are

hardly worth worrying about.  If you want to see how small 0.02 second is,

try reading numbers in the second decimal place in one of TIMESET's

running time displays.  You can't.  They change so fast that they blend

into each other.  We included the second decimal place in those displays

just to remind you how unimportant 0.02 sec is for most purposes and to

persuade you that there's no sense in going overboard with line delay

measurements.  Why go to all that time and trouble for a a couple

hundredths of a second more accuracy when your computer clock could drift

that much in the next several minutes?  Furthermore, computer clock

updating can take place only when the system timer ticks, every 0.055 sec,

which means you can only be confident that your clock is set somewhere

within 0.055 second of true time, a range of error nearly three times

larger than that extra 0.02 second of accuracy you're striving for.

Clearly, measured line delay is useful only with satellite communications,

- 16 -

where line delays are substantial, or for calibrating external equipment

in real time.

Another thing to consider with line delay measurement is the extra time

on line that it requires, as indicated in the following table:

Connect time typically required (seconds)

------------------------------------------------------------

Delay measurement              No delay measurement

-----------------------------      ---------------------------

Naval Observatory   21                            2

NIST                12                            6

The size of these connect time differences suggests that if a lot of

people get in the habit of doing pointless line delay measurements they

will effectively reduce the capacity of these services.  The result for

everybody could be a lot of frustrating busy signals.

Be scientific.  Use it sensibly.

A Note About MNP Modems

TIMESET 6.00A will work if you are using an MNP modem and it's initially

set for MNP error checking.  However, MNP isn't needed because neither the

NIST nor the Naval Observatory uses it.  Furthermore, tests show that MNP

action can increase the line delay that is measured by the NIST, since it

delays the return of echoed characters at the same time the NIST is

measuring the echo time to determine one-way line delay.  This should not

affect accuracy, because the delay is real and the NIST adjusts the

departure of the time-mark character to compensate for it if you request

line delay measurement.  Still it's something you need to know about.  If

you don't request line delay measurement and use MNP, the nominal 45 ms

standard advance used by the NIST, or TIMESET's 50 ms standard adjustment

for Naval Observatory signals, will provide even less compensation for

delay than they otherwise would, because of the slowing action of MNP.

MNP action could also require you to be online a few seconds longer than

with an ordinary modem or an MNP modem with MNP turned off, because the

first thing an MNP modem does is check for MNP action and MNP level in the

remote modem.  If it doesn't detect MNP in the remote modem, it reverts to

standard operation (provided it's configured to do so, which it should be

if you are calling one of the time services).  All that can take a few

seconds, and during that time data strings being transmitted are not being

seen by TIMESET.  You are wasting time on line, in other words.  If you

happened to call NIST at a time of day when there was a lot of phone

traffic, a few seconds worth of wasted data strings could make a

- 17 -

difference, especially if there was also line noise to corrupt data

strings and cause their rejection by TIMESET.  Since the NIST may limit

your connection time to 15 seconds during busy periods, you could be cut

off before your computer clock gets set.  For this reason you might need

to consider turning off MNP when doing line delay measurements.

THE FINAL DISPLAY

As soon as the time is received and the computer clock is set, TIMESET

jumps to its final screen, which displays the NIST or Naval Observatory

data string that was used to set the computer's clock, along with

extracted information about UTC time and date, daylight/standard time

announcements if the NIST was used, a table showing the computer's time

and date before and after the call, the estimated or measured line delay

correction, and the time it took TIMESET to process the data and set the

clock (internal delay).

The internal delay is an actual self-measurement by TIMESET of the time

it took to process the string of data it received.  There's a lot to do

with that string -- parse it, convert its components to numeric form,

calculate the date from the julian day, adjust for local time zone and

season, create new strings from numbers for summarizing results on the

screen, etc.  Practically all the internal delay is due to string

manipulation, one of the slowest processes in a computer.  With our 8086

computer the internal delay is always about 0.01 second, but it might well

be too small to measure with a very high-speed computer.  The value for

internal delay is added to the time that is sent to the computer clock.

If the call was to the Naval Observatory, the estimated or measured line

delay is also added to the time that was sent to the computer clock.  With

a call to the NIST, only the internal delay will be added (line delay will

not be included, because the NIST compensated for it by advancing the time

mark's departure so it could arrive on time).

Current system time to two decimal places is shown in a running display

in the middle of the final screen if TIMESET 6.00A is configured for

interactive mode.  The main menu screen's large clock display is available

at this point by pressing the backspace key to go back to the main menu.

MORE ABOUT ACCURACY

It's useful to remember that even though TIMESET 6.00A links you to the

most accurate clocks in the world, your computer's clock can never be set

to their accuracy.  The main reason is that a computer clock is not

continuously receptive to timesetting.  Because of the way standard

computers are designed, their clocks "tick" (advance) about 18.2 times a

second, with variations among computers, which means they are updated in

- 18 -

approximately 0.0549-second increments (1 divided by 18.2), although they

can display time to 0.01 second.  So even if a time mark arrives within

+-2 ms of true time, this "granularity" in computer clocks means you can

only be sure that your clock has been set within about 55 ms (55/1000 sec)

of true time, which is very accurate indeed but not perfect.

Repeatability of timesetting with TIMESET 6.00A is very good.  In tests

where one service or the other is called a few times in quick succession,

before the computer clock has time to drift to any measurable extent, the

"Computer time was:" item on the final screen is almost always within 0.01

second of what it was after the previous call.  Usually it's identical.

It's important to remember, however, that you shouldn't expect your

computer clock to stay highly accurate very long.  No computer clock does.

(If your computer time is wildly unstable and you have a battery-powered

clock in it, it could mean the battery needs replacing.)  Get to know your

computer clock's stability by running TIMESET a few times over a period of

days.  How often you need to run it thereafter depends on how stable your

clock is and the degree of inaccuracy you can tolerate.

If you are a scientist or engineer and need accurate time for physical

measurements, you already know you'd need to run it just before a

procedure.  Same thing if you are part of a far-flung organization and

need to have computers in the outposts synchronized with a reliable and

mutually accessible standard (but don't forget to pay for each copy of

TIMESET).  If you are part of a radio, TV, or cable network you might need

to run it at least a couple times a day so you won't switch into the

network in the middle of somebody's sentence.  If you are satisfied to

have your clock accurate within a few seconds, a run every day or two

should be sufficient.  If accuracy within 1 minute is enough, you probably

need to run it only once every few days.  If you're satisfied to be within

5 or 10 minutes, you can probably wait weeks, but you might want to

consider just checking the clock on the wall and setting your computer

clock manually.  If clock accuracy doesn't matter to you, then of course

you won't need TIMESET at all.  We definitely recommend that these

valuable time services be used no more than necessary, and we urge people

not to abuse them.

A Recommendation for People with CMOS Clocks:  Get RIGHTIME.COM

If your computer has a CMOS clock, we strongly recommend that you get hold

of RIGHTIME.COM.  It's a remarkable memory-resident program written by Tom

Becker.  RIGHTIME not only keeps your DOS clock continuously synchronized

with your more accurate CMOS clock, it learns the drift characteristics of

your CMOS clock and automatically corrects for it.  Each time you run a

program like TIMESET, it notices the extent of your clock drift and

refines the correction until eventually it can keep your CMOS and DOS

clocks within 0.5 second of true time.  Thus it can greatly reduce the

frequency of having to run TIMESET.  RIGHTIME, a shareware program, can be

found on Compuserve's IBMHW forum and computer bulletin board systems

around the country.

- 19 -

THE WORDPERFECT LIBRARY

TIMESET 6.00A contains assembly language routines for automatic detection

and full compatibility with the WordPerfect Library, an excellent program

for task-switching made by the WordPerfect Corporation.  It behaves

exactly like WPCorp's own programs when managed by the Library, allowing

you to:  (1) load TIMESET memory-resident or remove it from memory, both

invisibly; (2) directly access it from other programs in the Library menu

by pressing hot keys; and (3) directly access the other programs from

TIMESET by pressing hot keys.  Depending on how you load the Library, it

swaps TIMESET with the other programs by using either expanded memory

(EMS) or a temporary hard or RAM disk file as the storage area for the

program's memory image.  TIMESET 6.00A takes about 136 kbytes of EMS or

disk memory under the Library and about 135 kbytes under DOS.

- 20 -

PROGRAMMING NOTE,

ACKNOWLEDGEMENTS

November 19, 1990

TIMESET 6.00A was written with Microsoft's BASIC Professional Development

System version 7.0 and contains scores of linked commercial assembly

routines as well as several special purpose assembly routines I developed

when no commercial alternatives were available.

It's doubtful that using another higher-level language to write the

program would have improved anything, and I thank the folks at MicroHelp

Inc., Crescent Software, and Microsoft for transforming BASIC into a truly

professional language.

My special thanks go to a group of special people who tested TIMESET

6.00A for me during its development and made many helpful suggestions:

Tom Strickland, Virginia

Bob Germer, New Jersey

Bruce Felstein, New Jersey

Jim Luhman, Illinois

Stu Rothman, Alaska

Tom Takesian, Maryland

Julia Petrakis (my wife), Washington

The CLKDRV10.SYS driver for expansion slot add-on clocks was created by

Tom Strickland of Alexandria, Virginia and fills an important void.  It

has been part of the TIMESET package for over two years and is an

outstanding piece of work.  I'm very grateful to Tom for letting me

include it.

Pete Petrakis, Life Sciences Software

Stanwood, Washington (Compuserve 76555,1175)

- 21 -

CUSTOM PROGRAMMING SERVICES

LIFE SCIENCES SOFTWARE

If you need custom programming, write or phone Life Sciences Software,

tell us what you want, and find out if we can help you.  The chances are

good that we can, and with the same care and experience that has gone into

TIMESET and a lot of other very well received programs we've written.  But

you won't know unless you ask.  Our rates are competitive.

You can telephone us at 206-387-9788 between 8 a.m. and 5:30 p.m.

Pacific time, Monday through Friday

Our address is:

Life Sciences Software

8925 271st N.W., Suite 112

P.O. Box 1560

Stanwood, Washington 98292

- 22 -

DISCLAIMER

TIMESET is a mature program that has been thoroughly tested through

several revisions since its inception in early 1987.  Tremendous effort

has been expended to make it work properly with as many systems as

possible.  This includes the bewildering variety of modems that exist,

some of which may be described charitably as balky and idiosyncratic.  The

number of modem brands that TIMESET cannot work with has declined with

each refinement over the years, and dramatically since the release of

version 5.3 in the fall of 1989.  Only a few exceptionally stubborn ones

have proved unyielding so far.  It is our hope that version 6.00A will

work for everybody no matter what kind of equipment they have, but of

course we cannot guarantee it.  The virtue of shareware is that you can

determine a program's usefulness to you before spending any money on it.

Nor can we accept any responsibility or liability for any harm the program

might cause to your system or your data.  We have never heard of any bad

effects, have never experienced any ourselves, and cannot imagine how

there could be any, but, as with all programs, the user must assume any

risk.

YOUR SUPPORT IS IMPORTANT

We hope you like Professional TIMESET 6.00A and find it useful.  Please

register and pay for it if you do.  It has taken a lot of hard work and

study; many, many hours (surely thousands since the first version in

1987); and considerable expense to develop TIMESET into the highly

sophisticated program it is and the best of its kind.  If it's useful to

you or your organization, it merits your financial support.

It's also advantageous to you to register TIMESET.  Registration brings

you fresh copies of TIMESET and its supporting programs (including any

interim fixes before the next major upgrade), a printed manual, free

technical support, and a 20 percent discount on the next major revision.

Handling and mailing are included in the $35 per unit registration price.

Please fill out the form on the next page and mail it to us with payment,

or register by modem with TSREGSTR and be invoiced if you prefer.

By the way, we always like to get reports from TIMESET users --

favorable or unfavorable.  They are what have driven the program's

evolution from the beginning.

- 23 -

TIMESET MAIL REGISTRATION FORM

For Individuals and Organizations

Notice to companies and other organizations:  Registration and payment is

required for each copy of TIMESET in use within an organization.  The

price is $35 per copy for up to six copies.  Please call Life Sciences

Software at 206-337-7328 for discount information on more than six copies

(the phone number will be 206-387-9788 after December 17, 1991).

Notice to all:  If you prefer to register TIMESET by modem and be invoiced

for payment, use the TSREGSTR program to send the registration information

to our electronic mailbox.  The information TSREGSTR will prompt you for

and send to us is essentially what you see below.

TO:  LIFE SCIENCES SOFTWARE

BOX 1560

8925 271st, N.W., SUITE 112

STANWOOD, WASHINGTON 98292

I wish to become a registered user of TIMESET.

My name: _____________________________________________________________

Organization name (if any): __________________________________________

Street address: ______________________________________________________

Additional address: __________________________________________________

City: _______________________  State: __________________ ZIP: ________

Phone number: ___ ___ ____

Number of copies being registered: ___

PAYMENT:

Note:  As a licensed business in the State of Washington we are required

to charge residents of the state an additional 8.1 percent for sales tax.

___ I am enclosing payment for ___ copy(ies) of TIMESET at $35.00

per copy (Washington residents please add 8.1 percent for sales

tax).  Total enclosed:  U.S. $ ________

Please make your check or money order payable to Life Sciences Software.

Thank you for supporting shareware!

- 24 -