A EYE OPENING VIEW OF INFORMATION COLLECTED BY signal level meters

*COMSONICS WINDOW II signal level meter or *TRILITHIC TRICORDER

By using software furnished by the manufacture to make a

"WR.###" file, which will be a single lotus or 123 file of all

BINS.  CDB will make new files ALLOWING THE VIEWING OF VIDEO

AND AUDIO LEVELS WITH LINE LEVELS AND GRAPHICS OF PEAKS.

Levels which MAY NOT MEET FCC SPECIFICATIONS are INDICATED.

Readings may be SEARCHED and COMPARED. The SCALE can be FIXED or

AUTO SCALE for COMPARISON on screen display.  The MENU is

Displayed around the screen for fast and EASY USE. There are

several special keys not menued. "F" toggles FIXED and AUTO SCALE.

"I" stores a displayed reading as and output level and then use

"I" again to store a different reading as the INPUT to the next

amplifier so an evaluation of the cable between the two can be

made. The tab key will show level of high and low pilot. End key

or Home key will clear and redraw the last level. The (c) Dif_lev_

function has also been added. "d" stores the reading to be COMPARED

and "D" toggles ON and OFF if a reading is stored.

Sample readings will show COMMENTS/LOCATIONS which were added

when a WR.* FILE was CONVERTED to files that are smaller for use

by the viewer PROGRAM. The files maybe typed to the screen or printed.

IF GRAPHICS.COM is LOADED before starting, SHIFT PRINT SCREEN will

produce the current view on some PRINTERS, as it is on the SCREEN.

This program has been developed over a period of many years and

has evolved to this latest version.  With the FCC SPECIFICATIONS

changing, YOUR personal CDB.DAT file must be kept current.

To make use of different versions of DOS, support programs which

sort and find readings are used by CDB. Because the data files are

small, the complete system and data files will fit on a single disk.

If it's preferred that no outside files be put on a company computer,

a version is available that will only work on an external disk drive.

CDB is for ANYWHERE USE and CDBDISK will work only on a DISK DRIVE.

If the support files are not in the "path", the program will abort.

THIS AND ALL VERSIONS MAY BE USED WITHOUT MODIFICATION UNTIL

THEY DIE...USUALLY ONE YEAR!

REQUIRED UTILITIES THAT MUST BE IN PATH:

SORT.*   COMMAND.COM   GRAPHICS.*  WHERE.COM

For more information write: E H ELLZEY

RT 1 BOX 211C

ELLISVILLE MS 39437

or call 601-649-5391 or leave a message on the HAWK'S NEST

BBS - 601-649-0596 - 24 hours - 2400/14400-N-8-1