Program GEL  :  Version 2/17/89 JRT

PROGRAM GEL takes a set of standard DNA fragment sizes and mobilities and

predicts the sizes of unknown fragments.  GEL applies a least squares fit to

the reciprocal relationship between mobility and fragment length originally

detailed by Southern (Anal. Biochem 100,319-323[79]).  This program is a Turbo

Pascal(tm) implementation of the FORTRAN program described by Schaffer and

Sederoff (Anal. Biochem 115,113-122[81]).  Please refer to those references

for complete details of the calculations performed by this program.

This version allows the user to edit individual standards and

recalculate without the need to re-enter unchanged data.  Additionally,

standard fragment data may be stored in and read from a disk file.  This menu

driven looping structure makes it easy to exclude obviously aberrant data and

re-calculate.

ASSESSING GOODNESS OF FIT:  As detailed by Schaffer and Sederoff the

goodness of fit can be judged by the parameter C(i) which is a constant and

should theoretically be the same for all samples (see exapmle output below).

Thus, samples with vastly different values for C(i) may logically be

questioned.  In addition, several summary statistics are reported.  The

Standard Deviation of C(i) values (reported as "SD(C)") provides an overall

measure of the goodness of fit for the whole curve and the value reported as

"SD(Dev)" provides a measure of accuracy in terms of fragment length (Standard

deviation of values in the size deviation column).  Finally, and perhaps most

practically, you may simply compare the real and predicted sizes of your

standard fragments to get a feeling for how good the fit is.

PROGRAM USAGE NOTES

GEL is entirely menu driven and usage is largely self-explanatory.  Type

GEL at the DOS prompt to begin.  A main menu is presented and items are

selected by a single keystroke (see main menu below).  Some options have

suboptions requiring further responses.  The user is prompted at each level.

Normally, the user will select G or I options first to load a standard set.

Then select the C option to preview just the standard curve data or the U

option to enter mobilities for unknown fragment size prediction.

Program output is sent to the screen by default but may be directed to

printer or disk using the O option.  The O option redirects output from the

C and U functions only; all menu prompts still display on the screen.

=========================================================================

GEL : MAIN MENU

Select Function By First Letter

Documentation Screen

Input standard fragments and mobilities

Get standards from a disk file

Edit standard curve data

Save standard curve data to disk

Calculate standard curve

Unknown fragment size calculation

Output Device Selection [Currently = SCREEN]

Quit the Program and Exit to DOS

?

=========================================================================

SAMPLE OUTPUT

STANDARD CURVE  : Program GEL  :  Version 4/14/87 JRT

Std size  : Mobility : Pred. Size : Deviation :  %DEV  :  C(i)

5241.000       3.03     5245.058      -4.058    -0.08     26082.97

5070.000       3.11     5046.383      23.617     0.47     26173.47

4257.000       3.46     4280.093     -23.093    -0.54     26012.10

3510.000       3.89     3519.436      -9.436    -0.27     26057.64

1995.000       5.17     1979.983      15.017     0.75     26176.19

1880.000       5.30     1863.921      16.079     0.86     26183.95

1584.000       5.63     1592.661      -8.661    -0.55     26045.72

1350.000       5.95     1357.550      -7.550    -0.56     26049.75

941.000       6.60      948.387      -7.387    -0.79     26045.94

843.000       6.80      837.840       5.160     0.61     26131.94

M0 = -0.17186        L0 = -2905.20        C(Avg) = 26095.97

SD(Dev) = 16.448                SD(C) =  64.39

===============================================================

UNKNOWNS SIZE PREDICTION

Frag1: Size = 5322.14 For a Dist. of   3.00

Frag2: Size = 2140.56 For a Dist. of   5.00

=========================================================================

Some effort has been made to anticipate and trap for user input errors.

You should find this Program difficult to crash accidently.  If a given output

file name already exists the program will simply append new information to the

bottom.  All numeric entry is trapped so that inappropriate responses are

simply ignored.  Most options allow you to return to the main menu without

changing anything in case an item is selected by mistake.  Typing Ctrl C will

terminate the program and return control to DOS.

INSTALLATION OF PROGRAM GEL

Program GEL will run on any Monchrome or Color hardware setup.  The

program is designed for an 80 column display.  To install the program simply

place the file GEL.EXE on your working disk.  For Hard Disk users the GEL.EXE

file should be placed in one of the sub-directories specified by the DOS PATH.

When placed in the DOS PATH, program GEL may be run from any directory.

COLOR CONFIGURATION

Program GEL runs in monochrome mode by default.  If you wish to use

colors you need to add a single line to your AUTOEXEC.BAT file (usually found

in the root directory C:\).  Add the following line:

SET GEL=FGCOLOR:BGCOLOR

Substitute the desired foreground color for FGCOLOR and background color for

BGCOLOR.  For example:

SET GEL=LIGHTCYAN:BLACK

specifies lightcyan text on a black background.

Allowed Background colors are:

BLUE GREEN CYAN RED MAGENTA BROWN LIGHTGRAY

Foreground colors include the above colors as well as:

DARKGRAY LIGHTBLUE LIGHTGREEN LIGHTCYAN LIGHTRED LIGHTMAGENTA YELLOW WHITE

Update record:

4/9/87  First version released for BIONET distribution.

12/6/87

Added traps for printer off line condition, bad input data files

and several other possible user errors.

Added support for color systems via DOS SET Command.

Modified for Turbo Pascal 4.0

2/17/89  Corrected bug that cause garbage output if more than 24 unknowns

were entered.

Added a new feature to allow reading of unknown data from a text

file.  File Format: Each line of the text file must contain a

mobilty value, a space, and a Text label for the unknown (up to

30 characters).  A mobilty value of 0 on a line by itself

terminates the input.  Comments may be added below the data.

This routine is not error trapped very well; Pressing the

carriage return without entering a filename will cause a program

crash.  Also, Omission of the zero "end of data" marker will

cause a program crash as well.

Program Gel now has space allocated for 23 Standards and 100

Unknowns.

Installation Note:  If you have used previous versions of Program

Gel, you must delete the old program (GEL.COM) or it will

interfere with the new program (GEL.EXE).

This program was Written by:

John R. Thompson

7105 Hana Rd.

Edison, NJ 08817

Direct inquires/suggestions to JRT

Email Address is WOOLFORD.THOMPSON@BIONET-20.ARPA