MAKING A REFERENCE BLOCK FRAME FOR MERGEIT

MergeIt uses black reference blocks in the upper left and right corners of the

scanned image segments to calculate the best image cut and perform the merge.

For best and consistent results, these blocks should be very black so that the

boundary to the adjacent white regions are distinct. Write protect stickers

for 5 1/4" floppies are often excellent, as long as one of the long edges is

cut back to remove the rounded corners. When placing a document on the frame,

the thickness of these stickers makes it easy to align the document with the

bottom edges.

Frame layout for letter length images:

�<......3.5"......>�<......3.5".......>�<......3.5".......>�

Black=>        �����Ŀ            �����Ŀ             �����Ŀ            ���Ĵ

Reference      �������            �������             �������            �����

Blocks         | ~1" |            | ~1" |             | ~1" |            1/4"

(height                                                                 (width)

> 0.1")                                                               (or more)

Hand Scanner GuideBar Setup:

�������������������������������������������������������������������������Ŀ

���������������������������������������������������������������������������

optional top slide guide

............... ���Ŀ

.  optional    � L �Ŀ             �����������������������������Ŀ

.   back      � E � �             �  Hand Scanner               �

.   triangle � F � �i            �                             �

.   support � T � �m            �                             �

.          �   � �a            �������Ŀ             ���������

.         � G � �g                    �             �

.        � U � �e                    �             �

.       � I � �                     �             �

.      � D � �c                    �             �

.     � E � �u                    �             �

.    �   � �t             ����������������������Ŀ

.   �   � �              ������������������������

.  �   � �e              hand scanner bottom guide

. �   � �d

.�   � �g                          � �

�   ���e             scan       \ � � /

�����                  image     \� �/

down      .

using left guide against

scanner body and bottom

guide.

Attaching a bottom guide to the hand scanner is extremely helpful in getting

accurate image scanning. Any turning of the scanner will distort the image

in ways that are not correctable by any software. The hand scanners should be

made with an easy way to add an edge guide. They aren't..., I use an aluminum

rod, 1/4" square and 6" long, epoxied to the bottom edge of the handle.

The left guide may be a straight piece of trim wood, a piece of utility

aluminum angle, or ?. It must be straight and long enough to guide the

scanner to the bottom of its scanning sweep.

The image cut edge is a piece of stiff white paper secured to the bottom of

the left guide.

Position the image cut edge so that it just slightly intrudes into the scanned

image. This will help you to best align the left guide when scanning in an

image. It will also define the left image edge since it will cut off the top

left reference block. The image cut edge is not essential (in which case the

scanner will naturally cut off the reference block at the left edge of the

image it scans).

Orienting paper for landscape scanning and merging:

Start scan

D               C              B     here     A

black   ���Ŀ reference ���Ŀ  blocks  ���Ŀ downward ���Ŀ

����������������������������������������������������Ĵ

�       ...    LEFT MARGIN EDGE .       . . .        �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .     T  �

�       ...     ..             ..       . . .     O  �

�       ...     ..             ..       . . .     P  �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .        �

�               D O C U M E N T                      �

������������������������������������������������������

Where possible, adjust document to put natural breaks in the reference

block regions. Typically this will be between text lines. Start scanning down

the document on the right side (at the top of the document) with the left

edge of the scanned image covering part of reference block "B" at its top edge.

Shift to the left so that the next scanned image covers part of the reference

block "C" at its top edge.

Finally, shift to the left so that the next scanned image includes part of the

reference block "D".

The width of each of the black reference blocks is written in the initial

values file. See "MergeIt.Doc".