THE INVISIBLE LINK

Copyright (C) 1987

Robert Best and Garland Wong

Santa Ana, CA and La Jolla, CA

All Rights Reserved

I. ABOUT INVISIBLE LINK

The Invisible Link is a general purpose asynchronous communications

program.  However, it differs from an ordinary communications program in

that the Invisible Link is memory-resident and designed to function as a

background task executing concurrently with any MS-DOS application

program.  This means you can dial or transfer files in the background

without tying up your computer system.  Imagine, you can simultaneously be

working in your favorite word processor or spreadsheet and still be

dialing a busy number or receiving those important files.  Best of all,

you won't even notice that IL is running or "doing its thing".  In terms

of time savings alone, IL will "pay for itself" during its first hour of

use.  IL was written in assembly language for optimum size and speed.  It

represents the result of a thorough six month period of development and

testing.  It has proven to run faster and more reliably than the

competition (who charge from $100 - $200 for their product), as any

thorough benchmarking will testify.  When you are transferring a file in

the background, there is no noticeable degradation in your foreground

application program.  Lastly, you can use IL's background transfer or dial

capability along with your favorite communications program.  This means

you don't have to give up your favorite communications program because IL

was designed to work within or along side them.

The Invisible Link has the following features:

-      Communicate at speeds up to 9600 baud (limited by the speed

of your serial port and modem.)

-      Exchange binary or text files in the background using

Xmodem CRC or Xmodem Checksum Protocol.

-      Queue up to 10 files to upload or download for unattended

operation.

-      Automatically send a "goodbye" command to hang up the phone

when the file transfer has completed

-      Queue up to 5 numbers and dial in the background.  IL will

prompt you when a connection is made.

-      You can manually dial or choose from a 44 number dialing

directory.

-      All commands in IL are invoked using simple one or two key

sequences.

-      Dynamically alter communication parameters.  You can change

communication speeds and choose between no parity and 8 data bits

or even parity and 7 data bits.

-      Dynamically alter auto redial rate.  You can vary the time between

redialing anywhere from 10 seconds to one minute.

In short, the Invisible Link is a powerful yet unobtrusive communications

program.   It is both a timesaving utility and a productivity tool.

II. GETTING STARTED

2.1 What You Need (Basic Requirements)

The Invisible Link requires the following as a minimum:

-     IBM PC, XT, AT, or 100% Compatible.

-     DOS 2.0 or Later.

-     IL requires about 26K of RAM when loaded.

-     One Disk Drive or Hard Drive.

-     Serial I/O Port with a Modem.

-     CGA, EGA, IBM Monochrome, or Hercules Video Adapter.

-     Any Monitor Capable of Displaying 80 Characters Per Line.

2.2 Loading Invisible Link

Loading The Invisible Link is a simple one step process.  You pass IL

various configuration information via command line arguments.  First, all

you need is the file IL.COM.  There are no overlays to worry about.

Second, IL needs to know which communications port you will be using.

Third, IL also needs to know where IL.COM is stored.  This is because when

you change your directory, IL needs to know where to update the directory.

By default, IL will assume that you will be using communications port 1

and that IL is stored on the current drive in the current subdirectory.

The format of the command line is as follows:

IL [COMM PORT] [FULL PATHNAME OF WHERE IL IS STORED]

Note that you can specify the pathname first and then the COMM port or

vice versa.  The order does not matter.

EXAMPLES OF LOADING THE INVISIBLE LINK

A>IL<RETURN> <RETURN> means hit a carriage return.  If you load IL this

way, COM1 will be used and IL will look for IL.COM on the current drive in

the current subdirectory.

A>IL 2<RETURN> IL will use COM2 and update its directory on the current

drive in the current subdirectory.

A>IL 1 A:\IL.COM<RETURN> IL will use COM1 and update its directory on

drive A: in the root directory.

Note that when you are specifying a path that it must be a full path

including the three letter extension.  For example, C:\IL\IL is not

valid but C:\IL\IL.COM is valid.

A>IL C:\IL\IL.COM 2<RETURN> IL will use COM2 and update its directory on

drive C: in the subdirectory IL.

If IL is already loaded, you will get the "INVISIBLE LINK USING ALT-RIGHT

ALREADY LOADED -- ABORTING!" message.  You will then return to DOS.

NOTE:  It is strongly recommended that IL be loaded last in the chain of

memory resident programs.  In particular, it must be loaded after CED in order

to properly enable the switches installed to prevent system crashes.

III. YOUR BASIC COMMANDS

3.1 Invoking Invisible Link

ALT - RIGHT-SHIFT => Invoke The Invisible Link.  To invoke The Invisible

Link, simply hold down the <ALT> and <RIGHT-SHIFT> key simultaneously.

The Invisible Link was designed to be very robust so there are times when

IL will not let you "pop in" because it is not safe. Usually, it is not

safe when DOS is doing critical disk activity such as during a COPY or

TYPE command.  Just wait until your disk drive light goes off and then hit

<ALT> and <RIGHT-SHIFT> key.  When you "pop into" IL, you will be

presented with the following message, "TERMINAL MODE, HIT ALT - F10 FOR

HELP!".  You are now ready to begin your communication session.

3.2 Terminal Commands

3.2.1 Setting Communication Parameters

ALT - P => Set Up Communication Parameters.  Use this command when you

want to set the baud rate or choose the data, parity, and stop bits.  The

Invisible Link supports the two most popular communication configurations:

even parity, 7 data bits, and 1 stop bit; and no parity, 8 data bits and 1

stop bit.  When you hit ALT - P, will be presented with the following

prompt at the bottom of the screen.

Baud Rate For 8, 1, No Parity:   0) 300 1) 1200 2) 2400 3) 4800 4) 9600

Baud Rate For 7, 1, Even Parity  5) 300 6) 1200 7) 2400 8) 4800 9) 9600 |

At this point, type the number corresponding to the configuration you

wish.  For example, hit 6 if you want 7 data bits, 1 stop bit, even

parity, and 1200 baud.  An invalid entry will leave the configuration

unchanged.  The user selected configuration will remain in effect until

you change it or if you reboot the computer.

3.2.2 Clearing The Screen

ALT - C => Clear The Screen.  At any time you can clear the screen by

holding down <ALT> and C key simultaneously.

3.2.3 Hanging Up The Phone

ALT - H => Hang Up The Phone.  You may hang up the phone my hitting the

<ALT> and H key simultaneously.  After the phone is hung up, IL will

return you to terminal mode and prompt you with OK.

3.2.4 Auto-Redial Rate

ALT - R => Alter the Auto-Redial Rate.  You will be prompted to enter a

number between 0 and 9 which causes the redialing rate to be adjusted

between 10 seconds and one minute, in 5 second intervals.

3.3 Making A Connection

3.3.1 Using The Dialing Directory

ALT - D => Access The Invisible Link Dialing Directory.  Invisible Link's

dialing directory can be brought up by holding down the ALT and D key

simultaneously.

If the directory does not come up after hitting ALT-D, chances are your

modem may not be turned on.  Try turning the modem on and restriking

ALT-D.  The Invisible Link was designed to dial only if a modem is

connected.

3.3.1 Using The Dialing Directory

ALT - D => Access The Invisible Link Dialing Directory.  Invisible

Link's dialing directory can be brought up by holding down the ALT

and D key simultaneously.

If the directory does not come up after hitting ALT-D, chances are

your modem may not be turned on.  Try turning the modem on and

restriking ALT-D.  The Invisible Link was designed to dial only if

a modem is connected.

3.3.1.1 Dialing A Number Or Queueing Numbers

After hitting ALT-D,  IL will prompt you for the following: an

entry to dial, whether you wish to revise an entry, or whether

you wish to delete an entry.  Type a number between 1 - 44 and IL

will acknowledge your valid entry by putting '>' by that entry.

You may queue up to 5 numbers.  If you have less than 5 numbers

to dial, hit a empty <RETURN> and you will return to terminal

mode.  At which point, IL will dial your numbers once every X

seconds (where X is the Auto Redial Rate) until a connection is made.

Furthermore, you may return to your application.  When a connection

is made, IL will beep and inform you that a connection has been made.

Note, that hitting ALT-D again (or ALT-M) will stop any subsequent

dialing.

3.3.1.2 Adding New Entries Or Revising Existing Entries

By typing "R" at the above prompt, you may add new entries or

edit existing entries.  A window will pop up and IL will prompt

you for an entry number.  If the number you type has no existing

entry, then IL will bring up a blank entry.  You may type in the

name and telephone number in the respective fields.  For the baud

rate the following conventions are used:

Type 0 for 300  Baud         Note that IL uses the data bits, stop

Type 1 for 1200 Baud         bits, and parity from the last ALT-P

Type 2 for 2400 Baud         command, but will use  the baud rate

Type 4 for 4800 Baud         specified in the directory for the

Type 9 for 9600 Baud         given entry.

If the entry was an existing one, IL will display the old name,

telephone number, and baud rate.  You may now input these

parameters.  Note, if you hit a blank <RETURN> on the baud rate

field, IL assumes 1200 baud by default.  After you have input all

the information for the directory, hit a blank <RETURN> and IL

will update the directory.

3.3.1.3 Deleting Entries

By typing "D", IL will prompt you for the number of the entry you

wish to delete.  IL will then ask you "Are You Sure?".  Typing

'Y' will delete the entry and 'N' will not delete the entry.  To

return to the dialing menu, hit a blank <RETURN>

3.3.2 Manually Dialing A Number

ALT - M => Manually Dial A Number.  You can manually queue up to 5 numbers

to dial.  Hit Alt-M, and you will be prompted to input the telephone

numbers.  Hit a blank <RETURN> when you are done.  At this point, IL will

dial your numbers continuously in sequence until a connection is made.  By

default, IL will wait 30 seconds between each redial, however, this rate

can be adjusted using ALT-R.  If you want, you can now return to your

interrupted program and dial in the background.  IL will beep when a

connection is made.  Note, hitting ALT-M again while you are in terminal

mode will cancel the current dialing session independent of whether it was

started via ALT-D or ALT-M.

3.3.3 Terminating A Background Dial

ALT - LEFT-SHIFT => Terminate Background Dial.  If you are not in terminal

mode (i.e. you are in your foreground application program), you can

terminate a background dial by holding down the ALT and LEFT-SHIFT

simultaneously.  IL will acknowledge by beeping and displaying the

"BACKGROUND DIALING TERMINATED!" message.

3.4 Transmitting A File

3.4.1 Uploading A File

PAGE UP KEY => Start An Upload.  To start an upload, hit the PAGE UP key

(it is located on the numeric keypad and is the number 9 on a standard IBM

PC).  IL will prompt you for the name of the file you wish to upload.  You

may specify full pathnames if your files reside in other directories other

than the current one (i.e.  C:\IL\FOO.EXE is a valid path name).  Next, IL

will prompt you for the command that will be sent to the host.  For

example, U X FOO.EXE is a command that tells the host that you will be

uploading file FOO.EXE with XMODEM protocol.  For an upload, IL will

automatically determine if CRC or Checksum protocol will be used.  You

will then be prompted for a description of the file to be uploaded.  This

will be transmitted to satisfy the request given by most current Bulletin

Board Systems.  If desired, this request can be ignored by entering a

carriage return at the prompt.  After entering the description,  you will

again be prompted for another file to upload.  With IL, you can queue up

to ten files to upload.  That means you can enter ten files and ten upload

commands. If you will be uploading less than ten files, hit a blank

<RETURN> on the file prompt.  IL will then ask you for a goodbye command.

This is a convenient way to hang up the phone after a transfer is

completed.  Enter a "G" for goodbye.  IL automatically sends a "Y' to

confirm that you want to hangup.  At this point, the transfer will begin

in the background.

Note that once a transfer is going, you are returned to the foreground

program and locked out of the terminal.  This means you can not hit ALT -

RIGHT-SHIFT to enter IL's terminal.  Only after you see the "INVISIBLE

LINK TRANSFERS COMPLETED!" message and the bell rings is it safe to pop

into IL.

3.4.2 Downloading A File

PAGE DOWN KEY => Start An Download.  To start a download, hit the PAGE

DOWN key (it is located on the numeric keypad and is the number 3 on a

standard IBM PC).  First, IL will prompt you for the protocol that you

wish, CRC or Checksum.  Select 1 for XMODEM-CHECKSUM or 2 for XMODEM-CRC.

CRC is less prone to errors in transmission but tends to slow the computer

down a little more than Checksum*.  Next, IL will prompt you for the name

of the file you wish to download.  You may specify full pathnames if your

files reside in other directories other than the current one (i.e.

C:\IL\FOO.EXE is a valid path name).  Next, IL will prompt you for the

command that will be sent to the host.  For example, D X FOO.EXE is a

command that tells the host that you want to download file FOO.EXE with

XMODEM protocol.  After entering the download command,  you will again be

prompted for another file to download.

If a file already exists, IL will prompt you if you wish to overwrite.

You may enter 'Y' if you wish to overwrite or 'N' if you do not.  If after

5 seconds, you do not respond, IL will assume that you do want to

overwrite the file and the transfer will start.  This feature was

incorporated into IL for the sole purpose of permitting unattended

transfers.

With IL, you can queue up to ten files to download.  That means you can

enter ten files to download and ten download commands. If you will be

downloading less than ten files, hit a blank <RETURN> on the file prompt.

IL will then ask you for a goodbye command.  This is a convenient way to

hang up the phone after a transfer is completed.  Enter a "G" for goodbye.

IL automatically sends a "Y' to confirm that you want to hangup.  At this

point, the transfer will begin in the background.

Note that once a transfer is going, you are returned to the foreground

program and locked out of the terminal.  This means you can not hit ALT -

RIGHT-SHIFT to enter IL's terminal.  Only after you see the "INVISIBLE

LINK TRANSFERS COMPLETED!" message and the bell rings is it safe to pop

into IL.

3.4.3 Invoking The Status Screen

ALT - LEFT-SHIFT => View File Transfer Status.  At any time during a file

transfer, you can view the status of the current file being transferred by

hitting ALT - LEFT-SHIFT.  You have access to the following information:

upload or download, full pathname of file, mode of transmission (CRC or

Checksum), Current Block Number, consecutive error count, and total error

count.  If you are doing an upload, you will also see the total number of

blocks for that file.  Once in the status screen, hitting 'Q' will abort

the transfer of the current file and proceed with the next file in the

queue.  Hitting any other key will return you to the foreground program.

You can pop into the status screen as many times as you wish.

IL will abort the transfer itself after 10 consecutive errors and proceed

with the next file in the queue if there is one.  Please note that your

total error count will be higher than when you are using a conventional

communications program.  This is because IL is transferring in the

background.  So, if you see a high total error count, don't be alarmed.

IL was designed with a robust error recovery system, so chances are the

file was sent properly.

3.5 Exiting Invisible Link

ALT - X => Exit The Invisible Link But Leave Online.  You can exit IL by

hitting ALT-X.  This will leave you online and return you to the suspended

foreground application program.  You can enter and leave the terminal as

many times as you wish.

ALT - Q => Exit The Invisible Link And Hang Up The Phone.  You can exit IL

and hang up the phone by hitting the ALT and Q key simultaneously.  IL

will prompt you with "Are You Sure You Want To Disconnect?".  Typing 'Y'

will hang up the phone and return you to your suspended foreground

application.  Hitting any other key will return you to the terminal.

IV. SUMMARY OF COMMANDS

KEYSTROKE                    COMMAND

ALT - RIGHT-SHIFT            ENTER INVISIBLE LINK.

ALT - X                      EXIT INVISIBLE LINK BUT LEAVE ONLINE.

ALT - Q                      EXIT AND HANG UP PHONE (DROP DTR).

ALT - P                      CHANGE COMMUNICATION PARAMETERS.

ALT - H                      HANG UP PHONE AND RETURN TO TERMINAL.

PG UP                        START AN UPLOAD.

PG DN                        START A DOWNLOAD.

ALT - D                      DIAL A NUMBER USING A DIRECTORY.

ALT - M                      MANUALLY DIAL A NUMBER.

ALT - R                      ALTER THE AUTO-REDIAL RATE.

ALT - LEFT-SHIFT             TERMINATE BACKGROUND DIAL OR ENTER

FILE TRANSFER STATUS SCREEN.

V. TIPS ON USING THE INVISIBLE LINK

5.1 Getting The Most Out Of The Invisible Link

As with any memory resident program, contention may occur when running

with other memory resident programs.  The only way to check compatibility

is to test IL with that particular memory resident program.  IL has been

successfully tested with SIDEKICK(tm), SQZ(tm), CED(tm), and PRINT.COM to

name a few. It is quite a sight to see a background transfer going on

simultaneously with a background printing with no noticeable degradation

to the foreground application program.  Note that with disk intensive

application programs such as DBASE(tm), you will probably notice more

degradation than if you were using LOTUS 1-2-3(tm).

Also, application programs that take over certain interrupts and are not

well behaved may interfere with the background transfer.  The only way to

check compatibility is to try a background transfer or dial while using

that particular program.  You will probably know right away if IL is

compatible with that program.  IL has been extensively tested with DBASE

III+(tm), LOTUS 1-2-3(tm) (ALL VERSIONS), MICROSOFT(tm) COMPILERS AND

ASSEMBLER, TURBO PASCAL(tm), MANUSCRIPT(tm), AND NOVELL ADVANCED NETWARE

286(tm) VERSION 2.0.

VI. ERROR MESSAGES

INCORRECT DOS VERSION -- USE 2.0 OR ABOVE -- ABORTING! - IL requires DOS

2.0 or later.

IL.COM NOT FOUND, DIRECTORY NOT UPDATED! - When loading IL, you probably

did not specified the correct path (including extension) of where IL.COM

is stored.

TOO MANY ERRORS -- ABORTING! - IL has received 10 consecutive errors and

is aborting the transfer of the current file in the queue.

CANNOT OPEN FILE -- ABORTING! - IL could not open the file you wish to

transfer.  Make sure you have specified a valid path.

COMM PORT OUTPUT ERROR -- ABORTING! - IL encountered a COMM port error.

ERROR WRITING FILE -- ABORTING! - IL encountered a error while writing the

transmitted file.  This error usually occurs when you delete a file while

it is being transmitted.

ERROR READING FILE -- ABORTING! - IL encountered a error while reading the

transmitted file.  This error usually occurs when you delete a file while

it is being transmitted.

** THE INVISIBLE LINK LICENSING AGREEMENT **

The fully supported release version of THE INVISIBLE LINK is copyrighted

material intended for use in unaltered form.  Distribution of the release

version of THE INVISIBLE LINK in any form is a violation of copyright. The

program is provided "AS IS" without any warranty of any kind. Should the

program prove defective, the user shall assume the entire cost of all

necessary servicing, repair or correction.  Site licensing of the release

version of THE INVISIBLE LINK is available by arrangement with Robert

Best. The release version of THE INVISIBLE LINK is intended for use on a

single machine. Making copies for use on other machines represents a

violation of this licensing agreement.

A donation of $20.00 would be appreciated.  This program is not public domain.

The Invisible Link is marketed under the shareware concept. Send check or

money order payable to Garland Wong in the amount of $20.00 to:

Garland Wong

8663 Via Mallorca #86

La Jolla, Ca, 92037

If any party is interested in purchasing the full source code to THE

INVISIBLE LINK, as well as the exclusive marketing rights to any

derivative thereof, please contact Garland Wong at the above address to

discuss terms.