Zmax is fairly easy to use, just follow the parameter guide in

the Zmax.doc file.

Zmax is designed to replace Zmodem. It's faster and handles noisy

lines much better. Zmax isn't the fastest protocol around, Tmodem

is considerably faster but Zmax handles noisy lines better than

Tmodem.

To use Zmax to its fullest, you'll need to setup several

different definitions for it in your terminal program for

receiving or downloading files.

(NON-MNP Connects)

I would suggest that you setup one that gives you 1024 byte

Blocks in stream mode. This is for the systems you call and

you always get good lines and rarely, if every, have any bad

blocks.

I would then setup one that used 512 byte blocks in stream mode.

This is for those system that you call but you do on occation get

resend requests. This will save you about 2.2 seconds if you get

ONE resend request and about 5 seconds if you get 2 of them. 5

seconds can make a BIG difference in your thoughput.

Finally, setup one that uses 128 byte blocks in NON-STREAM mode.

This is for systems you call that always have bad lines. Your

thoughput will be much better.

If your calling a system that supports MNP and so do you, then I

would go so far as to setup one that always requested 2048 byte

blocks in STREAM mode. The likely hood that you'll get any errors

is so small its not really worth worrying about.