HOW TO STUFF YOUR OWN BOARDS WITH CHIPS

By Louis A. Warner, P. E.

PEOPLE SYSTEMS LTD. -- SOFTWARE (C) 1983

You have just purchased some of the finest devices

that have ever been produced. These chips need

special attention if you want to get the best out

of them, because they are very easily damaged by

static electricity, that you and I wouldn't even

notice. No snap, crackle or pop, but zowie, there

goes a good chip, destroyed by static electricity.

There are ways to protect your investment against

such damage, if you're careful and take your time.

Before you unpack the chips think of a pettigree

puppy that you just purchased, and treat it

tenderly and you'll enjoy. Oh, yes, don't open up

the chips until you're ready to install them.

Remember static electricity hates humidity, but it

loves nylon and synthetic clothing, soft rugs and

slippery shoes. Static is worse in the dry winter

weather, and least in the humid summer time.

Before you unpack your chips, have you read your

owners manuals for adding chips to the main memory

or an expansion board? Refresh your memory and

plan your project and you'll do it easily. It's

really not very hard to do and unless you get

rough with your equipment there is little you can

harm if you take precautions. You will need a big

sheet of aluminum foil, a spray bottle of water, a

boiling teakettle and some light hobby tools,

flashlight, magnifier, pad and pencil along with

the manuals etc.

A clear, clean kitchen table on a linoleum floor

where the light is very good is a fine place to

operate on the patient. Take your computer cabinet

off to get at the board you're going to stuff.

Read your manuals or ask your buddy's help to get

it unbuttoned. On the IBM-PC there are two large

screws in the lower rear corners, near the curve

at the bottom of the back. Slide the cover forward

and put it aside. Vacuum the fan and inlet areas

carefully, but don't let the air from the vacuum

stir up any dust in the room. Wiping the fan

blades with a tiny brush will help it do its job.

Remove the rear hold down screw from the expansion

board and gently raise the board from the bus

socket on the mother board. Place a large piece of

aluminum foil flat on the table to touch and

ground yourself on when you're working on the

chips. If you have a battery powered board (clock

etc) remove the battery. You are now ready to open

up your chips and set them into the open sockets.

Review your conditions again. Shoes off, tea

kettle boiling, dog and cat out of the house, kids

gone to the movies, you are ready to operate,

Doctor.

What do you do in the winter or when the humidity

is real low? Wait for a rainy day or raise the

humidity with a vaporizer in the room. Don't wear

synthetic clothes. But use a spray bottle to keep

from getting too dry. Mist your clothes and the

floor and chair seat (usually plastic too) with

water to keep it from generating static. Don't

work on rugs, if possible, or else spray them with

static guard and water mist. Look-out for nylon

lingerie whether you are stuffing a board or

whatever. Don't get your computer wet.

You have unwrapped your chips and the board is

sitting on the aluminum foil waiting for the

chips. Stand your chips, pins down on the foil and

examine one carefully, while touching the foil.

There may be a production number marked on the

underside, in case you need a reference mark. They

can also be marked with ordinary pencil lead if

you write small enough. Note the indentation at

one end that indicates the number one pin end.

Find out which way it should go in your board. The

pins taper outwards and need to be gently squeezed

together into two straight lines.

Press the side of the pins (lengthwise) against

the table to align and narrow down the spacing

between the lines. Do it again for the pins on the

opposite side. Now set it on the socket gently,

check direction for the dimpled end (socket covers

have been removed, if they existed). Inspect it

carefully, with a magnifying glass, if available.

If the pins line up with the contact holes

properly, then use your two thumbs and steadily

press it into the socket. You will feel the socket

grab the pins, but dont try to test its holding

power. It's hard to get them in but much harder to

take chips out of the sockets. There are special

chip tools that are available and are well worth

the money, if you are going to do much pushing and

pulling. Or you can use a couple of small

screwdrivers as wedges. But gently! Now take your

glass and inspect the chip pins for one that might

have been folded under when you pressed it in and

it was not in line with the hole. Inspect each pin

carefully for a few chips and develop a secure

feeling that you are doing it right before you

proceed too far.

When the board is stuffed with one full set of

eight or nine chips, (populated as they say) and

you have reset your switches properly, it is best

to test, doctor. But first, look around and read

the labels and signs so that the inside of your

computer isn't such a strange place next time you

have to dig under its covers. You probably feel

like the radio receiver owner, when he first

changed a tube, years ago. But it's just another

piece of hardware to get familiar with. Don't

touch but do look, ask questions, and read the

manuals. Take a photo or two.

Now, doctor, lets plug it in and test each set

before we suture up the patient, and put on the

cabinet again. Install the operating cables and

put the boot disk in the A: drive and turn it on.

Remember that with more memory its going to take

more self- diagnostic time, so dont be surprised

if 256K takes over a minute before the CPU

beeps,"All present and accounted for, Sir". Then

your normal boot routine should begin. Watch

carefully for error messages on the screen and

write them down. If you have parity lights on the

board note them and shut the power off in case

there is not a normal boot sequence (except for

the longer memory check at the beginning.)

DON'T PANIC -- SLOW AND EASY IS THE GAME -- THIS

HAS HAPPENED BEFORE AND PEOPLE LIVE THROUGH IT.

Here's a list of what could be causing a

malfunction, IF THERE IS ONE!  The switches on the

board and in the computer could be set wrong,

recheck them in writing. Have someone else double

check you. There could be a bad chip. (We'll tell

you how to look for it). You may have disturbed

something in your cleaning or reassembling the

cables and case. Make sure everything is properly

turned on for the test. There is also the remote

possibility that the expansion board is defective

or intermittant in the section you have just

stuffed. Examine the solder joints on the printed

circuit board in these areas. Retrace your steps

and restudy the manuals, check the switches and

then try it again after a short period. Shut it

down if it still doesn't boot properly and once

more check all possibilities. The chip failure

probability is less than any of the other

possibilities, especially wrong switch settings.

Remember there are totals of the PC mother board

chips, total of the expansion board's memory,

total of other boards like a C/PM adapter memory

and then the grand total of all RAM installed.

Check them by writing them down and see if the

switches are correctly set, one by one. Then if,

you find that everything is O.K. but it still

doesn't work the way it should, continue on.

Let's see if there is a bad chip. This is usually

the first assumption, but most often it is switch

settings or bent pins or bad sockets that make

intermittant contact or some silly thing like

leaving the keyboard unplugged. Since taking the

chips out is a stubborn operation that doesn't

improve with brute force, let's plan our attack.

If we have a couple of spare chips we can try them

in another working circuit on the board and reset

the switches. Go back to where you started. Ground

Zero we shall call it. Usually, you dont have to

pull the chips just reset the switches and try out

the old chips under the original conditions

(before stuffing). Put the board in and test it

again. It's probably O.K. and you haven't done any

damage. But electronic gremlins aren't called

glitches for nothing.

Now swap the new spares to determine if they work

in the working circuit. Yup, they work O.K. Now

let's start a planned procedure to reduce the

confusion and the number of times we have to push

and pull the same sockets and chips. We don't want

to damage our board or the chips with too much

handling or abuse. If you stuffed three sets of

chips without testing each set and the last one

shows the parity error then take those out to

test. Work your way backwards, one set at a time,

until you get to a working state. Remember to

reset the switches correctly each time you test

another full set of chips. You can not test

partial sets although they can usually remain in

the sockets if you know they are good. But, when

in doubt, take them out.

Now test three chips at a time and mark them. Put

nine that tested good in the old circuit into the

next set of sockets. If you only have one extra

chip, borrow the parity chip, and turn off the

parity reporting until you locate the bad chip

(probably only one, IF ANY). By testing your chips

two or three at a time in the working circuit, you

can isolate the bad group then test them one by

one to find the culprit, if there is one.

When you get through testing and stuffing, you'll

find there is nothing wrong with any of the chips

and they all work fine. There apparently was a

temporary glitche the first time around that

issued the parity error and it took a while to

discharge a condensor or change a flag or

something. That's why, when you buy a pocket

calculator, they tell you to turn it off and on

several times, if it doesn't work properly the

first time.

Try to keep track of your tests and actions on

paper so that you can isolate a bad chip or

convince yourself that it's something else that's

wrong. Mark the chips with a pencil or note the

production marks that should identify them

uniquely. For your own piece of mind when you

think that you have found a faulty chip, after all

else is done, test it to prove it's bad in a

working circuit, one chip at a time.

Don't be surprised if it tests good now. That's why

manufacturers call it "burning in". (Finding the

good and the bad actors). But usually the

percentage of bad chips is miniscule unless there

is a run that quality control didn't catch before

it left the plant, which is almost unheard of. If

it now works, burn it in. Leave a colored mark on

any suspicious ones, just in case you need to

locate an intermittent one in a couple of days or

so. Keep a written record of them also.

If you are using the same set of sockets to test a

lot of chips, alternate the sockets so that the

same ones don't get all the wear. Be careful not

to pry against any of the small electronic

components and damage them.

Good luck stuffing -- you'll find the experience

worth while and not hard at all once you get the

knack of it.