INTRODUCTION

============

PIBASYNC.ARC is an archive file which contains a set of routines for

implementing fully-buffered asynchronous serial port I/O from Turbo Pascal

on the IBM PC and compatibles.  The routines have been extracted

from my PibTerm v4.0 program and converted for used with Turbo Pascal v4.0.

Features of PibAsync:

-- Both input and output buffering for the serial port, resulting

in improved performance especially under multitaskers

-- Buffer sizes are user-definable up to 32K bytes each for

the input and output ring buffers

-- Speeds up through 38400 baud

-- Support for Com1: through Com4:

-- The ability to re-define port addresses, base RS 232 addresses,

and IRQ lines for non-standard ports

-- Optional XON/XOFF handling at the interrupt level

-- Optional CTS/DSR handling at the interrupt level

-- Machine code (via INLINE) provides respectable speed for

the basic functions

Limitation:

-- Only one serial port is supported at a time.  It would be

an interesting project for someone to provide multiple

simultaneous port access.

ARCHIVE CONTENTS

================

The following 18 files should be present in PIBASYNC.ARC:

ASCII.GLO     --- Some Ascii character set definitions

ASYINT.ASM    --- ASM source for asynchronous handler

ASYNCREC.ASM  --- ASM source for receive-a-character routine

ASYNCSEN.ASM  --- ASM source for send-a-character routine

DUMBTRM.PAS   --- Sample dumb terminal emulator

FDELAY.ASM    --- ASM source for 1 ms delay calculation

FIXBRACK.PAS  --- Routine for neatening .OBJ files produced

by the INLINE assembler.

GLOBTYPE.GLO  --- Global declarations for non-async-related variables

needed by async code

GLOBTYPE.PAS  --- Main program for unit GLOBTYPE.TPU.

MAKEDUMB.BAT  --- Batch file which calls TPC to compile DUMBTRM.PAS

PIBASYN1.MOD  --- Asynchronous I/O routines, part 1

PIBASYN2.MOD  --- Asynchronous I/O routines, part 2

PIBASYN3.MOD  --- Asynchronous I/O routines, part 3

PIBASYNC.GLO  --- Global declarations for async-related variables

PIBASYNC.PAS  --- Main program for PIBASYNC.TPU.

PIBTIMER.PAS  --- Time of day interface routines

README.DOC    --- What you're reading now

TIMEREC.ASM   --- ASM source for receiving character with delay

The .ASM files can be assembled with Dave Baldwin's program INLINE.COM,

which converts the assembler source to Turbo Pascal INLINE statements.

(You will need at least v2.11 of INLINE.COM to correctly assemble this code.)

The PIBASYN*.MOD files already contain the processed assembler source,

so you do not need to run INLINE on the .ASM files unless you want to alter

them.

COMPATIBILITY WITH PREVIOUS VERSIONS

====================================

The calling sequences of these revised routines are largely compatible

with those previously available as part of earlier versions of

PibTerm.  If you used those earlier routines, you should be able

to convert to these (with a resulting increase in efficiency) with

very little trouble.  The major important changes are that the

routines Async_Init and Async_Close now take arguments.

In the conversion to Turbo Pascal v4.0, I took advantage of the

new availability of the WORD type to add 38400 baud.

Hence, routines in which the baud rate is passed as an argument

(Async_Open, Async_Reset) now have the baud rate as a WORD

instead of an INTEGER.

DOCUMENTATION

=============

See the documentation block headers for individual routines in PIBASYN*.MOD

for details on the calling sequences and usage of the routines.

SAMPLE PROGRAM

==============

The DUMBTRM.PAS routine demonstrates how to use the PibAsync routines to

build a dumb terminal emulation program.

USE OF THE ROUTINES

===================

You may use these routines for any NON-COMMERCIAL purpose whatsoever.

If you change these routines, you MUST make the revised source

available.  Many individuals used the code from previous versions

of PibTerm and incorporated interesting changes in the async routines,

but did NOT make the resulting changes available in source form

so that all of us could benefit.  I take a dim view of that;  I have

made this source code available so that others can learn from what

I have done (and correct my mistakes).  I expect you to provide

the same courtesy in return.

SUPPORT

=======

Please realize that these routines are not a commercial effort and that

I simply cannot afford the time or money to individually support users.

I WILL NOT return phone calls about these routines, and I WILL NOT

fix your code for you.

There are a number of commercial packages of asynchronous routines

available for Turbo Pascal.  If you need a package for commercial

development, you should consider one of them.

You may send messages to me as indicated below, if you wish.  I will not

necessarily answer all queries individually, however.

COMMENTS, ETC.

==============

You may send comments to me via one of the following means.

I may not answer all messages individually.  Also, I don't check for

mail in these systems every day, so don't expect that a message

will reach me in less than a couple of weeks (except via BITNET).

BITNET:     PIB@NUACC.BITNET

CompuServe: CIS ID  72437,62

BBSs:

Gene Plantz's BBS   (subscription only)

Ron Fox's BBS       (312) 940 6496  (open)

Bob Blacher's BBS   (202) 547 2008  (open)

-- Thanks,

Phil Burns   11/01/86

Changes for v4.1 of these routines:

===================================

I corrected delay loops by avoiding the use of a Turbo run-time library

variable.  This 1-millesecond delay variable changed meaning at

v3.02 of Turbo, causing the various delay loops in PibAsync to

run incorrectly (for too short a time).  I now calculate and use

my own delay loop value, independent of Turbo's.

-- Phil Burns   12/27/86

Changes for v4.2 of these routines:

===================================

A couple of routines incorrectly left a word on the stack that should

have been popped in cases when the circular buffers were full.  Those

problems have been fixed in this version.

-- Phil Burns   01/24/86

Changes for v4.3 of these routines:

===================================

The send routine incorrectly tested for the wrong circular buffer pointer

when the output buffer was full.  This resulted in output buffer overruns

with lots of attendant nasty problems.

-- Phil Burns   03/25/87

Changes for v4.4 of these routines:

===================================

The code to setup mark and space parity was incorrect.  This has been

corrected in this version.

-- Phil Burns   07/20/87

Changes for v4.5 of these routines:

===================================

I have converted the async routines to a UNIT for v4.0 of Turbo Pascal.

To use the async routines, just do a

USES

DOS, PIBASYNC;

You don't need to include the code at all.  Just compile PIBASYNC.PAS

and place the resulting PIBASYNC.TPU in your unit include subdirectory.

38400 baud is now supported.

I've altered the character send routine to wait for a slot before

overwriting an existing buffer character when the output buffer is full,

rather than overwriting and then waiting.

The code to support serial ports 3 and 4 has been corrected.  Make sure

to call Async_Setup_Port before using ports 3 and 4, or when you're using

a non-standard port address on ports 1 and 2.  Async_Setup_Port will

now insert the port address you specify at the proper offset from

$0:$400, the address of the 8250.  If you set port address = -1, the

default port address is used; if you set irq = -1, then the default

IRQ is used.  The new function Async_Port_Address_Given can be used to

check if the RS232 base address is installed in memory or not.

The preprocessor symbol MTASK should be defined if you are using

these routines in conjunction with the PIBMDOS routines for interfacing

to multitaskers.

The routines DOS_Set_Intrpt and DOS_Get_Intrpt have been removed and

replaced by the standard TP4 routines GetIntVec and SetIntVec, respectively.

The batch file MAKEDUMB.BAT makes the DUMBTRM.EXE sample program

using the command line compiler (TPC.EXE).

-- Phil Burns   11/11/87