��������������������������������������ͻ

�                                      �

�            PEAK ENTERPRISES          �

�              P.O. Box 120            �

�            Hadlock,Wa.98339          �

�                                      �

�                 SETIT                �

�                                      �

�  A Program to Set Epson FX Printers  �

�                                      �

��������������������������������������ͼ

Page 1

OVERVIEW

This program to set Epson FX series printers is very easy to use as

it is completely menu driven. Just follow the screen prompts and you will

have no problems. It will set left and right margins, line spacing,

print style,etc. I am kind of a printer freak, and I use my printer

quite a lot, and I like to take advantage of all the options that

are available. Presto Chango -- SETIT appeared.

FUNCTION

SETIT is written in GW-basic and should run on most machines able

to use it. The registered version is compiled to produce a stand alone

program that will run on most MS /PC-DOS machines. I have an FX-86e,so the

codes used are for the FX series printers, specifically the FX-86e and

the FX-286e.  At one time I used an Epson MX series printer. I feel

that this program will probably work with the MX and RX printers

also, although I do not know for certain, not having the opportunity

to test this. All I can say is try it and see.

On to business.  SETIT can be run from any drive,including a

hard disk. Hard disk use will be covered later. First, make a backup

copy of your program disk.  DO NOT use the original to work with.

Put the original in a safe place.

Place the disk in the default drive, and from basic,load SETIT

and type RUN <ENTER>. First, a couple of notices will appear (3 to be

exact). Please read them and follow the screen prompts. (By the way,

if you like this program, thank Bessie, my Siamese  cat, who must

feel that I am not qualified to do anything by myself,as She insists

on sitting in my lap and overseeing everything I do.)

MAIN MENU

There are eight sections in the main menu.

1. Printer Operations

2. Overall Printing Style

3. Word Processing

4. Vertical Motion

5. Horizontal Motion

6. Print Size and Enhancement

7. Character Sets

8. End Program

Select #1, Printer Operations.  The menu will change to  the

Printer Operations Menu.  In this menu, there are Seven options

listed:

1. Enable Paper Out Sensor		4. Disable Paper Out Sensor

2. Turn Half Speed Mode On		5. Turn Half Speed Mode Off

3. Turn Auto Sheet Feed On	        6. Turn Auto Sheet Feed OFF

The options will be covered in order.

1. Enable Paper Out Sensor.  This is the default when your

Page 2

printer is powered up.  It is mostly used if you are using single

sheet paper, either with the auto sheet feeder or manually. It

sounds a beeper in the printer, and causes a little red light to

blink on the printer console.It also stops operation of the printer

until a new piece of paper is inserted, or this function is set to

off. It goes into operation approximately three-quarters of an

inch from the end of the paper. I strongly reccomend using this

function if you are using single sheets, as it will keep the

printhead pins from contacting the bare platten.

Obviously, option #4 disables this function.  I myself can see no

need to do so, but included it anyway.

Option # 2 is Turn Half Speed Mode On.  This function slows

the speed of printing. This means that in draft elite, you will

get about 120 CPS instead of 240, about 100 CPS in draft Pica

instead of 200, and around 20 CPS in Near Letter Quality Pica.

Option # 5 turns this mode off, and puts the printer back to

the default.

Option # 3 is Turn Automatic Sheet Feeder On.  This command

should only be used if you have the optional automatic sheet feeder

installed in your printer. This command overrides the setting of

DIP switch 2-2 on the printer, which also sets this command on or

off.  Option #6 turns  the automatic sheet feeder off.

Option #7 will return you to the Main Menu.

Back at the Main Menu now, select Option #2, Overall Printing

Style. Up comes the overall style printing Menu. In this menu, you

have the option of selecting either draft or Near Letter Quality

printing mode.  Selection of draft gives you the fastest printing

speed, about 240 characters per second. The printer prints each line

once, in a Sans Serif style print. This mode can be used in either

elite or pica print style. Elite print is slightly smaller in width

than pica. Printing in draft uses whatever print style the printer

happens to be in.  (pica or elite)  Selection of NLQ allow you to

also set which print style you want. When you select NLQ, you will

see another menu, which asks which print style you want. The default

print style is set with DIP switch 1-5.  This command will override

the setting of this switch.

When you select either one of these options, the printer will

print a test string to show you what you have selected and how it

looks.This holds true with almost all the options in this program

that make some sort of visible change in the style or format of

your printing, SO MAKE SURE YOUR PRINTER IS ON AND READY TO GO,

OR THE PROGRAM WILL PROMPT YOU TO DO SO. THERE IS ALSO THE CHANCE

THAT THE PROGRAM WILL ABORT AND YOU WILL HAVE TO START OVER.

However, if this happens, no damage or problems will occur. On my

computer, the program prompts -- on some compatibles it may abort.

Once you have selected NLQ, the menu will change to a choice

of either Roman style print, or Sans Serif.  Try both of them, and

see what the test string on the printer looks like.  Roman is like

the type you get on a typewriter, and will print in either pica or

elite, whichever is the current default. Sans Serif is more like

computer printing, in that the characters are somewhat squared.

This also will print in whatever is the default, pica or elite.

Option # 3 will return you to the Main Menu.

Page 3

Back at the Main Menu, select Option # 3, Word Processing.

Once again, you are now at another menu.  This menu lists

three options, one of which is return to menu. The two main options

are:

1. NLQ Justification

2. Set Intercharacter Space.

Select #1, NLQ Justification.  Here is another Menu. This one lists

four main options. They are:

1. Left Justification

2. Centering

3. Right Justification

4. Full Justification

NOTE: Justification is only operable in NLQ mode. It does not work

in DRAFT. If you want to print in draft, exit this menu.

1. Left Justification : This will print every line up against the

left margin default setting. Look at the test string, and you will see

that the tail end of the line is printed over on the left margin. When

you are composing your document or letter, you should run a test print

after setting the justification to see how it will come out. If your

lines are too long,using anything but the default left justification

can produce some real strange results.

2. Center Justification: This prints the tail end of the line

in the center. Try it and see how it looks. This function is best used

for documents that are specifically formatted to use center justification.

3. Right Justification :  The tail of the test string will be

printed on the right margin setting. Once agin, this is best used

for documents specifically formatted for this.

4. Full Justification: Means what it says. Try it and see what

it does.

NOTE: The default setting is for left justification.  Full justification

is performed when the printer buffer becomes full, or when a carriage

return, line feed, vertical tab, or a form feed is recieved by the

printer.  Horizontal tabs are invalid except in left justification

mode. Justification of any kind will not work in draft mode.

Back to the Word Processing Menu. Choose #2, Set Intercharacter

Space.  This function sets the amount of space added to the right of

each character as it is printed. This space is in ADDITION to the space

already allowed in the basic design of the character.  You may enter

a number from  1  to  63.  The higher a number you enter, the more space

will be printed between characters.  Try a couple of experiments to see

what kind of results you get. Each number higher that you enter increase

the amount of space between characters by  1 / 120 th of one inch. If

you enter 63, the space will be 63 /120 ths of an inch, in addition to

the design space.  This comes out to about one - half an inch between

characters. In effect, you will get about sixteen characters on one line

of print.  Have fun  ! !

Page 4

At the Main Menu again.  Select Option #4, Vertical Motion.

This section allows you to send a form feed to the printer, and to

set different line spacings. You can choose line spacings of 1/8,

7/72 inches,or reset to the default of 1/6 inch from whatever it was.

Try each one, and see what it looks like. Once again it is suggested

that you make a test print of your document after changing the line

spacing, to make sure you like it, and that it does what you want.

Main Menu again.  Select # 5, Horizontal Motion.  This section

allows setting the left and right margins, up to twenty spaces on

either end of the line.  Twenty spaces was arbitrarily selected by

me.  If feedback from you users indicates a need for more, that will

be expanded in future updates.

Try the left margin settings first.  Set it at ten and then twenty

and look at the test string printed. Now select right margin and set

that at twenty and see what you get.  For normal use, a setting of

between five and ten on the left margin, and zero on the right margin

is recommended. This will print a line of seventy five spaces long

without overlap.

Back to the Main Menu.  Select Option # 6, Print Size and Enhancment.

You are now at another Menu.

Option #1 : Select  Pica/Elite

This function selects either pica print, or elite print. Pica is larger

than the elite.  Pica runs ten characters to the inch, while elite runs

at twelve characters per inch. Using elite produces a line that can

squeeze a few more characters into the line. Pica is the default setting

on power up of the printer. Try each one and see how it looks.  Elite

can be somewhat difficult for people  like me who need glasses to read.

Select # 2, Proportional Print.

The width of proportional print varies from character to character.

Therefore, a narrow character such as an "i" will recieve less space

than a wide character such as a "w".  This function will not work on

FX-86e printers, but if used, WILL OVERRIDE THE CONDENSED PRINT FUNCTION.

If your printer will not go into condensed, set the proportional to OFF.

On FX-286e printers, this function works,and also overrides the condensed

function.

Select # 3, Condensed Mode.

This command sets the printer to the condensed mode.  In this mode, the

print is approximately 60%  of normal width.  For example, the condensed

pica print has 17 characters per inch as against 12 characters per inch

in normal width.  This mode cannot be selected in the printer is in the

proportional mode. Proportional mode cannot be condensed. It does, however,

work in both pica and elite print styles, as well as NLQ and Draft.

Select # 4, Double Wide Mode.

This command sets the printer to Double Wide.  In effect, each characte

is printed twice as wide as in the normal width. Look at the test string

Page 5

printed to see what it looks like. This command can be combined with others

in this section to produce some very striking printing. For example, try

this: Select # 4, Double Wide, and then select #5, Double High. Check the

test string.  How about that ?

Another one you can try is to select #4, Double Wide, #6, Emphasized,

and #3, Condensed. If you are in NLQ mode, this prints large, very nice

looking characters which are easy to read. I like this one because I can

read it without my glasses. Feel free to experiment with the various

commands and combinations.  You will not hurt your printer. If it does,

you had better return it to your dealer. It isn't right.

Select # 5, Double High.  Check the test string. This prints the

characters about twice as tall as normal. The width remains the same.

If you choose to use this, then make sure you double space what you are

printing, otherwise there will be no space between lines.

Select # 6, Emphasized Mode.

This command puts your printer into the emphasized print mode.  What this

does is the print head will make two passes at each line, and on the second

pass, it prints slightly out of register to the right with the first pass

which gives your print a bolder look.

Select # 7, Double Strike Mode.

This command does basically the same as the emphasized mode, in that on

the second pass, it prints slightly out of register with the first pass.

The difference between the two is that Double Strike prints out of register

below the first pass instead of to the right.  Using both of these commands

produces a very bold print.  In the NLQ mode, this function is not

available, but is not cancelled.  This is a normal function of the NLQ

mode, as the print head makes two passes with the second slighty out of

register.

Select # 8, Superscipt Mode.

This command sets the printer to print letters about two - thirds the

normal height, and prints them in the upper portion of the character

space. If you have good eyes, and really want to cram a lot of text onto

one page, try this.

Select # 9, Subscript Mode

This is basically the same as the superscript mode, except it prints in

the lower portion of the character space. Try it and check your test string.

Select # 10, Underline Mode.

This command prints in normal or condensed mode, pica,elite,NLQ,draft,etc.

It prints an underline on the whole line, including all spaces. Try it

to see what it does.

Back to the Main Menu !

The last option is # 7, Character Sets. This menu is used to select the

following:

Page 6

1. Character Graphics  (ASCII codes 128 - 255)

2. Italics

3. International Character Sets.

Option # 1: Character Graphics.

This command duplicates the setting of DIP switch 1-3. It also will

override the setting of this switch. This command does not disable the

printing of Italic characters, as italic printing can be selected by option

# 2.  What this function does is enables your printer to print the ASCII

codes from 128 to 155 as graphic characters . Instead of treating these

codes as control codes, your printer will print them.  Notice the cover

sheet of this document. The border around the printed information was

created using these codes.  If this function was disabled, what you would

get instead of the nice double line borde would be letters of the alphabet

printed in Italics. In normal usage, you will see no difference in the

print, unless the program you are running uses these codes. In some of

the form printing programs, it will print nice lines instead of dotted

lines.

Select Option # 2, Italics.

This command will cause your printer to print the letters of the

alphabet, numbers, puncuation marks,brackets,parentheses,etc., in Italics.

This command is valid even if Character Graphics has been selected, but

the character graphics printed will NOT be italicized.

Select Option # 3, International Character Sets.

This command enables you to select one of thirteen different character

sets tailored for different countries. There is one each character set

for countries such as  USA, France, Germany, UK, Sweden, Italy, Japan,

Norway, and Latin America. There are two each sets for Denmark, and Spain.

The difference between these sets is small, mostly in punctuation, and the

dollar sign. The USA set uses the regular dollar sign, while the UK set

uses the pound sign.Check the test string printed and you can see the

differences.

System Requirements

This program was written for  MS-DOS / PC-DOS machines with a monochrome

monitor, at least one disk drive, and at least 128K of system memory.

Computers that are not very closely compatible may or may not run this

program. I make no promises.

Hard Disk

Copy this program to your hard disk, preferably in your boot directory

and then load SETIT and RUN it.  As far as I know, there should be no

problems running from a hard disk.  If there is, let me know. SETIT can

also be used in your AUTOEXEC.BAT file to bring it up first, whether

you are using a hard disk or not. Consult your manual for the proper syntax.

Page 7

ACKNOWLEDMENTS

EPSON, FX-86e, FX-286e are registered Trademarks of Epson America,Inc.

MS-DOS is a registered Trademark of Microsoft, Inc.

PC-DOS is a registered Trademark of IBM, Inc.

GW-Basic is a registered Trademark of Microsoft,Inc.

DISCLAIMER

Neither the author, Robin Peak, nor Peak Enterprises makes any waranty

of any kind.  This program is provided "AS IS" and the entire risk of use

or quality or performance lies with the user. No representations as to

SETIT 's suitability for any purpose are made. Robin Peak and/or Peak

Enterprises will not be responsible for any loss or damage of any kind

whatsoever, even if Robin Peak and/or Peak Enterprises has been notified

of same. SETIT is a copyrighted work and is supplied under the User

supported Software concept, and all rights remain the property of

Robin Peak and Peak Enterprises.

REGISTRATION OF SETIT

Registration fee is Twenty Five Dollars in U.S.funds.

For registering, you get the following

1. A registered, serialized copy of SETIT with all the warning screens

removed. It is compiled to run stand alone, and is much faster and

easier to start and run. No loading of basic and all that stuff.

2. Twenty Five Dollar discount on the purchase of updates.

3. Purchase of Source Code (either printed or on disk)

for the sum of Fifteen Dollars in U.S. Funds

4. Free telephone support.  (You pay for the phone call)

5. Free written support.

6. Any registered user who submits suggestions for changes that

are used will recieve a free copy of updates including that change.

Registered copies of SETIT are not copy protected. I have no desire to

go into the problems of copy protection. Registered users may make as

many copies of their registered disk as is needed for backup, etc. The

registered disk may NOT be sold, given or lent to any person who has

the opportunity to make an illegal copy of same.  Source code is strictly

for the individuals use.  It may not be sold,given away or lent to any

one. It may not be used for any commercial purpose, nor can portions

of it be included in any other program except for personal use only.

It is wholly owned by Robin Peak and Peak Enterprises and all rights

are reserved. No representations of suitability for any purpose whatsoever

is made regarding SETIT or the source code thereof.

Page 8

Send registration fee to:

Robin Peak

Peak Enterprises

P.O.Box 120

Hadlock, Wa. 98339

When you register, you get my phone number. If you are not a res

registered user, don't bother to call me.  All you will get is a lecture

regarding TANSTAAFL.  (There Ain't No Such Thing As A Free Lunch ....

Robert A. Heinlein) Any one who has any suggestions as to changes or

upgrades, please feel free to write to me at the above address.

USER SUPPORTED SOFTWARE

Writing software takes a lot of time and effort on the part of the author.

By registering your shareware copy with the author and making a suitable

contribution, you are doing your share to keep this concept viable.

Software prices are quite reasonable under this method. Help keep

prices down.

You can also help by making your shareware version accesible to as

many people as possible. If you like the program, share it. Pass it around.

Write revues for your computer club magazine.  Put it on the bulletin

boards, for others to download.

U P D A T E S

I have a number of plans for this program, some of which are to

include a section to cover IBM Printer Emulation.  A lot of the control

codes are the same, and SETIT as it is now will work on IBM printers.

The commands that will NOT work are as follows:

1. Set left and right Margins.

2. Select Pica / Elite Pitch

3. Turn Proportional Mode  ON / OFF

4. Cancel Condensed Mode

5. Select Double High Printing.

6. Select International Character Sets.

Use of SETIT on IBM Printers may produce some very strange results

and all I can say is if you want to try it, it is your choice and your

responsibility. However, if you should run into problems, you can always

shut the printer off and re - power up. This will reset the printer to

its' default values.

I plan to look into using windows in future versions of this program.

There are some commands not included in this program , such as setting

non standard line spacing that I intend to include in future updates.

Any suggestions for changes or additions to SETIT are actively

encouraged.  After all, SETIT was written for the user, so any feature

the user would like, I will do my best to accomodate.

Copyright (c) By Peak Enterprises   1988

=============================================================================

E N D   O F   S E T I T   D O C U M E N T A T I O N