SKO1430.COM file Information

======================================================================

�

Seiko CM-1430  Monitor

======================

�

We have developed a solution to the problem that some Seiko Monitor

owners are finding. The problem occurs during operation of the Seiko

1430 monitor with the 800 x 600 modes. The solution is SKO1430.COM, a

program residing on the Utility Diskette included with your ProDesigner.

To install the solution, simply copy the program to your hard disk

(assuming drive C:) by placing the Utility Diskette in drive A: and

typing: �

COPY A:SKO1430.COM C: <press ENTER>

You can run the program from your DOS prompt, by typing:

SKO1430  <press ENTER>

You will only need to run SKO1430 one time during your computer

session, before you use 800 x 600 modes.

If you use 800 x 600 modes regularly you may want to run SKO1430.COM

from your AUTOEXEC.BAT file so that the SKO1430 program will

automatically run each time you boot up your computer. You will need

to edit the AUTOEXEC.BAT file with an ASCII text editor (such as

EDLIN), or an editor that will save an edited file in ASCII format.

See your DOS manual for more information on the DOS text editor EDLIN

and the formation and use of an AUTOEXEC.BAT file.

Please note: If you find problems while using a mouse or other memory

resident programs, try loading SKO1430 first before you load any

resident memory programs.

The following questions and answers will help to explain SKO1430.COM:

Question:  What is SKO1430.Com?

Answer:  SKO1430.Com is a program designed to permit the ProDesigner

to run the Seiko 1430 monitor in 800 x 600 modes. It is RAM

resident software (referred to as a TSR), approximately 12K

in size.

Question:  Why do we need SKO1430.Com?

Answer:  Although the Seiko 1430 supports 1024 x 768 modes, it uses

values which do not support the popular 800 x 600 scanning

frequencies, such as ProDesigner modes 29 (800 x 600, 16

colors) and 30 (800 x 600, 256 colors). Orchid Technology

is one of the few companies who have developed the software

necessary to support 800 x 600 on the Seiko 1430.

Question:  How do we recognize the Seiko monitor problem?

Answer:  If you are using the Seiko 1430 monitor in 800 x 600 mode

the screen may distort or blank out. In either case you

need to use the SKO1430.Com program to correct the problem.

Question:  How does the SKO1430.Com program work?

Answer:  The SKO1430.Com program is loaded into memory at the DOS

prompt (or in a batch file) where it remains to intercept

and change the Prodesigner signals before they reach the

Seiko monitor.