VIPER VLB Installation/Utilities/Drivers Diskette

Release Notes

***************************************************************************

** IMPORTANT VIPER VLB SOFTWARE INSTALLATION NOTES **

1.      DO NOT INSTALL THE VIPER VLB WINDOWS 3.1 DRIVERS INTO YOUR WINDOWS

DIRECTORY. DUPLICATE FILES NAMES EXIST!!

2.      It is best to use the VPRMODE.EXE utility to properly configure

the Viper VLB for your monitor BEFORE using Windows 3.1 or AutoCAD.

!!!  BEFORE USING THE VIPER VLB WINDOWS 3.1 OR AUTOCAD DRIVERS, USE  !!!

!!!  GO.EXE TO INSTALL THE VIPER VLB UTILITIES, AND USE VPRMODE.EXE  !!!

!!!  TO PROPERLY CONFIGURE THE VIPER VLB FOR YOUR MONITOR            !!!

** IMPORTANT WINDOWS 3.1 DRIVER INSTALLATION/CONFIGURATION NOTES **

1.      Start by running the GO utility from the floppy disk.  If this

is your initial install of the Viper utilities or if the system

finds an old version of the Viper.ini file, you will be prompted

to enter a path for the new Viper utilities.  The default is:

C:\VIPER.

2.      Once this procedure is complete, you can select the Windows 3.1

option.  The GO utility will then copy all the Windows files to

your hard drive.

3.      When GO has finished copying the files, you will be prompted

for the location of your Windows sub-directory.  The GO utility

will then modify the [MMCPL] section of your Control.ini file with

the line:  "TurboWin=<Path of Viper Windows files>\TURBOWIN.DLL".

4.      Now entering Windows using the standard VGA driver supplied by

Microsoft.  The Diamond Viper icon will now appear in the Control

Panel and allow you to change resolutions, colors, and fonts.

UTILITY FILES

VPRMODE.EXE     command line and menu driven mode and monitor setting

utility.  Use VPRMODE.EXE to configure the Viper VLB

for your particular monitor, to change the VIPER VLB

memory address, to select a text display mode, and to

run Viper VLB diagnostics.

VPRMODE saves monitor timing information to the

VIPER.INI file.

NOTES:  1. use VPRMODE ? for command line option list.

2. the VPRMODE monitor configuration save screen

may be displayed after running some of the

VPRMODE commands.  If this happens, select

ENTER to save. The screen will not appear again.

3. KNOW YOUR MONITOR! Please refer to your monitor

manual or manufacturer for correct monitor timing

information before running VPRMODE, and before

contacting Diamond for technical assistance. If

you do not have this information, DIAMOND WILL

NOT BE ABLE TO HELP YOU with VPRMODE problems.

VIP.EXE         utility which will display or write to a file important

system configuration information which can be used as

an aid in trouble-shooting. Type VIP to display to screen,

and VIP <filename> to save to a file.

NOTES:  1. the VPRMODE save configuration screen may be

displayed after running VIP.  If this happens,

select Enter to save.  The screen will not

appear again.

VDIAG.EXE       diagnotic utility which will display color test patterns

in each resolution supported.

NOTES:  1. to test extended modes, load the VESA BIOS

extentions before running VDIAG.  This can

be done by typing  VPRMODE VESA at the command

prompt.

CHKPATH.EXE     utility which checks the PATH environment variable for the

specified string.  Is used in the AUTOEXEC.BAT file with

the VIPERPATH statement.

VIPER.INI       this file is used to store the monitor and configuration

information for the Viper VLB, and is updated each

time you choose to Save Configuration from the VPRMODE.EXE

program.  Drivers find the location of the VIPER.INI file

by checking the VIPERPATH environment variable.

VIPERPATH       an environment variable which is used by device drivers

(Windows 3.1, AutoCAD) to locate the VIPER.INI file.

This variable is set by the DOS SET command, and is

automatically appended to your AUTOEXEC.BAT file during

Viper VLB Utility installation.

!!!!!  IMPORTANT !!!!!

THE VIPER.INI FILE AND THE VIPERPATH STATEMENT ARE

NEEDED FOR PROPER USE OF THE VIPER VLB WINDOWS AND

AUTOCAD DRIVERS!  DO NOT REMOVE THEM FROM YOUR SYSTEM.