Program: Olivetti JP for WB 1.0

Author:  Peter Hutchison

Date:    26/04/99

Version: 1.0

Requirements: Any Amiga, WB 1.3 or better

Introduction

------------

This is my ninnth driver to be released . This is a colour printer driver for

the Olivetti JP series of inkjet printers. This uses the HP Deskjet 500C PCL

command language

Features

--------

Colour: CMYK (Colour AND Black) dual cartridges

Photo cartridges are NOT supported. Sorry.

Resolution: 75 to 300 dpi.

Graphics: Upto 300 x 300 colour, b/w

Aspect:  Portrait and Landscape

Output:  B&W and Colour Text,

B&W, greyscale and colour graphics

Typefaces: Courier, CG Times, Letter Gothic, Universal, LinePrinter,

Pica, Prestige, Elite, Script, Helvetica, Presentations.

(Unfortunately, no room for 7 other typefaces)

Uses character symbol sets fonts for other typefaces.

*NEW - Optimised for Speed and Versions for '000 and '020 processors.

Compiled with SAS C 6.58

Densities

---------

0 75 x 75

1 75 x 75

2 150 x 150

3 150 x 150

4 300 x 300

5 300 x 300

6 600 x 300

7 600 x 600

NB: 600x300 may produce crushed output in some cases.

US LEGAL supports 14in paper.

Supported Printers

------------------

Olivetti JP 50, 70, 90, 150, 170, 190, 250, 270,

350, 360, 570, 450, 470.

NB: Use Olivetti_JP790 driver for the 790 printer.

HP PCL 3 Emulation (HP 500C).

Installation

------------

Run the Install program and choose processor type and model closest to your

printer from the list given.

Select your printer with the Printer preferences program. Select Single for

Paper Type and DIN A4 for Paper Format and set lines to 66, then set Margins

to around 2 and 80. For other paper sizes, change values to suit your

paper. Select Save.

Load PrinterGfx and set Floyd-Steinburg dithering, Integer Scaling, Positive

Image, Horz aspect, Color and Ignore limits. Using a Threshold of around 8 or less to

minimise use of ink and density 4 for general use. Select Save.

If you are experiencing problems with this driver send me email (see below)

and specify the version of Driver you are using (see this doc), make of

printer and what program you are printing from.

For extra features try the new v42+ printer.device from Aminet.

History

-------

1.0   First Release with Typeface support.

Giftware

--------

This driver is Giftware, you are not obliged to send anything but if you

want to contribute to further driver development for this printer and others

then send me either money (5UKP) or something 'Amiga' to me at:

Email: pjhutch@blizzard.u-net.com

Web:   http://www.blizzard.u-net.com

or

Peter Hutchison

75 Lower Skircoat Green

Copley Lane

Halifax

W Yorks

England

HX3 0TG