PORT FINDER                                      December 1, 1989

Port Finder is a COM and LPT port utility.  Its primary  function

is to activate nonstandard serial COM3 and COM4 port  addresses.

COM3 and COM4 serial port cards have been around for a while, but

it usually has taken special software to utilize them.  When  IBM

came  out with the PS/2 line of computers they required the  then

new  IBM  DOS 3.3 to work properly.  Both DOS 3.3  and  the  PS/2

computers support COM3 and COM4 serial ports, however IBM decided

to  use  COM port addresses that where different  from  any  port

address  used  by  the clone computer makers.   In  addition  the

detection of all COM ports is done by the BIOS.  Because IBM AT's

and PC's did not look for a COM3 or COM4 port, and all clone PC's

and  AT's  are based on the IBM design, none of these  older  ma-

chines look for the COM3 or COM4 addresses.  Both IBM DOS and  MS

DOS  3.3  and 4.0 support COM3 and COM4 serial ports.   DOS  only

provides half of the solution however and will not use a COM3  or

COM4  serial port if the BIOS does not establish it's (COM ports)

presence.  So even though you can purchase and run DOS 3.3 or 4.0

on  your system, your COM3 or COM4 serial port (if you have  pur-

chased  and installed such a port card in your system)  will  not

work with DOS unless you have a PS/2 computer.

Port Finder provides a simple solution to this problem.  It looks

for several non IBM COM3 and COM4 port's addresses. When it finds

the port address is present, it takes the port address found  and

places  it in the low memory BIOS area.  In addition Port  Finder

will  update the serial port count which is also kept in the  low

memory  BIOS  area.   You would normally place  the  Port  Finder

program name (PF) in your AUTOEXEC file so that the COM3 and COM4

port  will be enabled each time you boot your system.  You  would

further need to use IBM DOS or MS DOS versions 3.3 or 4.0 for the

complete  solution.  If you install a COM3 or COM4 port, use  DOS

3.3  or  4.0, and run Port Finder, you would find that  DOS  does

recognize  the new port and you will be able to use the DOS  MODE

command to set the baud rate and any other DOS command that could

be used with a serial port will work.

Port  Finder provides a second function with the ability to  pro-

duce  all possible COM and LPT port swaps.   Port swapping  would

be used to keep from swapping cables for programs you are  unable

or unwilling to change the port the program is setup to use.  You

may  also  have a program that allows you to only enter  COM1  or

COM2  but  you would like it to use COM3 or  COM4.   Port  Finder

provides full port swapping capability for LPT ports as well.

One  potential problem trying to use or swap a COM3 or COM4  port

is  that not all programs go through DOS to use a COM port.   Any

program  that by-passes DOS will be unaffected by any Port Finder

action.   Many  communications programs such as  Procom Plus does

by-pass DOS and read and write directly to the serial port.  Many

programs  will do this if they need a higher port speed than  DOS

can  supply.  However most new programs such as Procom  Plus  can

still work with COM3 and COM4 so you do not give up your COM3 and

COM4  port  if  present.  If you should  forget  what  ports  are

swapped,  Port Finder provides a reset command for both  COM  and

LPT ports that will restore the normal port locations.

In addition to COM port activation and COM and LPT port swapping,

Port  Finder can be used as a diagnostic tool.  In this  capacity

it  informs  you of responding ports along with the  actual  port

address  in HEX which can save you time when setting up a COM  or

LPT port in your system.  Port Finder informs you of all possible

memory  types with total installed memory, your DOS version,  EMS

version,  display  adapter type, and your BIOS  date.   One  more

strange  feature  is that it will install your  Joystick  in  the

equipment  list if present.  This is also a BIOS  function  which

IBM did use in it's PC computer BIOS's, but they decided to leave

this function out when they made the IBM AT computer.  Not having

your  Joystick  in  the BIOS equipment list does  not  keep  your

Joystick from working, however many diagnostic programs will look

at  this information and will tell you that your system does  not

have a Joystick when if fact it does.

Port  Finder  is very easy to use.  You can run it from  the  DOS

command line if you like or you can use it in a batch file.  Port

Finder  can also be used in a batch file to determine  which  COM

ports  are  present as it generates an Errorlevel  based  on  all

possible  COM  port combinations.  Port Finder  includes  a  nice

built in help screen.  You bring it up by entering "PF H" at  the

DOS  prompt.  All Port Finder commands are listed and basic  pro-

gram operation is discussed.  All program documentation is locat-

ed on the program disk.

Port  Finder is a Shareware program which costs $12.50  including

shipping.   If  you would like to try it before you  buy  it,  an

older  version of Port Finder is available on Compuserve  in  the

IBMHW  section, general hardware library.  The program is  called

PF24.COM  at this writing.  If you decide to purchase the  latest

version  of the program you can order it from  mcTRONic  Systems,

7426 Cornwall Bridge Ln., Houston, Texas 77041, phone number 713-

462-7687.   Considering  what the program does it  seems  like  a

bargain to me.