WINH19/WINH19K/WINKER

Emulation of the Heath/Zenith H/Z-19 Terminal

with the Kermit File Transfer Protocol

Copyright(c), 1988, 1989 by

William S. Hall

3665 Benton Street, ##66

Santa Clara, CA 95051

All rights reserved

INTRODUCTION

Winh19, Winh19k, and Winker are communications programs

running under Microsoft Windows and emulating the Heath/Zenith

H/Z-19 terminal.  The programs provide both Heath and ANSI modes.

Consequently, the main features of  the Heath/Zenith series

terminals are available in the Windowing environment.

The emulation features a very precise rendering of the

H/Z-19 keypad as well as a font containing the original graphics

characters.  Certain improvements have also been made over the

original design.  In particular, some function keys can be

programmed, default settings can be made from the program, and

the number of lines and columns the terminal has can be preset in

win.ini.

Winh19 contains only the terminal emulator.  At run time, it

uses the least memory of the three programs.  Winh19K and WinKer,

which are functionally equivalent, additionally contain the

Kermit file transfer protocol.  Having most of the basic features

of any Kermit implementation, the user can transfer files and

control the host whenever the latter is also capable of running

Kermit.  The programs are distributed as the pair Winh19/Winh19k

or as WinKer alone.

Since these are  Windows programs, the user can have up to

two transactions in progress simultaneously, and file transfers

can take place while other programs are active.

Only a few features of the H19 are not available.  Later

we discuss the precise differences, but briefly they are: no key

click control, no Enter key emulation on the keypad, and the

special keys on the top row have been relabeled.

The current version also lacks a few standard features of

Windows.  There is no selective copy-to-clipboard function.

However the entire contents of the video buffer can be slapped to

the clipboard, and text can be pasted into the program.  In

addition some keys do not have a corresponding menu item.

UNIX users can specify h19 in their TERM variable to obtain

Heath mode.  If this is not available, vt52 may suffice.

Alternatively, one can specify h29, h19a, or  ansi.  Depending on

how well the termcap file has been written, such features as

cursor key operation may or may not be recognized.

The programs will run only in Windows 2.03, Windows/286,

or Windows/386.  Windows 1.x is already obsolete.  If you are a

registered owner of Windows, you can receive an upgrade from

Microsoft at a nominal price.

To appreciate fully the features of this terminal, the user

should refer to the original Heathkit documentation.  We have

summarized most of the commands in the Appendix, but the

information is abbreviated.  The best source of detailed

information is in "Heathkit Manual for the Video Terminal, Model

H19, Operation", Part Number 595-2284-01, Heath Company, Benton

Harbor, MI 49022.

TERMINAL OPERATION

The programs are very easy to use.  Simply copy the one you

wish to your Windows directory and run it from Windows.  You will

be asked if the default entries should be added to win.ini.  If

you agree, win.ini will be modified and you can then continue.

Otherwise the program will exit.  If you agree to continue, the

program will try to open the first communications port specified

in win.ini.  If this is not successful, it will try the next one

listed.  If this one also fails, the program will exit with a

message.  We assume only COM1: or COM2: are available.

If a port is opened and successfully initialized, you will

find yourself already on-line and ready to communicate.  Most

communications and terminal parameters may be changed during the

session using the Set Communications and Set Terminal options in

the Emulator menu.  The values you choose can be saved

permanently by checking the 'Save' item in the dialog box.  In

addition, by checking the 'Default' item in the communications

dialog box, the selected port becomes the first port tried when

the program is run.

With the Edit menu you can paste text from the clipboard

when on line, and you can copy the contents of the entire screen

to the clipboard.  No attempt has been made to preserve such

attributes as inverse video or the graphics characters during

copy.

If you plan to use the graphics character set of the H19

terminal, you should install the font Winh19.fon with the Windows

control panel.  Otherwise, you will get the normal Windows

terminal (OEM) font, and if you go into the terminal's graphics

mode, you will see the usual characters supplied by IBM for the

control characters.  You may want to install the extra font at

any rate if you are using a display such as the EGA where the OEM

font is really too small, or the AT&T/Olivetti 640x400 screen

where the OEM font is a bit too large to get the entire H19

window on the screen. Winh19.fon is in the Windows 2.0 format.

Basic Functions

It is typical of terminals of this type that every operation

is performed by its receiving commands as sequences of characters

prefixed by the escape character.  Such sequences can originate

from the keyboard if the terminal is off-line or can be received

from the host.  Only one setting cannot be changed in this way,

and that is whether the terminal is to run in local-echo mode or

not.  All other operations are obtained via escape sequences.

Typically, however, a user needs to know very few of these

commands; the host computer, if it knows the terminal type, will

perform them as required.  However, most basic operations can be

carried out using the keyboard and/or the menu.

The Off Line/On Line menu item can be used to toggle between

line and local modes.  Either click on the item with the mouse or

simply type ALT-O (Letter O).  In local mode you can explore the

entire range of the terminal's capabilities including the

behavior of the keypad, which has been organized exactly as the

keypad on the H19.  In particular you can make mode and setting

changes by typing the corresponding escape sequence.  For

example, to set the terminal to 300 baud when in Heath mode, go

off-line and type ESC r C.  To set a block cursor,use the

sequence ESC x 4.  See the Appendix for a complete list of such

commands.

Function keys F1 to F5 and F7 to F9 can be programmed to

send user-defined strings if the control, shift, or control-shift

keys are also actuated.  Function key F10, and if you have an

extended keyboard, F11 and F12, can additionally be programmed in

unshifted mode as well.  F6, which plays the role of the ERASE

key on the H19 cannot be altered.  The ability to program the

function keys is NOT part of the standard capabilities of the H19

but is an added feature.  See the win.ini discussion below for

more information on setting these keys.

Some terminal commands are available directly from the

keypad or the function keys.  For example, F6 will clear the

screen from the present cursor position to the end of the screen,

and Shift F6 will clear the entire screen.  If the terminal has

shifted keypad mode set, then the keypad performs editing

functions.  In particular, cursor movement is as expected.

However, the rest of the keypad may function differently from the

actions marked on the keys as its action in this program is

exactly as on a real H19.  A complete account of keyboard usage

is in the Appendix.

Note that all of the editing key actions such as clear

screen, delete a line, move cursor, etc., will not be transmitted

to a host when on-line if the control key is held down.

You can send a break with CTRL-BREAK, or on an extended

keyboard, CTRL-SCROLLOCK.  The break signal is active as long as

the key remains depressed.  CTRL-BACKSPACE will send a DELETE

character to the host.  CTRL-J will send a line feed character.

The emulator can be reset to its 'power-up' values by using the

Reset selection in the Emulator menu.

Finally, you can use the Scroll Lock key to conveniently

stop and start the display.  Hold screen is also available.

Simply go off line and type the appropriate escape sequence for

the emulation mode in use.  Then when a carriage return is

received on the bottom line, the display will pause until you

strike the Scroll Lock.  This will allow the next line to

display.  You can see the entire screen by holding down the shift

key when depressing Scroll Lock.  To end the hold screen mode,

again go off line and type the correct sequence.

Win.ini Settings

Here is a typical set of values in win.ini along with some comments.

[Winh19]

port1=COM2:		; try to open this port first.

port2=COM1:		; if unsuccessful, try this one next.

COM1:=1200,e,7,1	; set this port to 1200 baud

;    even parity, 7 bit word, and 1 stop bit.

COM2:=9600,n,8,1	; set this one at 9600 baud, no parity,

;    8 bit word, and 1 stop bit.

BlockCursor=0		; set to 1 for a block, 0 for an underline cursor.

Keyclick=1		; currently not implemented.

WrapAround=1		; terminal does line wrap if set to 1

AutoLFonCR=0		; does LF on receipt of CR if set to 1

AutoCRonLF=0		; does CR on receipt of LF if set to 1

ANSIMode=0		; Heath mode if 0, ANSI mode if 1

ShiftedKeypad=1	; Keypad is shifted if set to 1 (keypad sends

;    edit function commands).

FullDuplex=1		; Terminal is in local echo if 0.

lines=24		; 24 lines plus status.  Maximum is 40

columns=80		; 80 columns.  Maximum is 95

SF1=ATDT 5555555	; Shift F1 sends this Hayes-like dialing sequence.

SF2=whall		; Shift F2 sends my login command.

CF1=ats16=5d		; Control F1 turns on analog loopback on a

;     U. S. Robotics Courier modem.

CSF2=atz		; Control Shift F2 resets U. S. Robotics Courier modem.

CSF3=dir\M		; Command to display a directory on host.

The settings for COM1 and COM2 are in the format for an

MS-DOS mode command.  In particular, you cannot currently select

either a default parity of mark or space.  However, you can

configure the terminal for such a parity type from within the

program.

The number of lines or columns can only be set by editing

win.ini. You may want to experiment with suitable values.  On an

EGA display, 27 lines works well.  30 lines gives a nice display

on the AT&T/Olivetti 640x400 monitor.  Finally, you can use up to

36 lines comfortably on the VGA.  95 is the upper limit for the

number of columns.  Don't forget to inform the host computer

about the number of lines on your terminal if you select

non-standard values.

The keystrings and corresponding keys are as follows:

F10 to F12		F10 to F12

SF1 to SF12		Shift F1 to Shift F12

CF1 to CF12		Control F1 to Control F12

CSF1 to CSF12		Control-shift F1 to Control-shift F12

You can change these strings from the program with the Edit

Function Keys menu.  Note that when editing, you have complete

capability to cut, copy, and paste the strings from one selection

to the next.  However, you must use the keyboard interface at

present, although the mouse can be used to select text for a copy

or cut operation, as well as to save the string to win.ini.

Control characters ^@ (NUL) thru ^[ (ESC) can be used in

strings by prefixing with a backslash '\'.  Thus '\M' means a

carriage return and '\[' is an escape.  To place a backslash in a

string use '\\'.

Note that key F6 is currently always grayed and cannot be

selected in the dialog box.

Winh19.fon

WINH19.FON is an OEM type font using a Modern typeface.

Characters are 7x9 with extenders in an 8x10 cell.  Characters 32

to 126 are standard ASCII but are not faithful reproductions of

the original H19 forms because they were found to be too light on

a Windows screen to be easily readable.  However, characters 0 to

31 and 127, the  graphics characters, are exactly the same as the

original in shape and composition.  In order to see the graphics

characters, the user must first set the terminal to graphics

mode.  Then the lower case letter keys plus some of the

non-alphabetic keys will display the graphics set.  See the Heath

manual for complete information including bitmaps of each

character.

Differences with an actual H19

The following differences exist between our emulation and

the real machine:

1.  Key click cannot be turned on or off.  This is simply

regarded as too hardware-dependent to implement.

2.  An extension has been made to the H19 Scroll key.  When

in hold screen mode, the Scroll Lock key functions just as on the

H19.  However, in normal mode, Scroll Lock can also be used to

stop and start the display if you have XON-XOFF enabled on the

host.  The Scroll Lock key prevents any further reading of the

communications port when toggled on.  The effect is that the

Windows communications driver sends XOFF to the host, effectively

stopping the display.

3.  Since many IBM-style, 83-key keyboards lack a keypad

Enter key, the corresponding H19 functionality is also absent.

Unfortunately, even on an extended (101/102-key) keyboard, it is

impossible to determine in Windows whether the keypad or main

ENTER key has been depressed.

4.  On an extended keyboard, the Break key does not send

a key released signal.  So Windows simply returns both a keydown

and keyup message when the key has been depressed.  So you must

use Control Scrollock to set and reset the communications break

state on an extended keyboard.

5.  Keyboard correspondence with the 83-key keyboard is

as follows:

HEATH H-19			WinH19

Main keyboard:

Off Line			ALT-O (menu)

f1				F1

f2				F2

f3				F3

f4				F4

f5				F5

Erase				F6

Shift Erase			Shift F6

BLUE				F7

RED				F8

WHITE (GRAY)			F9

Reset				ALT-E-R (menu)

Break				Control Break (see note 4 above)

Scroll				Scroll Lock (see note 2 above)

Line Feed			Control J

Delete				Control Backspace

Keypad:

Insert Character (7)		Home (7)

Up (8)				Up (8)

Delete Character (9)		PgUp (9)

Left (4)				Left (4)

Home (5)			(5)

Right (6)			Right (6)

Insert Line (1)			End (1)

Down (2)			Down (2)

Delete Line (3)			PgDn (3)

Zero (0)				Ins (0)

Decimal (.)			Del (.)

Enter				None

All other keys are the same.  For more information, see the Appendix.

KERMIT OPERATION

Winh19k and Winker contain an implementation of the Kermit

protocol developed by the Center for Computing Activities at

Columbia University.  The following information will help you use

Kermit.

The Kermit protocol provides a way to effect an error-free

file transfer files between two, possibly very different

computers provided each is capable of running Kermit.  Kermit

also has several other features which allow one machine to act as

a server while the other computer controls the server's actions

by means of remote commands.

If you are a regular user of Windows, then there are several

advantages to using a version of Kermit directly from Windows.

First, since Windows is multi-tasking, you can run simultaneous

transfers from up to two communications ports (but, see the notes

below) and continue to use other applications.  Secondly,

transfers continue even when the window is iconic.  Finally,

programs written for Windows can be controlled by its memory

management system for optimum memory usage.

Windows Kermit currently can send and receive files and has

a fairly complete set of commands for interacting with a remote

server.  The program cannot act as a server (yet), and, at this

time, only a few file-transfer parameters can be set.  In

addition, file compression is not yet implemented, and only the

standard 1-byte checksum is available.  The eight-bit quoting

mechanism is in place so that binary file transfer over 7-bit

connections is possible.  But, there is no turn-around character

mechanism, so certain IBM mainframe Kermits may not work with

this implementation.

File transfers frequently fail because of incompatible

parity and word size settings between the host and the local

Kermit.  You will usually get your best results by using eight

bits and no parity.  Any other settings will cause binary file

transfers to be slower since the eight-bit quoting mechanism will

go into action.

There are currently no DOS services allowing you to change

directory, display a directory, type a file, etc.  However, you

can easily use the MSDOS.EXE window to show the files in a given

directory.  And, you can take advantage of some features in the

SEND dialog box to help you move to another directory and to list

the files there.  After you are done, just push the CANCEL

button.  For information on using SEND, see below.

Basic Usage

Kermit is very easy to use.  To send a file from the local

computer (usually the controlling microcomputer) to the remote

(host) machine the steps are:

1.  Run Kermit on the remote.  At the prompt type

'receive'

2.  Choose the 'send' option on the local machine, select

the files, and confirm the selection.  The transfer will begin

and continue until all files selected have arrived at the host or

the transfer is canceled.

To receive a file:

1.  Run Kermit on the remote machine and select 'send'

along with the target file names.

2.  Select 'receive' on the local machine.  The files

will continue to arrive until the transfer is complete or  is

canceled.

To use remote operations:

1.  Run Kermit on the remote machine and select 'server'.

2.  Issue all commands from the local machine.  Typical

commands are send files, get files, get help, type or delete a

file on the host, etc..

3.  Terminate remote operations by issuing the 'finish',

'bye', or 'logout' command.

During a transaction, the terminal display window is removed

from the screen and various items reflecting the state of the

Kermit process are shown.  If a file is being transferred,

statistics about the transfer are displayed.  If a remote command

is being executed, any information is written to the bottom line

of the window and is scrolled upward.  At the end of the

transaction, a dialog box appears announcing success or failure.

The display may be obscured by the box, but it can always be

moved out of the way.  However, once 'OK' has been pressed, the

terminal display window is restored and the old information is

lost.

Once a file transfer has begun, you can minimize the window

and have the transfer continue 'in the background'.  The icon

will display the packet types and numbers for both the outbound

and inbound packets.

Warnings

File transfers usually fail because of differences between

the local and remote parity and word size.  If you cannot get

even the first packet across, then try changing these parameters.

Windows is multitasking and you can run several instances of

the same program.  If you are running two copies, watch out for

possible file conflicts such as trying to transfer files to the

SAME file from two different remote Kermits.  The DOS file system

does not always protect you from this kind of error. It can

happen that while you are receiving a file from one source, the

same file can be successfully opened for sending. It usually

means disaster for the incoming file.

Similarly, if the target file is a floppy disk, do not

remove the disk until the transfer has completed.

Detailed Information about the Commands

Three menu selections control the operation of the Kermit

protocol.  They are: Protocol, Files, and Remote.  Files handles

send, receive, get, and the various means for canceling the

transfer.  Remote contains all commands which can be issued to a

remote server.  Finally, Protocol allows a limited selection of

options.

All menus are provided with accelerators as indicated by the

underlined character.  Please see your Windows reference manual

for instructions on using accelerator keystrokes.  We have listed

below when applicable the accelerator keystroke sequence for each

command.

The Protocol Menu

Currently you may select the following items:

Timeout on:  If this item is checked, then after a

certain number of seconds have elapsed with no packets received,

a retry will be attempted.  If the number of retries exceed a

given threshold, then the protocol will exit.  Currently the

elapsed time is fixed at 5 seconds and the number of retries is

also 5.  Neither can be varied in the current implementation.

Duplicate filename warning:  If checked, then an incoming

file with the same name as an existing one will be renamed so as

not to overwrite the old one.  The new name appends or replaces

part of the old name with a unique generation number.

Discard incomplete file:  If checked, then any file being

received in which the protocol has been interrupted by an error

condition or by the user will be deleted.

Bell at end of transfer:  If checked, the bell will sound

when a transaction is complete or canceled.

If the 'Save settings' box is also checked, then win.ini

will be modified to reflect the new defaults under [Kermit].

The Files Menu

Using these selections, you can send or receive a file or a

group of files.  Before a transaction begins, you can select

either of Send, Receive, or Get.  After the transfer has begun,

you may interrupt the transfer in various ways.  Of course, you

should have already set the remote Kermit as described above.

Thus, before you attempt to receive files you should have already

set the remote Kermit to send them.  Similarly, if you are

sending files to the remote Kermit, you must first command it to

receive or place it into server mode.  Finally, server mode on

the remote host must be active in order to use the Get command.

Send (ALT-F-S):  When Send is selected, a dialog box with

several windows appears.  Two are list boxes; the one on the left

contains all files matching the search specification shown in the

edit box at the top.  The right list box contains a list of paths

and disk drives which can be accessed.  The push buttons allow

you to select a list of files based on the search specification,

send the selected files, or cancel the operation.

The basic principle is that a file name must be highlighted

in the list box to be sent.  Thus, you may send several files.

However, you may select only those files from the current

directory.  Hence files from more than one directory cannot be

sent during one transaction.

A file can be selected in several ways.  The usual method is

to click on the name with the mouse.  More than one file can be

highlighted by holding down the shift key during selection.  You

can also deselect one of several highlighted items in the list by

again depressing the shift-key and clicking on the item.  If you

do not hold down the shift key, all items will be deselected

except the one to which the mouse is pointing.

You can also use the keyboard to select files.  Use the tab

key to move to the files list box.  There you can select files by

using the arrow keys to move through the list and the shift key

to highlight contiguous items in the list.

The default search specification is "*.*".  You can choose

another string to limit the files displayed in the list box.  For

example, if you tab to the edit box (or point to it with the

mouse), type "*.txt", and press the RETURN key or the SELECT

pushbutton, then not only will all files having that extension be

displayed, but all will be selected as well.  You can then send

the entire list by activating the SEND button.

At any time, you can deselect all files but one in the list

box by simply pointing and clicking on that file.  You can

deselect all file in the box by deleting the contents of the edit

box and pressing RETURN.

You can move to another drive or directory by typing it into

the edit box and pressing RETURN or double-clicking on the

corresponding item in the right list box.  The current search

specification is appended to the path, the program moves to the

new directory, and all matching files are displayed.  A second

RETURN or a push of the SELECT button then highlights the files

list.

Finally, at any time, a single file located anywhere in the

system can be selected and sent.  Simply type its name, along

with any required drive and/or path information and hit RETURN.

If the file can be found, then a switch is made to that directory

and the file name is displayed and selected.  If the file cannot

be found,  no action takes place.

You may cancel the operation at any time by pressing the

CANCEL button.  Pushing SEND with no files highlighted has the

same effect.

Receive (ALT-F-R):  If the Receive menu item is chosen,

then the program enters a state where it is awaiting the arrival

of an initialization packet from the sender.

Get (ALT-F-G):  If the remote host is in server mode, you

may use this command to receive files.  A dialog box appears in

which you can type a single file name or a search specification.

After the OK button has been pressed, the program sends a

initialization packet and then waits for the file(s).

Exclude file (ALT-F-X):  Once a transaction has begun,

you can exclude the file currently being transferred by selecting

this menu item.  The rest of the files will not be affected. Note

that if you have elected to keep partial files, the current file

will NOT be deleted if you are receiving.

Zap batch (ALT-F-Z):  Once the transaction has begun, you

can kill the remaining transaction by making this selection.

Only the current and remaining files in the list are affected.

The comment about partial files in the paragraph above also

applies.

Error abort (ALT-F-E):  When this item is selected, an

error packet is sent to the remote Kermit and the transaction is

terminated.  Incomplete files are discarded (or saved, if the

partial file flag is set).  Use this method to cleanly terminate

a transaction once the exchange has started.

Cancel protocol (ALT-F-C):  This selection is most useful

as a way to cancel a transaction before any exchange of packets

has begun.  Note that the remote Kermit will attempt to continue

if it has already started and you will may have to use other

means to terminate the remote program, or, you can simply let it

time out.

The Remote Menu

You may control many actions of the remote host by placing

its Kermit into server mode and then using the various commands

available under this menu item.  Many of the commands will

present you with a dialog box, although in some cases, the

information is optional.  In any case, you confirm your selection

by pushing the OK button.  A touch of the space bar is

sufficient.  Press the CANCEL button and the transaction will not

start.

Some remote commands may return many lines of information.

You can stop the display (for a limited time, as the remote

Kermit may time out) and restart it by toggling the Scroll Lock

key.  Finally, during the transaction, you can use the various

cancel options to abort the operation.

Help (ALT-R-H):  This item will request the host to

display the remote commands available.  Some remote Kermits may

be able to respond to help about a particular subject, and a

dialog box is available to add the additional string.

Host Command (ALT-R-O):  Use this item to have the host

perform a shell command.  An entry in the dialog box is

mandatory.

Finish (ALT-R-F):  Select this item to cause the remote

Kermit to exit the server mode.

Logout (ALT-R-L):  As the name implies, this item is used

to log off the remote machine from server mode.

Bye (ALT-R-B):  This selection logs off the user from the

remote and exits the local Kermit.

Chdir (ALT-R-C):  You can change directory on the remote

server with this command.  A dialog box will appear allowing you

to enter the new directory, and, if needed, password.  The

password will not be displayed.  On some remote Kermits, if you

make no entries, the current directory is returned.

Directory (ALT-R-D):  This item lists the files located

on the remote host.  An optional search specification can be

given.

Type (ALT-R-T);  You can type a file or group of files

located on the remote host with this command.

Delete (ALT-R-D):  This menu item allows you to delete

files on the remote host.

Who (ALT-R-W):  You can list the users on the system with

this command.  An optional string can also be sent.

Space (ALT-R-U):  This command lists the disk space

available on the host.  An optional string can be sent.

APPENDIX

SUMMARY OF WINDOWS MAIN KEYBOARD USAGE FOR THE WINDOWS H19 EMULATOR

Note:  Keyboards covered include IBM-style 83 and 101-key (extended)

models.  F10, F11, F12 are available only on 101 key keyboards.

Other differences are noted below.

Key		Heath		ANSI		Terminal action if received

Scroll Lock					Display pauses

Ctrl Break (Ctrl Scrollock on ext kbd)	 	Sends break until released

Ctrl Backspace					Sends delete (127)

F1		ESC S		ESC O S		None

F2		ESC T		ESC O T		None

F3		ESC U		ESC O U		None

F4		ESC V		ESC O V		None

F5		ESC W		ESC O W		None

F6		ESC J		ESC [ J		Clear to end of display

F7		ESC P		ESC O P		None

F8		ESC Q		ESC O Q		None

F9		ESC R		ESC [ R		None

F10 to F12	none		none		User defined (1)

Shift F1 to F5	 				User defined (2)

Shift F6	ESC E		ESC [ 2 J	Clear display and home cursor

Shift F7 to F12					User defined (2)

Ctrl F1 to F5					User defined (3)

Ctrl F6						Same as F6 but not transmitted

Ctrl F7 to F12					User defined (3)

Ctrl Shift F1 to F5				User defined (4)

Ctrl Shift F6					Same as shift F6

Ctrl Shift F7 to F12				User defined (4)

(1) Set Fn=string (max of 80 chars) in win.ini under [Winh19].

(1) Set SFn=string (max of 80 chars) in win.ini under [Winh19].

(2) Set CFn=string (max of 80 chars) in win.ini under [Winh19].

(3) Set CSFn=string (max of 80 chars) in win.ini under [Winh19].

KEYPAD KEYS

Shifted keypad mode OFF, Alternate keypad OFF

Key		  Heath		ANSI		Terminal action if received

Ins		  0		0		Displays 0

End		  1		1		Displays 1

Down Arrow	  2		2		Displays 2

PgDn		  3		3		Displays 3

Left Arrow	  4		4		Displays 4

5 key		  5		5		Displays 5

Right Arrow	  6		6		Displays 6

Home		  7		7		Displays 7

Up Arrow  	  8		8		Displays 8

PgUp		  9		9		Displays 9

Del		  .		.		Displays .

Shift Ins	  0		0		Displays 0

Shift End	  ESC L		ESC [ L		Insert Line

Shift Down Arrow  ESC B		ESC [ B		Cursor down 1 line

Shift PgDn  	  ESC M		ESC [ B		Delete Line

Shift Left Arrow  ESC D		ESC [ D		Cursor left 1 character

Shift 5 key	  ESC H		ESC [ H		Home cursor

Shift Right Arrow ESC D		ESC [ D		Cursor right 1 character

Shift Home	  ESC @		ESC [ 4 h	Enter insert mode (1)

ESC O		ESC [ 4 l	Exit insert mode (1)

Shift Up Arrow	  ESC A		ESC [ A		Cursor up 1 line

Shift PgUp 	  ESC N		ESC [ P		Delete character

Shift Del	  .		.		Displays .

Shifted keypad mode ON, Alternate keypad OFF

Key		  Heath		ANSI		Terminal action if received

Ins		  0		0		Displays 0

End		  ESC L		ESC [ L		Insert Line

Down Arrow 	  ESC B		ESC [ B		Cursor down 1 line

PgDn		  ESC M		ESC [ B		Delete Line

Left Arrow  	  ESC D		ESC [ D		Cursor left 1 character

5 key	  	  ESC H		ESC [ H		Home cursor

Right Arrow 	  ESC D		ESC [ D		Cursor right 1 character

Home	  	  ESC @		ESC [ 4 h	Enter insert mode (1)

ESC O		ESC [ 4 l	Exit insert mode (1)

Up Arrow	  ESC A		ESC [ A		Cursor up 1 line

PgUp 	  	  ESC N		ESC [ P		Delete character

Del		  .		.		Displays .

Shift Ins	  0		0		Displays 0

Shift End	  1		1		Displays 1

Shift Down Arrow  2		2		Displays 2

Shift PgDn	  3		3		Displays 3

Shift Left Arrow  4		4		Displays 4

Shift 5 key	  5		5		Displays 5

Shift Right Arrow 6		6		Displays 6

Shift Home	  7		7		Displays 7

Shift Up Arrow	  8		8		Displays 8

Shift PgUp	  9		9		Displays 9

Shift Del	  .		.		Displays .

Shifted keypad mode OFF, Alternate keypad mode ON

Key		  Heath		ANSI		Terminal action if received

Ins		  ESC ? p	ESC O p		Displays p

End		  ESC ? q	ESC O q		Displays q

Down Arrow	  ESC ? r	ESC O r		Displays r

PgDn		  ESC ? s	ESC O s		Displays s

Left Arrow	  ESC ? t	ESC O t		Displays t

5 key		  ESC ? u	ESC O u		Displays u

Right Arrow	  ESC ? v	ESC O v		Displays v

Home		  ESC ? w	ESC O w		Displays w

Up Arrow	  ESC ? x	ESC O x		Displays x

PgUp		  ESC ? y	ESC O y		Displays y

Del		  ESC ? n	ESC O n		Displays n

Shift Ins	  ESC ? p	ESC O p		Displays p

Shift End	  ESC L		ESC [ L		Insert Line

Shift Down Arrow  ESC B		ESC [ B		Cursor down 1 line

Shift PgDn	  ESC M		ESC [ B		Delete Line

Shift Left Arrow  ESC D		ESC [ D		Cursor left 1 character

Shift 5 key	  ESC H		ESC [ H		Home cursor

Shift Right Arrow ESC D		ESC [ D		Cursor right 1 character

Shift Home	  ESC @		ESC [ 4 h	Enter insert mode (1)

ESC O		ESC [ 4 l	Exit insert mode (1)

Shift Up Arrow	  ESC A		ESC [ A		Cursor up 1 line

Shift PgUp 	  ESC N		ESC [ P		Delete character

Shift Del	  ESC ? n	ESC O n		none

Shifted keypad mode ON, Alternate keypad mode ON

Key		  Heath		ANSI		Terminal action if received

Ins		  ESC ? p	ESC O p		Displays p

End		  ESC L		ESC [ L		Insert Line

Down Arrow  	  ESC B		ESC [ B		Cursor down 1 line

PgDn		  ESC M		ESC [ B		Delete Line

Left Arrow	  ESC D		ESC [ D		Cursor left 1 character

5 key		  ESC H		ESC [ H		Home cursor

Right Arrow	  ESC D		ESC [ D		Cursor right 1 character

Home		  ESC @		ESC [ 4 h	Enter insert mode (1)

ESC O		ESC [ 4 l	Exit insert mode (1)

Up Arrow	  ESC A		ESC [ A		Cursor up 1 line

PgUp 		  ESC N		ESC [ P		Delete character

Del	  	  ESC ? n	ESC O n		Displays n

Shift Ins	  ESC ? p	ESC O p		Displays p

Shift End	  ESC ? q	ESC O q		Displays q

Shift Down Arrow  ESC ? r	ESC O r		Displays r

Shift PgDn	  ESC ? s	ESC O s		Displays s

Shift Left Arrow  ESC ? t	ESC O t		Displays t

Shift 5 key	  ESC ? u	ESC O u		Displays u

Shift Right Arrow ESC ? v	ESC O v		Displays v

Shift Home	  ESC ? w	ESC O w		Displays w

Shift Up Arrow	  ESC ? x	ESC O x		Displays x

Shift PgUp	  ESC ? y	ESC O y		Displays y

Shift Del	  ESC ? n	ESC O n		Displays z

(1) Sends first sequence shown if not in insert character mode.

Sends second sequence if in insert mode.

SUMMARY OF ESCAPE SEQUENCES RECOGNIZED BY THE WINDOWS H19 EMULATOR

CURSOR FUNCTIONS			HEATH			ANSI

Cursor home				ESC H			ESC [ H

ESC [ 0;0 H

ESC [ 1;1 H

ESC [ f

ESC [ 0;0 f

ESC [ 1;1 f

Cursor forward 1 character		ESC C			ESC [ C

ESC [ 1 C

Cursor forward n characters		none			ESC [ n C

Cursor backward 1 character		ESC D			ESC [ D

ESC [ 1 D

Cursor backward n characters		none			ESC [ n D

Cursor down one line			ESC B			ESC [ B

ESC [ 1 B

Cursor down n lines			none			ESC [ n B

Cursor up 1 line			ESC A			ESC [ A

ESC [ 1 A

Cursor up n lines			none			ESC [ n A

Reverse line feed			ESC I			ESC M

Cursor position report (1)		ESC n 			ESC [ 6 n

Save cursor position			ESC j			ESC [ s

Set to saved postion			ESC k			ESC [ u

Address cursor (2)			ESC Y a b		ESC [ r;c H

ESC [ r;c f

(1) Heath mode replies with ESC Y a b, where a and b are characters

whose ASCII values are, respectively, the row and column position of the

cursor plus 31.  ANSI mode replies with ESC [ r;c R where r and c are

character strings of digits for the row and column position of the cursor.

(2) In the Heath mode, a and b are ASCII characters whose values are the

respective row and column postions offset by 31 (if the first row or

column is regarded as 1).  In ANSI mode, r and c are character strings

of digits for the row and column position.

ERASING AND EDITING			HEATH			ANSI

Clear display and home cursor		ESC E			ESC [ 2 J

Erase to top of display			ESC b			ESC [ 1 J

Erase to end of display			ESC J			ESC [ J

ESC [ 0 J

Erase entire line			ESC l			ESC [ 2 K

Erase to beginning of line		ESC o			ESC [ 1 K

Erase to end of line			ESC K			ESC [ K

ESC [ 0 K

Insert one line				ESC L			ESC [ L

ESC [ 1 L

Insert n lines				none			ESC [ n L

Delete line				ESC M			ESC [ M

ESC [ 1 M

Delete n lines				none			ESC [ n M

Delete character			ESC M			ESC [ P

ESC [ 1 P

Delete n characters			none			ESC [ n P

Enter insert character mode		ESC @			ESC [ 4 h

Exit insert character mode		ESC O			ESC [ 4 l

CONFIGURATION and MODES			HEATH			ANSI

Reset to Power-up state			ESC z			ESC [ z

Set baud rate (3)			ESC r char		ESC [ r integer

Enable status line			ESC x 1			ESC [ > 1 h

Disable status line			ESC y 1			ESC [ > 1 l

No key click (4)			ESC x 2			ESC [ > 2 h

Key click (4)				ESC y 2			ESC [ > 2 l

Hold screen mode			ESC x 3			ESC [ > 3 h

ESC [

Exit hold screen mode			ESC y 3			ESC [ > 3 l

ESC \

Block cursor				ESC x 4			ESC [ > 4 h

Underscore cursor			ESC y 4			ESC [ > 4 l

Cursor off				ESC x 5			ESC [ > 5 h

Cursor on				ESC y 5			ESC [ > 5 l

Shifted keypad				ESC x 6			ESC [ > 6 h

ESC t

Unshifted keypad			ESC y 6			ESC [ > 6 l

ESC u

Alternate keypad mode			ESC x 7			ESC [ > 7 h

ESC =			ESC =

Exit alternate keypad mode		ESC y 7			ESC [ > 7 l

ESC >			ESC >

Auto LF on CR				ESC x 8			ESC [ > 8 h

Auto CR on LF				ESC x 9			ESC [ > 9 h

ESC [ 20 m

Reverse video mode			ESC p			ESC [ 7 m

Exit reverse video mode			ESC q			ESC [ m

ESC [ 0 m

Graphics mode				ESC F			ESC [ 10 m

Exit graphics mode			ESC G			ESC [ 11 m

Keyboard disabled			ESC }			ESC [ 2 h

Keyboard enabled			ESC {			ESC [ 2 l

Wrap at end of line			ESC v			ESC [ ? 7 h

Discard at end of line			ESC w			ESC [ ? 7 l

Enter ANSI mode				ESC <			none

Enter Heath mode			none			ESC [ ? 2 h

(3) 	Baud Rate 		Heath mode		Ansi Mode

110			A			1

150			B			2

300			C			3

600			D			4

1200			E			5

1800			F			6

2000			G			7

2400			H			8

3600			I			9

4800			J			10

7200			K			11

9600			L			12

19200			M			13

(4) Not yet implemented.

MISCELLANEOUS FUNCTIONS			HEATH			ANSI

Identify as VT52 (5)			ESC Z			none

Transmit status line			ESC ]			ESC [ q

Transmit screen				ESC #			ESC [ p

(5) Replies with ESC \ Z.