GENERIC FIDO DRIVER INTERFACE

T. Jennings 12 July 1985

FIDO_GEN.EXE is a "generic" Fido for otherwise unsupported

machines. It will not work until you write a device driver

or terminate-stay-resident program to provide the following

functions. The format of this driver will remain independent

of the Fido version; once written it will work with all

future Fido versions.

All IO is done through an IBM PC like Interrupt 14 hex. Many

calls are directly compatible, but you will have to extend

it to support the added features. Note that FIDO_GEN will

NOT run on an IBM PC as is.

The interface is very consistent; AX and DX are used for

all functions.

It is possible to use an entirely polled IO system for

Fido; Fido does "typeahead" in the higher level software,

so everything but ASCII Upload will work fine polled, and

even that may work OK on most machines. Interrupt character

input is very desireable. Interrupt output is not needed

at all, unless you wish to run under a multi-tasker such

as Multilink.

Please note that I cannot help you write drivers, nor will

I help you debug them. You are on your own! Extensive

assembly language experience is needed, and a good degree

of familiarity with your hardware, so please do not

underestimate the difficulty of this task.

Assumptions here are that you can run MSDOS or PCDOS

version 2 or 3; Fido does not support ANY other operating

system.

Either a device driver or a terminate-stay-resident type

structure for your driver set is reccomended. Device drivers

obviously go in your CONFIG.SYS on the boot disk; term stay

res programs should be put in AUTOEXEC.BAT, to be run only

ONCE. Every time you run a term stay res, it consumes more

memory, unless special provision is made in your program

to detect whether it was previously loaded. Keep in mind that

in normal operation, Fido is run more than once, ie. you may

Control-C Fido to copy files, run in Test Mode, etc.

On the subject of Test Mode, an important note: Fido does not

make ANY HARDWARE DEPENDENT CALLS in /T test mode. This means

that ANY version of Fido will work on ANY other machine in

test mode. You must run it without /T specified in order to

get it to use your drivers. Test mode is useless as a test of

your drivers. It means nothing.

It is probably a good idea to test each and every single

function before attempting to use Fido with your driver set;

I repeat, I cannot help you with your drivers in any way.

The following is the function call used internally to Fido

to access the drivers you write. This is provided for your

information only.

val= int14(ah,al,dx);

int val;		/* integer return value */

int ah;			/* command byte, 0 - N */

int al;			/* argument */

int dx;			/* I/O device, ie. channel */

For all calls, DX is the IO device from the /1, /2, etc

command line switch. This can be used or ignored as desired.

It will be valid for all calls at all times.

AH is the command type; 0 - 3 are very similar to IBM PC

INT 14 calls. 4 and up are extentions to support further

functions. (Note that the IBM version of Fido does NOT

use INT 14.)

For all calls, your driver MUST preserve all segment

registers, plus BP and SP. All other registers can be

modified. Interrupts should be left ENABLED after

returning. There is adequate stack space to do almost

anything you need; there should be at least a kilobyte

of stack space.

AH = 0:	Set baud rate

This command is almost identical to the IBM PC equivalent

call. The upper three bits indicate the baud rate; the

lower ones indicate parity (always none) and stop bits

(always 1). You can always assume that NO PARITY and ONE

STOP BIT are used throughout Fido.

Baud	Value

300	043h

1200	083h

2400	0a3h

9600	0e3h

The sample driver strips off all but the upper three

bits. You can do this in your driver also, but Fido

will always pass the byte patterns as defined above.

AH = 1:	Write a character to the serial port

Output the character in AL to the serial port. Wait for

output ready as necessary.

AH = 2:	Read a character from the serial port

Return one character from the serial port in AL. Wait for

one if necessary.

AH = 3:	Return status

This command returns various status bits back to Fido in AH

and AL. There are three status bits you must return in this

call; two have predetermined values, and the other you can

assign to any other bit, as it is tested with the command

line /V value. (See below) These are bit patterns, not

numeric values; you can (and will) have more than one bit

set at a time.

AH:	20h		TBE	Transmit Buffer Empty

01h		RDA	Receive Data Available

TBE means the transmitter is ready to accept a character.

RDA means there is a character from the modem waiting.

The other bit the the Carrier Detect (CD) bit. This can be

returned in either AH or AL. The command line switch /V

defines the mask bit used to test this port. For example,

128/V

Defines the CD mask bit to be 128, or 0080h. This is ANDed

with the value returned by the STATUS call, and if the

result is true (bit is set) then Fido assumes the modem is

connected.

For making this test, Fido treats AH and AL as a 16 bit

integer, in the usual fashion: AH is the most signifigant

and AL the least.  Therefore, 32768/V tests the MSB in

AH, and 1/V test the LSB in AL. You can't use 8192/V

or 256/V, since those are TBE and RDA, respectively.

Unused bits may take on any value as you see fit.

AH = 4:	One time initialization

Perform any necessary one time initialization here. Fido

will make this call only ONCE before any IO is done to

the serial port. NO OTHER call will ever be made before

this call. If not needed, ignore.

AH = 5:	One time UN initialization

Do one time un-initialization; this is called ONCE, just

before Fido exits to DOS. Remove any interrupt structures

or whatever were installed by AH = 0. NO OTHER call will be

mode to the IO drivers after this call.

It is general practice when exiting Fido to leave the

DTR line low so that the modem is disabled when Fido

is not running. Fido lowers DTR before calling this

function and exiting, but you might want to make sure that

this function does not raise it again inadvertantly.

AH = 6:	Raise/Lower Data Terminal Ready

This function controls the state of the DTR line output to

the modem, usually on Pin 20. AL = 0 means lower DTR;

AL = 1 means raise it true. True enables the modem; false

disables it.

No other functions in your drivers should change the state

of the DTR line, except AH = 4 and AH = 5. See the note

in AH = 5 about exiting Fido.

AH = 7:	Return Timer Tick parameters

This function is used to tell Fido about the timer tick

to be used for marking time. This is an extremely critical

function, and luckily, fairly simple. The hard part will

be getting the information.

AH = 4 is called before this call, so you can use that to

start any timer running if necessary; if you do, use AH = 5

to remove it.

AH = 7 returns three numbers:

AL = 	Timer Tick Interrupt Number	0 - 255

AH =	Ticks Per Second

DX = 	Timer Tick Rate, Milliseconds

Note that this function merely returns fixed values to

Fido; Fido will do all the work of operating the timer

interrupt, and pass control to the original interrupt

service routine. The following information is given

as background only.

Fido's timer tick is done very simply by inserting itself

in series with the interrupt numbe you specify in AL. When

Fido has done its timer counter incrementing, it chains

via a long jump to the original interrupt vector, thereby

preserving the original interrupt vector. Fido does not

defer the interrupt by more than a few hundred microseconds,

so there are no problems with interrupt acknowledges

regardless of the interrupt system type. The DEC Rainbow,

for instance, has a low tolerance for interrupt deferrment;

it has hardware to check for interrupt loss, and the screen

refresh depends on a repeatable clock; Fido's timer tick

method presents no problem.

The reason for both ticks per second and milliseconds per

tick is that frequently the timer is at an inconvenient rate;

the IBM PC for example is approx. 55 mS, which does not make

for easy counting. (Note that if you work it out, the

sample drive as supplied is very inaccurate. Oh well.)

Fido counts both milliseconds and hours minutes seconds; these

two parameters let you choose values that will result in the

highest accuracy. It is not necessary to have extreme accuracy.

Fido does its millisecond counting by adding the number you

return in DX to a 32 bit counter in memory; it is used for

XMODEM, etc timeouts, and does not need to be highly accurate;

10% is fine, but try for the most accurate possible.

Hour minute second counting is done with the number your return

in AH; every (AH) timer tick Fido increments the seconds counter.

This value should be as accurate as is possible, since this is

what controls each users time limit on the system.