J M O D E M

*  The Microsoft C Version  *

February 17, 1990

Richard B. Johnson

405 Broughton Drive

Beverly Massachusetts 01915

BBS (508) 922-3166

Introduction.

JMODEM was first introduced about two years ago. It has en-

joyed a steady increase in popularity around the world. It

has even been explained in some detail in John Dvorak's book

on PC communications; Dvorak's Guide to PC Telecommunica-

tions, 1990, Osborne-McGraw-Hill, 2600 Tenth Street,

Berkely, CA.

JMODEM was first written in assembly language. Since this

language is hard to read and use, it has been difficult for

communications program developers to incorporate it into

their programs directly. Instead it must be executed as an

external protocol. Now JMODEM has been written in the C

Language. C has become the de-facto standard for portable

code development even though there isn't presently any code

that is truly portable across many different machines. The

writing of JMODEM in C will make it easier for software de-

velopers throughout the world to use this very useful pro-

tocol.

You can use this new version of JMODEM just as the older

versions written in assembly. It has a new color sign-on

screen and status-block windows that overlap. Although the

new JMODEM.EXE code is larger than the assembly-language

version, JMODEM.COM, the program still executes very fast

because much effort has been taken to streamline the C code.

JMODEM.COM required 64 k of RAM (one segment) to execute

properly. JMODEM.EXE requires 79 k to allow the screens to

be written properly, and 66 k of free RAM to execute without

aborting although the previous screen content will be lost.

Like all versions of JMODEM, this version is 100% compatible

with all previous version including Beta version 1.00. From

it's inception, the essential structure of JMODEM has never

been changed.

How to install JMODEM.

JMODEM executes best from a batch file as an external pro-

tocol for any of the communications programs that have ex-

ternal-protocol capability. A typical communications program

is TELIX.

- 1 -

JMODEM, the C Version

Here is a batch file used with TELIX for uploads:

@ECHO OFF

Rem * JMODEM TELIX Upload batch file.

C:\TELIX\JMODEM S1 %3

Rem           | ||  |_________ file name (passed by TELIX)

Rem           | ||____________ COM port (1 - 4)

Rem           | |_____________ Send

Rem           |_______________ Path and name of JMODEM

This is a batch file used with TELIX for downloads:

@ECHO OFF

Rem * JMODEM TELIX Download batch file.

C:\TELIX\JMODEM R1 %3

Rem           | ||  |_________ file name (passed by TELIX)

Rem           | ||____________ COM port (1 - 4)

Rem           | |_____________ Receive

Rem           |_______________ Path and name of JMODEM

This is a batch file used for PCPLUS (PROCOMM) uploads:

@ECHO OFF

Rem * JMODEM PCPLUS Upload batch file.

C:\TELIX\JMODEM S1 %1

Rem           | ||  |_________ file name (passed by PCPLUS)

Rem           | ||____________ COM port (1 - 4)

Rem           | |_____________ Send

Rem           |_______________ Path and name of JMODEM

This is a batch file for PCPLUS (PROCOMM) ownloads:

@ECHO OFF

Rem * JMODEM PCPLUS Download batch file.

C:\TELIX\JMODEM R1 %1

Rem           | ||  |_________ file name (passed by PCPLUS)

Rem           | ||____________ COM port (1 - 4)

Rem           | |_____________ Receive

Rem           |_______________ Path and name of JMODEM

You can chop this text out with an editor and create your

batch files quickly and correctly. PCPLUS passes the file-

name as the %1 parameter and TELIX uses the %3 parameter.

JMODEM does not need to know anything else about your system

to operate. It does not even need to know the baud-rate or

other communications parameters. All it needs to know about

is the file name and the communications adapter port to use.

- 2 -

JMODEM, the C Version

JMODEM doesn't care if you are using a 19,200 baud modem

with the DTE locked at high-speed or a 300 baud modem on a

lamp-cord. It is very smart about communications and handles

lost carriers (hangup) and flow-control in a very simple way

which will be explained in detail later.

If you only want to try JMODEM without having to install it,

you need only to activate the JMODEM protocol on a BBS sys-

tem, then "shell to DOS" and execute it manually from the

DOS prompt. You will be pleasantly surprised at how rapidly

it can transfer a file and how well it executes over net-

works.

How JMODEM works.

JMODEM uses variable-length records called blocks. These

blocks start with 512 data-bytes and increase in length to a

maximum of 8192 bytes per block. There is a 6-byte overhead

associated with each block so the percentage of overhead

starts at a fairly high 0.1 percent and decreases to a very

low 0.07 percent as the transmission progresses. The block

length will increase in 512-byte increments as long as there

are no errors requiring retransmission. Should an error

occur, the block-size is cut in half. This continues until

the block-size is as short as 64 bytes.

The blocks may actually be of any length but never exceed

the maximum allowed. An attempt is made to reduce the amount

of data that needs to be transmitted by compressing each

block before transmission. Since much data is already com-

pressed, being from ".ZIP" files and such, it is possible

that "compression" may actually cause an increase in block-

length. JMODEM will send a compressed block only if it is

shorter than the non-compressed block.

Since it takes time to compress and expand data, only the

simplest and quickest compression method is used. It is very

effective with ".EXE" files that contain large blocks of

binary nulls and most text files but it is not very effec-

tive with highly compressed archive files. The compression

method is called "run-length-limited" or more explicitly

"how many of what kind". Basically, the block is searched

for groups if similar characters. If many similar characters

are found, compression consists of sending a sentinel byte

of hexadecimal BB, followed by a two-byte amount, and this

followed by the byte to be repeated.

In the following example we have a string of spaces (20

hex).

- 3 -

JMODEM, the C Version

Before compression:

( 18 bytes )

20 20 20 20 20 20 20 20 20 20 20 34 37 87 EF FF 3A 23

After compression:

( 11 bytes )

BB 0B 00 20 34 37 87 EF FF 3A 23

The blocks could actually get longer because the sentinel

byte could be present in the data:

Before compression:

( 9 bytes )

BB 00 BB AF EF BB 00 AE EF

After compression:

( 16 bytes )

BB 01 00 BB 00 BB 01 00 BB AF BB 01 00 BB AE EF

As soon as the encoded length exceeds the data block length,

compression is abandoned and the non-compressed block is

sent. JMODEM sends a byte that tells the receiver if the

data is compressed or not. This same byte tells the receiver

when the end-of-file has occurred so JMODEM is able to

preserve exact file-length.

The JMODEM block.

The JMODEM block looks like this:

|< --------- First byte sent / received

20 00 01 01 .. .. .. .. .. AE 01

|  |  |  |        |        |  |___ CRC (high byte)

|  |  |  |        |        |______ CRC (low byte)

|  |  |  |        |_______________ data bytes

|  |  |  |________________________ type of block

|  |  |___________________________ block number

|  |______________________________ block length (high byte)

|_________________________________ block length (low byte)

The Block length:

The block length is a WORD (16 bits). This allows the blocks

to be 65,535 bytes long although in practice the length is

not allowed to exceed 8192 bytes (plus overhead). The block-

length is the length of the entire JMODEM block, not just

the data-bytes.

- 4 -

JMODEM, the C Version

The Block number:

The block number is a BYTE. It starts at 1 and becomes zero

again after block 255. It is used to make certain that all

receiption errors are detected.

The Block type:

This BYTE is bit-mapped to tell the receiver what kind of

block has been sent. Presently there are three kinds of

blocks:

00000000B       Hex 00  ( Normal data )

00000001B       Hex 01  ( Compressed data )

00000010B       Hex 02  ( End of file )

The CRC:

The CRC is a WORD (16 bits). It is not a "standard" type of

CRC because, unless done with hardware, a standard 16-bit

polynoimial would take several seconds to calculate for a

long block. Instead it uses a rapid rotate and sum algoritm

that is probably just as effective as the more "standard"

polynominials.

The CRC is generated using this polynomial:

X =  X + X^(2(n-mod 7)).......  Where n = t(n-1)

And t = string length

It has the advantage of simplicity in assembly-language

programming and will detect errors with a probability of

about  one undetected error in 2^132 (which is a very large

number). It does not correct errors so its not important to

use some "standard" function to generate the CRC.

The C code shows how easy it is to create and check this

kind of CRC.

- 5 -

JMODEM, the C Version

The JMODEM Communications

Hardware Control

When JMODEM is first loaded for execution it checks the

state of the modem-control leads. If there is no modem

carrier detected, it assumes that you have connected two PCs

together without a modem and will not bother to check for a

dropped carrier during execution.

Since JMODEM must exercise flow-control so it can be used

with high-speed modems with fixed baud rates, it also as-

sumes that at the time at which it is first executed, the

modem will be requesting data because there will have been

very little I/O over the previous few seconds. JMODEM stores

the state of RTS/CTS and DTR/DSR and uses this as a refer-

ence. When transmitting data, should JMODEM find that the

state of these modem-control leads has changed, it waits

until the modem-control leads have reverted back to the

initial state before sending any more data. This allows ANY

modem to exercise flow-control with JMODEM, even those that

use "pin 4" instead of "pin 20". When JMODEM is waiting, it

checks to verify that the modem carrier has not been drop-

ped. If the carrier is dropped, or if the user aborts,

JMODEM will exit, setting a DOS ERRORLEVEL code.

You can abort JMODEM at any time by hitting Ctrl-Break or

Ctrl-C. It will take several seconds for JMODEM to abort

because it does not continually check these keys. Unlike

previous versions, JMODEM erases the files from aborted

downloads. Also, JMODEM will abort if it is unable to rename

a file in the following example.

Suppose you wish to download a file called VIRUS.EXE. Sup-

pose also that VIRUS.EXE already exists. Instead of over-

writing your previous version of VIRUS.EXE, JMODEM has al-

ways been nice and renamed it to VIRUS.OLD before creating a

new file. With previous versions of JMODEM, if VIRUS.OLD

already existed, JMODEM would have deleted it before renam-

ing the present file to ".OLD". This no longer is done.

JMODEM never deletes ANYTHING anymore except it's own abort-

ed download. This should reduce the number of threats I have

received!

- 6 -

JMODEM, the C Version

The C programmer's

Guide to JMODEM

Included within this package should be the following files:

JMODEM          This is the MAKE file

JMODEM_A  C     Contains the _main() routine entry point.

JMODEM_B  C     Allocates memory, parses input strings.

JMODEM_C  C     All of the file I/O is in this file.

JMODEM_D  C     Data compression/expansion, CRC calculation

JMODEM_E  C     Communications I/O interrupt service, etc.

JMODEM_F  C     Screen I/O (windows, etc)

JMODEM    H     Contains JMODEM globals and data structures.

SCREEN    H     Screen definitions, prototypes, structures

UART      H     Definitions for the 8250 UART

If you have MicroSoft 'C' version 5.0 or later, and if your

environment and paths are properly set up, you should be

able to type:

MAKE JMODEM

... and a brand new version of JMODEM will be created.

Just as in the MASM language files, the source code is

strongly-typed. All function prototypes are declared and no

defaults are used. You can compile at warning-level 3, the

most stringent level available with Microsoft compilers, and

you will get no errors at all.

You may modify JMODEM for your own use, but  P L E A S E  do

NOT distribute the modified version on BBS systems because I

do not wish to support 222,500 versions of JMODEM! If you

find a bug, or wish to improve or add something that will

increase the value of JMODEM without making it incompatible

with previous versions, please upload your improvements to

my BBS system and I may include the revisions (with your

name attached) in an upcoming version. Note that there are 6

"type" bits available in the JMODEM control-byte that could

be used to tell the receiver that another file is coming,

etc., (for batch). Things like this could be added without

destroying compatibility with previous versions.

MicroSoft C seems to have several bugs one of which affects

files that are very long (over 256k). My first attempts to

use C files under MicroSoft for JMODEM used the stream-I/O

(FILE *) type of files. This resulted in corruption of long

files. Therefore I implemented the UNIX/DOS type of file-I/O

that uses handles rather than file-control blocks. These

work rather well.

Another bug is in the SMALL C run-time library. Although you

can compile the source to use the SMALL library version, and

- 7 -

JMODEM, the C Version

the compiler / linker execute without errors, the resulting

program will not work because the memory allocation routines

dump the allocated buffers on top of each other! This re-

sults in VERY corrupt files, bearing no resemblance to the

originals! Stay with the medium models and you will have no

problems.

- 8 -