1 Introduction

E.CIRC: Electrical Circuits Calculator and Teaching Aid

Version 3.2

(c) Written by KCA Smith

Published by Granta Electronics Ltd. 1993

This program has been written with the aim of providing assistance to

the engineering or science student in working through the examples and

problems to be found in textbooks on electrical circuit theory. In

particular, it will reduce some of the tedium associated with the handling

of the complex arithmetic required in a.c. circuit problems, and it will

obviate the difficulties of polynomial factorization when using Laplace

transform techniques.

The four programs contained in E.CIRC (CALC, TEEPI, SIMUL and ROOTS)

follow the basic pattern of those described and listed in Appendix C of

the book: 'Electrical Circuits' by KCA Smith and RE Alley, Cambridge

University Press (1992).

These original programs were written in BBC/IBM BASIC; the derivative

programs in E.CIRC have been written using QuickBASIC. Advantage has been

taken of the power and flexibility of this language to provide greater

ease of use and improved linking among the programs.

Version 3.2 of E.CIRC differs mainly  from  the earlier version 2.1 with

respect to the inclusion of facilities for plotting graphs of real and

complex variables

Files included with E.CIRC v3.2

1. ECIRC.EXE.....Machine-code executable version of E.CIRC.

2. EHELP32.QKH...On-line 'Help' file which MUST be in the same

directory as ECIRC.EXE at runtime.

3. EINFO.QKH.....On-line 'Information' file which MUST be in the

same directory as ECIRC.EXE at runtime.

4. EHELP32.DOC...ASCII version of the on-line 'Help' file.

5. EGRAPH. DOC...ASCII file containing information on graph routines

Please ensure that all files are included when making copies.

Disclaimer of Warranty: The author and publishers of E.CIRC make no

warranty of any kind that the program is free of error or that it is

consistent with any standard of merchantability, or that it will meet your

requirements for any particular application. The program should not be

relied on for solving a problem whose incorrect solution could result in

injury to persons or property. The author and publishers of E.CIRC

disclaim liability for direct, incidental, or consequential damage

resulting from its use or application.

2.1 Program CALC: general information

CALC contains two linked programs: RLC, which computes the properties

of branches containing RLC elements connected either in series or in

parallel; and CMPLX, for performing chained complex arithmetic. Results

from RLC (effective series resistance Rs and reactance X) may, optionally,

be transferred to CMPLX. Results from both programs are stored in a Global

array, which allows calculated values to be used by other programs or

restored to CMPLX.

2.2 Program  RLC

Data for up to three circuit branches at a time may be entered and

calculated. An optional 'Clear' facility is also provided. If not cleared,

calculated data remains in place while new branch parameters are entered.

Values of effective series resistance [Rs] and reactance [X] may be

transferred to CMPLX. Note that this operation will overwrite data held in

the 'ENTER' registers of CMPLX. (If required for later computations, such

data may be transferred to the 'STORE' registers using the block transfer

facility available in 'Global' (see section 8).

Calculated values of Rs and X are automatically stored in 'Global' and

are there available for printing out or saving to disk. Other calculated

circuit parameters may be printed using the 'Print screen' key.

2.3 Program CMPLX

CMPLX contains three groups of three registers: 'Enter', 'Store' and

'Result'. These labels are for reference purposes only; they do not

indicate any restriction on which registers may be used for data entry,

storage or display of results. It is recommended, however, that input is

generally directed to the 'Enter' registers and output to the 'Result'

registers. The default for the display of calculations is Reg# 9.

The contents of the registers in CMPLX are automatically transferred to

the 'Global' array as registers are filled. If CMPLX is exited then re-

entered during calculations, 'Restore' will recall values held in the

'Global' array.

All calculations are effected using the Expression Evaluator (see

sections 5,6,7)

===================================================================

3 Program RLC

Data for up to three circuit branches at a time may be entered and

calculated. An optional 'Clear' facility is also provided. If not cleared,

calculated data remains in place while new branch parameters are entered.

Values of effective series resistance [Rs] and reactance [X] may be

transferred to CMPLX. Note that this operation will overwrite data held in

the 'ENTER' registers of CMPLX. (If required for later computations, such

data may be transferred to the 'STORE' registers using the block transfer

facility available in 'Global' (see section 8); alternatively the 'copy'

[=] command available within CMPLX may be used to transfer the contents of

registers.)

Calculated values of Rs and X are automatically stored in 'Global'

where they are available for printing out or storing to disk. Other

calculated circuit parameters may be printed using the 'Print screen' key.

RLC [N]ew Branch: Select Branch number and whether circuit is series or

parallel. Values of R,L,C and f are then entered/edited.

RLC [C]lear: Select which of three branches, or all, are cleared. (Data

in 'Global' are unaffected.)

RLC [T]ransfer: allows transfer of Rs and X from selected Branch(s)

and effects entry to program CMPLX.

===================================================================

4.1 Program CMPLX

CMPLX contains three groups of three registers: 'Enter', 'Store' and

'Result'. These labels are for reference purposes only; they do not

indicate any restriction on which registers may be used for data entry,

storage or display of results. It is recommended, however, that input is

generally directed to the 'Enter' registers and output to the 'Result'

registers. The default for the display of calculations is Reg# 9.

The contents of the registers in CMPLX are automatically transferred to

the 'Global' array as registers are filled. If CMPLX is exited then re-

entered during calculations, 'Restore' will recall values held in the

'Global' array.

All calculations are effected using the Expression Evaluator (see

sections 5-7).

CMPLX [E]nter: Allows manual entry of complex quantities. Use the

Up/Down cursor keys or number keys to select registers; use Left/Right

keys to select entry in Cartesian or Polar coordinates. When entering a

quantity whose real or imaginary part is zero, it is unnecessary to enter

the zero explicitly, pressing 'Enter' <CR> will suffice. Step to a previous

entry by using the 'Up' cursor key.

CMPLX e[V]aluate: Enter expression to be evaluated using register

numbers as operands. Delete, Home, End and Backspace keys may be used when

editing the expression. (See sections 5-7 for a complete description of

the Expression Evaluator.)

CMPLX [C]lear/[R]estore: Clears screen to remove unwanted data. Note

that values are still held in registers after the 'Clear' operation; the

'Restore' facility will recall data if required.

CMPLX [Z]ero: Places zeros in selected registers. Global values are

unaffected.

CMPLX [G]lobal: Block transfers may be effected as indicated in the

Global Submenu (see section 8).

[M]ain-Menu: Returns to Main menu. (Alternatively, press 'Esc')

[P]lot-Graph: Graphics routines.

===================================================================

5.1 The Expression Evaluator

The Expression Evaluator is designed primarily to handle complex

numbers held in registers, but it contains also features that make it

useful as a general purpose calculator. For complex number calculations

register numbers are used as operands in expressions; real numbers may be

handled in the same way by placing imaginary parts to zero. The real

integers 1...9 may also be incorporated into expressions directly by

prefixing with the hash symbol, eg #4, #8 etc. The range of integers

usable in this way may be further extended by the means of the multiplier

'M'(=10), eg (#4*M)=40, but normally it will be found more convenient to

enter numbers other than single integers via the registers.

With the exception of the reciprocal operator 'R' and the complex

conjugate operator '�', all of the built-in functions (root, log, sin,

etc.) operate only on the real part of the contents of a register, the

imaginary part is ignored.

Operator priority follows the order: 'quantities in brackets';

exponentiation; multiplication; division; addition, and subtraction. Note

that this differs from the conventional rules of algebra and from standard

BASIC, in which equal priority is specified for multiplication/division

and for addition/subtraction. Great attention must therefore be paid to

operator priority when constructing expressions, particularly those

containing mixed positive and negative terms. For example, if registers

1,2,3 contain the real numbers 1,2,3, then the expression 1-2+3-1 is

interpreted as 1-(2+3)-1 and produces the result -5 (not 1 as might be

expected). In expressions of this type, therefore, positive and negative

terms must be grouped together, negative terms being enclosed within

brackets, viz: 1+3-(2+1). Brackets may be nested to any depth and must be

employed wherever necessary to avoid ambiguity. Examples on the use of the

expression evaluator are given in section 7. (Note: unary minus is not

available in this expression evaluator.)

Expressions entered in lower-case symbols are converted automatically

to upper case during evaluation.

5.2 Operating procedure/keys

Edit mode:

Left/right arrow keys...normal function

Home/End keys...........normal function

Delete key..............normal function

Backspace key...........normal function

Ctrl-Backspace key......deletes entire expression

F1......................recalls expression (automatic save)

F5=�; F6=�; F7=�

Edit mode: macro keys F1-F4

Alt-Fn...assigns expression to key Fn

(Use 'Enter' key first to record expression)

Fn.......recalls expression assigned to key Fn

Ctrl-Backspace + Alt-Fn...gives null Fn key

Fn + Alt-S...saves assigned expression to disk

(Use Ctrl-Backspace prior to this sequence)

Fn + Alt-L...assigns and displays an expression saved to disk

(Use Ctrl-Backspace prior to this sequence)

Enter mode:

Ctrl-Enter...direct entry to 'Enter' sequence

Use the Up/Down cursor keys or number keys to select

registers; use Left/Right keys to select entry in

Cartesian or Polar coordinates. When entering a

quantity whose real or imaginary part is zero, it is

unnecessary to enter the zero explicitly, pressing

'Enter' <CR> will suffice.

===================================================================

6 Expression Evaluator: specification

OPERATORS:

Multiplication [*]   Real and complex numbers

Division       [/]            ditto

Addition       [+]            ditto

Subtraction    [-]            ditto

Reciprocal     [R]            ditto

Exponentiation [^]   Real numbers only

Conjugate      [�]   Complex numbers only

Copy           [=]   Register/register contents

OPERATOR PRIORITY:

[(..)]  [^]  [*]  [/]  [+]  [-]

OPERANDS:

Register numbers  1...9

Real integers     #1...#9

Multiplier        M (=10)

Pi                �

FUNCTIONS (real numbers only):

Square root x     [�x]

e to power x      [Ex]

sin x             [Sx]        (x in degrees)

cos x             [Cx]        (x in degrees)

tan x             [Tx]        (x in degrees)

arcsin x          [ASx]

arccos x          [ACx]

arctan x          [ATx]

log base e        [Lx]

log base 10       [LGx]

PROGRAMMED FUNCTION KEYS:

F1 Expression entered

F5 Square root    [�]

F6 Pi             [�]

F7 Conjugate      [�]

F1-F4 Macro keys

===================================================================

7 Expression Evaluator: examples

Register contents: Reg#1 (1+j0);   Reg#4 (10+j0)

Reg#2 (2+j0);   Reg#5 (20+j0)

Reg#3 (3+j3);   Reg#6 (45+j0)

Expression       Result           Note

Complex       1+2-3            0-j3             +  priority

numbers       1-2+3            -4-j3            +  priority

1-(2+3)          -4-j3            () priority

(1-2)+3          2+j3             () priority

1+2*3            7+j6             *  priority

(1+2)*3          9+j9             () priority

(1/2)*3          1.5+j1.5         () priority

3/2+3/2          3+j3             /  priority

r(r3+r3+r3)      1+j1             Reciprocal

2*�3             6-j6             Conjugate

Real          2*�*4            62.8318          2�x10

numbers       #2*�*m           62.8318          2�x10

�(2^2+#2^#2)     2.82842          �(2�+2�)

Functions     lgm              1                log 10

(real only)   lm               2.30258          ln 10

t6               1                tan 45

at#1             45               arctan 1

s(4+5)           0.5              sin (10+20)

s4*c5+c4*s5      0.5              sin10cos20+cos10sin20

r(�(#2*�))       0.39894          1/�(2�)

r(e(1^#2/#2))    0.60653          exp(-(1)�/2)

===================================================================

8.1 Global: Storage and Display

Global is used to store and display the results from programs RLC, TEEPI

and CMPLX. Results from SIMUL and ROOTS are stored in the array locations

corresponding to the CMPLX registers. The basis of 'Global' is an array

capable of holding 15 complex numbers, and which is common to all

programs. Menu driven commands allow results, which are automatically

stored in this array, to be transferred between programs.

The contents of Global may also be saved to disk and recalled using

Save/Load commands. After loading from disk, the contents of Global may be

entered into the registers of CMPLX by means of the Restore command

(within CMPLX).

8.2 Global: operating procedure/keys

Transfer results from RLC to CMPLX 'Enter'.....key 'R'.

Transfer results from TEEPI to CMPLX 'Enter'...key 'T'.

Transfer values in CMPLX 'Enter' to 'Store'....key 'E'.

Save values to disk......key 'S' + Function keys F1-F8.

(The contents of Global are saved to files GLOBAL1-GLOBAL8.)

Load files GLOBAL1-8.....key 'L' + Function keys F1-F8.

Print the currently displayed values...key 'P'.

===================================================================

9 Examples on the use of CALC

Example 1

Find the impedances of each of the following series-connected branches

at a frequency of 50 hertz:

1. 1000 ohm; 4 microfarads

2. 300 ohm; 0.5 henry

3. 10,000 ohm; 3 microfarads; 2 henry.

Solution

1. Program RLC; select Series; Branch# 1.

Enter: R=1000; L=0; C=4E-6 (or 4e-6); f=50

Answer: 1000-j795.7747

2. and 3: repeat using Branch# 2 and 3 respectively.

Answers: 300+j157.0796; 10,000-j432.7144

Example 2

Find the effective impedance of the three branches in Example 1 when

they are connected in parallel. (Use the relation: 1/Z=1/Z1+1/Z2+...)

Solution:

Transfer 'All' values to CMPLX registers 1,2,3.

Use the Expression Evaluator to evaluate the expression directly.

Enter..Expression...# R(R1+R2+R3)     [ or r(r1+r2+r3) ]

Enter.. Results Reg# (1..9)..# 7

Answer: 280.9+j74.124 in Reg# 7.

Example 3

A 240 volt supply is connected to the above parallel combination; find

the current drawn.

Solution:

Enter..240+j0 - Register 4.

Enter..Expression...# 4/7

Enter..Results Reg# # 8

Answer: 0.826, angle -14.78 (0.798-j0.2107) in Reg# 8.

Check:

Compute individual branch currents then add.

With impedances held in registers 1,2,3 and voltage in 4

use expressions 4/1; 4/2; 4/3 to put individual currents into

registers 7,8,9 respectively.

Answers: 0.187 angle 38.51; 0.708 angle -27.63; 0.0239 angle 2.477

Adding these currents gives the main current as before.

===================================================================

10 Theory used in program RLC

Definitions

(Numbers in brackets refer to equations and page numbers in Smith and

Alley - see above)

Resistance R(Rs) ohm �             Conductance G  siemens S

Inductance  L  henry H             Inductive reactance Xl=wL

Capacitance C  farad F             Capacitive reactance Xc=1/wC

Frequency   f  herz Hz             Reactance X=(wL-1/wC) �

Angular (radian) frequency         Susceptance B=(wC-1/wL) S

w=2�f rad/s

Series connected elements  (Rs = R)

G = effective parallel conductance

B = effective parallel susceptance

Complex Impedance Z = Rs+jX = Rs+j(wL-1/wC)                   (3.28)

Impedance Z (or �Z�) = �(Rs�+X�); Angle Z = arctan(X/Rs)

Complex Admittance Y = 1/Z = 1/(Rs+jX) = G+jB                 (3.46)

Admittance Y (or �Y�) = �(G�+B�); Angle Y = arctan(B/G)

G = Rs/(Rs�+X�); B = -X/(Rs�+X�)                           [125]

Parallel connected elements  (G = 1/R)

Rs = effective series resistance

X = effective series reactance

Complex Admittance Y = G+jB = G+j(wC-1/wL)

Admittance Y (or �Y�) = �(G�+B�); Angle Y = arctan(B/G)

Complex Impedance Z = 1/Y = 1/(G+jB) = Rs+jX

Impedance Z (or �Z�) = �(Rs�+X�); Angle Z = arctan(X/Rs)

Rs = G/(G�+B�);  X = -B/(G�+B�)

Resonance: series circuit                             [151]

Condition for resonance: X = wL-1/wC = 0

Resonant frequency fo = 1/2��(LC)             (3.90)

Impedance at resonance = Rs

Quality factor Q = wL/Rs;                     (3.91)

Bandwidth = fo/Q                              (3.98)

Resonance: parallel circuit                           [164]

Condition for resonance: B = wC-1/wL = 0

Resonant frequency fo = 1/2��(LC)

Impedance at resonance = R

Quality factor Q = R/wL

Bandwidth = fo/Q

===================================================================

11 Program TEEPI

Given the impedances ZA,ZB,ZC connected in a T (or star or Y)

configuration, or the impedances Z1,Z2,Z3 connected in a PI (or delta)

configuration, program TEEPI computes the transformation:

Z1=ZA+ZC+(ZAZC/ZB);        ZA=Z1Z2/(Z1+Z2+Z3)

Z2=ZB+ZA+(ZBZA/ZC);        ZB=Z2Z3/(Z1+Z2+Z3)        (8.25)

Z3=ZC+ZB+(ZCZB/ZA);        ZC=Z3Z1/(Z1+Z2+Z3)

Example 1: Three equal impedances of 3+j3 ohms are connected in T. Find

the equivalent PI impedances.

Solution: Select 'Tee to Pi' from the menu and enter impedances as

directed to give PI impedances of 9+j9 ohms.

Example 2: Check that the program gives the correct reverse

transformation.

Solution: Re-run TEEPI this time using the results stored in Global.

Press key 'G' to select Global and transfer the 'TEEPI' results to CMPLX.

Return to TEEPI, select Pi To Tee, then select 'global values' (key 'C')

Example 3: A circuit connected in delta (�) contains series resistance

and inductance in each branch: (1) 1� + .01H; (2) 2� + .02H; (3) 3� +

.03H. Find the impedance of each branch and the impedances of the

equivalent star- (T) connected circuit if the operating frequency is 50

Hz.

Solution: Use program RLC to calculate the branch impedances; the

results will be automatically stored in Global. Transfer to TEEPI, select

Pi to Tee (for delta configuration), and use 'RLC results' (key 'R') from

the submenu.

Answers: Delta impedances: 1+j3.14; 2+j6.28; 3+j9.42.

Star impedances:  .33+j1.047; 1+j3.14; .5+j1.57.

==================================================================

12 Program SIMUL

Program SIMUL solves the set of equations:

Z11.X1 + Z12.X2 +...+ Z1N.XN = C1/�1

Z21.X1 + Z22.X2 +...+ Z2N.XN = C2/�2

...      ...          ...     ...

ZN1.X1 + ZN2.X2 +...+ ZNN.XN = CN/�N

where the coefficients Z = (A + jB) are complex.

Select real or complex coefficients and enter the number of equations N

(Nmax = 4). The real (A11...) and imaginary (B11...) parts of each

coefficient are entered separately, as are the magnitudes (C1...) and

angles (�1...) of the independent variables on the RHS of the equations.

After entry, check coefficients carefully. A facility is provided for

editing the table displayed. This same facility may be used at the end of

the program to modify the coefficients so that the effects of changing

circuit parameters may be ascertained.

Example: A two-mesh circuit is described by the following equations:

(24+j18)I1 + (-20)I2 = 415/90

(-20)I1 + (60-j30)I2 = 415/150

Find the change in the magnitude of the current I1 if the phase of the

voltage in mesh 1 is changed from 90 degrees to 30 degrees

Solution: With the phase set at 90 deg the table of coeffs displayed is:

Row 1 A11 24     B11 18

A12 -20    B12 0

C1  415    �1  90

Row 2 A21 -20    B21 0

A22 60     B22 -30

C2  415    �2  150

which gives I1 = 18.21 angle 66.89.

To change the phase to 30 deg, edit the table putting �1=30, which

gives I1 = 13.21 angle 2.93. The change in magnitude is, therefore, 18.21-

13.21 = 5 A.

===================================================================

13 Theory used in program SIMUL

Program SIMUL is designed to solve the circuit mesh and nodal equations

The mesh equation for a two-mesh network is of the form:

Z11.I1 + Z12.I2 = V11           (B1)[545]

Z21.I1 + Z22.I2 = V22

where Z11, Z22 are the self-impedances of meshes 1 and 2; Z12 = Z21 is the

mutual impedance; I1 and I2 are the mesh currents, and V11, V22 are the

net emfs acting in meshes 1 and 2 respectively.

The nodal equations for a network having two independent nodes are:

Y11.I1 + Y12.I2 = I11

Y21.I1 + Y22.I2 = I22

where the Ys are admittances (with similar definitions as above); V1 and

V2 are the node voltages, and the Is are the currents injected at the

nodes For purely resistive networks resistance R replaces Z in the mesh

equations; conductance G replaces Y in the nodal equations, and Is and Vs

are interchanged in both.

===================================================================

14 Program ROOTS

Program ROOTS finds the roots of polynomials of the form:

A(M+1)x^M + A(M)x^(M-1) + ... + A(2)x + A(1) = 0

where M is the degree of the polynomial, and the coefficients A may be

complex. In linear electrical circuit theory the coefficients are in

general real.

Example: Find the roots of:

s^4 + 2.613s^3 + 3.414s^2 + 2.613s + 1 = 0

(Denominator polynomial for 4th-order Butterworth filter - see page 372

Smith & Alley)

Solution: At the prompt put M = 4 and select real coefficients. Enter:

A5=1; A4=2.613; A3=3.414; A2=2.613; A1=1. The roots are then:

s1=-0.3826+j0.9239; s2=-0.3826-j0.9239;

s3=-0.9238+j0.3827; s4=-0.9238-j0.3827.

Notice that the roots are complex conjugate pairs.

As an exercise, try substituting these roots back into the polynomial.

Transfer the roots to CMPLX using the [R]estore command to display roots

s1-s4 in Reg# 1-4 respectively. Use Reg# 5 and 6 to hold the coefficients

2.613 and 3.414. Finally (for the first root) enter the expression:

1*1*1*1+5*1*1*1+6*1*1+5*1+#1

The result of this operation (5.960464E-08 + j0) provides an indication

of the accuracy of the computations. Note that complex quantities are

raised to a power by repeated multiplication, NOT by means of the

exponentiation operator.

Repeat for the other roots by copying them in turn into Reg# 1 using

the copy commands [1=2], [1=3], [1=4]. (Store the expression first on key

F2, otherwise it will be lost when you use the copy command.)

===================================================================

15 Symbols and constants

A symbol consisting of up to two characters in length may be assigned to

any of the nine registers. Symbols are entered when using the 'Enter'

routine, either from the CMPLX sub-menu (key E), or from the Evaluator

(Ctrl-Ent). After setting up an expression in the Evaluator in the usual

way, a separate display line will display the expression in symbolic form

when the 'Ent' <CR> key is pressed.

The following physical constants may be entered into the REAL parts of

registers by using the Shift-key combination:

Velocity of light  c  Shift-c

Planck constant    h  Shift-h

Boltzmann constant k  Shift-k

Electronic charge  e  Shift-e

Mass of electron   me Shift-m

Permaeability free space (mu)  Shift-u

Permittivity free space  (epsilon) Shift-p

===================================================================

16 Graph: general information

This version of the program differs from the previous version (v 2.1)

mainly in that it includes facilities for plotting graphs. To run the

graphics routines a VGA adapter is required (neither CGA nor EGA will

suffice). All other routines should run on a minimal PC without an

adapter. Graphs may be output to a printer by running GRAPHICS.COM from

DOS prior to loading E.CIRC. This activates the Print-Screen key.

Three main graphics modes are available:

1. Graph: functions

Up to four functions of a real variable may be plotted. Only the real

parts of registers are operative for this mode. Normally the independent

variable, which is plotted on the x-axis, is assigned to Reg#1; the

dependent variable (y-axis) is normally assigned to Reg#9. Functions are

entered into the Expression Evaluator and are stored on the F1-F4 keys.

The Graph functions routine is invoked from the CMPLX Sub-Menu. You are

prompted to enter the number of functions to be plotted.

This graphics mode also contains associated routines for

plotting and displaying Parametric Functions, Bode and Nyquist diagrams.

2. Graph: parameters

A single function may be plotted for up to four values of a selected

parameter. The function to be plotted is entered into the Expression

Evaluator and stored on the F1 key. The independent variable may be the

real, imaginary, magnitude, or angle components of any register (normally

Reg#1); the dependent variable (always the y-axis) is normally chosen from

one component of Reg#9. Normally the x-axis is assigned to the independent

variable, but it may, alternatively, be assigned to any one of the four

components of the dependent variable, so that, for example, the magnitude

of the independent variable may be plotted on the y-axis against the angle

of the independent variable on the x-axis. The Graph parameters routine is

invoked from the CMPLX Sub-Menu.

3 Graph: Argand

This routine displays the contents of the 9 registers in program CMPLX

===================================================================

17 Graph: functions

Sub-menu

[N]ew Graph: Enter dialogue box by pressing key N (response to

Y/N?). Step forward through list to change default values by

pressing 'Ent' <CR>. Use the 'Up' cursor key to step backwards

through the list. Return to the Sub-menu by pressing 'Esc'.

When your plotting parameters are satisfactory, press key Y.

At prompt, choose between true and false origins.

Read off values from graphs using Left/Right Up/Down cursor keys

Press 'Ctrl'-L/R cursor keys to magnify cursor shift.

Press 'Esc' to return to Sub-menu

[R]e-plot Graph: Use this key to re-plot data held in registers.

(Note: once the Sub-menu is left, data is lost.)

[S]ave: Current graph is saved to a disk file - select keys F1-F8.

[L]oad: Recalls graphs saved on F1-F8 keys.

[E]nter: Use to change values of constants held in registers.

CMPL[X]: Returns to CMPLX Sub-Menu. (Or use 'Esc')

===================================================================

18 Graph: parameters

Sub-menu

[N]ew Graph: Enter dialogue box by pressing key N (response to

Y/N?). Step forwards through list to change default values by

pressing 'Ent' <CR>. Use the 'Up' cursor key to step backwards

through the list. Return to Sub-menu by pressing 'Esc'.

When your plotting parameters are satisfactory, press key Y.

At prompt, choose between true and false origins.

Read off values from graphs using Left/Right Up/Down cursor keys

Press 'Ctrl'-L/R cursor keys to magnify cursor shift.

Press 'Esc' to return to Sub-menu

[R]e-plot Graph: Use this key to re-plot data held in registers.

(Note: once the Sub-menu is left, data is lost.)

[S]ave: Current graph is saved to a disk file - select keys F1-F8.

[L]oad: Recalls graphs saved on F1-F8 keys.

[E]nter: Use to change values of constants held in registers.

CMPL[X]: Returns to CMPLX Sub-Menu.

===================================================================

19 Graph: phasor example using  Graph: parameters

Phasor voltages V1 and V2 are connected in series opposition across a

resistor R so that the resulting current is given by (V1-V2)/R. If V1=10/0

and V2=10/A, plot the magnitude of the current as a function of angle A

over the range 1<A>359 and for values of R = 2,4 6,8. (Note that the phase

of the current is indeterminate for angles 0 and 360.)

Set up the function as (1-2)/3 in the evaluator with:

Reg#1 containing  Mag=10, Angle=0, symbol=V1

Reg#2 containing  Mag=10, Angle=45, symbol=V2

Reg#3 containing  Real=2, Imag=0, symbol=R    (parameter)

This will evaluate to 3.8268/-67.5 if set up correctly.

Invoke the New Graph dialogue box and enter the following data:

Col.1 Variable Reg#=2; Complex Pt=a; Min value=1; Max value=359.

Col.2 x-axis Reg#=2; Complex Pt=a; y-axis Reg#=9; Complex Pt=m; p-label=R.

Col.3 Parameter Reg#=3; Complex Pt=r; Num Parameters=4, Values=2,4,6,8.

Default values may be used for all other settings.

In this example we have plotted the magnitude of the current (y-axis)

against the phase angle of V2 (x-axis). Try re-plotting with the x-axis

Reg#=9 and the Complex Pt=a. Now the x-axis represents the angle of the

current; observe that this angle changes from -89.5 to +89.5 as the phase

angle of V2 (independent variable x') changes over the range 1-359. As x'

approaches 180, x approaches 0.

(At this angle the magnitude of the voltage across the resistor is 20,

which results in a current of 10 (R=2) - as indicated on the graph.)

As a further exercise, try re-plotting with x-axis Reg#=9, Complex Pt=r

and with y-axis Reg#=9, Complex Pt=i. These plots give the real and

imaginary parts of the complex current as a function of the phase angle of

V2 with resistance R as the parameter.

===================================================================

20 Graph: Bode, Nyquist, Parametric, Argand

20.1 Bode plot

The Bode plot displays Gain (decibels) and Phase (degrees) plotted

against a logarithmic frequency scale (log base 10). Gain and phase

functions must be stored on the F1 and F2 keys respectively from the

Evaluator. Real parts of registers only are used for this routine.

Sub-menu: This is identical to the 'Graph Functions' sub-menu (see

section 17). The dialogue box is also similar.

20.2 Nyquist (polar) plot

This diagram displays Gain (loop-gain) as the magnitude  of a vector

(linear scale) against Phase (degrees) as the angle of the vector. This is

essentially the same as a polar (R,�) plot. Gain and phase functions of

frequency (R,� functions of the independent parameter) must be stored on

the F1 and F2 keys respectively from the Evaluator. Real parts of registers

only are used for this routine.

Sub-menu: This is identical to the 'Graph Functions' sub-menu (see

section 17). The dialogue box is also similar.

20.3 Parametric

This routine plots functions of an independent variable along the x- and y-

axes. The function stored on the F1 key is plotted on the x-axis, while the

function stored on the F2 key is plotted on the y-axis. Real parts of

registers only are used for this routine.

Sub-menu: This is identical to the 'Graph Functions' sub-menu (see

section 17). The dialogue box is also similar.

20.4 Argand diagram

This routine displays the contents of registers 1...9 in CMPLX. All of the

register contents may be selected or only those held in specified

registers. A non-linear display option is available which allows a wider

range of values to be displayed on the same diagram.

Example: Display on the Argand diagram the roots of the polynomial:

s^4 + 2.613s^3 + 3.414s^2 + 2.613s + 1 = 0

(Denominator polynomial for 4th-order Butterworth filter - see page 372

Smith & Alley)

Solution: Find the roots as detailed in the example contained in section

14. Transfer the values of the roots to Reg# 1...4 in CMPLX by using

[R]estore from the CMPLX sub-menu. (Note: anything held in these registers

will be overwritten.) Select these registers and the linear option after

invoking [A]rgand.

===================================================================

1 Software from Granta Electronics

Granta Electronics Ltd. publishes scientific and engineering software

which has been developed mainly within the University of Cambridge for

educational purposes and for university and industrial R&D. All

educational programs are designed to run on IBM PC or compatible

computers. The following programs in the E.CIRC series will be released

during 1994 (E.CIRC v2.1 was published, August 1992):

1. E.CIRC v2.2

As version 2.1 but with improved facilities for manipulating complex

quantities. Will run on minimal PC.

(Scheduled for release second quarter 1994.)

2. E.CIRC v3.3

As E.CIRC v3.2 but with improved facilities for plotting graphs. Will

run only on machines with VGA graphics.

(Scheduled for release second quarter 1994.

===================================================================

Registration Order Form

------------------------------------------------------------------------

To: Granta Electronics Ltd., 50 Selwyn Road, Cambridge CB3 9EB, UK.

I wish to be registered as an authorised user of E.CIRC v3.2.

Name (block capitals).........................................

Address.......................................................

.......................................................

.............................Post Code.................

Registration fee �7.50

Please send me the latest edition of E.CIRC v3.2 @ �3.50.

(UK only; includes VAT and P & P)

DISK SIZE 5.25" [  ]   3.5" [  ] (tick one)

I enclose cheque/money order for:

Registration fee only........ �7.50  [  ]

Disk only (registered users)..�3.50  [  ]

Registration fee + disk......�11.00  [  ] (tick one)

(Cheque/money order payable to Granta Electronics Ltd.)

The registration fee and disk price given above will remain valid for

UK users until 31 December 1994.

-----------------------------------------------

Users of E.CIRC in countries other than the UK will be sent the latest

information on E.CIRC, and details of the registration procedure, on

request.

-----------------------------------------------