The information on this file is primarily intended to aid the electronics

enthusiast turn more of his/her ideas into reality by making the manufacture

of home P.C.Bs a little easier and more flexible.

You are free to copy this file for others but I would prefer the whole file

be copied rather than portions of it.

Although the information given below details how to make a single sided

PCB I am (when commitments allow) working on a technique to produce

double sided P.C.Bs. At the present time the main difficulty is in getting

two 'transparencies' to align accurately enough using the tools and skills

available to the average home constructor.

The basic principles detailed below are comparatively straightforward,

however as with most manuals, actually writing the procedure has the effect

of making the technique appear to be more complicated than it really is.

P.D.S.S                                      Mr Adrian Peirson

Winscombe House                              133 runley Road

Beacon Road                                  Luton

Crowborough                                  Bedfordshire

Sussex                                       LU1 1TX

TN6 1UL                                      England

England

A Simple P.C.B manufacturing process for the electronics enthusiast.

One of the most off putting things about being an electronics enthusiast

is that having thought out a design sufficiently enough to warrant

the building of a prototype, there then comes the problem of making the base

board. This is one of the main causes of project abandonment as far as

the home constructor is concerned.

With simple circuits the use of strip board is perfectly adequate, however

as circuits become more complex, so does the amount of wiring that needs

to be added to the strip board to make all the required connections.

To have a PCB made proffesionally usually costs a small fortune.

The only option remaining for the experimenter/enthusiast is to make

his/her own PCB.

This usually involves placing pads on a copper clad board then adding

tracks etc to complete the circuit.

Whilst this method can produce excellent results it is rather time consuming

and not very versatile, any errors in tracking can result in having to

scrap that board and start again. Another drawback to this method is that

you are only making one board at a time. If its not quite right

a complete board may have to be redesigned.

Alternatively the layout may be made onto transparent 'acetate' sheets,

this is then termed a transparency, using a transparency allows copies of

the board to be made over and over again.

However should the finished design need modification, removal of tracks

and pads etc is to say the least, fiddly.

Detailed below is a comparatively simple technique that will enable the

home constructor to produce medium qaulity P.C.Bs relatively simply and

cheaply.

Try this method with a simple circuit to get the idea.

i      Use a CAD package to draw out the circuit you wish to make

There are many excellent shareware electronics CAD packages

available (do'nt forget to register these).

NB the printout needs to be an inverted copy

Print this out on a good quality printer, those of you fortunate

enough to have laser or inkjet printers may print this out on a

1:1 scale. The reasoning here is that laser and inkjet printers

can produce a finer, more detailed printout.

The less fortunate of us will have to print out at

2:1 scale, this is because sometimes a line may be printed that

should actually be 1 and a half 'printer pins' wide so the printer

makes it two 'pins' wide and the line comes out too wide.

If we print out at 2:1 then that line will be printed out three

'pins' thick which, in relation to the rest of the layout (at this

scale,)is the correct thickness.

ii     Photocopy this, reducing it back to 1:1 scale if you've used

an enlarged printout. You may need to experiment with the

contrast setting of the photocopier, what you want is a very

black circuit layout on a very white background, without spots

or 'blurring' of the tracks which some copiers seem to do.

A few spots in areas not used might be OK but make sure that these

blemishes do not occur between tracks or pads as these will lead to

shorts when the board is manufactured.

Also some photocopiers are better than others, you may need to

try several the next time you visit town, if you're unable to use

the works one.

The reason for photocopying is to prevent the ink from 'running'

during the next phase.

iii    You should now have on white paper, the circuit you wish to place

on a copper clad board.

iv      Buy a piece of Ultra violet sensitive copper clad board of suitable

size. This should be shielded until required  from sources of UV

light, eg sunlight, flourescent lights, sunbeds etc

v      Soak the photocopy in a transparent oil such as baby oil, wiping off

any excess.

The photocopy is now transparent.

Those of you with experience making P.C.B s do'nt really need to

read further, you now have a transparency.

vi     Place the transparent photocopy ( transparency ) on the sensitized

copper clad board, cover this with a thin sheet of plastic or

glass and apply pressure, this is to make sure the transparency

is pressed hard against the surface of the board.

Perhaps the best method of ensuring good all over contact between

board and transparency is by using a glass picture frame.

The reason for the inverted printout is that the circuit tracks

(or rather, the photocopy 'ink') needs to be pressed hard against

the board to prevent the UV light 'getting underneath'.

If the printout were not inverted there would be a layer of paper

between the photocopy image and the UV sensitive board, thus allowing

light to expose the board directly underneath the photocopy tracks.

This will lead to 'blurring' of the track layout.

vii    Expose the board to a U.V. light source. There are many sources of UV

light, the wifes sunbed for example. Try to ensure that illumination

is directional.

By this I mean ensure that only one source of UV light is used for

the exposure. This will help to reduce 'blurring' of the edges of

the tracks and pads.

You'll have to experiment with the exposure time, it depends on lots

of factors distance, glass thickness etc, try 5-10 mins to begin

with.

Although I've never tried it I'm sure with a little

experimentation with exposure time, the sun could be utilised here.

Also many hardware stores have small battery operated UV light

sources, the type used for identifying items marked with UV

flourescent marker pens.

A relatively inexpensive one may be obtained in UK for about 6 pounds.

*******************

viii   WARNING.  WEAR RUBBER GLOVES, SODIUM HYDROXIDE CAN BE DANGEROUS

READ & HEED THE WARNINGS ON THE PACKAGE.

After exposure, the board should be screened from any strong UV

source but this doesn't involve working in the dark.

I usually find that simply closing the curtains to exclude direct

sunlight is perfectly adequate.

Wash the board with a little soapy water to remove any oil.

The board should now be placed in Sodium Hydroxide solution.

Use a weak solution, test the edge of the board first of all, if

the solution is too strong it will remove all the UV sensitive

coating off the board very quickly, you do not want this.

What you want at this stage is for the board to appear unnafected

initially, after about 30 seconds you should see the circuit

layout 'developing' slowly.

Try initially approx 1 teaspoon per litre of cold water.

Periodically rub the surface of the board with a finger of the

rubber glove (that you are wearing) to speed up the development

process.

This process may take fifteen minutes or more to dissolve away

the UV exposed coating and leave only the unexposed portions

of the circuit.  The exact amount of time depends on the concentration

of the solution, on the exposure time, on the distance between

the board and source during exposure, glass thickness etc,etc.

Hopefully, by this time you should have a copper clad board with

the actual circuit protected by the remaining coating.

Wash the board.

If the track and pad layout appears 'blurred' there are two possible

reasons for this:

The UV light source was placed too close to the board during exposure.

Or the transparency was not in contact with the board.

A 'blurred' board may or may not be acceptable/useable.

** ** **

WARNING....FERRIC CHLORIDE STAINS EVERYTHING, THESE STAINS ARE

VIRTUALLY IMPOSSIBLE TO REMOVE.

ANOTHER UNUSUAL PROPERTY OF FERRIC CHLORIDE IS ITS ABILITY TO

MATERIALISE AS AN ORANGE STAIN IN PLACES OF THE HOUSE YOU'RE SURE

YOU NEVER VISITED THAT NIGHT, AND ON ITEMS OF CLOTHING YOU

SWEAR YOU WERN'T WEARING AT THE TIME. MOST WIVES ARE RATHER

SCEPTICAL ABOUT THIS PROPERTY, SO BE WARNED.

ix     Place the board in a solution of Ferric Chloride to etch away

the unprotected copper areas. This part of the process can take

an hour or more, periodically the (plastic) container should be

agitated to aid the etching process. Keep an eye on this stage

don't bring it out too early as you will only later find short

circuits remaining. Bring out the board too late and you'll find

the solution has etched away underneath the protective coating.

Good P.C.B manufacture by any method is a skill that requires

experimentation and experience.

x      Remove the board from the solution, washing it in cold water.

Hopefully by now you have the required copper tracks/pads etc,

the rest of the copper having been etched away

xi     Scour away the protective coating from the etched board and inspect

the board for shorts and open circuits.If none, then the board is now

ready for drilling.

In essence what I'm saying here, for those of you with a little experience

in making P.C.B s is make a photocopy of the printout, soak it in transparent

oil and hey-presto   instant transparecy, now that's not quite what

Mr Daniels would call magic, but it is cheap, quick and if you want more

boards you still have the transparency.

Another advantage is that should the inital prototype require modification

this may be carried out on the C.A.D package rather than by hand.

Should you regularly make P.C.B s and find this method an improvement or

cheaper than the one you would normally use, a small donation ( 5 pounds)

to my favourite charity, namely me, would be very much appreciated.

(Wife and sixteen goldfish to support (more on the way (goldfish, that is))

This is however not a condition of usage, you are free to use the information

and techniques contained in this file and make copies to give to friends

collegues etc without obligation.

A contribution might allow me to replace those nice towels the wife bought,

you know, the ones with the mysterious orange stains.

One final point, many electronics magazines often carry P.C.B track layouts

of their projects for the home experimenter to copy, or alternatively they

offer a P.C.B manufacturing service for these.

Reasonable results can be obtained by photocopying these and using the above

method to produce the transparency required.

The magazines however do not always print them out full scale and so a

scaling will have to be carried out during the photocopying process.

Also they are generally not inverted, this means that UV light can get

under the photocopied tracks sometimes producing a 'blurred' PCB.

This effect may be reduced by placing the UV source further away from

the board (increase the exposure time) but will not completely eliminate this.

Again, experimentation is the key.

Good Luck.

*****************************************************************************

Printout looking a little pale

This suggestion applies to fabric impact printer ribbons and is not suitable

for plastic printer ribbons.

Generally only a small part of the ribbon is impacted, the middle, look at

an old printer ribbon you'll find it paler than the outside of the ribbon.

You can of course buy reinking spray but I have found earosol oil sprays

a little easier,      and cleaner.

Oil has the effect of emulsifying the ink and allowing the ink remaining

at the edges to be redistributed.

It is of course necessary to open up the ribbon case to spray the whole

ribbon, best results are obtained if the 'rejuvenated ribbon' is allowed

to stand for a day or two befor use. Do'nt soak the ribbon when spraying

use just enough to dampen it.

The result can be a little patchy, this method is probably not suitable

where high quality printout is needed, but for the average home user

it should give reasonable results.

A word of warning here, some oils may have an adverse effect on some types of

plastic, experiment first with a small amount of spray.

The best advice is, use the oil sparingly and carefully, do'nt allow the

spray to contact anything other than the ribbon by doing this away from

the printer, that way the worst that can happen is that you have to buy a

new printer ribbon.

fs