PLOTTER.DOC

Environmental Optics Inc. Mark V Photoplotter Specifications:

Photoplotters represent a superior technique for the preparation of

phototooling for the fabrication of printed circuits.  Compared to pen

plotters, photoplotters draw with a beam of light and do not skip or smear.

Since they produce one to one art-work directly, further photographic reduction

is not required, eliminating the possibility that lens distortions will produce

misalignments.  Because this is a flat bed plotter, there is no accumulated

skew as can occur in pen plotters which move the paper back and forth under a

pen.

For several reasons, a photoplotter can keep up in performance with a much

faster pen plotter.  Since plotting is the final size there is an immediate

gain over doing art-work 2X or 4X.  Further, this photoplotter contains

apertures where a whole object, i.e. a component pad, is flashed onto the film

at once.

The Mark V is designed as a compromise between pen plotters which have the

above problems, and traditional photoplotters which are ten to fifty times as

expensive.  While of lower precision than more expensive photoplotters, we

believe it offers sufficient precision for multi-layer and surface mount

boards.

The Mark V is light tight so that operation is possible with minimal dark room

facilities.   All that is needed is a broom closet or similar space that can be

made dark while film is loaded.

The package includes a complete IBM compatible XT type computer which with the

addition of a color monitor is suitable for operation of the DROEGE software.

Included are a disk drive and adapter, 640k memory, serial and parallel ports,

a clock calender, and a color adapter.

Effective Plotting Area: ............. 17" x 22"

(430mm x 560mm)

Maximum Plotting Speed: .............. 1 ips (25mm/second), axially

Non-Plotting Moves:  ................. 2 ips (50mm/second), axially

Distance Accuracy: ................... 0.05% of distance moved.

(calibrated by software to 0.01%)

Repeatability: ....................... +/- 0.002" (0.05mm)

Step Size: ........................... 0.001" (0.025mm)

Apertures: ........................... 16 Electronicly Selected

Aperture Size: ...................... 0.005" to 0.150" (.127mm to 3.81mm) or

to customer specification.  Apertures

may be used as draws or flashes.

Aperture plate may be changed in the

field.

Size: ................................ 32" x 25" x 7" (812mm x 635mm x 178)

Interface: ........................... Direct drive from plot files stored on

floppy disk.  Files may be generated

from the DROEGE CAD program or down

loaded from the RS-232 interface.  Files

may also be ported by floppy disk.

Film Required: ....................... LPF-7 or equal speed lithographic film.

Bearings: ............................ Hardened and ground precision steel

shafts with linear ball bearing

bushings.

Note: Specifications are preliminary.