Esc			Open the popmenu

t Ins arrows Esc	draw

=			edit mode

= next + et - arrows: 	correction  (++++/ ---- jump to next/last vectors)

= next - Ins +++ or --	arrows : Observe the difference. THIS IS THE MOST

(AFTER -)		 POWERFULL FUNCTION IN LAYO1 !  Please do some

exercises with simple tracks on empty screen.

= next + et -		Del   delete

s   	INS		Pads (INS to confirm, to repeat after arrow)

ss	ESC		pads, change to another pad with arrows

Ins/Voo   		add a vector (=   y - Ins  arrows   enter)

Esc/ooV			confirm

Esc/ooV   		ending an operation

Del			delete track under cursor (confirm), delete

F1  	ATTENTION	Help (last moment updates ONLY in F1 on menulines

AND DO THIS BEFORE enter.)

F2			via

F3    			load component (in CNF, a CMP will be created)

F4			save (save NET & CMP explicitely in popmenu

F5   			refresh/re-design screen

F6			set ruler. see 'r'

F7 & F8   		select layer

Shift F7 & F8		selection of layers to select with F7-F8

F9 & F10   		decrease & increase stepcurseur

Shift F9 & F10		define this step (mode mm)

Shift F2 (after 'b' enter	Mirror of  SMB-component

to solder side of circuit

as follows:		Pads from l-1 to 2;

shapes from 8 to 10;

Text from 9 to 11

Shift-F5   		jump between 2 different displays

Ctrl-F2			design ratsnest from pad

Ctrl-F3			design ratsnest fromcomponent

Ctrl-F4 		design complete ratsnest

Ctrl-F6 		vrify all nets ( little cross on errors)

a & q   		cross orthogonal / diagonal

Shift_a   or A		changr formecursor

b   (block)   		select  component/text

b puis Ctrl-F3		change component

b nexr Del		destroy component

c 			center

d 			display / hide layer

d next  a or c		show or hide all layers

f			cursor centered on screen.

Shift-f			fills up a box or other surface .

l    			change layer

i / m     		modeinch / millimtre.

n			jump numrotation of pads

n   (in router)		choose  NET

p   (pen)   		select pen-number.

r   			ruler.

r1 = 0,0;

r2 = left-up-on screen:

r3 = temporaly on cursorposition but

relative to coordinates previously fixed with F6

s / VVo 		pad ss change pad

t / Voo  		draw but in s-mode  TOOL  !!!

u      			move component to actual cursorposition (U5 etc)

w / VoV 		window

x / y			places cursor on X/Y axis

z / oVo  		zoom 0...9  (modify possible, see / )

* *  			places cursor on grid (gg)

*       g   		places on  stepsize only

:  			jumps the miniwindow   (in CNF_PLY)

.  >       		jumps grid /  squares (sqs_> follows mode mm)

"   			jumps: ref, type, housing-shape-name, nothing

,  et  ;    		jumps housing / text (layer 8)

{  et  }   		select datalines / vectors (while drawing)

(must be placed on begin/end of track -

if not there will be a connection after copying.)

------------------------------------------------------------------------------

/	SPECIAL COMMANDS

/get U1 		cursor on component

(place on pad 1 next  b)

/del R3    		delete component after confirmation (like b DEL but in

CNF mode. If you do not save the CMP list explicitely

the unmodified CMP will load later the component again)

/set zoom		change zoom-factor

/set panauto		pan automatique (if not type c)

/quit 			without save   ! !  !  (if you do exercise)

------------------------------------------------------------------------------

Special:

Shift_Shift_?		shows the memory left,

with CHKDSK in DOS		read last line, you need 550 K

if not: change config.sys / autoexec

This is almost the only question on our HOT-LINE. Another user changed

the CONFIG or AUTOEXEC: the Layo-user was not informed and meets diffi-

culties. If short only little bit of memory: Layo functions but makes

all kind of mistakes. So check CHKDSK regulary if several users.

Special: In CNF the REF displayed and switchable with ' are in memory only.

That is to save datalines/vectors specially in the limited version.

In  Menu  Draw   Text:

draw ref   		(this is a 'command' in lowercase

Places  REF/rfrences on layer  9

draw typ		Same for types for  7400 etc

This is hided somewhat because it

consumes vectors

------------------------------------------------------------------------------

***** 	COMMANDS IN DOS:

Load  (existing) or Create (new) CNF_PLY files

LAYO1E CNF\BARTELS.CNF (if loaded: next (macro): Type:    ALT_X   )

LAYO1E CNF\FILENAME.CNF [/U01]

LAYO1E  A:\DIRECT\DIRECT2\TEST7.CNF /U05

LAYO1E CNF\NAME.CNF /c

------------------------------------------------------------------------------

******	 	STRUCTURE

@SUBKOP = ACTIVE

Diodes, Transistors, CI (SIL - DIL), Leds, Afficheurs

PASSIVE

Rsistances, Condensateurs, Bobinages

DEVICES

Transformateurs, Potentiomtres, Fusibles, Quartz, LCD

CONNECT

Connecteurs

MECANIQ

Pices mcaniques

SMB SMB Active  - passive

------------------------------------------------------------------------------

*****		CODES:

S/W

Prfixe de grille: 1,778 mm / 5,08 mm

DIL

Dual in Line: 22

Number of Pins

S/M/L/H/V

Distance entre lignes 7,62/10,16/15,24/19,05/22,86

C

Condensateurs

E

Electrolytique

20

Diamtre en mm

-12

Largeur x 1/10 inch

A/R

Axial/Radial (placement hor/ placement vert)

------------------------------------------------------------------------------

*****	 CREATION COMPONENTS

Place pads in sequence.

Use GRID if possible

You must use one track at least on layer 8

avoid the centers and tools if that is possible.

F1	HELP YOU MUST BE ON THE SPECIFIC

LINE OF THE POPMENU BEFORE TYPING 'ENTER'.

THESE TELEX-LINES CONTAIN RECENT UPDATES !

OF LAYO1 . YOU MAY CHANGE THEM (LAYO1HLP.0??

F1	ON SCREEN: MOUSE POSITIONS

F1	IN 'TRACEURS' INFO PLOTTERMENU  (LAYO_PST.HLP)

English manual in: \LAYO1EE\LAYODOC\MANUAL (It is NOT updated: see F1 first)

------------------------------------------------------------------------------

LAYO FRANCE,

CHATEAU GARAMACHE

VALLEE DE SAUVEBONNE

83400    HYERES,

FRANCE.

TEL	+33 	94282259

FAX 	+33 	94482216 or

94482312,

Server:	Videotext only: 3614  LAYOFRANCE

Downloading:		3617  LAYO

BIX:  LAYO

MCI:  LAYO

------------------------------------------------------------------------------

OPEN LAYO1 FOR THE FIRST TIME:  From DOS

LAYO1E CNF\COG2.CNF

b  enter on pad 1 of component

re-place components with arrows Esc

NEXT type Alt_X

------------------------------------------------------------------------------

Command to generate netlistin Orcad SDT (tm.) (if LAYO-MAMAGER not available):

NETLIST NAME.SCH NAME.NET calay /s

Result NAME.NET     (may be typed/created in/with a simple texteditor)

NAME.CMP       "         "           "        "         "

PLACE BOTH FILES IN \LAYO1EE\CNF

If no fieldstuff was used in Orcad: Prepare NAME.LIB

Place this NAME on top in Pop-Menu  User  Library

Next type from DOS

LAYO1E CNF\NAME.CNF  (or Load from Popmenu  Files  New design)

------------------------------------------------------------------------------

IF YOU WANT TO WORK MANUALLY ONLY:

t, s, or F3 just on an empty screen. Route manual possible: Ins on pads. F4