Dear PADS User:

PADS Software, Inc. is proud to announce the migration

between PADS-LOGIC and the MINC PLDesign Package.

The migration includes a 60 part library and the capability

of Futurenet enhanced output formatted files which will be

adjusted to feed into MINC PLDesign.

Customers who are interested can receive further information

from our Bulletin Board service or from MINC Incorporated.

The file MINC.EXE will be available on our BBS which will

include instructions on how to migrate, as well as, include

an expandable 60 part library.  Customers who have received

the MINC Library via our Bulletin Board Service should

follow the instructions defined below beginning with item

#1.

Customers who have received PADS-LOGIC Version 2.X should

note the MINC Library files have already been included in

the \PADS\LIB Directory, and are labeled: MINC.PT, MINC.PD,

MINC.LD, and MINC.LN.  To utilize the MINC Library,

customers should automatically move to item #4 (defined

below) and follow the instructions from there on in.

NOTE: Customer must have PADS-LOGIC Version 2.X in order to

create a compatible netlist which can be converted into the

proper MINC format.

MINC LIBRARY INSTALLATION

To install the necessary files, simply follow the steps

outlined below:

1.) Copy MINC.EXE into your \PADS\LOGFIL Directory.

2.) From the \PADS\LOGFIL Directory, type "MINC" (this will

unpack the MINC file into many files).

3.) Copy the MINC.* files from the \PADS\LOGFIL to the

\PADS\LIB Directory.

4.) Go into PADS-LOGIC Version 2.X, under the SETUP

(F8)\LIBPATH (F4) menu, and add a new library called MINC.

5.) You should now be able to call parts from the MINC

library and use them on your schematic.

6.) At this point, you may remove any of the MINC related

files from the \PADS\LOGFIL directory that are not being

used.

NOTE: If the schematic you are generating is going to be

ported over to MINC, make sure that all your parts are taken

out of the MINC library only.  To insure this, make sure

your MINC library is the first library PADS-LOGIC searches.

CREATING MINC PARTS IN PADS-LOGIC

To insure your parts are set up for MINC translation, you

must create them in the following manner:

1.) Make sure you create, save, and use only the parts which

will be converted to MINC, in the MINC library, and are

identified in the MINC PLDesigner manual.

2.) Make sure the MINC library is the first sequence number

identified in the SETUP (F8)\LIBPATH (F4) menu.

3.) Make sure the PKGTYP.DAT file contains the "I$  ASC"

statement and is located in the \PADS Directory.

4.) When you create the parts, make sure the "family"

category contained in the electrical window is set to ASC.

5.) After all these items are completed, you may create your

part types and decals per the instructions in the PADS-LOGIC

and MINC PLDesigner manuals.

INSTRUCTIONS FOR OUTPUTTING A FILE TO THE MINC PLDESIGN

PACKAGE

1.) Make sure all components in the schematic are created

based on the MINC requirements and come from the special

MINC library (60 parts included in basic library).

2.) Select IN/OUT (F1), REPORTS (F7), NETLIST (F1), and

FUTNETENH.

3.) Enter name to save as the following:

<DRIVE><DIRECTORY><FILENAME><EXTENSION>

Example: D:\MINC\TOM.NV4

NOTE 1: If the file is saved without the NV4 extension it

will have a .NET extension.  The .NET extension must be

renamed to .NV4 for proper MINC compiling.  The file must

then be copied from the \PADS\LOGFIL directory to the \MINC

directory, if not previously directed.

NOTE 2: The name of the NV4 file cannot contain a dash.

EXAMPLE: H-8.NV4 is incorrect and will not allow the

PLDesigner to compile.

EXAMPLE: H8.NV4 is correct and will allow the PLDesigner to

compile.

4.) Enter in the # "4" for MFG 1 attribute.

5.) Enter the # "5" for MFG 2 attribute.

6.) Enter in the # "1" for PART DESC attribute.

INSTRUCTIONS FOR CONVERTING A PADS .NV4 FILE TO A PLDESIGNER

FORMAT.

1.) Setup PLDesigner to operate with Futurenet.

2.) Type the following to produce a .DRW file for compiling:

PLSCH_<NAME>_FUTURE

NOTE 1: "_" is a space between characters.

NOTE 2: <CR> refers to the "enter" key on most keyboards.

NOTE 3: Do not type the .NV4 extension after the name.  The

PLDesigner program assumes that the filename has the .NV4

extension.

NOTE 4: The expected output and messages of this program are

listed later in this document file.

3.) Enter: "PLD" <CR>.

NOTE: The PLDesigner program will start.

4.) Select "Current File" with the keyboard or mouse and hit

<CR>.

5.) Enter the file name in the blue box and hit <CR>.

6.) Use the mouse or keyboard to select "Compile Design" and

hit <CR>.

NOTE: The following are the programs which will run using

PLDesigner, and the expected output from these programs.

a.) The message "BAD COMMAND OR FILE NAME" will appear

three times as the program attempts to access

Futurenet.  These errors do not affect the compiling of

the PADS schematics.

b.) The PLSCHematic program will run again and will

produce the .DRW output as previously encountered.  The

program will read:

"Initialize 11"

"Ports

"Instances"

"NETS"

Any output ports not connected will give warning

messages, but this will not affect compiling.  Error

messages will be given at this point and these will not

permit a .DRW file to be produced for compiling.

After the warning messages two other non-error messages

will appear.

"Processing"

"Writing"

After "Processing" and "Writing", the PLSCHematic

process is complete.

c.) The second program "PLCompile" will give the

following messages.

"Reading"

"No error detected, translation complete" - once

compiling is accomplished.

If errors exist, then the program will give error

messages after "Reading" appears.  The error messages

will pertain to the .DRW file and the faulty line of

the .DRW file will be listed by the compiler in

parenthesis.

d.) The third program to run is PLReduce.  PLReduce

will list all the schematic outputs.

e.) The fourth program to run is PLSimulate and the

following messages will appear:

"....No input date...."

"Print............"

f.) The last program to run is PLDocument.  One message

should be given.

"Print............"

7.) Once the compiling process is complete with no errors,

then the schematic should be processed correctly.

SITUATIONS WHICH WILL CAUSE ERRORS DURING THE COMPILING

PROCESS

1.) NET names should not end or begin with brackets "[]" or

backslash "\".

2.) Input and Output ports must have net names (not assigned

net numbers by PADS-LOGIC).  Components may have PADS-LOGIC

assigned net numbers if they are not connected to ports.

3.) Net names cannot have more than one occurrence on a

schematic unless the nets are physically connected together.

4.) Components must not have more than one occurrence of a

pin name (per part) or the Futurenet output may assign the

same pin twice in the netlist.