A letter to interested parties...

Dear Engineer,

On this disk or residing in this directory is a demo version and

instructions with 2 examples from Engineering Software's HYDROFLO.

It performs the functions of a hydraulic system calculator and runs

on the IBM PC and the related family of PC/MS-DOS microcomputers.

We are excited about this software because there are no comparable

packages on the market today in terms of its capabilities and price.

The design engineer essentially works in a worksheet type

environment, analyzing hydraulic elements and observing the bottom

line effects.  HYDROFLO's extensive flexibility starts with the fact

that calculations can be performed in English or metric units and

use any type of fluid.  All of Engineering Software's packages have

this basic capability.  The full featured version includes the

UTILITY functions where with a single command the entire system's

diameter or friction can be changed to a new value and the system's

new operating point and pump specification be determined.

The engineer can specify that a pump be prescribed for a system

to meet a specified flow.  Cavitation calculations are made and

reports detailing the system's head losses for various flows are

provided.  This demo is a working version in that your own hydraulic

calculations can be performed and verified.  It is a good way to

evaluate HYDROFLO risk free.  Full screen graphics of the system

and pump operation are accessible along with the full functions

in the REPORT option.

The full featured version costs $195 and includes a User's Guide

with numerous examples and a hydraulic reference handbook.  Tele-

phone support is available at no cost to registered users.  Thank

you for your interest in our products.

If you still do your pump and pipeline designs with paper and pencil...

you need HYDROFLO!

HYDROFLO is hydraulic design software for the IBM PC/XT/AT and compatibles

that substantially reduces the time involved in the design of pump and

piping systems.               HYDROFLO...

- Will analyze systems using ANY type of fluid.

- Can use differing units of head and flow within the English or metric

systems.

- Specifies the pump needed for any any design.

- Can determine possible cavitation situations.

- Provides graphics output of pump and system curves to screen and printer.

- Has a full help facility available anywhere in the program.

- Uses the Lotus 123 type menu interface.

- Has large-scale editing of system element diameters and friction.

- Automatically uses fitting and valve friction coefficients from menus.

- Uses math co-processor if available.

- Provides each user with their own configuration at start-up.

All these time savers for only $195

(includes User's Guide with examples and hydraulic reference handbook)

In a matter of minutes an engineer can layout and analyze a system and

store the work in a compact database.  Runs on any PC/MS-DOS compatible

microcomputer and comes with a 30 day money back guarantee.  Developed and

written by engineers in compiled "C" code for speed of execution.  A demo

disk is available on request.

____________________________________________________________________________________________

ORDER FORM

Name: ____________________________________  Title: __________________________

Firm: ____________________________________  Phone: __________________________

Address: ____________________________________________________________________

City, St, Zip: ______________________________________________________________

Quantity

_____      HYDROFLOs at $195 (US)       ______

California residents add 6%      ______

Outside US add $10 airmail       ______

Total       ______

Check enclosed _____        or COD delivery _____

Make checks payable and mail to:

ENGINEERING SOFTWARE

P. O.  Box 1450

Nevada City, California  95959

Phone (916)  288 - 3470

USING THE HYDROFLO DEMO

Provided here is enough information for the user to run the demo

problems successfully.  Much more information is found in the USER'S GUIDE

included with the full featured version of HYDROFLO.

To start the demo, simply place the demo disk in floppy drive A:,

type  A:DEMO  and the program will load.  The first prompt asks for

the type of display adapter you have in your system.  This is to assure

that the correct type of graph is displayed in the REPORT GRAPH option

of HYDROFLO.  You will then receive an opening screen displaying important

information and listing the name and address of a Engineering Software

dealer.  Press RETURN and the HYDROFLO opening screen will be displayed.

The top line of the main HYDROFLO screen displays the HYDROFLO copyright

notice on the left and the current system filename on the right (if known).

The bottom line of the screen has the computer time and date on the left

and the right has an error space reserved for when errors occur during

system operation.  The various keystrokes available as responses for any

HYDROFLO prompt is displayed on the line above the bottom line of the screen.

HYDROFLO uses the following Lotus-style menus and keystroke syntax.

To select an option from a displayed menu:

- Use the space bar or cursor arrows to hi-light the option

and press RETURN.

- or press the first character of the option name.

To exit from a lower menu or operation:

- Press the escape key (Esc).

(Note:  pressing escape repeatedly will eventually

bring you back to the main menu but never let you

fully exit to DOS.   You must press "Y" at QUIT).

A help facility is available by pressing the F1 (function) key anywhere

during program execution.

When entering characters or numbers a toggle between insert mode and

overtype mode is available by pressing the insert (Ins) key.

DESIGN METHODS

A general design methodology for using HYDROFLO follows:

1.)  Enter the STATUS option and set the head loss equation, units,

fluid parameters, design flow and pressures.

2.)  Enter the DEFINE option and enter all element data starting with

the inlet.

3.)  When all data is entered, use the  FILE - SAVE  option to save

the current data.  Perform file saves often so that work

is not lost.

4.)  Use the ANALYZE option to identify any incomplete or inaccurate

data.

5.)  Use the REPORT option to obtain a summary of system head losses,

detailed reports of all element data and a graph of the

system and pump curves.  Ascertain NPSHA problems and potential

low or very high velocity levels.

6.)  Use the DEFINE option to edit individual element data and the

UTILITY option to edit all element diameters and friction

for observing the end effect of pump specification.

7.)  Once a desired system design is obtained and an actual pump

curve obtained from a manufacturer that is close in operating

point with the theoretical one, the actual curve can be entered

by positioning at the pump and pressing F2, selecting a pump

"to be defined" instead of "to be prescribed" and entering the

points off the curve.  The system can then be analyzed and

adjusted for the actual operating of the system.

DESIGN TIPS...

- If a pump is to be prescribed for the system, the design flow is the

desired flow for the system.  If the pump curve is known (ie, an existing

system analysis) the design (analysis) flow should be near the

operating point of the system.

- When a pump is restricted to being positioned at a certain location

and the NPSHA is low, the diameters of the suction side lines can be

increased to minimize these intake losses.

- When a pump is not restricted to a specific location, the insert element

function (F4) and delete element function (F3) can be used to move the

pump to various spots in the line to provide adequate NPSHA.

- Trade offs existing between system element diameters (pipe and fitting

costs) and pump size (pump costs) can be made by using the  UTILITY -

DIAMETER option and letting HYDROFLO prescribe the pump.  The optimal

solution would be find the minimum total cost for the system while

meeting the design flow and providing an adequate NPSHA (Net Positive

Suction Head Available).

DEFINE (excerpt from the HYDROFLO User Guide)

The DEFINE option on the main menu allows you to ENTER and EDIT new system

elements.  This section details the type of elements available for defining.

ELEMENTS AVAILABLE FOR DEFINING ...

- INLETS

The first element in a design is an inlet.  The user can select the

type of inlet from the inlet menu or press F2 and input custom values for

description and resistance coefficient K.  In both cases, the inlet

elevation, diameter and pressure are needed.  A non-zero diameter is

needed to set up the default system diameter.  The pressure value is the

absolute atmospheric pressure at the surface of liquid supply level

(barometric pressure from an open tank or absolute

pressure existing in a closed tank).  This value of pressure is needed for

the NPSHA calculation.  If using HYDROFLO to obtain simple head loss values

for other element than inlets, a zero value of K for the inlet will omit

inlet losses from REPORT option totals.

- PIPES

The data entered for pipes include description, diameter, length and

friction factor.  The optional descriptions for all elements are for

the use of the user for identification purposes.  The friction factor that

appears will be either the default specific roughness or the default H-W

coefficient and is available for editing.  The user is free to press

RETURN and use this value or input another friction value.

- VALVES & FITTINGS

The fitting menu lists numerous types of valves and fittings.  Use the

cursor arrow keys to hi-light the desired fitting and press return to select

it.  The user will then be prompted for the fitting diameter and HYDROFLO

will use the appropriate K value for the fitting in its head loss

calculations.  The user has the option of pressing F2 on the fitting menu

and then inputing custom descriptions and K values.  NOTE: when custom

values are input and when then the  UTILITY - DIAMETER  option is used,

HYDROFLO will not be able to update the fitting's K value accordingly and

the user must remember to update the value.

- PUMPS

The user has the choice of defining a pump curve for analysis or letting

HYDROFLO prescribe one for the system.  If a curve is defined, the

user must describe four equal distant points off the curve as follows.  The

flow increment sets the distance between the four points and is in units of

flow.  The four points of head begin first with the shutoff head (The head

at which no flow can be pushed through the pump).  The remaining three points

from the curve are the head values at the flow increment, twice the flow

increment and three times the flow increment.  Once a pump is defined, the

user can position the element pointer at the pump and press F5 to obtain a

plot of the pump curve.  The pump elevation is needed for the NPSHA

calculation and the suction side calculations.  The atmospheric pressure,

needed for NPSHA, suction and discharge calculations when pressure tanks are

used, is input in the  STATUS  option.

- EXITS

The last element in a hydraulic system is an exit.  The K value for

exits is normally 1.0 but the user has the option of inputing any value.

The pressure at the exit will default to the pump's atmospheric pressure.  If

the discharge is to be a substantially different elevation or a closed tank,

be sure to update this value to the correct absolute pressure.

EDITING (excerpt from the HYDROFLO User Guide) ...

EDIT (F2)

Use the cursor arrow keys to position the pointer at a defined element and

press F2.  The original data is displayed and available for editing by

inserting or overtyping text or numeric characters.  Pressing RETURN or

pressing arrow down at a data position retains the original values shown.

Pressing escape jumps out of the editing of the current element and

moves to the next element.

DELETE (F3)

When positioned at a defined element, pressing the F3 key prompts the

user for deletion of that element.  If the user confirms the request the

element is removed.  Pressing "N" or Esc retains the element.

INSERT (F4)

Pressing the F4 key prompts the user for inserting an element before

the current element.  Again the user can confirm the request and select the

type of element to insert.  Pressing "N" or Esc aborts the operation.

PG-UP  and  PG-DN

These keys allow movement by screens within the pages of defined elements.

DEMO PROBLEM 1

An engineer is to provide the pump curve and system pipe size needed for

the hydraulic system shown opposite.  A flow of 250 gpm is desired.  The

Hazen-Williams equation will be used to compute pipe head losses in this water

system.

The file DEMO1.FLO on the demo disk contains the data for this problem

and can be retrieved using the  FILE - RETRIEVE  option.  The following

text describes the step by step procedure for creating this file.

First, enter HYDROFLO's  STATUS  option by pressing "S" at the opening

screen or press the space bar once to move to and hi-light the  "STATUS"

phrase, then press the RETURN (ENTER) key.  Once in the STATUS option,

press the arrow down key to move past the DATA FILE DIRECTORY field to the

HEAD CALCULATION EQUATION field.  If  "Hazen-Williams"  is not displayed,

press the arrow right or left key to switch from the displayed  "Darcy-

Wiesbach" statement and press the arrow down key to move to the UNIT SYSTEM

field.  It should say  "English"  and if not, pressing arrow left or right

will switch to the desired unit system.  FLOW UNITS are selected as the

above parameters were, and we use  "gpm"  not  "cfs" .  The DESIGN (ANALYSIS)

FLOW is a string input field and so type in "250." and press the arrow down

(or RETURN) key.  If any parameter field is not correct, press the arrow up

key until arriving at the desired field, then edit it accordingly.  For HEAD

UNITS we want "feet of liquid"  not  "psi"  and it is selected as FLOW UNITS

was above.  Type in  "100."  and press RETURN for the DEFAULT H-W

COEFFICIENT.  This system uses water and to set the SPECIFIC GRAVITY at 1.0,

just press RETURN.  The VAPOR PRESSURE of water at 60 degrees F is ".25"

psia so that value is entered and RETURN is pressed.  Enter "14.2" for the

atmospheric pressure at the pump and press RETURN.  The following prompt

will now appear...

Update current system to new values:   NO   YES

If the values set are correct, press RETURN or " Y " to accept the new

values.  If they aren't, we could press RETURN anyway to accept the values,

and then go back to the STATUS option again to edit desired fields.  If we

chose not to accept the new values at all, we could press "N" or Esc to abort

the process.  When the values are correct and are accepted, a prompt to

update the default parameter file appears.  If we wanted to use

the new values each time HYDROFLO was started, we would answer with a "Y",

but for these demonstration purposes we instead press RETURN.

Discharge elev  =  1080.63'

Supply elev  =  1012.51'

PRESSURES:

pump elev = 1005.74'

press. at pump = 14.2 psia

PIPES:

Steel sched-40 4"(est)

H-W coef = 100

(page 3-8 Cameron)

Spec Rough = .00015

(page 3-5 Cameron)

FLUID:

Water at 60 deg. F.

Vap. press. = .25 psia

(Page 4-20 Cameron)

Kin. vis  = .00001216 ft2/sec

(pg 4-4 & 4-25 Cameron)

DETAILED REPORT - INDIVIDUAL ELEMENTS

(System: DEMO1.FLO) @ flow = 250.00 gpm

El# Description                            Fric    Dia     Vel      HL

(inches) (ft/s) (feet)

1 Inlet Flush - sharp edged               0.50   4.00   6.38    0.32

Elev = 1012.51 feet   Press = 14.20 psia

2 Pipe                       l=  50.00  100.00   4.00   6.38    3.35

3 Fitt  Std. elbow-long radius 90 deg.    0.27   4.00   6.38    0.17

4 Pipe                       l=  25.00  100.00   4.00   6.38    1.67

5 Fitt  Stop check valves                 6.80   4.00   6.38    4.31

6 Fitt  Plug valve Straightway            0.31   4.00   6.38    0.20

7 Pipe                       l=   6.00  100.00   4.00   6.38    0.40

8 Pump  pump prescribed for system

Elev = 1005.74 feet   Press = 14.20 psia

9 Pipe                       l=   6.00  100.00   4.00   6.38    0.40

10 Fitt  Butterfly valve                   0.77   4.00   6.38    0.49

11 Pipe                       l=  50.00  100.00   4.00   6.38    3.35

12 Fitt  Std. elbow-long radius 90 deg.    0.27   4.00   6.38    0.17

13 Pipe                       l= 500.00  100.00   4.00   6.38   33.48

14 Fitt  Stop check valves                 6.80   4.00   6.38    4.31

15 Pipe                       l=1000.00  100.00   4.00   6.38   66.97

16 Exit                                    1.00   4.00   6.38    0.63

Elev = 1080.63 feet   Press = 14.20 psia

Back at the main HYDROFLO menu, we select the  DEFINE  option to

perform data input of the elements shown on the opposite of this side

of the page.  Starting with the inlet (the element data is listed in

output report opposite side of paper).  When  DEFINE  is selected, the

Inlet menu appears and the description  "Flush - sharp edged"  is hi-

lighted by pressing arrow down once, then pressing RETURN to select.  A

smaller sub-window appears that displays the inlet description and

resistance value K, and the cursor is positioned at the elevation field.

An elevation of "1012.51" is input and the user presses RETURN.  Our

initial estimate of system diameter is 4" and that value is entered thus

setting up the default diameter for the rest of the system.  The pressure

at the inlet is the next input and since this is an open surface intake,

press RETURN to use the default atmospheric pressure (set up for the pump

in the  STATUS  option).

The element menu is automatically displayed after the inlet data input

and the position pointer is set to element #2.  Element #2 is a pipe so

RETURN is pressed and an optional description can be entered.  The next

field displays the default diameter set up for the inlet and RETURN can

be pressed to accept the value.  A length of  "50."  feet is entered for

the pipe and the RETURN key is pressed.  The default Hazen-Williams coef-

ficient appears and the RETURN key is again pressed to accept the value.

The element menu is displayed and the pointer is positioned at element

#3.  This element is a fitting and the word "Fitt" is hi-lighted by press-

ing the arrow down key and the RETURN key is pressed to select it and the

fitting and valve menu appears.  Again using the cursor arrow keys, the

statement  "Std. elbow long radius 90 deg"  is hi-lighted and the RETURN

key is pressed.  The default diameter is displayed and RETURN is pressed

to accept the default value.  HYDROFLO automatically uses the correct

resistance coefficient K, for inlets, exits, valves and fittings selected

from it's internal menus.

The data for elements #4 through #7 are input in the same manner as

the previous elements.

When pump position #8 is reached, the phrase  " pump "  is hi-lighted

on the element menu and the RETURN key is pressed.  The pump window appears

and offers the choice of  "define a pump for analysis"  or  "let HYDROFLO

prescribe a pump"  for the system.  In this problem, a pump is to be

prescribed so press the arrow down key and then press RETURN.  The pump

elevation field is displayed and the value  "1005.74"  is entered an

RETURN is pressed.

The data for elements #9 through #15 is entered (according to data

listed on opposite side, detailed report) up to exit element #16.

At exit element #16, the phrase  " exit "  is hi-lighted on the element

menu and the RETURN key is pressed.  A window is displayed with the exit's

data and the cursor is positioned at the description field.  The description

is optional.  The next input field is the resistance coefficient, K.  For

most exits a value of 1.0 is used but the value may be edited.  In the

elevation field a value of  "1080.63" is entered the RETURN key is

pressed.  We press RETURN again to accept the default diameter value and

again to accept the default pressure since this is an open surface exit.

The  "defined"  mode is entered where by pressing the arrow left or

right (or page-up, page-dn) key can position the pointer at an element and

display it's data.  We can edit an individual element's data by positioning

the pointer at the element and pressing F2, insert an element by pressing

F4 or delete an element by pressing F3.

With the full featured version we could now select  FILE , then  SAVE

and specify a filename for our data to reside in.  At the main menu run the

ANALYZE  option to determine if any potential problems exist in the system.

From the main menu, the  REPORT  option can be used to preview results

of the analysis.  The operating point is determined to be at 250 gpm and

188 feet head with a NPSHA (Net Positive Suction Head Available) of 28.5

feet and a general system velocity of 6.38 ft/sec.  Other system diameters

can be tested with the full featured of HYDROFLO using the  UTILITY -

DIAMETER option but 4" appears to be the best.  With the demo disk, the

DEFINE  option is used to edit individual element diameters using the F2 key.

The graph obtained with the  REPORT - GRAPH  option (CAPACITY vs. HEAD

graph, following) and the NPSHA value (end of SUMMARY OF PUMPING SYSTEM DATA)

would be taken to a pump manufacturer to find an appropriate pump.  Once a

close operating pump is found, it's curve is input to the system by

positioning the DEFINE pointer to the pump position and pressing F2,

selecting  "define a pump"  (the top line option) then inputing it's data

according to the previously discussed DEFINE - PUMPS section and

the actual operating point will be found.

SUMMARY OF PUMPING SYSTEM DATA  (System: DEMO1.FLO)

CONFIGURATION:   Hazen-Williams Eq.

Flow                =     250.00 gpm

FLUID PROPERTIES:

Specific Gravity    =      1.000

Vapor Pressure      =       0.25 psia

STATIC HEADS:

Suction Elev Head   =       6.77 feet

Discharge Elev Head =      74.89 feet

Exit - inlet press. =       0.00 feet

Total Static Head   =      68.12 feet

NPSHA CALCULATION:

Supply pressure     =      14.20 psia (abs)

Vapor pressure      =       0.25 psia (abs)

Suction static head =       6.77 feet

Suction dynamic hd  =      10.41 feet @ design flow

NPSHA               =      28.58 feet

HEAD LOSSES (in feet)

FLOW (gpm)

SUCTION SIDE                125         250         375

MINOR LOSSES:

Valves & fittings                    1.25        4.99       11.23

MAJOR LOSSES:

Pipe HL using Hazen-Williams Eq      1.50        5.42       11.48

STATIC SUCTION HEAD:

Inlet - pump elev + pressures        6.77        6.77        6.77

TOTAL SUCTION HEAD:

Head (+), lift (-)                   4.02       -3.64      -15.94

DISCHARGE SIDE

MINOR LOSSES:

Valves & fittings                    1.40        5.60       12.59

MAJOR LOSSES:

Pipe HL using Hazen-Williams        28.90      104.20      220.61

STATIC DISCHARGE HEAD:

Exit - pump elev + pressures        74.89       74.89       74.89

TOTAL DISCHARGE HEAD:

Static head plus friction losses   105.19      184.69      308.10

TOTAL SYSTEM LOSSES

TOTAL SUCTION HEAD:

Static head + friction losses        4.02       -3.64      -15.94

TOTAL DISCHARGE HEAD:

Static head + friction losses      105.19      184.69      308.10

TOTAL SYSTEM HEAD:

Suction and discharge heads        101.18      188.33      324.04

Pump Prescription:

Design operating point:  FLOW =      250.00 gpm

HEAD =      188.33 feet

NPSHA =      28.58 feet

DEMO PROBLEM 2

A pump is to be specified for the oil conveyance system displayed

opposite.  A flow of .4 m3/sec is to be delivered by the 30 cm. diameter

system.  The Darcy-Wiesbach equation will be used to calculate pipe

head losses and the valves will be modeled as pipes using the equivalent

length method.  The file DEMO2.FLO on the demo disk contains the data for

this problem and can be retrieved using the FILE - RETRIEVE option.

Using HYDROFLO's  STATUS  option, we set the Darcy-Wiesbach equation

for use in calculating pipe head losses.  The DESIGN (ANALYSIS) FLOW will

be set at .4 m3/sec.  Within the metric unit system we setup flow units

of m3/sec and head units of meters of liquid.  We will also set a default

specific roughness of .03 mm for the steel pipe.  Specific gravity and

kinematic viscosity are set to the values listed opposite.

Using the  DEFINE  option, we give the inlet a K value of

0.0 because all fittings (except the pump) are being modeled as equivalent

lengths of pipe.  On the inlet data input, press arrow up when at the

elevation field and enter "0.0" for the friction K value.  All the other

data for the inlet is needed so that static head and NPSHA calculations are

performed correctly.  Using the chart on page 3-121 in Cameron (included

with full featured version) we line up the diameter 11.8 inches (30 cm.) and

the point for an ordinary entrance and we get 17 feet of pipe length which

works out to 5.18 meters.  Pipe element #2 has a length of 30 meters so we

have added this length to it.  The same process is used for the exit and

its equivalent length is add to pipe #24 and a K value of 0.0 is given to

the exit.

The rest of the data is entered (pipes entered normally and valves as

pipes) along with the pump which we request to be prescribed.

The  ANALYZE  option will tell us that the inlet and exit have zero

for coefficients.  This is as intended.  The losses for the inlet and

exit are included in the adjacent pipes length.

The prescribed pump's and system's curve along with the system summary

of head losses is shown on the back of this page.

Tank B elev  =  178.15 m

Tank A elev  =  132.63 m

Pump elev = 125.79 m

PIPING SYSTEM:

Steel sched-40  30 cm.

Specific Rough = .03 mm

PRESSURES:

Atm press @ pump = 99.285 Kpa

Tank A = 121.39 Kpa (closed)

Tank B = 103.54 Kpa (closed)

FLUID:

SG = .86 Crude oil at 120 deg. F.

Kinematic Vis = 4.1 centistokes = 4.1e-6 m2/sec

Vap. press. = .5 KPa

DETAILED REPORT - INDIVIDUAL ELEMENTS

(System: DEMO2.FLO) @ flow = 0.40   m3/sec

El# Description                            Fric    Dia     Vel      HL

(cm.) (mtr/s) (meters)

1 Inlet Inward projecting                 0.00  30.00   5.66    0.00

Elev =  132.63 meters   Press = 121.39 Kpa

2 Pipe                     l=  35.18 3.00e-002  30.00   5.66    2.83

3 Pipe  Fitt -elbow-45 deg l=   4.27 3.00e-002  30.00   5.66    0.34

4 Pipe                     l=  30.00 3.00e-002  30.00   5.66    2.41

5 Pipe  Fitt -Tee-thruflo  l=   5.79 3.00e-002  30.00   5.66    0.47

6 Pipe                     l=  15.00 3.00e-002  30.00   5.66    1.21

7 Pipe  Fitt -Tee-thruflo  l=   5.79 3.00e-002  30.00   5.66    0.47

8 Pipe                     l=  47.00 3.00e-002  30.00   5.66    3.78

9 Pipe  Fitt -Sw chk val   l=  22.86 3.00e-002  30.00   5.66    1.84

10 Pipe  Fitt -Plug Val Str l=   4.27 3.00e-002  30.00   5.66    0.34

11 Pipe                     l=   4.00 3.00e-002  30.00   5.66    0.32

12 Pump  pump prescribed for system

Elev =  125.79 meters   Press = 99.29 Kpa

13 Pipe                     l=   4.00 3.00e-002  30.00   5.66    0.32

14 Pipe  Fitt -Plug val str l=   4.27 3.00e-002  30.00   5.66    0.34

15 Pipe                     l= 150.00 3.00e-002  30.00   5.66   12.07

16 Pipe  Fitt -elbow lr 90  l=   7.62 3.00e-002  30.00   5.66    0.61

17 Pipe                     l=  85.18 3.00e-002  30.00   5.66    6.85

18 Exit                                    0.00  30.00   5.66    0.00

Elev =  178.15 meters   Press = 103.54 Kpa

SUMMARY OF PUMPING SYSTEM DATA  (System: DEMO2.FLO)

CONFIGURATION:   Darcy-Wiesbach Eq.

Flow                =       0.40 m3/sec

FLUID PROPERTIES:

Specific Gravity    =      0.860

Vapor Pressure      =       0.50 Kpa

Kinematic Viscosity =   4.1e-006 m2/sec

STATIC HEADS:

Suction Elev Head   =       6.84 meters

Discharge Elev Head =      52.36 meters

Exit - inlet press. =      -2.12 meters

Total Static Head   =      43.40 meters

NPSHA CALCULATION:

Supply pressure     =     121.39 Kpa (abs)

Vapor pressure      =       0.50 Kpa (abs)

Suction static head =       6.84 meters

Suction dynamic hd  =      14.01 meters @ design flow

NPSHA               =       7.16 meters

HEAD LOSSES (in meters)

FLOW (m3/sec)

SUCTION SIDE                  0.20        0.40        0.60

MINOR LOSSES:

Valves & fittings                       0.00        0.00        0.00

MAJOR LOSSES:

Pipe HL using Darcy-Wiesbach Eq         3.87       14.01       30.05

STATIC SUCTION HEAD:

Inlet - pump elev + pressures           9.46        9.46        9.46

TOTAL SUCTION HEAD:

Head (+), lift (-)                      5.59       -4.55      -20.59

DISCHARGE SIDE

MINOR LOSSES:

Valves & fittings                       0.00        0.00        0.00

MAJOR LOSSES:

Pipe HL using Darcy-Wiesbach            5.58       20.20       43.32

STATIC DISCHARGE HEAD:

Exit - pump elev + pressures           52.86       52.86       52.86

TOTAL DISCHARGE HEAD:

Static head plus friction losses       58.44       73.07       96.19

TOTAL SYSTEM LOSSES

TOTAL SUCTION HEAD:

Static head + friction losses           5.59       -4.55      -20.59

TOTAL DISCHARGE HEAD:

Static head + friction losses          58.44       73.07       96.19

TOTAL SYSTEM HEAD:

Suction and discharge heads            52.85       77.62      116.78

Pump Prescription:

Design operating point:  FLOW =        0.40 m3/sec

HEAD =       77.62 meters

NPSHA =       7.16 meters