Atmosphere Calculator Documentation

HyperStuffWare - Copyright (c) 1988

ATMCALC  has  been  developed  to perform most of  the  routine  atmospheric

calculations  used in aviation and aeronautical  engineering.   The  program

contains extensive error checking and has the following options.

[A] User input of pressure altitude (Hc) in feet.   After input, the program

updates  all other altitude dependant variables such as Mach number but does

not change the calibrated airspeed (Vc) which is independent of Hc.

[B]  User  input of calibrated airspeed  (Vc).   After  input,  the  program

updates all airspeed dependant variables at the existing altitude.

[C] User input of true airspeed (Vt).  After input, the program calculates a

new  Vc  at  the  existing  altitude  and  updates  all  airspeed  dependant

variables.

[D]  User  input  of equivalent airspeed (Ve).   After  input,  the  program

calculates  a  new  Vc at the existing altitude  and  updates  all  airspeed

dependant variables.

[E]  User input of the local speed of sound (a).   After input,  the program

calculates  a  new  Vc at the existing altitude  and  updates  all  airspeed

dependant variables.

[F]  User  input of the ambient pressure (Pa).   After  input,  the  program

calculates  a new altitude and updates all other variables while maintaining

the same Vc.

[G]  User input of the dynamic pressure (q) which is the velocity  component

of the impact pressure (Qc).   After input,  the program calculates a new Vc

at the existing altitude and updates all airspeed dependant variables.

[H]  User input of the impact pressure (Qc) which is the sum of the  ambient

pressure  and  dynamic  pressure  components.    After  input,  the  program

calculates  a  new  Vc at the existing altitude  and  updates  all  airspeed

dependant variables.

[I]  User input of the static temperature (TAS).   After input,  the program

calculates a new altitude and updates all other variables while  maintaining

the same Vc.

[J]  User input of the total temperature (TAT).   After input,  the  program

calculates  a  new  Vc  at the existing altitude and  updates  all  airspeed

dependant variables.

[K]  User input of the ambient pressure ratio  (delta).   After  input,  the

program  calculates  a  new altitude and updates all other  variables  while

maintaining the same Vc.

[L]  User  input of the ambient density ratio  (sigma).   After  input,  the

program  calculates  a new altitude and updates all  other  variables  while

maintaining the same Vc.

[M]  User input of the static temperature ratio (theta).   After input,  the

program  calculates  a new altitude and updates all  other  variables  while

maintaining the same Vc.

[N]  User input of the Mach number.   After input,  the program calculates a

new  Vc  at  the  existing  altitude  and  updates  all  airspeed  dependant

variables.

[O] User input of the air density.   After input,  the program calculates  a

new altitude and updates all other variables while maintaining the same Vc.

[P]  User input of the ram air temperature recovery factor.   This value  is

typically  0.80  for  aircraft  pitot-static systems.   It is  used  in  the

relationship between static and total air temperature  calculations.   After

input,  the  program  uses  the  existing  Vc and Hc  to  update  all  other

variables.

[R]  User input of Ve and Mach to calculate the altitude at which  they  are

equal.  After input, the program calculates a new Hc and Vc then updates all

other variables.

[S]  User input of a temperature differential from the standard  atmosphere.

After  input,  the program calculates a new static temperature then  updates

the remaining variables at the existing Vc and Hc.

[Q] Quit AtmCalc.

[Z] User input to reinitialize AtmCalc to zero conditions.